ANNEX  V.     PART - SPA

SPECIFIC    APPROVALS

INTENTIONALLY            LEFT            BLANK

ANNEX  V.     PART - SPA    /    SPECIFIC  APPROVALS

CONTENT

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   5

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   7

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   9

SUBPART  A.   GENERAL  REQUIREMENTS    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     11

SPA. GEN. 100    Competent  Authority   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  11

SPA. GEN. 105    Application  for  a  Specific  Approval  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    11

SPA. GEN. 110    Privileges  of  an  Operator  Holding  a  Specific  Approval  .  .  .  .  .  .  .  .  .  .    11

SPA. GEN. 115    Changes  to  a  Specific  Approval  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  11

SPA. GEN. 120    Continued  Validity  of  a  Specific  Approval    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   11

SUBPART  B.   PERFORMANCE - BASED  NAVIGATION  OPERATIONS   .  .  .  .  .  .  .  .  13

SPA. PBN. 100    PBN  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   13

SPA. PBN. 105    PBN  Operational  Approval   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   13

SUBPART  C.   OPERATIONS  with  SPECIFIED  MINIMUM  NAVIGATION

PERFORMANCE  ( MNPS )   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    15

SPA. MNPS. 100   MNPS  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   15

SPA. MNPS. 105   MNPS  Operational  Approval  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  15

SUBPART  D.   OPERATIONS  in  AIRSPACE  with  REDUCED  VERTICAL

SEPARATION  MINIMA  ( RVSM )  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

SPA. RVSM. 100   RVSM  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

SPA. RVSM. 105   RVSM  Operational  Approval   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

SPA. RVSM. 110   RVSM  Equipment  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

SPA. RVSM. 115   RVSM  Height - keeping  Errors   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

SUBPART  E.    LOW  VISIBILITY  OPERATIONS  ( LVO )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   .   19

SPA. LVO. 100   Low  Visibility  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  19

SPA. LVO. 105   LVO  Approval  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

SPA. LVO. 110   General  Operating  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

SPA. LVO. 115   Aerodrome  Related  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    20

SPA. LVO. 120   Flight  Crew  Training  and  Qualifications  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  20

SPA. LVO. 125    Operating  Procedures   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   21

SPA. LVO. 130    Minimum  Equipment   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   21

SUBPART  F.   EXTENDED  RANGE  OPERATIONS  with  TWO-ENGINED

AEROPLANES  ( ETOPS )   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

SPA. ETOPS. 100   ETOPS   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  23

SPA. ETOPS. 105   ETOPS  Operational  Approval   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

SPA. ETOPS. 110   ETOPS  En - route  Alternate  Aerodrome   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

SPA. ETOPS. 115   ETOPS  En - route  Alternate  Aerodrome  Planning  Minima  .  .  .  .  .  .  .  .    24

SUBPART  G.   TRANSPORT  of  DANGEROUS  GOODS   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  25

SPA. DG. 100   Transport  of  Dangerous  Goods   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    25

SPA. DG. 105    Approval  to  Transport  Dangerous  Goods   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    25

SPA. DG. 110    Dangerous  Goods  Information  and  Documentation  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 25

SUBPART  H.   HELICOPTER  OPERATIONS  with  NIGHT  VISION

IMAGING  SYSTEMS  ( NVIS )  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   27

SPA. NVIS. 100   Night  Vision  Imaging  System  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   27

SPA. NVIS. 110   Equipment  Requirements  for  NVIS  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 27

SPA. NVIS. 120   NVIS  Operating  Minima .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   28

SPA. NVIS. 130   Crew  Requirements  for  NVIS  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   28

SPA. NVIS. 140   Information  and  Documentation   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  28

SUBPART  I.   HELICOPTER  HOIST  OPERATIONS  ( HHO )  .  .  .  .  .  .  .  .  .  .  .  .  .  .    31

SPA. HHO. 100   Helicopter Hoist  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  31

SPA. HHO. 110   Equipment  Requirements  for  HHO   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

SPA. HHO. 115   HHO  Communication  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  31

SPA. HHO. 125   Performance  Requirements  for  HHO  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

SPA. HHO. 130   Crew  Requirements  for  HHO  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   32

SPA. HHO. 135   HHO  Passenger  Briefing  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   33

SPA. HHO. 140   Information  and  Documentation   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  33

SUBPART  J.   HELICOPTER  EMERGENCY  MEDICAL  SERVICE

OPERATIONS   ( HEMS )  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

SPA. HEMS. 100   Helicopter  Emergency  Medical  Service  Operations .  .  .  .  .  .  .  .  .  .  .  .  .  35

SPA. HEMS. 110   Equipment  Requirements  for  HEMS  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .   35

SPA. HEMS. 115   Communication  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  35

SPA. HEMS. 120   HEMS  Operating  Minima  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  35

SPA. HEMS. 125   Performance  Requirements  for  HEMS  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .   36

SPA. HEMS. 130   Crew  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   37

SPA. HEMS. 135   HEMS  Medical  Passenger  and  other  Personnel  Briefing   .  .  .  .  .  .  .  .  .   38

SPA. HEMS. 140   Information  and  Documentation   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   38

SPA. HEMS. 145   HEMS  Operating  Base  Facilities  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  39

SPA. HEMS. 150   Fuel  Supply   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   39

SPA. HEMS. 155   Refuelling  with  Passengers  Embarking,  on  Board  or  Disembarking   .  .  .   39

GAIDANCE  MATERIALS  &  ACEPTABLE  MEANCE  and  COMPLIANCE   .  .  .  .  .  .  41

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE | TO PAGE |  |  |  | EFFECTIVE DATE |
| --- | --- | --- | --- | --- | --- | --- |
| CONTENT | 3 | 4 |  |  |  | 01 - November - 2014 |
| RECORDS of REVISION | 5 | 6 |  |  |  | 01 - November - 2014 |
| RECORDS of TEMPORARY REVISION | 7 | 8 |  |  |  | 01 - November - 2014 |
| LIST of EFFECTIVE PAGE | 9 | 10 |  |  |  | 01 - November - 2014 |
| SUBPART A. GENERAL REQUIREMENTS | 11 | 12 |  |  |  | 01 - November - 2014 |
| SPA. GEN. 100 Competent Authority | 11 18 |  |  |  |  | 01 - November - 2014 |
| SPA. GEN. 105 Application for a Specific Approval | 11 |  |  |  |  | 01 - November - 2014 |
| SPA. GEN. 110 Privileges of an Operator Holding a Specific Approval | 11 224 |  |  |  |  | 01 - November - 2014 |
| SPA. GEN. 115 Changes to a Specific Approval | 11 33 |  |  |  |  | 01 - November - 2014 |
| SPA. GEN. 120 Continued Validity of a Specific Approval | 11 |  |  |  |  | 01 - November - 2014 |
| SUBPART B. PERFORMANCE - BASED NAVIGATION OPERATIONS | 13 |  | 14 |  |  | 01 - November - 2014 |
| SPA. PBN. 100 PBN Operations | 13 |  |  |  |  | 01 - November - 2014 |
| SPA. PBN. 105 PBN Operational Approval | 13 |  |  |  |  | 01 - November - 2014 |
| SUBPART C. OPERATIONS with SPECIFIED MINIMUM NAVIGATION PERFORMANCE | 15 |  |  | 16 |  | 01 - November - 2014 |
| SPA. MNPS. 100 MNPS Operations | 15 |  |  |  |  | 01 - November - 2014 |
| SPA. MNPS. 105 MNPS Operational Approval | 15 |  |  |  |  | 01 - November - 2014 |
| SUBPART D. OPERATIONS in AIRSPACE with REDUCED VERTICAL SEPARATION MINIMA | 17 |  |  |  | 18 | 01 - November - 2014 |
| SPA. RVSM. 100 RVSM Operations | 17 |  |  |  |  | 01 - November - 2014 |
| SPA. RVSM. 105 RVSM Operational Approval | 17 |  |  |  |  | 01 - November - 2014 |
| SPA. RVSM. 110 RVSM Equipment Requirements | 17 |  |  |  |  | 01 - November - 2014 |
| SPA. RVSM. 115 RVSM Height - keeping Errors | 18 |  |  |  |  | 01 - November - 2014 |
| SUBPART E. LOW VISIBILITY OPERATIONS | 19 |  | 22 |  |  | 01 - November - 2014 |
| SPA. LVO. 100 Low Visibility Operations | 19 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 105 LVO Approval | 19 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 110 General Operating Requirements | 19 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 115 Aerodrome Related Requirements | 20 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 120 Flight Crew Training and Qualifications | 20 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 125 Operating Procedures | 21 |  |  |  |  | 01 - November - 2014 |
| SPA. LVO. 130 Minimum Equipment | 21 |  |  |  |  | 01 - November - 2014 |
| SUBPART F. EXTENDED RANGE OPERATIONS with TWO - ENGINED AEROPLANES | 23 | 24 |  |  |  | 01 - November - 2014 |
| SPA. ETOPS. 100 ETOPS | 23 |  |  |  |  | 01 - November - 2014 |
| SPA. ETOPS. 105 ETOPS Operational Approval | 23 |  |  |  |  | 01 - November - 2014 |
| SPA. ETOPS. 110 ETOPS En - route Alternate Aerodrome | 23 |  |  |  |  | 01 - November - 2014 |
| SPA. ETOPS. 115 ETOPS En - route Alternate Aerodrome Planning Minima | 24 |  |  |  |  | 01 - November - 2014 |
| SUBPART G. TRANSPORT of DANGEROUS GOODS | 25 |  | 26 |  |  | 01 - November - 2014 |
| SPA. DG. 100 Transport of Dangerous Goods | 25 |  |  |  |  | 01 - November - 2014 |
| SPA. DG. 105 Approval to Transport Dangerous Goods | 25 |  |  |  |  | 01 - November - 2014 |
| SPA. DG. 110 Dangerous Goods Information and Documentation | 25 |  |  |  |  | 01 - November - 2014 |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE | TO PAGE |  | EFFECTIVE DATE |
| --- | --- | --- | --- | --- |
| SUBPART H. HELICOPTER OPERATIONS with NIGHT VISION IMAGING SYSTEMS | 27 | 30 |  | 01 - November - 2014 |
| SPA. NVIS. 100 Night Vision Imaging System Operations | 27 |  |  | 01 - November - 2014 |
| SPA. NVIS. 110 Equipment Requirements for NVIS Operations | 27 |  |  | 01 - November - 2014 |
| SPA. NVIS. 120 NVIS Operating Minima | 28 |  |  | 01 - November - 2014 |
| SPA. NVIS. 130 Crew Requirements for NVIS Operations | 28 |  |  | 01 - November - 2014 |
| SPA. NVIS. 140 Information and Documentation | 28 |  |  | 01 - November - 2014 |
| SUBPART I. HELICOPTER HOIST OPERATIONS | 31 |  | 34 | 01 - November - 2014 |
| SPA. HHO. 100 Helicopter Hoist Operations | 31 |  |  | 01 - November - 2014 |
| SPA. HHO. 110 Equipment Requirements for HHO | 31 |  |  | 01 - November - 2014 |
| SPA. HHO. 115 HHO Communication | 31 |  |  | 01 - November - 2014 |
| SPA. HHO. 125 Performance Requirements for HHO | 31 |  |  | 01 - November - 2014 |
| SPA. HHO. 130 Crew Requirements for HHO | 32 |  |  | 01 - November - 2014 |
| SPA. HHO. 135 HHO Passenger Briefing | 33 |  |  | 01 - November - 2014 |
| SPA. HHO. 140 Information and Documentation | 33 |  |  | 01 - November - 2014 |
| SUBPART J. HELICOPTER EMERGENCY MEDICAL SERVICE OPERATIONS | 35 | 40 |  | 01 - November - 2014 |
| SPA. HEMS. 100 Helicopter Emergency Medical Service Operations | 35 |  |  | 01 - November - 2014 |
| SPA. HEMS. 110 Equipment Requirements for HEMS Operations | 35 |  |  | 01 - November - 2014 |
| SPA. HEMS. 115 Communication | 35 |  |  | 01 - November - 2014 |
| SPA. HEMS. 120 HEMS Operating Minima | 35 |  |  | 01 - November - 2014 |
| SPA. HEMS. 125 Performance Requirements for HEMS Operat-s | 36 |  |  | 01 - November - 2014 |
| SPA. HEMS. 130 Crew Requirements | 37 |  |  | 01 - November - 2014 |
| SPA. HEMS. 135 HEMS Medical Passenger and other Personnel Briefing | 38 |  |  | 01 - November - 2014 |
| SPA. HEMS. 140 Information and Documentation | 38 |  |  | 01 - November - 2014 |
| SPA. HEMS. 145 HEMS Operating Base Facilities | 39 |  |  | 01 - November - 2014 |
| SPA. HEMS. 150 Fuel Supply | 39 |  |  | 01 - November - 2014 |
| SPA. HEMS. 155 Refuelling with Passengers Embarking, on Board or Disembarking | 39 |  |  | 01 - November - 2014 |
| GAIDANCE MATERIALS & ACEPTABLE MEANCE and COMPLIANCE | 41 | 134 |  | 01 - November - 2014 |

SUBPART   A.     GENERAL    REQUIREMENTS

SPA. GEN. 100     Competent  Authority  -  GDCA  of  RA

a )   The  GDCA  for  issuing  a  specific  approval  shall  be :

1 )  for  the  Commercial  Air  Transport  Operator  the  authority  of  the  State  of  the  Operator  has  its  principal  place  of  business ;

2 )   for  the  Non - commercial  Operator  the  authority  of  the  State  of  the  Operator  is  established   or  residing.

b )   Notwithstanding  ( a )( 2 ),  for  the  Non - commercial  Operator  using  aircraft  registered  in  a third  country,  the  applicable  requirements  under  this  Annex  for  the  approval  of  the  following operations  shall  not  apply  if  these  approvals  are  issued  by  a  third  country  State  of  Registry :

1 )   Performance - based  Navigation  ( PBN ) ;

2 )   Minimum  Operational  Performance  Specifications  ( MNPS ) ;

3 )   Reduced  Vertical  Separation  Minima  ( RVSM )  airspace.

SPA. GEN. 105     Application   for   a   Specific   Approval

a )    The  Operator  applying  for  the  initial  issue  of  a  specific  approval  shall  provide  to  the GDCA  the  documentation  required  in  the  applicable  Subpart,  together  with  the  following  information :

1 )   the  name,  address  and  mailing  address  of  the  applicant ;

2 )   a  description  of  the  intended  operation.

b )    The  Operator  shall  provide  the  following  evidence  to  the  GDCA :

1 )   compliance  with  the  requirements  of  the  applicable  Subpart ;

2 )   that  the  relevant  elements  defined  in  the  mandatory  part  of  the  operational  suitability  data  established  in  accordance  with  CR - EU No 748 / 2012  are  taken  into  account.

c )    The  Operator  shall  retain  records  relating  to  ( a )  and  ( b )  at  least  for  the  duration  of  the  operation  requiring  a  specific  approval,  or,  if  applicable,  in  accordance  with   Annex  III             ( Part -ORO ).

SPA. GEN. 110     Privileges  of  an  operator  holding  a  specific  approval

The  scope  of  the  activity  that  an  Operator  is  approved  to  conduct  shall  be  documented  and specified :

a )   For  Operators  holding  an  Air  Operator  Certificate  ( AOC )  in  the  Operations  Specifications  to  the  AOC ;

b )   For  all  other  operators  in  the  list  of  specific  approvals.

SPA. GEN. 115     Changes   to   a   Specific   Approval

When  the  conditions  of  a  specific  approval  are  affected  by  changes,  the  Operator  shall  provide the  relevant  documentation  to  the  GDCA  and  obtain  prior   approval  for  the operation.

SPA. GEN. 120     Continued  Validity  of  a  Specific  Approval

Specific  approvals  shall  be  issued  for  an  unlimited  duration  and  shall  remain  valid  subject  to the  Operator  remaining  in  compliance  with  the  requirements  associated  with  the  specific approval  and  taking  into  account  the  relevant  elements  defined  in  the  mandatory  part  of  the operational  suitability  data  established  in  accordance  with  CR - EU  No 748 / 2012.

SUBPART   B.     PERFORMANCE  -  BASED    NAVIGATION   (  PBN  )

OPERATIONS

SPA. PBN. 100      PBN   Operations

Aircraft  shall  only  be  operated  in  designated  airspace,  on  routes  or  in  accordance  with procedures  where  Performance -  Based  Navigation  ( PBN )  specifications  are  established  if  the Operator  has  been  granted  an  approval  by  the  Competent  Authority  to  conduct  such  operations.                                                                                                                                                                No  specific  approval  is  required  for  operations  in  Area  Navigation  5  ( RNAV 5 ( Basic  Area   Navigation,  B - RNAV ) )  designated   airspace.

SPA. PBN. 105     PBN   Operational   Approval

To  obtain  a  PBN  operational  approval  from  the  Competent  Authority,  the  Operator  shall  provide  evidence  that :

a )    the  relevant  airworthiness  approval  of  the  RNAV  system  has  been  obtained ;

b )    a  training  programme  for  the  flight  crew  members  involved  in  these  operations  has  been established ;

c )    operating  procedures  have   been  established  specifying :

1 )   the  equipment  to  be  carried,  including  its  operating  limitations  and  appropriate  entries  in  the  Minimum  Equipment  List  ( MEL ) ;

2 )   flight  crew  composition  and  experience  requirements ;

3 )   normal  procedures ;

4 )   contingency  procedures ;

5 )   monitoring  and  incident  reporting ;

6 )   electronic  navigation  data  management.

INTENTIONALLY            LEFT            BLANK

SUBPART   C.      OPERATIONS   with    SPECIFIED    MINIMUM

NAVIGATION    PERFORMANCE   (  MNPS  )

SPA. MNPS. 100     MNPS   Operations

Aircraft  shall  only  be  operated  in  designated  Minimum  Navigation  Performance  Specifications           ( MNPS )  airspace  in  accordance  with  regional  supplementary  procedures,  where  minimum navigation  performance  specifications  are  established,  if  the  Operator  has  been  granted  an approval  by  the  Competent  Authority  to  conduct  such  operations.

SPA. MNPS. 105     MNPS   Operational   Approval

To  obtain  an  MNPS  operational  approval  from  the  Competent  Authority,  the  Operator  shall provide  evidence  that :

a )    the  navigation  equipment  meets  the  required  performance ;

b )    navigation  displays,  indicators  and  controls  are  visible  and  operable  by  either  pilot  seated at  his  /  her  duty  station ;

c )    a  training  programme  for  the  flight  crew  members  involved  in  these  operations  has  been established ;

d )    operating  procedures  have  been  established  specifying :

1 )   the  equipment  to  be  carried,  including  its  operating  limitations  and  appropriate  entries  in  the  MEL ;

2 )   flight  crew  composition  and  experience  requirements ;

3 )   normal  procedures ;

4 )   contingency  procedures  including  those  specified  by  the  authority  responsible  for  the airspace  concerned ;

5 )   monitoring  and  incident  reporting.

INTENTIONALLY            LEFT            BLANK

SUBPART   D.     OPERATIONS   in   AIRSPACE   with   REDUCED

VERTICAL   SEPARATION   MINIMA   (  RVSM  )

SPA. RVSM. 100     RVSM   Operations

Aircraft  shall  only  be  operated  in  designated  airspace  where  a  Reduced  Vertical  Separation Minimum  of  300 m  ( 1 000 ft )   applies   between  Flight  Level ( FL )  290  and   FL  410,  inclusive,  if  the  Operator  has  been  granted  an  approval  by  the  Competent  Authority  to  conduct  such operations.

SPA. RVSM. 105     RVSM   Operational   Approval

To  obtain  an  RVSM  operational  approval  from  the  Competent  Authority,  the  Operator  shall provide  evidence  that :

a )    the  RVSM  airworthiness  approval  has  been  obtained ;

b )    procedures  for  monitoring  and  reporting  height - keeping  errors  have  been  established ;

c )    a  training  programme  for  the  flight  crew  members  involved  in  these  operations  has  been established ;

d )    operating  procedures  have  been  established  specifying :

1 )   the  equipment  to  be  carried,  including  its  operating  limitations  and  appropriate  entries  in  the  MEL ;

2 )   flight  crew  composition  and  experience  requirements ;

3 )   flight  planning ;

4 )   pre - flight  procedures ;

5 )   procedures  prior  to  RVSM  airspace  entry ;

6 )   in - flight  procedures ;

7 )   post - flight  procedures ;

8 )   incident  reporting ;

9 )   specific  regional  operating   procedures.

SPA. RVSM. 110     RVSM   Equipment   Requirements

Aircraft  used  for  operations  in  RVSM  airspace  shall  be  equipped  with :

a )    two  ( 2 )  independent  Altitude  Measurement  Systems ;

b )    an  Altitude  Alerting  System ;

c )    an  Automatic  Altitude  Control  System ;

d )    a  Secondary  Surveillance  Radar  ( SSR )  Transponder   with  Altitude  Reporting  System  that

can  be  connected  to  the  Altitude  Measurement  System  in  use  for  altitude  control.

SPA. RVSM. 115     RVSM   Height - keeping   Errors

a )    The  Operator  shall  report  recorded  or  communicated  occurrences  of  height - keeping  errors caused  by  malfunction  of  aircraft  equipment  or  of  operational  nature,  equal  to  or  greater  than :

1 )   a  Total  Vertical  Error   (  TVE  )    of   ±  90  m   (  ±  300  ft  ) ;

2 )   an  Altimetry  System  Error   (  ASE  )   of   ±  75  m   (  ±  245  ft  ) ;                      and

3 )   an  Assigned  Altitude  Deviation   (  AAD  )   of   ±  90  m   (  ±  300  ft  ).

b )    Reports  of   such  occurrences  shall  be  sent  to  the  Competent  Authority  within  72  hours. Reports  shall  include  an  initial  analysis  of  causal  factors  and  measures  taken  to  prevent  repeat occurrences ;

c )    When  Height - keeping  Errors  are  recorded  or  received,  the  Operator  shall  take  immediate action  to  rectify  the  conditions  that  caused  the  errors  and  provide  follow - up  reports,  if requested  by  the  Competent  Authority.

SUBPART   E.      LOW    VISIBILITY    OPERATIONS   (  LVO  )

SPA. LVO. 100     Low   Visibility   Operations

The  Operator  shall  only  conduct  the  following  Low  Visibility  Operations  ( LVO )  when  approved  by  the  Competent  Authority :

a )    Low  Visibility  Take - off  (  LVTO  )  Operation ;

b )    Lower  Than  Standard  Category  I   (  LTS  CAT  I  )  Operation ;

c )    Standard  Category  II  (  CAT  II  )  Operation ;

d )    other  than  standard  category  II   (  OTS  CAT  II  )  Operation ;

e )    Standard  Category  III   (  CAT  III  )  Operation ;

f )    approach  operation  utilizing  Enhanced  Vision  Systems  (  EVS  )  for  which  an  operational credit  is  applied  to  reduce  the  Runway  Visual  Range  ( RVR )  Minima  by  no  more  than  one   third   of   the   published   RVR.

SPA. LVO. 105     LVO   Approval

To  obtain  an  LVO  approval  from  the  Competent  Authority,  the  Operator  shall  demonstrate compliance  with  the  requirements  of  this  Subpart.

SPA. LVO. 110    General   Operating   Requirements

a )    The  Operator  shall  only  conduct  LTS  CAT  I  Operations  if :

1 )   each  aircraft  concerned  is  certified  for  operations  to  conduct  CAT  II  operations ;     and

2 )   the  approach  is  flown :

( i )   auto-coupled  to  an  Auto-land  that  needs  to  be  approved  for  CAT  III A  operations ;   or

( ii )  using  an  approved  Head - up  Display  Landing  System  ( HUDLS )  to  at  least  150  ft  above  the   threshold.

b )    The  Operator  shall  only  conduct  CAT  II,  OTS  CAT  II  or  CAT  III  Operations  if :

1 )   each  aircraft  concerned  is  certified  for  operations  with  a  Decision  Height  ( DH )  below  200  ft,  or  no  DH,  and  equipped  in  accordance  with  the  applicable  airworthiness  requirements ;

2 )   a  system  for  recording  approach  and  / or  automatic  landing  success  and  failure  is

established  and  maintained  to  monitor  the  overall  safety  of  the  operation ;                                                3 )   the  DH  is  determined  by  means  of  a  Radio  Altimeter ;

4 )   the  flight  crew  consists  of  at  least  2  ( two )  pilots ;

5 )   all  height  call - outs  below  200  ft  above  the  aerodrome  threshold  elevation  are  determined   by  a  Radio  Altimeter.

c )    The  Operator  shall  only  conduct  approach  operations  utilizing  an  EVS  if :

1 )   the  EVS  is  certified for  the  purpose  of  this  Subpart  and  combines  infra - red  sensor  image  and  flight  information  on  the  HUD ;

2 )   for  operations  with  an  RVR  below 550 m,  the  flight  crew  consists  of  at  least  two  pilots ;

3 )   for  CAT I  operations,  natural  visual  reference  to  runway  cues  is  attained  at  least  at               100  ft  above  the  aerodrome  threshold  elevation ;

4 )   for  approach  procedure  with  Vertical  Guidance  ( APV )  and  Non-precision  Approach                       ( NPA )  Operations  flown  with  CDFA  technique,  natural  visual  reference  to  runway  cues  is  attained  at  least  at  200 ft  above  the  aerodrome  threshold  elevation  and  the  following requirements  are  complied  with :

( i )    the  approach  is  flown  using  an  approved  vertical  flight  path  guidance  mode ;

( ii )   the  approach  segment  from  Final  Approach  Fix  ( FAF )  to  runway  threshold  is  straight  and  the  difference between  the  final  approach  course  and  the  runway  centreline  is  not  greater than  2o ;

( iii )  the  final  approach  path  is  published  and  not  greater  than  3,7o ;

( iv )  the  maximum  cross-wind  components  established  during  certification  of  the  EVS  are  not  exceeded.

SPA. LVO. 115     Aerodrome   Related   Requirements

a )    The  Operator  shall  not  use  an  aerodrome  for  LVO’s  below  a  visibility  of  800 m  unless :

1 )   the  aerodrome  has  been  approved  for  such  operations  by  the  State  of  the  aerodrome ;                                                                                                          and

2 )   Low  Visibility  Procedures  ( LVP )  have  been  established.

b )    If  the  Operator  selects  an  aerodrome  where  the  term  LVP  is  not  used,  the  Operator  shall ensure  that  there  are  equivalent  procedures  that  adhere  to  the  requirements  of  LVP  at  the aerodrome.  This  situation  shall  be  clearly  noted  in  the  Operations  Manual  or  Procedures  Manual  including  guidance  to  the  flight  crew  on  how  to  determine  that  the  equivalent  LVP  are  in  effect.

SPA. LVO. 120     Flight   Crew   Training   and   Qualifications

The  Operator  shall  ensure  that,  prior  to  conducting  an  LVO :

a )    each  flight  crew  member :

1 )   complies  with  the  training  and  checking  requirements  prescribed  in  the  Operations  Manual,  including  Flight  Simulation  Training  Device ( FSTD )  Training,  in  operating  to  the  limiting values  of  RVR / VIS  ( visibility )  and  DH  specific  to  the  operation  and  the  aircraft  type ;

2 )   is  qualified  in  accordance  with  the  standards  prescribed  in  the  Operations  Manual ;

b )    the  Training  and  Checking  is  conducted  in  accordance  with  a  detailed  syllabus.

SPA. LVO. 125     Operating   Procedures

a )    The  Operator  shall  establish  procedures  and  instructions  to  be  used  for  LVO’s.                          These  procedures  and  instructions  shall  be  included  in  the  Operations  Manual  or  Procedures Manual  and  contain  the  duties  of  flight crew  members  during  taxiing,  take-off,  approach,  flare, landing,  rollout  and  missed  approach  operations,   as  appropriate ;

b )    Prior  to  commencing  an  LVO,  the  Pilot - in - Command  /  Commander  shall  be  satisfied that :

1 )   the  status  of  the  visual  and  non-visual  facilities  is  sufficient ;

2 )   appropriate  LVP’s  are  in  force  according  to  information  received  from  Air  Traffic  Services   ( ATS ) ;

3 )   flight  crew  members  are  properly  qualified.

SPA. LVO. 130     Minimum   Equipment

a )    The  Operator  shall  include  the  minimum  equipment  that  has  to  be  serviceable  at  the commencement  of  an  LVO  in  accordance  with  the  Aircraft  Flight  Manual  ( AFM )  or  other approved  document  in  the  Operations  Manual  or  Procedures  Manual,  as  applicable.                                 b )    The  Pilot - in - Command  /  Commander  shall  be  satisfied  that  the  status  of  the  aircraft  and  of  the  relevant  airborne  systems  is  appropriate  for  the  specific  operation  to  be  conducted.

INTENTIONALLY            LEFT            BLANK

SUBPART   F.     EXTENDED   RANGE   OPERATIONS   with

TWO - ENGINED   AEROPLANES   (  ETOPS  )

SPA. ETOPS. 100     ETOPS

In  Commercial  Air  Transport  Operations,  two - engined  aeroplanes  shall  only  be  operated  beyond  the  threshold  distance  determined  in  accordance  with  CAT. OP. MPA. 140  if  the  Operator  has been  granted  an  ETOPS  operational  approval  by  the  Competent  Authority.

SPA. ETOPS. 105     ETOPS   Operational   Approval

To  obtain  an  ETOPS  operational  approval  from  the  Competent  Authority,  the  Operator  shall provide  evidence  that :

a )    the  aeroplane  /  engine  combination  holds  an  ETOPS  type  design  and  reliability  approval for  the  intended  operation ;

b )    a  training  programme  for  the  flight  crew  members  and  all  other  operations  personnel involved  in  these  operations  has  been  established  and  the  flight  crew  members  and  all  other operations  personnel  involved  are  suitably  qualified  to  conduct  the  intended  operation ;

c )    the  Operator’s  organization  and  experience  are  appropriate  to  support  the  intended operation ;

d )    operating  procedures  have  been  established.

SPA. ETOPS. 110     ETOPS   En - route   Alternate   Aerodrome

a )    An  ETOPS  En-route  alternate  aerodrome  shall  be  considered  adequate,  if,  at  the  expected time  of  use,  the  aerodrome  is  available  and  equipped  with  necessary  ancillary  services  such  as Air  Traffic  Services ( ATS ),  sufficient  lighting,  communications,  weather  reporting,  navigation aids  and  emergency  services  and  has  at  least  one  instrument  approach  procedure  available ;

b )    Prior  to  conducting  an  ETOPS  flight,  the  Operator  shall  ensure  that  an  ETOPS  En-route alternate  aerodrome  is  available,  within  either  the  Operator’s  approved  diversion  time,  or  a diversion  time  based  on  the  MEL  generated  serviceability  status  of  the  aeroplane,  whichever  is shorter ;

c )    The  Operator  shall  specify  any  required  ETOPS  En-route  alternate  aerodrome(s)  in  the Operational  Flight  Plan  and  ATS  Flight  Plan.

SPA. ETOPS. 115     ETOPS  En - route   Alternate   Aerodrome   Planning   Minima

a )    The  Operator  shall  only  select  an  aerodrome  as  an  ETOPS  En - route  alternate  aerodrome when  the  appropriate  weather  reports  or  forecasts,  or  any  combination  thereof,  indicate  that, between  the  anticipated  time  of  landing  until  one  hour  after  the  latest  possible  time  of  landing,  conditions  will  exist  at  or  above  the  planning  minima  calculated  by  adding  the additional  limits  of   Table 1 ;

b )    The  Operator  shall  include  in  the  Operations  Manual  the  method  for  determining  the operating  minima  at  the  planned  ETOPS  En - route  alternate  aerodrome.

Table  1.    Planning   Minima   for   the   ETOPS   En - route   Alternate   Aerodrome

| Type of Approach | Planning Minima |
| --- | --- |
| Precision Approach | DA / H + 200 ft, RVR / VIS + 800 m ( * ) |
| Non - precision Approach or Circling Approach | MDA / H + 400 ft, RVR / VIS + 1 500 m (*) |
| ( * ) VIS : Visibility ; MDA / H : Minimum Descent Altitude / Height. |  |

SUBPART   G.     TRANSPORT    of    DANGEROUS    GOODS

SPA. DG. 100     Transport  of  Dangerous  Goods

Except  as  provided  for  in  Annex  IV  ( Part - CAT ),  Annex  VI  ( Part - NCC ),  Annex  VII  ( Part - NCO )  and  Annex  VIII  ( Part - SPO ),  the  Operator  shall  only  transport  Dangerous  Goods  by air  if  the  Operator  has  been  approved  by  the  GDCA  of  RA.

SPA. DG. 105     Approval   to   Transport   Dangerous   Goods

To  obtain  the  approval  to  transport  dangerous  goods,  the  Operator  shall  in  accordance  with  the  technical  instructions :

a )    establish  and  maintain  a  training  programme  for  all  personnel  involved  and  demonstrate  to  the  Competent  Authority  that  adequate  training  has  been  given  to  all  personnel ;

b )    establish  operating  procedures  to  ensure  the  safe  handling  of  dangerous  goods  at  all  stages  of  air  transport,  containing   information  and  instructions  on :

1 )    the  Operator’s  policy  to  transport  dangerous  goods ;

2 )    the  requirements  for  acceptance,  handling,  loading,  stowage  and  segregation  of  dangerous  goods ;

3 )    actions  to  take  in  the  event  of  an  aircraft  accident  or  incident  when  dangerous  goods  are  being  carried ;

4 )    the  response  to  emergency  situations  involving  dangerous  goods ;

5 )    the  removal  of  any  possible  contamination ;

6 )    the  duties  of  all  personnel  involved,  especially  with  relevance  to  ground  handling  and aircraft  handling ;

7 )    inspection  for  damage,  leakage  or  contamination ;

8 )    dangerous  goods  accident  and  incident  reporting.

SPA. DG. 110     Dangerous   Goods   Information   and   Documentation

The  Operator  shall,  in  accordance  with  the  technical  instructions :

a )    provide  written  information  to  the  Pilot - in - Command  /  Commander :

1 )   about  dangerous  goods  to  be  carried  on  the  aircraft ;

2 )   for  use  in  responding  to  in-flight  emergencies ;

b )    use  an  acceptance  checklist ;

c )    ensure  that  dangerous  goods  are  accompanied  by  the  required  dangerous  goods  transport document(s),  as  completed  by  the  person  offering  dangerous  goods  for  air  transport,  except when  the  information  applicable  to  the  dangerous  goods  is  provided  in  electronic  form ;

d )    ensure  that  where  a  dangerous  goods  transport  document  is  provided  in  written  form,  a copy  of  the  document  is  retained  on  the  ground  where  it  will  be  possible  to  obtain  access  to it  within  a  reasonable  period  until  the  goods  have  reached  their  final  destination ;

e )    ensure  that  a  copy  of  the  information  to  the  Pilot - in - Command  /  Commander  is  retained  on  the  ground  and  that  this  copy,  or  the  information  contained  in  it,  is  readily accessible  to  the  aerodromes  of  last  departure  and  next  scheduled  arrival,  until  after  the  flight to  which  the  information  refers ;

f )    retain  the  acceptance  Checklist,  transport  document  and  information  to  the  Pilot - in - Command  /  Commander  for  at  least  3  ( three )  months  after  completion  of  the  flight ;

g )    retain  the  training  records  of  all  personnel  for  at  least  3  ( three )  years.

SUBPART  H.     HELICOPTER   OPERATIONS   WITH   NIGHT   VISION

IMAGING   SYSTEMS   (  NVIS  )

SPA. NVIS. 100     Night   Vision   Imaging   System   ( NVIS )   Operations

a )    Helicopters  shall  only  be  operated  under  VFR  at  night  with  the  aid  of  NVIS  if  the  Operator  has been  approved  by  the  Competent  Authority ;

b )    To  obtain  such  approval  by  the  Competent  Authority,  the  Operator  shall :

1 )  operate  in  Commercial  Air  Transport  ( CAT )  and  hold  a  CAT AOC  in  accordance  with  Annex  III  ( Part - ORO ) ;

2 )   demonstrate  to  the  Competent  Authority :

( i )   compliance  with  the  applicable  requirements  contained  in  this  Subpart ;

( ii )  the  successful  integration  of  all  elements  of  the  NVIS.

SPA. NVIS. 110     Equipment   Requirements   for   NVIS   Operations

a )    Before  conducting  NVIS  operations  each  helicopter  and  all  associated  NVIS  equipment  shall  have been  issued  with  the  relevant  airworthiness  approval  in  accordance  with  Regulation      ( EC )  No 1702 / 2003 ;

b )    Radio  Altimeter.   The  helicopter  shall  be  equipped  with  a  radio  altimeter  capable  of  emitting  an audio  warning  below  a  pre - set  height  and  an  audio  and  visual  warning  at  a  height  selectable  by  the pilot,  instantly  discernable  during  all  phases  of  NVIS  flight ;

c )    Aircraft  NVIS  Compatible  Lighting.    To  mitigate  the  reduced  peripheral  vision  cues  and  the  need to  enhance  situational  awareness,  the  following  shall  be  provided :

1 )   NVIS - compatible  instrument  panel  flood - lighting,  if  installed,  that  can  illuminate  all  essential flight  instruments ;

2 )   NVIS - compatible  utility  lights ;

3 )   portable  NVIS  compatible  flashlight ;                                                               and

4 )   a  means  for  removing  or  extinguishing  internal  NVIS  non - compatible  lights.

d )   Additional  NVIS  Equipment.   The  following  additional  NVIS  equipment  shall  be  provided :

1 )   a  back - up  or  secondary  power  source  for  the  Night  Vision  Goggles  ( NVG ) ;

2 )   a  helmet  with  the  appropriate  NVG  attachment.

e )    All  required  NVG’s  on  an  NVIS  flight  shall  be  of  the  same  type,  generation  and  model ;

f )    Continuing   Airworthiness

1 )   Procedures  for  continuing  airworthiness  shall  contain  the  information  necessary  for  carrying  out ongoing  maintenance  and  inspections  on  NVIS  equipment  installed  in  the  helicopter  and  shall  cover,  as  a  minimum :

( i )     helicopter   windscreens   and   transparencies ;

( ii )    NVIS  lighting ;

( iii )   NVG’s ;                                                                        and

( iv )   any  additional  equipment  that  supports  NVIS  operations.

2 )   Any  subsequent  modification  or  maintenance  to  the  aircraft  shall  be  in  compliance  with  the  NVIS  airworthiness  approval.

SPA. NVIS. 120     NVIS   Operating   Minima

a )    Operations  shall  not  be  conducted  below  the  VFR  weather  minima  for  the  type  of  night  operations  being  conducted ;

b )    The  Operator  shall  establish  the  minimum  transition  height  from  where  a  change  to / from  aided flight  may  be  continued.

SPA. NVIS. 130     Crew   Requirements   for   NVIS   Operations

a )    Selection.   The  Operator  shall  establish  criteria  for  the  selection  of  crew  members  for  the  NVIS task ;

b )    Experience.    The  minimum  experience  for  the  Commander  shall  not  be  less  than  20  hours  VFR  at  night  as  Pilot - in - Command  /  Commander  of  a  helicopter  before  commencing  training ;

c )    Operational   Training.    All  pilots  shall  have  completed  the  operational  training  in accordance  with  the  NVIS  procedures  contained  in  the  Operations  Manual ;

d )    Recency.    All  pilots  and  NVIS  technical  crew  members  conducting  NVIS  operations  shall have  completed  3  ( three )  NVIS  flights  in  the  last  90 days.   Recency  may  be  re - established  on  a  training  flight  in  the  helicopter  or  an  approved  Full  Flight  Simulator  ( FFS ),  which  shall include  the  elements  of  ( f )( 1 ) ;

e )    Crew  Composition.    The  minimum  crew  shall  be  the  greater  of  that  specified :

1 )   in  the  Aircraft  Flight  Manual   ( AFM ) ;

2 )   for  the  underlying  activity ;                                                  or

3 )   in  the  operational  approval  for  the  NVIS  operations.

f )    Crew  Training   and   Checking

1 )   Training  and  Checking  shall  be  conducted  in  accordance  with  a  detailed  syllabus

approved  by  the  Competent  Authority  and  included  in  the  Operations  Manual ;                                                               2 )   Crew  members :

( i )   crew  training  programmes  shall :  - improve  knowledge  of  the  NVIS  working  environment  and  equipment ;   -  improve  crew  coordination ;                                and                                        -  include  measures  to  minimize  the  risks  associated  with  entry  into  low  visibility  conditions  and  NVIS  normal  and  emergency procedures ;

( ii )  the  measures  referred  to  in  ( f )( 2 )( i )  shall  be  assessed  during :

( A )    night  Proficiency  Checks ;                                                    and

( B )    Line  Checks.

SPA. NVIS. 140     Information   and   Documentation

The  Operator  shall  ensure  that,  as  part  of  its  risk  analysis  and  management  process,  risks associated  with  the  NVIS  environment  are  minimized  by  specifying  in  the  Operations  Manual : -  selection,  composition  and  training  of  crews ;   -  levels of  equipment  and  dispatch  criteria ; and   -  operating  procedures  and  minima,  such  that  normal  and  likely  abnormal  operations  are described  and  adequately  mitigated.

INTENTIONALLY            LEFT            BLANK

INTENTIONALLY            LEFT            BLANK

SUBPART   I.     HELICOPTER   HOIST   OPERATIONS   (  HHO  )

SPA. HHO. 100     Helicopter  Hoist  Operations  (  HHO  )

a )    Helicopters  shall  only  be  operated  for  the  purpose  of  CAT  hoist  operations  if  the  operator  has  been  approved  by  the  Competent  Authority ;

b )    To  obtain  such  approval  by  the  Competent  Authority,  the  Operator  shall :

1 )   operate  in  CAT  and  hold  a  CAT  AOC  in  accordance  with  Annex  III ( Part - ORO ) ;

2 )   demonstrate  to  the  Competent  Authority  compliance  with  the  requirements  contained  in  this  Subpart.

SPA. HHO. 110     Equipment   Requirements   for   HHO

a )    The  installation  of  all  helicopter  hoist  equipment,  including  any  radio  equipment  to  comply  with  SPA. HHO. 115,  and  any  subsequent  modifications,  shall  have  an  airworthiness approval  appropriate  to  the  intended  function.  Ancillary  equipment  shall  be  designed  and  tested to  the  appropriate  standard  as  required  by  the  Competent  Authority ;

b )    Maintenance  instructions  for  HHO  equipment  and  systems  shall  be  established  by  the Operator  in  liaison  with  the  manufacturer  and  included  in  the  Operator’s  helicopter  maintenance  programme  as  required  by  CR - EC  No  2042 / 2003.

SPA. HHO. 115     HHO   Communication

Two - way  radio  communication  shall  be  established  with  the  organization  for  which  the  HHO is  being  provided  and,  where  possible,  a  means  of  communicating  with  ground  personnel  at the  HHO  site  for :

( a )    day  and  night  offshore  operations ;

( b )    night  onshore  operations,  except  for  HHO  at  a  Helicopter  Emergency  Medical  Services

( HEMS )  operating  site.

SPA. HHO. 125     Performance   Requirements   for   HHO

Except  for  HHO  at  a  HEMS  operating  site,  HHO  shall  be  capable  of  sustaining  a  critical engine  failure  with  the  remaining  engine(s)  at  the  appropriate  power  setting  without  hazard  to the  suspended  person(s)  /  cargo,  third  parties  or  property.

SPA. HHO. 130     Crew   Requirements   for   HHO

a )    Selection.    The  Operator  shall  establish  criteria  for  the  selection  of  flight  crew  members for  the  HHO  task,  taking  previous  experience  into  account ;

b )    Experience.    The  minimum  experience  level  for  the  Commander  conducting  HHO  flights shall  not  be  less  than :

1 )    Offshore :

( i )   1 000  hours  as  Pilot - in - Command  /  Commander  of  helicopters,  or  1 000  hours  as                    Co-pilot  in  HHO  of  which  200 hours  is  as  Pilot - in - Command  under  supervision ;

and

( ii )  50  hoist  cycles  conducted  offshore,  of  which  20 cycles shall  be  at  night  if  night  operations  are  being  conducted,  where  a  hoist  cycle  means  one  down - and - up  cycle  of  the  hoist  hook.

2 )    Onshore :

( i )    500  hours  as  Pilot - in - Command  /  Commander  of  helicopters,  or  500  hours  as  Co-pilot  in  HHO  of  which 1 00 hours  is  as  Pilot - in - Command  under  supervision ;

( ii )   200  hours  operating  experience  in  helicopters  gained  in  an  operational  environment  similar  to  the  intended  operation ;                                               and

( iii )  50  hoist  cycles,  of  which  20 cycles  shall  be  at  night  if  night  operations  are  being conducted.

c )    Operational   Training   and   Experience.    Successful  completion  of  training  in  accordance with  the  HHO  procedures  contained  in  the  Operations  Manual  and  relevant  experience  in  the role  and  environment  under  which  HHO  are  conducted ;

d )    Recency.    All  pilots  and  HHO  crew  members  conducting  HHO  shall  have  completed  in the  last  90 days :

1 )   when  operating  by  day :  - any  combination  of  3  ( three )  day  or  night  hoist  cycles,  each  of  which  shall  include  a  transition  to  and  from  the  hover ;

2 )   when  operating  by  night :   -  3  ( three )  night  hoist  cycles,  each  of  which  shall  include  a transition  to  and  from  the  hover.

e )    Crew  Composition.    The  minimum  crew for  day  or  night  operations  shall  be  as  stated  in the  Operations  Manual.  The  minimum  crew  will  be  dependent  on  the  type  of  helicopter,  the weather  conditions,  the  type  of  task,  and,  in  addition  for  offshore  operations,  the  HHO  site environment,  the  sea  state  and  the  movement  of  the  vessel.  In  no  case  shall  the  minimum crew  be  less  than  one  pilot  and  one  HHO  crew  member ;

f )    Training   and   Checking

1 )   Training  and  Checking  shall  be  conducted  in  accordance  with  a  detailed  syllabus  approved  by  the  Competent  Authority  and  included  in  the  Operations  Manual ;

2 )    Crew  members :

( i )   crew  training  programmes  shall :   -  improve  knowledge  of  the  HHO  working  environment  and  equipment ;   -  improve  crew  coordination ;             and                                     -  include  measures  to  minimize  the  risks  associated  with  HHO  normal  and  emergency  procedures  and  static  discharge ;

( ii )  the  measures  referred  to  in  ( f )( 2 )( i )  shall  be  assessed  during  Visual  Meteorological Conditions  ( VMC )  day  Proficiency  Checks,  or  VMC  night  Proficiency  Checks  when  night  HHO  are  undertaken  by  the  Operator.

SPA. HHO. 135     HHO   Passenger   Briefing

Prior  to  any  HHO  flight,  or  series  of  flights,  HHO  passengers  shall  have  been  briefed  and made  aware  of  the  dangers  of  static  electricity  discharge  and  other  HHO  considerations.

SPA. HHO. 140     Information   and   Documentation

a )    The  Operator  shall  ensure  that,  as  part  of  its  risk  analysis  and  management  process,  risks associated  with  the  HHO  environment  are  minimized  by  specifying  in  the  Operations  Manual : - selection, composition  and  training  of  crews ;  - levels  of  equipment  and  dispatch  criteria ;  and - operating  procedures  and  minima,  such  that  normal  and  likely  abnormal  operations  are described  and  adequately  mitigated ;

b )    Relevant  extracts  from  the  Operations  Manual  shall  be  available  to  the  organization  for which  the  HHO  is  being  provided.

INTENTIONALLY            LEFT            BLANK

SUBPART   J.     HELICOPTER   EMERGENCY   MEDICAL   SERVICE

OPERATIONS   (  HEMS  )

SPA. HEMS. 100     Helicopter  Emergency  Medical   Service  ( HEMS )  Operations

a )    Helicopters  shall  only  be  operated  for  the  purpose  of  HEMS  operations  if  the  Operator has  been  approved  by  the  Competent  Authority ;

b )    To  obtain  such  approval  by  the  Competent  Authority,  the  Operator  shall :

1 )   operate  in  CAT  and  hold  a  CAT  AOC  in  accordance  with  Annex  III  ( Part - ORO ) ;

2 )   demonstrate  to  the  Competent  Authority  compliance  with  the  requirements  contained  in  this  Subpart.

SPA. HEMS. 110     Equipment   Requirements   for   HEMS   Operations

The  installation  of  all  helicopter  dedicated  medical  equipment  and  any  subsequent  modifications  and,  where  appropriate,  its  operation  shall  be  approved  in  accordance  with                CR - EU No 748 / 2012

SPA. HEMS. 115     Communication

In  addition  to  that  required  by  CAT. IDE. H,  helicopters  conducting  HEMS  flights  shall  have communication  equipment  capable  of  conducting  two - way  communication  with  the  organization  for  which  the  HEMS  is  being  conducted  and,  where  possible,  to  communicate with  ground  emergency  service  personnel.

SPA. HEMS. 120     HEMS   Operating   Minima

a )   HEMS  flights  operated  in  Performance  Class  1  and  2  shall  comply  with  the  weather minima  in  Table 1.  for  dispatch  and  En-route  phase  of  the  HEMS  flight.                                               In  the  event  that  during  the  En-route  phase  the  weather  conditions  fall  below  the  cloud  base or  visibility  minima  shown,  helicopters  certified  for  flights  only  under  VMC  shall  abandon  the flight  or  return  to  base.                                                                                                                            Helicopters  equipped  and  certified  for  Instrument  Meteorological  Conditions  ( IMC )  operations may  abandon  the  flight,  return  to  base  or  convert  in  all  respects  to  a  flight  conducted  under Instrument  Flight  Rules  ( IFR ),  provided  the  flight  crew  are  suitably  qualified.

Table  1.    HEMS   Operating   Minima

| 2 Pilots |  | 1 Pilot |  |
| --- | --- | --- | --- |
| DAY |  |  |  |
| Ceiling | Visibility | Ceiling | Visibility |
| 500 ft and above | As defined by the applicable airspace VFR Minima | 500 ft and above | As defined by the applicable airspace VFR Minima |
| 499 - 400 ft | 1 000 m ( * ) | 499 – 400 ft | 2 000 m |
| 399 - 300 ft | 2 000 m | 399 – 300 ft | 3 000 m |
| NIGHT |  |  |  |
| Cloud Base | Visibility | Cloud Base | Visibility |
| 1 200 ft ( ** ) | 2 500 m | 1 200 ft ( ** ) | 3 000 m |

( * )   During  the  En-route  phase  visibility  may  be  reduced  to  800 m  for  short  periods  when  in  sight  of  land  if  the  helicopter  is  manoeuvred  at  a  speed   that  will  give  adequate   opportunity  to  observe  any  obstacles  in  time  to  avoid   a  collision ;

( ** )    During  the  En-route  phase,  cloud  base  may  be  reduced  to  1 000 ft  for  short  periods.

b )    The  weather  minima  for  the  dispatch  and  En - route  phase  of  a  HEMS  flight  operated  in Performance  Class  3  shall  be  a  cloud  ceiling  of  600  ft  and  a  visibility  of  1 500  m.                           Visibility  may  be  reduced  to   800  m   for  short  periods  when  in  sight  of  land  if  the  helicopter is  manoeuvred  at  a  speed  that  will  give  adequate  opportunity  to  observe  any  obstacle  and avoid  a  collision.

SPA. HEMS. 125     Performance   Requirements   for   HEMS   Operations

a )    Performance  Class  3  operations  shall  not  be  conducted  over  a  hostile  environment.

b )    Take - off  and  Landing :

1 )   Helicopters  conducting  operations  to / from  a  Final  Approach  and  Take-Off  Area ( FATO )  at  a  hospital  that  is  located  in  a  congested  hostile  environment  and  that  is  used  as  a  HEMS  operating  base  shall  be  operated  in  accordance  with  Performance  Class 1 ;

2 )   Helicopters  conducting  operations  to / from  a  FATO  at  a  hospital  that  is  located  in  a congested  hostile  environment  and  that  is  not  a  HEMS  operating  base  shall  be  operated  in  accordance  with  Performance   Class  1,  except  when  the  Operator  holds  an  approval  in accordance  with   CAT. POL. H. 225 ;

3 )   Helicopters  conducting  operations  to / from  a  HEMS  operating  site  located  in  a  hostile environment  shall  be  operated  in  accordance  with  Performance  Class  2  and  be  exempt  from the  approval  required  by  CAT. POL. H. 305 ( a ),   provided  compliance  is  shown  with   CAT. POL. H. 305 ( b )( 2 )  and  ( b )( 3 ) ;

4 )   The  HEMS  operating  site  shall  be  big  enough  to  provide  adequate  clearance  from  all obstructions.   For  night  operations,  the  site  shall  be  illuminated  to  enable  the  site  and  any  obstructions  to  be  identified.

SPA. HEMS. 130     Crew   Requirements

a )    Selection.    The  Operator  shall  establish  criteria  for  the  selection  of  flight  crew  members for  the  HEMS  task,  taking   previous  experience  into  account ;

b )    Experience.   The  minimum  experience  level for  the  Commander  conducting  HEMS  flights shall  not  be  less  than :

1 )    either :

( i )   1 000 hours  as  Pilot - in - Command  /  Commander  of  aircraft  of  which  500 hours  are  as  PIC  /  Commander  on  helicopters ;                                         or

( ii )  1 000 hours  as  Co-pilot  in  HEMS  operations  of  which  500 hours  are  as  Pilot - in -Command  under  supervision   and   100 hours  pilot - in - Command  /  Commander  of  helicopters ;

2 )    500 hours’  operating  experience  in  helicopters,  gained  in  an  operational  environment similar  to  the  intended  operation ;                                     and

3 )    for  pilots  engaged  in  night  operations,   20 hours  of  VMC  at  night  as  Pilot - in - Command  /  Commander.

c )    Operational  Training.    Successful  completion  of  operational  training  in  accordance  with the  HEMS  procedures  contained  in  the  Operations  Manual ;

d )    Recency.    All  pilots  conducting  HEMS  operations  shall  have  completed  a  minimum  of            30  minutes’  flight  by  sole  reference  to  instruments  in  a  helicopter  or  in  an  FSTD  within  the last  six  months ;

e )    Crew  Composition

1 )    Day  Flight.    The  minimum  crew  by  day  shall  be  one  pilot  and  one  HEMS  technical crew  member ;

( i )   this  may  be  reduced  to  one  pilot  only  when :

( A )   at  a  HEMS  operating  site  the  Commander  is  required  to  fetch  additional  medical supplies.  In  such case  the  HEMS  technical  crew  member  may  be  left  to  give  assistance  to  ill  or  injured  persons  while  the  Commander  undertakes  this  flight ;

( B )   after  arriving  at  the  HEMS  operating  site,  the  installation  of  the  stretcher  precludes  the  HEMS  technical  crew  member  from  occupying  the  front  seat ;    or

( C )   the  medical  passenger  requires  the  assistance  of  the  HEMS  technical  crew  member  in  flight.

( ii )    in  the  cases  described  in  ( i ),  the  operational  minima  shall  be  as  defined  by  the applicable  airspace  requirements ;   -  the  HEMS  operating  minima  contained  in  Table 1 of   SPA. HEMS. 120   shall  not  be  used ;

( iii )   only  in  the  case  described  in  ( i )( A )  may  the  Commander  land  at  a  HEMS  operating site  without  the  technical  crew  member  assisting  from  the  front  seat ;

2 )    Night  Flight.    The  minimum  crew  by  night  shall  be :

( i )    two  pilots ;                                      or

( ii )   one  pilot  and  one  HEMS  technical  crew  member  in  specific  geographical  areas  defined by  the  Operator  in  the  Operations  Manual  taking  into  account  the  following :

( A )    adequate  ground  reference ;

( B )    flight following  system  for  the  duration  of  the  HEMS  mission ;

( C )    reliability  of  weather  reporting  facilities ;

( D )    HEMS  Minimum  Equipment  List ;

( E )     continuity  of  a  crew  concept ;

( F )     minimum  crew  qualification,  initial  and  recurrent  training ;

( G )     operating  procedures,  including  crew  coordination ;

( H )     weather  minima ;                                                           and

( I )       additional  considerations  due  to  specific  local  conditions.

f )    Crew   Training   and   Checking

1 )   Training  and  Checking s hall  be  conducted  in  accordance  with  a  detailed  syllabus  approved  by  the  Competent  Authority  and  included  in  the  Operations  Manual ;

2 )   Crew   members :

( i )   crew  training  programmes  shall :  - improve  knowledge  of  the  HEMS  working  environment  and  equipment ;   -  improve  crew  coordination ;    and   -  include  measures  to  minimize  the  risks  associated  with  En-route  transit  in  low  visibility  conditions,  selection  of  HEMS  operating  sites and  approach  and  departure  profiles ;

( ii )  the  measures  referred  to  in  ( f )( 2 )( i )  shall  be  assessed  during :

( A )   VMC day  Proficiency  Checks,   or  VMC  night  Proficiency  Checks  when  night HEMS  operations  are  undertaken  by  the  Operator ;                            and

( B )    Line  Checks.

SPA. HEMS. 135     HEMS   Medical   Passenger   and   other   Personnel   Briefing

a )    Medical   Passenger.    Prior  to  any  HEMS  flight,  or  series  of  flights,  medical  passengers shall  have  been  briefed  to  ensure  that  they  are  familiar  with  the  HEMS  working  environment and  equipment,  can  operate  on-board  medical  and  emergency  equipment  and  can  take  part  in normal  and  emergency  entry  and  exit  procedures ;

b )    Ground   Emergency   Service   Personnel.    The  Operator  shall  take  all  reasonable  measures to  ensure  that  ground  emergency  service  personnel  are  familiar  with  the  HEMS  working environment  and  equipment  and  the  risks  associated  with  ground  operations  at  a  HEMS operating  site ;

c )    Medical   Patient.    Notwithstanding   CAT. OP. MPA. 170,  a  briefing  shall  only  be  conducted  if  the  medical  condition  makes  this  practicable.

SPA. HEMS. 140     Information   and   Documentation

a )    The  Operator  shall  ensure  that,  as  part  of  its  risk  analysis  and  management  process,  risks associated  with  the  HEMS  environment  are  minimized  by  specifying  in  the  Operations  Manual:  -  selection, composition  and  training  of  crews ;  - levels  of  equipment  and  dispatch  criteria ; and -  operating  procedures  and  minima,  such  that  normal  and  likely  abnormal  operations  are described  and  adequately  mitigated ;

b )    Relevant  extracts  from  the  Operations  Manual  shall  be  made  available  to  the  organization for  which  the  HEMS  is  being  provided.

SPA. HEMS. 145     HEMS   Operating   Base   Facilities

a )    If  crew  members  are  required  to  be  on  standby  with  a  reaction  time  of  less  than                     45  minutes,  dedicated  suitable  accommodation  shall  be  provided  close  to  each  operating  base ;

b )    At  each  operating  base  the  pilots  shall  be  provided  with  facilities  for  obtaining  current and  forecast  weather  information  and  shall  be  provided  with  satisfactory  communications  with the  appropriate  Air  Traffic  Services ( ATS )  unit.                                                                                         Adequate  facilities  shall  be  available  for  the planning  of  all  tasks.

SPA. HEMS. 150     Fuel   Supply

a )    When  the  HEMS  mission  is  conducted  under  VFR  within  a  local  and  defined geographical  area,  standard  fuel  planning  can  be  employed  provided  the  Operator  establishes final  reserve  fuel  to  ensure  that,  on  completion  of  the  mission  the  fuel  remaining  is  not  less than  an  amount  of  fuel  sufficient  for :

( 1 )    30  minutes  of  flying  time  at  normal  cruising  conditions ;             or

( 2 )    when  operating  within  an  area  providing  continuous  and  suitable  precautionary  landing

sites,  20  minutes  of  flying  time  at  normal  cruising  speed.

SPA. HEMS. 155     Refuelling  with  Passengers  Embarking,  on  Board  or  Disembarking

When  the  Commander  considers  refuelling  with  passengers  on  board  to  be  necessary,  it  can  be  undertaken  either  rotors  stopped  or  rotors  turning  provided  the  following  requirements  are met :

( a )    door(s)  on  the  refuelling  side  of  the  helicopter  shall  remain  closed ;

( b )    door(s)  on  the  non - refuelling  side  of  the  helicopter  shall  remain  open,  weather permitting ;

( c )    fire  fighting  facilities  of  the  appropriate  scale  shall  be  positioned  so  as  to  be immediately  available  in  the  event  of  a  fire ;                                           and

( d )    sufficient  personnel  shall  be  immediately  available  to  move  patients  clear  of  the helicopter  in  the  event  of  a  fire.

INTENTIONALLY            LEFT            BLANK