ANNEX  III.     PART - ORO

ORGANIZATION    REQUIREMENTS     FOR     AIR     OPERATIONS

COMMISSION    REGULATION   ( EU )   No   83 / 2014       of   29   January   2014    amending

CR - EU  No   965 / 2012   laying   down   technical   requirements   and administrative   procedures

related   to   Air   Operations   pursuant   to  CR - EC  No 216 / 2008   of   the   European  Parliament  and

of  the  Council.

In  Annex  III  to  CR - EU  No  965 / 2012,  the  following  Subpart  FTL  is  added .

INTENTIONALLY            LEFT            BLANK

ORGANISATION    REQUIREMENTS     FOR     AIR     OPERATIONS

CONTENT

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    7

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    11

ORO. GEN. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  15

SUBPART   GEN.   General  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    17

Section  I.     General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

ORO. GEN. 105   Competent  Authority    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    17

ORO. GEN. 110   Operator  Responsibilities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    17

ORO. GEN. 115   Application  for  an  AOC   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    18

ORO. GEN. 120   Means  of  Compliance   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   18

ORO. GEN. 125   Terms  of  Approval  and  Privileges  of  an  AOC  Holder   .  .  .  .  .  .  .  .  .  .    18

ORO. GEN. 130   Changes   Related  to  an  AOC  Holder   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    19

ORO. GEN. 135   Continued  Validity   of  an  AOC  Holder   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

ORO. GEN. 140   Access   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  19

ORO. GEN. 150   Findings   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  20

ORO. GEN. 155   Immediate  reaction  to  a  Safety  Problem   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  29

ORO. GEN. 160   Occurrence  Reporting   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    20

Section  II.    Management    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   21

ORO. GEN. 200   Management  System   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     21

ORO. GEN. 205   Contracted  Activities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     21

ORO. GEN. 210   Personnel   Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  21

ORO. GEN. 215   Facility  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    22

ORO. GEN. 220   Record - Keeping   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   22

SUBPART   AOC.   Air  Operator  Certification   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

ORO. AOC. 100   Application  for  an  Air  Operator  Certificate    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

ORO. AOC. 105   Operations   Specifications   and   Privileges   of   an   AOC   Holder   .  .  .  .  .   23

ORO. AOC. 110   Leasing   Agreement   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

ORO. AOC. 115   Code - Share   Agreements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   .  24

ORO. AOC. 120   Approvals  to  provide  Cabin  Crew  Training  and  to  issue  Cabin  Crew

Attestations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     24

ORO. AOC. 125    Non - Commercial  Operations  of  Aircraft  listed  in  the  Operations

Specifications  by  the  Holder  of  an  AOC    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    25

ORO. AOC. 130    Flight  Data  Monitoring  —  Aeroplanes   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    25

ORO. AOC. 135   Personnel  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  26

ORO. AOC. 140   Facility  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   26

ORO. AOC. 150   Documentation  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   26

SUBPART   DEC.    Declaration    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    27

ORO. DEC. 100  Declaration   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    27

SUBPART   SPO.    Commercial   Specialized   Operations    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  29

ORO. SPO. 100   Common  Requirements  for  Commercial  Specialized  Operators   .  .  .  .  .  .    29

ORO. SPO. 110   Authorization  of  High  Risk  Commercial  Specialized  Operations   .  .  .  .  .    29

ORO. SPO. 115   Changes   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   30

ORO. SPO. 120   Continued  Validity   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   30

SUBPART  MLR.   Manuals,  Logs  and   Records    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    31

ORO. MLR. 100   Operations   Manual  —  General   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     31

ORO. MLR. 101   Operations  Manual  —  Structure   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    32

ORO. MLR. 105   Minimum   Equipment   List   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   32

ORO. MLR. 110   Journey   Log   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   33

ORO. MLR. 115   Record - Keeping   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   33

SUBPART   SEC.    Security    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   35

ORO. SEC. 100. A   Flight   Crew   Compartment   Security    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

ORO. SEC. 100. H    Flight   Crew   Compartment   Security    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   35

SUBPART   FC.      Flight  Crew     .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    37

ORO. FC. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  37

Section  1.  Common  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    37

ORO. FC. 100   Composition   of   Flight   Crew   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   37

ORO. FC. 105   Designation  as  Pilot - in - Command  /  Commander   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  38

ORO. FC. 110   Flight  Engineer   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    38

ORO. FC. 115   Crew  Resource  Management  ( CRM )  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     38

ORO. FC. 120   Operator  Conversion  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .   .  .  .  .  .  .  .  .  .  .  .  .   38

ORO. FC. 125   Differences  Training  and  Familiarization  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   39

ORO. FC. 130   Recurrent  Training  and  Checking   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    39

ORO. FC. 135   Pilot  Qualification  to  Operate  in  Either  Pilot’s  Seat   .  .  .  .  .  .  .  .  .  .  .  .  .     39

ORO. FC. 140   Operation  on  More  than  One  Type  or  Variant   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     39

ORO. FC. 145   Provision  of  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   39

Section  2.  Additional  Requirements  for  Commercial  Air  Transport  Operations  .  .  .  .  .  41

ORO. FC. 200   Composition  of  Flight  Crew   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   41

ORO. FC. A. 201   In - Flight  Relief  of  Flight  Crew  Members  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     41

ORO. FC. 202   Single - Pilot  Operations  under  IFR  or  at  Night   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   42

ORO. FC. 205   Command  Course   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   43

ORO. FC. 215   Initial  Operator’s  Crew  Resource  Management  ( CRM )  Training   .  .  .  .  .  .    43

ORO. FC. 220   Operator  Conversion  Training  and  Checking   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   43

ORO. FC. 230   Recurrent  Training  and  Checking   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    44

ORO. FC. 235   Pilot  Qualification  to  Operate  in  Either  Pilot’s  Seat    .  .  .  .  .  .  .  .  .  .  .  .  .    45

ORO. FC. 240   Operation  on  more  than  One  Type  or  Variant   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     46

ORO. FC. A. 245   Alternative  Training  and  Qualification  Programme    .  .  .  .  .  .  .  .  .  .  .  .  .  46

ORO. FC. A. 250   Commanders  Holding  a  CPL ( A )   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   47

ORO. FC. H. 250   Commanders  Holding  a  CPL ( H )   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   48

Section  3.   Additional  Requirements  for  Commercial  Specialized  Operations  and

CAT  Operations  referred  to  in  ORO. FC. 005 ( b )( 1 ) and ( 2 )   .  .  .  .  .  .  .   49

ORO. FC. 330  Recurrent  Training  and  Checking - Operator  Proficiency  Check   .  .  .  .  .  .  .  .  49

SUBPART  CC.       Cabin  Crew    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    51

ORO.  CC.  005     Scope    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     51

Section  1.    Common  Requirements    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   51

ORO. CC. 100   Number  and  Composition  of  Cabin  Crew    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    51

ORO. CC. 110   Conditions  for  Assignment  to  Duties   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     51

ORO. CC. 115   Conduct  of  Training  Courses  and  Associated  Checking   .  .  .  .  .  .  .  .  .  .  .   52

ORO. CC. 120   Initial  Training  Course   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  52

ORO. CC. 125   Aircraft  Type  Specific  Training  and  Operator  Conversion  Training  .  .  .  .  .   53

ORO. CC. 130   Differences  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    54

ORO. CC. 135   Familiarization  -  Line  Training  under  Supervision    .  .  .  .  .  .  .  .  .  .  .  .  .  .   54

ORO. CC. 140   Recurrent  Training    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   54

ORO. CC. 145   Refresher  Training    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   56

Section  2.   Additional  Requirements  for  Commercial  Air  Transport  Operations   .  .  .  .   57

ORO. CC. 200   Senior  Cabin  Crew  Member   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   57

ORO. CC. 201   Cabin  Crew  Instructor   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  57

ORO. CC. 205   Reduction  of  the  number  of  Cabin  Crew  during  Ground  Operations

and  in  unforeseen  circumstances   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  58

ORO. CC. 210   Additional  Conditions  for  Assignment  to  Duties   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  58

ORO. CC. 215   Training  and  Checking  Programs  and  related  Documentation   .  .  .  .  .  .  .  .  .  59

ORO. CC. 250   Operation  on  more  than  One  Aircraft  Type  or  Variant   .  .  .  .  .  .  .  .  .  .  .  .   59

ORO. CC. 255   Single  Cabin  Crew  Member  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  59

SUBPART    FTL.    Flight  &  Duty  Time  Limitations  &  Rest  Requirements   .  .  .  .  .  .  .    61

Section  1.    General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    61

ORO. FTL. 005    General   &   Applicability   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   61

ORO. FTL. 010    General  Principles    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   61

ORO. FTL. 100     Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  62

ORO. FTL. 105     Definitions   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   62

ORO. FTL. 109     Responsibilities  -  General   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    67

ORO. FTL. 110     Operator   Responsibilities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  67

ORO. FTL. 115     Crew   Member   Responsibilities    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  69

ORO. FTL. 116     Pilot - in - Command   Responsibilities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   70

ORO. FTL. 120     Fatigue   Risk   Management   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    70

ORO. FTL. 125     Flight   Time   Specification   Schemes    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  71

Section  II.   Commercial  Air  Transport  ( CAT )  Operators   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  73

ORO. FTL. 200     Home   Base   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  73

ORO. FTL. 205     Maximum  Daily  Flight   Duty   Period   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    73

ORO. FTL. 210     Flight  Block  Times  and  Duty  Time  Limitations   .  .  .  .  .  .  .  .  .  .  .  .  .  .    76

ORO. FTL. 212     Cabin  Crew  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     79

ORO. FTL. 215     Positioning   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  79

ORO. FTL. 220     Split  Duty   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   80

ORO. FTL. 225     Standby  and  Duties  at  the  Airport    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  81

ORO. FTL. 230     Reserve   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    81

ORO. FTL. 235     Rest  Periods   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  82

ORO. FTL. 240     Nutrition   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  84

ORO. FTL. 242     FDP, and  Simulator  Flight  Time  Limitations  during  Simulator  Training

activities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  84

ORO. FTL. 245     Records of Home Base, FDP, Flight Times, Flight  Block  Times,

and  Rest Periods   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    85

ORO. FTL. 250     Fatigue  Management  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   86

SUBPART   TC.   Technical  Crew  in  HEMS,  HHO  or  NVIS  Operations    .  .  .  .  .  .  .  .  .  89

ORO.TC.100   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  89

ORO. TC. 105   Conditions  for  Assignment  to  Duties   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    89

ORO. TC. 110   Training  and  Checking    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   89

ORO. TC. 115   Initial  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   90

ORO. TC. 120    Operator  Conversion  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    90

ORO. TC. 125   Differences  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   90

ORO. TC. 130   Familiarization  Flights   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   90

ORO. TC. 135   Recurrent  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   91

ORO. TC. 140   Refresher  Training   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   91

Appendix   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    93

GUIDANCE   MATERIAL   &   ACCEPTABLE   MEANS   of   COMPLIANCE    .  .  .  .  .  .   95

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE |  | TO PAGE | EFFECTIVE DATE |
| --- | --- | --- | --- | --- |
| CONTENT | 3 |  | 6 | 01 - November - 2014 |
| RECORDS of REVISION | 7 |  | 8 | 01 - November - 2014 |
| RECORDS of TEMPORARY REVISION | 9 |  | 10 | 01 - November - 2014 |
| LIST of EFFECTIVE PAGE | 11 |  | 14 | 01 - November - 2014 |
| ORO. GEN. 100 Scope | 15 |  |  | 01 - November - 2014 |
| SUBPART GEN. GENERAL REQUIREMENTS Section I. General | 17 |  | 22 | 01 - November - 2014 |
| Section I. General | 17 |  | 20 | 01 - November - 2014 |
| ORO. GEN. 105 Competent Authority | 17 |  |  | 01 - November - 2014 |
| ORO. GEN. 110 Operator Responsibilities | 17 |  |  | 01 - November - 2014 |
| ORO. GEN. 115 Application for an AOC | 18 |  |  | 01 - November - 2014 |
| ORO. GEN. 120 Means of Compliance | 18 |  |  | 01 - November - 2014 |
| ORO. GEN. 125 Terms of Approval and Privileges of an AOC Holder | 18 |  |  | 01 - November - 2014 |
| ORO. GEN. 130 Changes Relation to an AOC Holder | 18 |  |  | 01 - November - 2014 |
| ORO. GEN. 135 Continued Validity of an AOC Holder | 19 |  |  | 01 - November - 2014 |
| ORO. GEN. 140 Access | 19 |  |  | 01 - November - 2014 |
| ORO. GEN. 150 Findings | 20 |  |  | 01 - November - 2014 |
| ORO. GEN. 155 Immediate reaction to a Safety Problem | 20 |  |  | 01 - November - 2014 |
| ORO. GEN. 160 Occurrence Reporting | 20 |  |  | 01 - November - 2014 |
| Section II. Management | 21 |  | 22 | 01 - November - 2014 |
| ORO. GEN. 200 Management System | 21 |  |  | 01 - November - 2014 |
| ORO. GEN. 205 Contracted Activities | 21 |  |  | 01 - November - 2014 |
| ORO. GEN. 210 Personnel Requirements | 21 |  |  | 01 - November - 2014 |
| ORO. GEN. 215 Facility Requirements | 22 |  |  | 01 - November - 2014 |
| ORO. GEN. 220 Record - Keeping | 22 |  |  | 01 - November - 2014 |
| SUBPART AOC. AIR OPERATOR CERTIFICATION | 23 |  | 26 | 01 - November - 2014 |
| ORO. AOC. 100 Application for an Air Operator Certificate | 23 |  |  | 01 - November - 2014 |
| ORO. AOC. 105 Operations Specifications and Privileges of an AOC Holder | 23 |  |  | 01 - November - 2014 |
| ORO. AOC. 110 Leasing Agreement | 23 |  |  | 01 - November - 2014 |
| ORO. AOC. 115 Code-Share Agreements | 24 |  |  | 01 - November - 2014 |
| ORO. AOC. 120 Approvals to provide Cabin Crew Training and to issue Cabin Crew Attestations | 24 |  |  | 01 - November - 2014 |
| ORO. AOC. 125 Non-Commercial Operations of Aircraft listed in the Operations Specifications by the Holder of an AOC | 25 |  |  | 01 - November - 2014 |
| ORO. AOC. 130 Flight Data Monitoring - Aeroplanes | 25 |  |  | 01 - November - 2014 |
| ORO. AOC. 135 Personnel Requirements | 26 |  |  | 01 - November - 2014 |
| ORO. AOC. 140 Facility Requirements | 26 |  |  | 01 - November - 2014 |
| ORO. AOC. 150 Documentation Requirements | 26 |  |  | 01 - November - 2014 |
| SUBPART DEC. DECLARATION | 27 |  | 28 | 01 - November - 2014 |
| ORO. DEC. 100 Declaration | 27 |  |  | 01 - November - 2014 |
| SUBPART SPO. COMMERCIAL SPECIALIZED OPERATIONS | 29 | 30 |  | 01 - November - 2014 |
| ORO. SPO. 100 Common Requirements for Commercial Specialized Operators | 29 |  |  | 01 - November - 2014 |
| ORO. SPO. 110 Authorization of High Risk Commercial Specialized Operations | 29 |  |  | 01 - November - 2014 |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE | TO PAGE |  |  | EFFECTIVE DATE |
| --- | --- | --- | --- | --- | --- |
| ORO. SPO. 115 Changes | 30 32 |  |  |  | 01 - November - 2014 |
| ORO. SPO. 120 Continued Validity | 30 |  |  |  | 01 - November - 2014 |
| SUBPART MLR. MANUALS, LOGS and RECORDS | 31 | 34 |  |  | 01 - November - 2014 |
| ORO. MLR. 100 Operations Manual - General | 31 |  |  |  | 01 - November - 2014 |
| ORO. MLR. 101 Operations Manual - Structure | 32 |  |  |  | 01 - November - 2014 |
| ORO. MLR. 105 Minimum Equipment List | 32 |  |  |  | 01 - November - 2014 |
| ORO. MLR. 110 Journey Log | 33 |  |  |  | 01 - November - 2014 |
| ORO. MLR. 115 Record - Keeping | 33 |  |  |  | 01 - November - 2014 |
| SUBPART SEC. SECURITY | 35 |  | 36 |  | 01 - November - 2014 |
| ORO. SEC. 100 Flight Crew Compartment Security - A | 35 |  |  |  | 01 - November - 2014 |
| ORO. SEC. 100 Flight Crew Compartment Security - H | 35 |  |  |  | 01 - November - 2014 |
| SUBPART FC. FLIGHT CREW | 37 |  |  | 50 | 01 - November - 2014 |
| ORO. FC. 005 Scope | 37 |  |  |  | 01 - November - 2014 |
| Section 1. Common Requirements | 37 35 |  |  | 40 | 01 - November - 2014 |
| ORO. FC. 100 Composition of Flight Crew | 37 |  |  |  | 01 - November - 2014 |
| ORO. FC. 105 Designation as Pilot-in-Command / CPT | 38 |  |  |  | 01 - November - 2014 |
| ORO. FC. 110 Flight Engineer | 38 |  |  |  | 01 - November - 2014 |
| ORO. FC. 115 Crew Resource Management Training | 38 |  |  |  | 01 - November - 2014 |
| ORO. FC. 120 Operator Conversion Training | 38 |  |  |  | 01 - November - 2014 |
| ORO. FC. 125 Differences Training and Familiarization Training | 39 |  |  |  | 01 - November - 2014 |
| ORO. FC. 130 Recurrent Training and Checking | 39 |  |  |  | 01 - November - 2014 |
| ORO. FC. 135 Pilot Qualification to Operate in either Pilots Seat | 39 |  |  |  | 01 - November - 2014 |
| ORO. FC. 140 Operation on more than one Type or Variant | 39 |  |  |  | 01 - November - 2014 |
| ORO. FC. 145 Provision of Training | 39 |  |  |  | 01 - November - 2014 |
| Section 2. Additional Requirements for Commercial Air Transport Operations | 41 | 48 |  |  | 01 - November - 2014 |
| ORO. FC. 200 Composition of Flight Crew | 41 |  |  |  | 01 - November - 2014 |
| ORO. FC. A. 201 In-flight Relief of Flight Crew Members | 41 |  |  |  | 01 - November - 2014 |
| ORO. FC. 202 Single-Pilot Operations under IFR or at Night | 42 |  |  |  | 01 - November - 2014 |
| ORO. FC. 205 Command Course | 43 |  |  |  | 01 - November - 2014 |
| ORO. FC. 215 Initial Operators CRM Training | 43 |  |  |  | 01 - November - 2014 |
| ORO. FC. 220 Operator Conversion Training and Checking | 43 |  |  |  | 01 - November - 2014 |
| ORO. FC. 230 Recurrent Training and Checking | 44 |  |  |  | 01 - November - 2014 |
| ORO. FC. 235 Pilot Qualification to Operate in Either Pilot’s Seat | 45 |  |  |  | 01 - November - 2014 |
| ORO. FC. 240 Operation on more than One Type or Variant | 46 |  |  |  | 01 - November - 2014 |
| ORO. FC. A. 245 Alternative Training and Qualification Programme | 46 |  |  |  | 01 - November - 2014 |
| ORO. FC. A. 250 Commanders Holding a CPL ( A ) | 47 |  |  |  | 01 - November - 2014 |
| ORO. FC. H. 250 Commanders Holding a CPL ( H ) Programme | 48 |  |  |  | 01 - November - 2014 |
| Section 3. Additional Requirements for Commercial Specialized Operations and CAT Operations referred to in ORO. FC. 005 ( b )( 1 ) and ( 2 ) | 49 | 50 |  |  | 01 - November - 2014 |
| ORO. FC. 330 Recurrent Training and Checking - Operator Proficiency Check | 49 |  |  |  | 01 - November - 2014 |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE |  |  | TO PAGE |  | EFFECTIVE DATE |
| --- | --- | --- | --- | --- | --- | --- |
| SUBPART CC. CABIN CREW | 51 |  |  | 60 |  | 01 - November - 2014 |
| ORO. CC. 005 Scope | 51 |  |  |  |  | 01 - November - 2014 |
| Section 1. Common Requirements | 51 |  | 56 |  |  | 01 - November - 2014 |
| ORO. CC. 100 Number and Composition of Cabin Crew | 51 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 110 Conditions for Assignment to Duties | 51 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 115 Conduct of Training Courses and Associated Checking | 52 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 120 Initial Training Course | 52 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 125 Aircraft Type Specific Training and Operator Conversion Training | 53 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 130 Differences Training | 54 82 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 135 Familiarization - Line Training under Supervis | 54 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 140 Recurrent Training | 54 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 145 Refresher Training | 56 |  |  |  |  | 01 - November - 2014 |
| Section 2. Additional Requirements for Commercial Air Transport Operations | 57 |  |  |  | 60 | 01 - November - 2014 |
| ORO. CC. 200 Senior Cabin Crew Member | 57 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 201 Cabin Crew Instructor | 57 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 205 Reduction of the number of Cabin Crew during Ground Operations and in unforeseen circumstances | 58 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 210 Additional Conditions for Assignment to Duti Air Transport Operations | 58 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 215 Training and Checking Programs and related Documentation | 59 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 250 Operation on more than One Aircraft Type or Variant | 59 |  |  |  |  | 01 - November - 2014 |
| ORO. CC. 255 Single Cabin Crew Member Operations | 59 |  |  |  |  | 01 - November - 2014 |
| SUBPART FTL. FLIGHT & DUTY TIME LIMITATIONS Duties | 61 |  |  | 88 |  | 01 - November - 2014 |
| Section 1. General | 61 |  |  | 72 |  | 01 - November - 2014 |
| ORO. FTL. 005 General & Applicability | 61 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 010 General Principles | 61 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 100 Scope & REST REQUIREMENTS | 62 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 105 Definitions | 62 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 109 Responsibilities - General | 67 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 110 Operator Responsibilities | 67 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 115 Crew Member Responsibilities | 69 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 116 Pilot-in-Command Responsibilities | 70 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 120 Fatigue Risk Management | 70 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 125 Flight Time Specification Schemes | 71 |  |  |  |  | 01 - November - 2014 |
| Section 2. Commercial Air Transport Operators | 73 | 88 |  |  |  | 01 - November - 2014 |
| ORO. FTL. 200 Home Base | 73 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 205 Maximum Daily Flight Duty Period | 73 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 210 Flight Block Times & Duty Time Limitations | 76 |  |  |  |  | 01 - November - 2014 |
| ORO. FTL. 212 Cabin Crew Requirements | 79 |  |  |  |  | 01 - November - 2014 |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE | TO PAGE | EFFECTIVE DATE |
| --- | --- | --- | --- |
| ORO. FTL. 215 Positioning | 79 82 |  | 01 - November - 2014 |
| ORO. FTL. 220 Split Duty | 80 |  | 01 - November - 2014 |
| ORO. FTL. 225 Standby and Duties at the Airport | 80 |  | 01 - November - 2014 |
| ORO. FTL. 230 Reserve | 81 |  | 01 - November - 2014 |
| ORO. FTL. 235 Rest Periods | 82 |  | 01 - November - 2014 |
| ORO. FTL. 240 Nutrition | 84 |  | 01 - November - 2014 |
| ORO. FTL. 242 FDP, and Simulator Flight Time Limitations during Simulator Training activities | 84 |  | 01 - November - 2014 |
| ORO. FTL. 245 Records of Home Base, FDP, Flight Times, Flight Block Times, and Rest Periods | 85 |  | 01 - November - 2014 |
| ORO. FTL. 250 Fatigue Management Training | 86 |  | 01 - November - 2014 |
| SUBPART TC. TECHNICAL CREW in HEMS, HHO or NVIS OPERATIONS | 89 | 92 | 01 - November - 2014 |
| ORO. TC. 100 Scope | 89 |  | 01 - November - 2014 |
| ORO. TC. 105 Conditions for Assignment to Duties | 89 |  | 01 - November - 2014 |
| ORO. TC. 110 Training and Checking | 89 |  | 01 - November - 2014 |
| ORO. TC. 115 Initial Training | 90 89 |  | 01 - November - 2014 |
| ORO. TC. 120 Operator Conversion Training | 90 0 |  | 01 - November - 2014 |
| ORO. TC. 125 Differences Training | 90 |  | 01 - November - 2014 |
| ORO. TC. 130 Familiarization Flights | 90 |  | 01 - November - 2014 |
| ORO. TC. 135 Recurrent Training | 91 |  | 01 - November - 2014 |
| ORO. TC. 140 Refresher Training | 91 |  | 01 - November - 2014 |
|  | 83 |  |  |
| Part - ARO, Annex - Declaration | 93 | 94 | 01 - November - 2014 |
| GUIDANCE MATERIAL & ACCEPTABLE MEANS of COMPLIANCE | 95 | 238 | 01 - November - 2014 |
|  | 84 |  |  |
|  | 84 |  |  |
|  | 84 |  |  |
|  | 85 |  |  |
|  | 85 |  |  |

ORO. GEN. 005     Scope

This  Annex  establishes  requirements  to  be  followed  by  an  Air  Operator  conducting :

a )   Commercial  Air  Transport  operations  ( CAT ) ;

b )   Commercial  Specialized  operations ;

c )   Non - commercial  Operations  with  complex  motor - powered  aircraft ;

d )   Non - commercial  Specialized  Operations  with  complex  motor - powered  aircraft.

INTENTIONALLY            LEFT            BLANK

SUBPART   GEN.       GENERAL   REQUIREMENTS

SECTION  I.     General

ORO. GEN. 105     Competent  Authority

For  the  purpose  of  this  Annex,  the  GDCA  exercising  oversight  over  operators  subject  to  a  certification   or  declaration   obligation  or  specialized  operation  authorization  shall  be  for  operators  having  their  principal  place  of  business  in  a  Republic  of  Armenia  the  authority  designated  by  Republic  of  Armenia  ( GDCA  of  RA).

ORO.GEN.110   Operator  Responsibilities

a )  The  Operator  is  responsible  for  the  operation  of  the  aircraft  in  accordance  with  Annex  IV ( Part - CAT ),  as  applicable,  the  relevant  requirements  of  this  Annex  and  its  Air  Operator  Certificate  ( AOC )  or  Specialized  Operation  Authorization  ( SPO  Authorization )  or  declaration ;

b )   Every  flight  shall  be  conducted  in  accordance  with  the  provisions  of  the  Operations  Manual ;

c )   The  Operator  shall  establish  and  maintain  a  system  for  exercising  operational  control  over  any  flight  operated  under  the  terms  of  its  Certificate   SPO  Authorization  or  Declaration;

d )   The  Operator  shall  ensure  that  its  aircraft  are  equipped  and  its  crews  are  qualified  as required  for  the  area  and  type  of  operation ;

e )   The  Operator  shall  ensure  that  all  personnel  assigned  to,  or  directly  involved  in,  ground  and  flight  operations  are  properly  instructed,  have  demonstrated  their  abilities  in  their  particular  duties  and   are  aware  of  their  responsibilities  and  the  relationship  of  such  duties  to  the  operation  as  a  whole ;

f )   The  Operator  shall  establish  procedures  and  instructions  for  the  safe  operation  of  each aircraft  type,  containing  ground  staff  and  crew  member  duties  and  responsibilities  for  all  types  of  operation  on  the  ground  and  in  flight.  These  procedures  shall  not  require  crew  members  to  perform  any  activities  during  critical  phases  of  flight  other  than  those  required  for  the  safe  operation  of  the  aircraft ;

g )   The  Operator  shall  ensure  that  all  personnel  are  made  aware  that  they  shall  comply  with  the  laws,  regulations  and  procedures  of  in  which  operations  are  conducted  and  that  are  pertinent  to  the  performance  of  their  duties ;

h )   The  Operator  shall  establish  a  Checklist  system  for  each  aircraft  type  to  be  used   by crew  members  in  all  phases  of  flight  under  normal,  abnormal  and  emergency  conditions  to  ensure  that  the  operating  procedures  in  the  operations  manual  are  followed.  The  design  and  utilization  of  Checklists  shall  observe  human  factors  principles  and  take  into  account  the  latest  relevant  documentation  from  the  aircraft  manufacturer ;

i )   The  Operator  shall  specify  flight  planning  procedures  to  provide  for  the  safe  conduct of  the  flight  based  on  considerations  of  aircraft   performance,  other  operating  limitations  and  relevant  expected  conditions  on  the  route  to  be  followed  and  at  the  aerodromes  or  operating  sites  concerned.  These  procedures  shall  be  included  in  the  Operations  Manual ;

j )   The  operator  shall  establish  and  maintain  dangerous  goods  training  programmes  for personnel  as  required  by  the  technical  instructions  which  shall  be  subject  to  review  and  approval  by  the  competent  authority.  Training  programmes  shall   be  commensurate  with  the  responsibilities  of  personnel ;

k )   Notwithstanding  ( j ),  the  Operator  of  a  sailplane  or  a  balloon  or  of  flights  taking  off  and landing  at  the  same  aerodrome  or  operating  site,  under  VFR  by  Day,  with

( i )   single-engined  propeller-driven  aeroplanes  having  a  Maximum  Certified  Take-Off  Mass  of  5 700 kg  or  less  and  a  MOPSC  of  5  or  less ;                 or

( ii )  other  than  complex  motor-powered  helicopters,  single-engined,  with  a  MOPSC  of  5  or less,  shall  ensure  that  the  flight  crew  has  received  an  appropriate  training  or  briefing  to  enable  them  to  recognize  undeclared  dangerous  goods  brought  on-board  by  passengers  or  as  cargo.

ORO. GEN. 115     Application  for  an  AOC

a )   The  application  for  an  Air  Operator  Certificate  or  an  amendment  to  an  existing  certificate shall  be  made  in  a  form  and  manner  established  by  the  GDCA,  taking  into  account  the applicable  requirements  of  this   Regulation   and  its  Implementing  Rules ;

b )   Applicants  for  an  initial  certificate  shall  provide  the  GDCA  with  documentation  demonstrating  how  they  will  comply  with  the  requirements  established  in  Air  Operations  Regulation   and  its  Implementing  Rules.  Such  documentation  shall  include  a  procedure  describing  how  changes  not  requiring  prior  approval  will  be  managed  and  notified  to  the  GDCA.

ORO. GEN. 120   Means  of  Compliance

a )   Alternative  means  of  compliance  to  those  adopted  by  the  GDCA  may  be used  by  an  Operator  to  establish  compliance  with  Air  Operations  Regulation  and  its  Implementing  Rules ;

b )   When  an  Operator  subject  to  certification  wishes  to  use  an  alternative  means  of compliance  to  the  Acceptable  Means  of  Compliance  ( AMC )  adopted  by  the   EASA  to  establish  compliance  with  Air  Operations  Regulation  and  its  Implementing  Rules,  it  shall,  prior  to  implementing  it,  provide  the  GDCA  with  a  full  description  of  the  alternative  means  of  compliance.  The  description  shall  include  any  revisions  to  manuals  or  procedures  that  may  be relevant,  as  well  as  an  assessment  demonstrating  that  the  Implementing  Rules  are  met.

The  Operator  may  implement  these  alternative  means  of  compliance  subject  to  prior  approval  by  the  GDCA  and  upon  receipt  of  the  notification  as  prescribed  in  ARO.GEN.120 ( d ).

c )   An  Operator  required  to  declare  its  activity  shall  notify  to  the  GDCA  the  list  of alternative  means  of  compliance  it  uses  to  establish  compliance  with  this  Regulation  and  its Implementing  Rules ;

d )   When  an  Operator  subject  to  SPO  authorization  wishes  to  use  alternative  means  of compliance,  it  shall  comply  with  ( b )  whenever  such  alternative  means  of  compliance  affects the  Standard  Operating  Procedures  that  are  part  of  the  authorization  and  with  ( c )  for  the declared  part  of  its  organization  and  operation.

ORO. GEN. 125   Terms  of  Approval  and  Privileges  of  an  AOC  Holder

A  certified  Operator  shall  comply  with  the  scope  and  privileges  defined  in  the  Operations Specifications  attached  to  the  Operator’s  Certificate.

ORO. GEN. 130   Changes  Related  to  an  AOC  Holder

a )   Any  change  affecting :

1 )   the  scope  of  the  certificate  or  the  operations  specifications  of  an  Operator ;        or

2 )   any  of  the  elements  of  the  operator’s  management  system  as  required  in  ORO. GEN.

200 (a) (1)  and  (a) (2),  shall  require  prior  approval  by  the  GDCA.

b )   For  any  changes  requiring  prior  approval  in  accordance  with  Air  Operations  Regulation  and  its  Implementing  Rules,  the  Operator  shall  apply  for  and  obtain  an  approval  issued  by  the  GDCA.  The  application  shall  be  submitted  before  any  such  change  takes  place,  in  order  to  enable  the  GDCA  to  determine  continued  compliance  with   this  Regulation   and  its  Implementing  Rules  and  to  amend,  if  necessary, the  operator  certificate  and  related  terms  of  approval  attached  to  it.

The  Operator  shall  provide  the  GDCA  with  any  relevant  documentation.

The  change  shall  only  be  implemented  upon  receipt  of  formal  approval  by  the  GDCA  in  accordance  with  ARO. GEN. 330.

The  Operator  shall  operate  under  the  conditions  prescribed  by  the  GDCA  during  such  changes,  as  applicable ;

c )   All  changes  not  requiring  prior  approval  shall  be  managed  and  notified  to  the  GDCA  as  defined  in  the  procedure  approved  by  the  GDCA  in  accordance  with  ARO. GEN. 310 (c).

ORO. GEN. 135   Continued  Validity  of  an  AOC

a )   the  Operator’s  certificate  shall  remain  valid  subject  to :

1 )   the  Operator  remaining  in  compliance  with  the  relevant  requirements  of  this  Regulation   and  its  Implementing  Rules,  taking  into  account  the  provisions  related  to  the  handling  of  findings  as  specified  under   ORO. GEN. 150 ;

2 )   The  GDCA  being  granted  access  to  the  Operator  as  defined  in  ORO.GEN.140  to  determine  continued  compliance  with  the  relevant  requirements  of  current  Regulation   and  its  Implementing  Rules ;                     and

3 )   The  certificate  not  being  surrendered  or  revoked.

b )   upon  revocation  or  surrender  the  certificate  shall  be  returned  to  the  GDCA  without  delay.

ORO. GEN. 140   Access

a )   For  the  purpose  of  determining  compliance  with  the  relevant   requirements  of  Air Operations  Regulation  and  its  Implementing  Rules,  the  Operator  shall  grant  access  at  any  time  to  any  facility,  aircraft,  document,  records,  data, procedures  or  any  other  material  relevant  to  its  activity  subject  to  certification,  SPO  Authorization   or  Declaration,  whether  it  is  contracted  or  not,  to  any  person  authorized  by  one  of  the  following  authorities :

1 )   The  GDCA  defined  in  ORO. GEN. 105 ;

2 )   The  authority  acting  under   the  provisions  of  ARO. GEN. 300 (d),  ARO. GEN. 300 (e)  or

ARO. RAMP.

b )   Access  to  the  aircraft  mentioned  under  ( a )  in  the  case  of  CAT  shall  include  the  possibility  to  enter  and  remain  in  the  aircraft  during  flight  operations  unless  otherwise  decided  by  the  Commander  for  the  flight  crew  compartment  in  accordance  with  CAT. GEN. MPA. 135  in  the  interest  of  safety.

ORO. GEN. 150   Findings

After  receipt  of  notification  of  findings,  the  operator  shall :

a )   identify  the  root  cause  of  the  non-compliance ;

b )   define  a  corrective  action  plan ;                                and

c )   Demonstrate  corrective  action  implementation  to  the  satisfaction  of  the  GDCA  within  a  period  agreed  with  GDCA  that   as  defined  in  ARO.GEN.350 (d).

ORO. GEN. 155   Immediate  reaction  to  a  Safety  Problem

The  Operator  shall  implement :

a )   any  safety  measures  mandated  by  the  GDCA  in  accordance  with  ARO.GEN.135(c) ;   and

b )   any  relevant  mandatory  safety  information  issued  by  the  EASA,  including  airworthiness

directives.

ORO. GEN. 160   Occurrence  Reporting

a )   The  Operator  shall  report  to  the  GDCA,  and  to  any  other  organization  required by  the  State  of  the  operator  to  be  informed,  any  Accident,  serious  Incident  and  occurrence  as  defined  in  Regulation ;

b )   Without  prejudice  to  point  ( a )  the  Operator  shall  report  to  the  GDCA  and  to  the organization  responsible  for  the  design  of  the  aircraft  any  incident,  malfunction,  technical defect,  exceeding  of  technical  limitations  or  occurrence  that  would  highlight  inaccurate, incomplete  or  ambiguous  information  contained  in  the  operational  suitability  data  established  in accordance  with  CR - EU No 748 / 2012  or  other  irregular  circumstance  that  has  or  may  have endangered  the  safe  operation  of  the  aircraft  and  that  has  not  resulted  in  an  accident  or  serious  incident ;

c )   Without  prejudice  to  Regulation ,  the  reports  referred  in  paragraphs  ( a )  and  ( b )  shall  be  made  in  a  form  and  manner  established  by  the  GDCA  and  contain  all  pertinent  information about  the  condition  known  to  the  Operator ;

d )   Reports  shall  be  made  as  soon  as  practicable,  but  in  any  case  within  72  hours  of  the Operator  identifying  the  condition  to  which  the  report  relates,  unless  exceptional  circumstances  prevent  this ;

e )   Where  relevant,  the  Operator  shall  produce  a  follow-up  report  to  provide  details  of  actions  it  intends  to  take  to  prevent  similar  occurrences  in  the  future,  as  soon  as  these  actions  have  been  identified.  This  report  shall  be  produced  in  a  form  and  manner  established  by  the  GDCA

SECTION  II.    Management

ORO. GEN. 200   Management  System

a )   the  Operator  shall  establish,  implement  and  maintain  a  management  system  that  includes :

1 )   clearly  defined  lines  of  responsibility  and  accountability  throughout  the  Operator,  including  a  direct  safety  accountability  of  the  Accountable  Manager ;

2 )   a  description  of  the  overall  philosophies  and  principles  of  the  Operator  with  regard  to  safety,  referred  to  as  the  safety  policy ;

3 )   the  identification  of  aviation  safety  hazards  entailed  by  the  activities  of  the Operator,  their  evaluation  and  the  management  of  associated  risks,  including  taking  actions  to  mitigate  the  risk  and  verify  their  effectiveness ;

4 )   maintaining  personnel  trained  and  competent  to  perform  their  tasks ;

5 )   documentation  of  all  management  system  key  processes,  including  a  process  for  making  personnel  aware  of  their  responsibilities  and  the  procedure  for  amending  this  documentation ;

6 )   a  function  to  monitor  compliance  of  the  Operator  with  the  relevant  requirements. Compliance  monitoring  shall  include  a  feedback  system  of  findings  to  the  Accountable  Manager  to  ensure  effective  implementation  of  corrective  actions  as  necessary ;      and

7 )   any  additional  requirements  that  are  prescribed  in  the  relevant  Subparts  of  this  Annex  or  other  applicable  Annexes.

b )   The  management  system  shall  correspond  to  the  size  of  the  Operator  and  the  nature  and  complexity  of  its  activities,  taking  into  account  the  hazards  and  associated  risks  inherent  in  these  activities.

ORO. GEN. 205     Contracted  Activities

a )   The  Operator  shall  ensure  that  when  contracting  or  purchasing  any  part  of  its  activity,  the contracted  or  purchased  service  or  product  conforms  to  the  applicable  requirements ;

b )   When  the  certified  Operator  or  the  SPO  Authorization  Holder  contracts  any  part  of  its activity  to  an  organization  that  is  not  itself  certified  or  authorized  in  accordance  with  this  Part to  carry  out  such  activity,  the  contracted  organization  shall  work  under  the  approval  of  the Operator.  The  contracting  organization  shall  ensure  that  the  Competent  Authority  is  given  access  to  the  contracted  organization,  to  determine  continued  compliance  with  the  applicable requirements.

ORO. GEN. 210   Personnel   Requirements

a )   the  Operator  shall  appoint   an  Accountable  Manager,  who  has  the  authority  for  ensuring  that  all  activities  can  be  financed  and  carried  out  in  accordance  with  the  applicable  requirements.  The  Accountable  Manager  shall  be  responsible  for  establishing  and  maintaining  an  effective  management  system ;

b )   a  person  or  group  of  persons  shall  be  nominated  by  the  Operator,  with   the responsibility  of  ensuring  that  the  Operator  remains  in  compliance  with  the  applicable  requirements.                        Such  person(s)  shall  be  ultimately  responsible  to  the  Accountable  Manager ;

c )   the  Operator  shall  have  sufficient  qualified  personnel  for  the  planned  tasks  and  activities  to  be  performed  in  accordance  with  the  applicable  requirements ;

d )   the  Operator  shall  maintain   appropriate  experience,  qualification  and  training  records  to  show  compliance  with  point  ( c ) ;

e )    the  Operator  shall  ensure  that  all  personnel  are  aware  of  the  rules  and  procedures  relevant  to  the  exercise  of  their  duties.

ORO. GEN. 215   Facility  Requirements

The  Operator  shall  have  facilities  allowing  the  performance  and  management  of  all  planned  tasks  and  activities  in  accordance  with  the  applicable  requirements.

ORO. GEN. 220   Record - Keeping

a )   the  Operator  shall  establish  a  system  of  record - keeping  that   allows  adequate   storage  and  reliable  traceability  of  all  activities  developed,  covering  in  particular  all  the  elements  indicated  in  ORO. GEN. 200 ;

b )   the  format  of  the  records  shall  be  specified  in  the operator’s  procedures ;

c )   records  shall  be  stored  in  a  manner  that  ensures  protection  from  damage,  alteration  and  theft.

SUBPART   AOC.     AIR   OPERATOR   CERTIFICATION

ORO. AOC. 100   Application  for  an  Air  Operator  Certificate

a )  Without  prejudice  to  this  Regulation,  prior  to  commencing  Commercial  Air  Transport Operations,  the  Operator  shall  apply  for  and  obtain  an  Air  Operator  Certificate  ( AOC )  issued  by  the  GDCA  of  RA ;

b )   The  Operator  shall  provide  the  following  information  to  the  Competent  Authority :

1 )  the  official  name  and  business  name,  address,  and  mailing  address  of  the  applicant ;

2 )  a  description  of  the  proposed  operation,  including  the  type(s),  and  number  of  aircraft  to  be  operated ;

3 )  a  description  of  the  management  system,  including  organizational  structure ;

4 )  the  name  of  the  Accountable  Manager ;

5 )  the  names  of  the  nominated  persons  required  by  ORO. AOC. 135 ( a )  together  with  their qualifications  and  experience ;

6 )  a  copy  of  the  Operations  Manual  required  by  ORO. MLR. 100 ;

7 )  a  statement  that  all  the  documentation  sent  to  the  Competent  Authority  have  been  verified  by  the  applicant  and  found  in  compliance  with  the  applicable  requirements.

c )   Applicants  shall  demonstrate  to  the  Competent  Authority  that :

1 )  they  comply  with  all  the  applicable  requirements  of  Annex  IV ( Part - CAT ),  this  Annex  III,  and  Annex  V  ( Part - SPA )  to  this  Regulation ;

2 )  all  aircraft  operated  have  a  certificate  of  airworthiness  ( CofA )  in  accordance  with  CR - EU  No 748 / 2012 ;                                                                                               and

3 )  its  organization  and  management  are  suitable  and  properly  matched  to  the  scale  and  scope  of  the  operation.

ORO. AOC. 105   Operations   Specifications   and   Privileges   of   an   AOC   Holder

The   privileges   of  the  Operator,  including  those  granted  in  accordance  with  Annex  V                  ( Part - SPA ),  shall  be  specified  in  the  Operations  Specifications  of  the  Certificate.

ORO. AOC. 110   Leasing   Agreement

Any   Lease - in

a )   Without  prejudice  to  Regulation,  any  lease  agreement  concerning  aircraft  used  by  an  Operator  certified  in  accordance  with   this  Part - ORO  shall  be  subject  to  prior  approval  by  the  GDCA  of  RA ;

b )   The  Operator  certified  in  accordance  with  this  Part  shall  only  Wet  Lease - in  aircraft  from  an  operator  that  is  not  subject  to  an  operating  ban  pursuant  to  Regulation ;

Wet   Lease - in

c )   The  applicant  for  the  approval  of  the  Wet  Lease - In  of  an  aircraft  of  a  third  country operator  shall  demonstrate  to  the  GDCA  that :

1 )   the  third  country  operator  holds  a  valid  AOC  issued  in  accordance  with  ICAO  Annex  6 ;

2 )   The  safety  standards  of  the  third  country  operator  with  regard  to  continuing  airworthiness  and  air  operations  are  equivalent  to  the  applicable  requirements  established  by  CR - EC  No  2042 / 2003  and  this  Regulation ;                                     and

3 )   the  aircraft  has  a  standard   CofA  issued  in  accordance  with  ICAO  Annex  8.

Dry   Lease - in

d )   An  applicant  for  the  approval  of  the  Dry  Lease - In  of  an  aircraft  registered  in  a  third country  shall  demonstrate  to  the  GDCA  that :

1 )   an  operational  need  has  been  identified  that  cannot  be  satisfied  through  leasing  an  aircraft  registered  in  the  EU ;

2 )   the  duration  of  the  Dry  Lease - in  does  not  exceed  7 ( seven )  months  in  any  12 consecutive  month  period ;                                                          and

3 )   compliance  with  the  applicable  requirements  of  CR - EC  No  2042 / 2003  is  ensured.

Dry   Lease - out

e )   The  Operator  certified  in  accordance  with  this  Part  intending  to  Dry  Lease - out  one  of  its  aircraft  shall  apply  for  prior  approval  by  the   GDCA.  The  application  shall  be  accompanied  by  copies  of  the  intended  lease  agreement  or  description  of  the  lease  provisions,  except  financial  arrangements,  and  all  other  relevant  documentation.

Wet   Lease - out

f )   Prior  to  the  Wet  Lease - out  of  an  aircraft,  the  Operator  certified  in  accordance  with  this Part  shall  notify  the  GDCA  of  RA.

ORO. AOC. 115   Code - Share   Agreements

a )   Without  prejudice  to  applicable  RA  safety   requirements  for  third   country  operators  and  aircraft,  an  operator  certified  in  accordance  with  this  Part  shall  enter  into  a  code-share  agreement  with  a  third  country  operator  only  after :

1 )   having  verified  that   the  third  country  operator  complies  with  the  applicable  ICAO  standards ;                                                                              and

2 )   having  provided  the  GDCA  with  documented  information  enabling  such  authority  to  comply  with  ARO. OPS. 105.

b )   When  implementing   the  Code - Share  agreement  the  Operator  shall  monitor  and  regularly assess  the  ongoing  compliance  of  the  third  country  operator  with  the  applicable  ICAO  standards ;

c )   The  operator  certified  in  accordance  with  this  Part  shall  not  sell  and  issue  tickets  for  a  flight  operated  by  a  third  country  operator  when  the  third  country  operator  is  subject  to  an  operating  ban   pursuant  to  CR - EC  No  2111 / 2005  or  is  failing  to  maintain  compliance  with  the  applicable  ICAO  standards.

ORO. AOC. 120   Approvals  to  provide  Cabin  Crew  Training  and  to  issue  Cabin  Crew

Attestations

a )   When  intending  to  provide  the  training  course  required  in  Annex  V ( Part - CC )  to  ARM - Air  Crew  Regulation,  the  Operator  shall  apply  for  and  obtain  an  approval  issued  by  the GDCA  of  RA.  For  this  purpose,  the  applicant  shall  demonstrate  compliance  with  the  requirements  for  the  conduct  and  content  of  training  course  established  in  CC. TRA. 215  and  CC. TRA. 220  of  that  Annex  and  shall  provide  the  GDCA  with :

1 )    the  date  of  intended  commencement  of  activity ;

2 )    the  personal  details  and  qualifications  of  the  instructors  as  relevant  to  the  training elements  to  be  covered ;

3 )    the  name(s)  and  address(es)  of  the  training  site(s)  at  which  the  training  is  to  be  conducted ;

4 )   a  description  of  the  facilities,  training  methods,  manuals  and  representative  devices  to  be  used ;                                           and

5 )   the  syllabi  and  associated  programmes  for  the  training  course.

b )   If  a  GDCA  decides,  in  accordance  with  ARA. CC. 200  of  Annex  VI  ( Part - ARA )  to  ARM - Air  Crew  Regulation,  that  Operators  may  be  approved  to  issue  cabin  crew  attestations,  the  applicant  shall,  in   addition  to  ( a ) :

1 )   demonstrate  to  the  GDCA   that :

( i )    the  organization  has  the  capability  and   accountability  to  perform  this  task ;

( ii )   the  personnel  conducting  examinations  are  appropriately  qualified  and  free  from  conflict  of  interest ;                                                                           and

2 )   provide  the  procedures  and  the  specified   conditions  for :

( i )    conducting  the  examination  required  by  CC. TRA. 220 ;

( ii )   issuing  cabin  crew  attestations ;                                              and

( iii )  supplying  the  GDCA  with  all  relevant  information  and  documentation  related  to  the  attestations  it  will  issue  and  their  holders,  for  the  purpose  of  record - keeping,  oversight  and  enforcement  actions  by  that  authority.

c )   The  approvals  referred  to  in  ( a )  and  ( b )  shall  be  specified  in  the  Operations Specifications.

ORO. AOC. 125    Non - Commercial  Operations  of  Aircraft  listed  in  the  Operations

Specifications  by  the  Holder  of  an  AOC

a )   The  holder  of  an  AOC  may  conduct  Non - commercial  Operations  with  an  aircraft  otherwise  used  for  Commercial   Air  Transport  Operations  that  is  listed  in  the  operations  specifications  of  its  AOC,  provided  that  the  operator :

1 )   Describes  such   operations   in  detail  in  the  Operations  Manual,  including :

( i )    identification  of  the  applicable  requirements ;

( ii )   a  clear  identification  of  any  differences  between  operating  procedures  used  when  conducting   Commercial  Air  Transport  and  Non - commercial   Operations ;

( iii )   a  means  of  ensuring  that  all  personnel  involved   in  the  operation   are  fully  familiar  with  the  associated  procedures ;

2 )  Submits  the  identified  differences  between  the  operating  procedures  referred  to  in  ( a )( 2 )  to  the  GDCA  for  prior  approval.

b )   An  AOC  Holder  conducting  operations  referred  to  in  ( a )  shall  not  be  required  to  submit a  declaration  in  accordance  with  this  Part.

ORO. AOC. 130   Flight  Data  Monitoring  —  Aeroplanes

a )   The  Operator  shall  establish  and  maintain  a  Flight  Data  Monitoring  System,  which  shall  be  integrated  in  its  management  system,  for  aeroplanes  with  a  maximum  certificated  take - off  mass  of  more  than  27 000 kg ;

b )   The  Flight  Data  Monitoring  System  shall  be  non - punitive  and  contain  adequate  safeguards  to  protect  the  source(s)  of  the  data.

ORO. AOC. 135   Personnel  Requirements

a )   In  accordance  with   ORO. GEN. 210 ( b ),  the  Operator  shall  nominate  persons  responsible for  the  management  and  supervision  of  the  following  areas :

1 )   Flight  Operations ;

2 )   Crew  Training ;

3 )   Ground   Operations ;             and

4 )   Continuing  Airworthiness  in  accordance  with  CR - EC  No  2042 / 2003.

b )    Adequacy  and  Competency  of  Personnel :

1 )   the  operator  shall  employ  sufficient  personnel  for  the  planned  ground  and  flight  operations ;

2 )   all  personnel  assigned  to,  or  directly  involved  in,  ground  and  flight  operations  shall :

( i )    be  properly  trained ;

( ii )   demonstrate  their  capabilities  in  the  performance  of  their  assigned  duties ;     and

( iii )  be  aware  of  their  responsibilities  and  the  relationship  of  their  duties  to  the   operation  as  a  whole.

c )   Supervision  of  Personnel :

1 )   the  Operator  shall  appoint  a  sufficient  number  of  personnel  supervisors,  taking  into  account  the  structure  of  the  operator’s  organization  and  the  number  of  personnel  employed ;

2 )   the  duties  and  responsibilities  of  these  supervisors  shall  be  defined,  and  any  other necessary  arrangements  shall  be  made  to  ensure  that  they  can  discharge  their  supervisory  responsibilities ;

3 )   the  supervision  of  crew  members  and  personnel  involved  in  the  operation  shall  be exercised  by  individuals  with  adequate  experience  and  the  skills  to  ensure  the  attainment  of  the  standards  specified  in  the  Operations  Manual.

ORO. AOC. 140   Facility  Requirements

In  accordance  with  ORO. GEN. 215,  the  operator  shall :

a )   make  use  of  appropriate  ground  handling  facilities  to  ensure  the  safe  handling  of  its  flights ;

b )   arrange  operational  support   facilities  at  the  main  operating  base,  appropriate  for  the  area  and  type  of  operation ;                                                 and

c )   ensure  that  the  available  working  space  at  each  operating  base  is  sufficient  for  personnel  whose  actions  may  affect  the  safety  of  flight  operations.  Consideration  shall  be  given  to  the  needs  of  ground  crew,  personnel  concerned  with  operational  control,  the  storage  and  display  of  essential  records  and  flight  planning  by  crews.

ORO. AOC. 150   Documentation  Requirements

a )   The  Operator  shall  make  arrangements  for  the  production  of  Manuals  and  any  other documentation  required  and  associated   amendments ;

b )   The  Operator  shall  be  capable  of  distributing  operational  instructions  and  other  information  without  delay.

SUBPART   DEC.         DECLARATION

ORO. DEC. 100  Declaration

The  Operator  of  complex  motor-powered  aircraft  engaged  in  Non - commercial  Operations  or Non - commercial  Specialized  Operations,  and  the  Commercial  Specialized  Operator  shall :

a )  Provide  the  GDCA  with  all  relevant  information  prior  to  commencing  operations,  using  the  Form  contained  in  Appendix  I  to  this  Annex  III ;

b )  Notify  to  the  GDCA  a  list  of  the  alternative  means  of  compliance  used ;

c )  Maintain  compliance  with  the  applicable  requirements  and  with  the  information  given  in  the  declaration ;

d )  Notify  the  GDCA  without  delay  of  any  changes  to  its  declaration  or  the  means  of compliance  it  uses  through  submission  of  an  amended  declaration  using  the  Form  contained  in Appendix  I  to  this  Annex  III ;                                                   and

e )  Notify  the  GDCA  of  RA  when  it  ceases  operation.

INTENTIONALLY            LEFT            BLANK

SUBPART   SPO.       COMMERCIAL   SPECIALIZED   OPERATIONS

ORO. SPO. 100     Common  Requirements  for  Commercial  Specialized  Operators

a )   A  Commercial  Specialized  Operator  shall  in  addition  to  ORO. DEC. 100  also  comply  with ORO. AOC. 135,  ORO. AOC. 140  and  ORO. AOC. 150 ;

b )   Aircraft  shall  have  a  Certificate  of  Airworthiness  ( CofA )  in  accordance  with  CR - EU  No 748 / 2012  or  shall  be  leased - in  in  accordance  with  ( c ) ;

c )   A  Commercial  Specialized  Operator  shall  obtain  prior  approval  of  the  Competent  Authority and  comply  with  the  following  conditions,  if :

1 )  Wet  Leasing - in  an  aircraft  of  a  third  country  operator :

( i )    the  safety  standards  of  a  third  country  operator  with  regard  to  continuing  airworthiness and  air  operations  are  equivalent  to  the  applicable  requirements  established  by  this  Regulation ;

( ii )   the  aircraft  of  a  third  country  operator  has  a  standard  CofA  issued  in  accordance  with  ICAO  Annex  8 ;

( iii )  the  duration  of  the  Wet  Lease - in  does  not  exceed  7  ( seven )  months  in   any  12  consecutive  month  period ;                                   or

2 )  Dry  Leasing - in  an  aircraft  registered  in  a  third  country :

( i )   an  operational  need  has  been  identified  that  cannot  be  satisfied  through  leasing  an  aircraft  registered  in  the   Republic  of  Armenia ;

( ii )   the  duration  of  the  Dry  Lease - in  does  not  exceed  7  ( seven )  months  in  any  12  consecutive  month  period ;

( iii )  compliance  with  the  applicable  requirements  of  CR - EC No 2042 / 2003  is  ensured ;

( iv )  the  aircraft  is  equipped  in  accordance  with  Annex  VIII  [ Part  SPO ].

ORO. SPO. 110  Authorization  of  High  Risk  Commercial  Specialized  Operations

a )   A  commercial  specialized  operator  shall  apply  for  and  obtain  an  authorization  issued  by the  Competent  Authority  of  the  operator  prior  to  commencing  a  high  risk  commercial specialized  operation :

1 )  that  is  carried  out  over  an  area  where  the  safety  of  third  parties  on  the  ground  is  likely  to  be  endangered  in  the  event  of  an  emergency ;                     or

2 )  that,  as  determined  by  the  competent  authority  of  the  place  where  the  operation  is conducted,  due  to  its  specific  nature  and  the  local  environment  in  which  it  is  conducted,  poses a  high  risk,  in  particular  to  third  parties  on  the  ground.

b )   The  Operator  shall  provide  the  following  information  to  the  Competent  Authority :

1 )  the  official  name  and  business  name,  address,  and  mailing  address  of  the  applicant ;

2 )  a  description  of  the  management  system,  including  organizational  structure ;

3 )  a  description  of  the  proposed  operation,  including  the  type(s),  and  number  of  aircraft  to  be operated ;

4 )  the  risk  assessment  documentation  and  related  Standard  Operating  Procedures,  required  by SPO. OP. 230 ;

5 )  a  statement  that  all  the  documentation  sent  to  the  GDCA  has  been  verified  by the  Operator  and  found  in  compliance  with  the  applicable  requirements.

c )   The  application  for  an  authorization  or  its  amendment  shall  be  made  in  a  form  and manner  established  by  the  GDCA,  taking  into  account  the  this  Regulation  and  its  Implementing  Rules.

ORO. SPO. 115     Changes

a )   Any  change  affecting  the  scope  of  the  authorization  or  the  authorized  operations  shall require  prior  approval  of  the  GDCA.  Any  change  not  covered  by  the  initial  risk  assessment, shall  require  the  submission  of  an  amended  risk  assessment  and  SOP  to  the  GDCA  of  RA ;

b )   The  application  for  approval  of  a  change  shall  be  submitted  before  any  such  change  takes place,  in  order  to  enable  the  GDCA  to  determine  continued  compliance  with  this  Regulation  and  its  Implementing  Rules  and  to  amend,  if  necessary,  the  authorization.  The  Operator  shall provide  the  GDCA  with  any  relevant  documentation ;

c )   The  change  shall  only  be  implemented  upon  receipt  of  formal  approval  by  the  GDCA  in

accordance  with  ARO. OPS. 150 ;

d )   The  Operator  shall  operate  under  the  conditions  prescribed  by  the  GDCA  during  such changes,  as  applicable.

ORO. SPO. 120     Continued  Validity

a )   An  Operator  holding  a  specialized  operation  authorization  shall  comply  with  the  scope  and privileges  defined  in  the  authorization ;

b )   The  Operator’s  authorization  shall  remain  valid  subject  to :

1 )  the  Operator  remaining  in  compliance  with  the  relevant  requirements  of  this  Regulation  and  its  Implementing  Rules,  taking  into  account  the  provisions  related  to  the  handling  of  findings  as  specified  under  ORO. GEN. 150 ;

2 )  the  GDCA  being  granted  access  to  the  operator  as  defined  in  ORO. GEN. 140  to  determine  continued  compliance  with  the  relevant  requirements  of  current  Regulation  and  its  Implementing  Rules ;                                                                        and

3 )  the  authorization  not  being  surrendered  or  revoked.

c )   Upon  revocation  or  surrender  the  authorization  shall  be  returned  to  the  GDCA  without

delay.

SUBPART   MLR.     MANUALS,   LOGS   and   RECORDS

ORO. MLR. 100   Operations   Manual  —  General

a )   The  Operator  shall  establish  an  Operations  Manual  ( OM )  as  specified  under  8. b  of Annex  IV  to  ARM - AIR  OPERATIONS  Regulation ;

b )   The  content  of  the  OM  shall  reflect  the  requirements  set  out  in  this  Annex  III,  Annex  IV  ( Part - CAT ),  Annex  V  ( Part - SPA ),  Annex  VI  ( Part - NCC )  and  Annex  VIII                     ( Part - SPO ),  as  applicable,  and  shall  not  contravene  the  conditions  contained  in  the  Operations  Specifications  to  the  Air  Operator  Certificate ( AOC ), the  SPO  authorization  or  the  declaration and  the  list  of  Specific  Approvals,  as  applicable ;

c )   The  OM  may  be  issued  in  separate  parts ;

d )   All  operations  personnel  shall  have  easy  access  to  the  portions  of  the  OM  that  are relevant  to  their  duties ;

e )   The  OM  shall  be  kept  up  to  date.  All  personnel  shall  be  made  aware  of  the  changes  that  are  relevant  to  their  duties ;

f )   Each  crew  member  shall  be  provided  with  a  personal  copy  of  the  relevant  sections  of  the  OM  pertaining  to  their  duties.  Each  holder  of  an  OM,  or  appropriate  parts  of  it,  shall  be  responsible  for  keeping  their  copy  up  to  date  with  the  amendments  or  revisions  supplied  by  the  Operator ;

g )   For  AOC  holders :

1 )   for  amendments  required  to  be  notified  in  accordance  with  ORO. GEN. 115 (b)  and  ORO. GEN. 130 (c),  the  Operator  shall  supply  the  GDCA  with  intended   amendments  in  advance  of   the  effective  date ;                                                          and

2 )   for  amendments  to  procedures  associated  with  prior  approval  items  in  accordance  with  ORO. GEN. 130,  approval  shall  be  obtained  before  the  amendment  becomes  effective.

g 1 )   For  SPO  authorization  holders,  any  amendment  associated  with  the  authorized  standard operating  procedures, prior  approval  shall  be  obtained  before  the  amendment  becomes effective;

h )   Notwithstanding  ( g ),  and  ( g1 )  when  immediate  amendments  or  revisions  are  required  in  the  interest  of  safety,  they  may  be  published  and  applied  immediately,  provided  that  any  approval  required  has  been  applied  for ;

i )   The  Operator  shall  incorporate  all  amendments  and  revisions  required  by  the  GDCA  of  RA ;

j )   The  Operator  shall  ensure  that  information  taken  from  approved  documents,  and  any amendment  thereof,  is  correctly  reflected   in  the  OM.  This  does  not  prevent  the  Operator  from  publishing   more  conservative  data  and  procedures  in  the  OM ;

k )   The  Operator  shall  ensure  that  all  personnel  are  able  to  understand  the  language  in  which  those  parts  of  the  OM  which  pertain  to  their  duties  and  responsibilities  are  written.                          The  content  of  the  OM  shall  be  presented  in  a  form  that  can  be  used  without  difficulty  and  observes  human  factors  principles.

ORO. MLR. 101   Operations  Manual  —  Structure  for  Commercial  Air  Transport

Except  for  operations  with  single-engined  propeller-driven  aeroplanes  with  a  MOPSC  of  5  or single-engined  non-complex  helicopters  with  a  MOPSC  of  5,  taking-off  and  landing  at  the same  aerodrome  or  operating  site,  under  VFR  by  Day,  and  for  operations  with  sailplanes  and balloons,  the  main  structure  of  the  OM  shall  be  as  follows :

a )    Part  A :   General / Basic,  comprising  all  non - type - related  operational  policies,  instructions   and  procedures ;

b )    Part  B :  Aircraft  operating  matters,  comprising  all  type - related  instructions  and procedures,  taking  into  account  differences  between  types / classes,  variants  or  individual  aircraft  used  by  the  Operator ;

c )    Part  C :   Commercial  Air  Transport   Operations,  comprising  route / role / area  and  aerodrome / operating  site  instructions  and  information ;

d )    Part  D :   Training,  comprising  all  training  instructions  for  personnel  required  for  a  safe operation.

ORO. MLR. 105   Minimum   Equipment   List

a )   A  Minimum  Equipment  List  ( MEL )  shall  be  established  as  specified  under  8. a. 3  of Annex  IV  to  ARM - AIR  OPERATIONS  Regulation,  based  on  the  relevant  Master  Minimum  Equipment  List  ( MMEL )  as  defined  in  the  data  mandatory  part  of  the  operational  suitability  established   in  accordance  with  CR - EC  No  748 / 2012 ;

b )   The  MEL  and  any  amendment  thereto  shall  be  approved  by  the  GDCA  of  RA ;

c )   The  operator  shall  amend  the  MEL  after  any  applicable  change  to  the  MMEL  within  the  acceptable  timescales ;

d )   In  addition  to  the  list  of  items,  the  MEL shall  contain :

1 )  a  preamble,  including  guidance  and  definitions  for  flight  crews  and  maintenance  personnel using  the  MEL ;

2 )  the  revision  status  of  the  MMEL  upon  which  the  MEL  is  based  and  the  revision  status  of  the  MEL ;

3 )  the  scope,  extent  and  purpose  of  the  MEL.

c )   The  operator  shall :

1 )   establish  rectification  intervals  for  each  inoperative  instrument,  item  of  equipment  or  function  listed  in  the  MEL.  The  rectification  interval  in  the  MEL  shall  not  be  less  restrictive  than  the  corresponding  rectification  interval  in  the  MMEL ;

2 )   establish  an  effective  rectification  programme ;

3 )   only  operate  the  aircraft  after  expiry  of  the  rectification  interval  specified  in  the MEL  when :

( i )    the  defect  has  been  rectified ;                      or

( ii )   The  rectification  interval  has  been  extended  in  accordance  with  ( f ).

f )    Subject  to  approval  of  the  GDCA,  the  Operator  may  use  a  procedure  for  the  one  time  extension  of  category  B,  C  and  D  rectification  intervals,  provided  that :

1 )   the  extension  of  the  rectification  interval  is  within  the  scope  of  the  MMEL  for  the  aircraft  type ;

2 )   the  extension  of  the  rectification  interval   is,  as  a  maximum,  of  the  same  duration  as  the  rectification  interval  specified  in  the  MEL ;

3 )   the  rectification  interval  extension  is  not  used  as  a  normal  means  of  conducting  MEL  item  rectification  and  is  used  only  when  events  beyond  the  control  of  the  Operator  have  precluded  rectification ;

4 )   a  description  of  specific  duties  and  responsibilities  for  controlling  extensions  is  established  by  the  Operator ;

5 )   the  GDCA  is  notified  of  any  extension  of  the  applicable  rectification  interval ;   and

6 )   a  plan  to  accomplish  the  rectification  at  the  earliest  opportunity  is  established.

g )   The  Operator  shall  establish  the  operational  and  maintenance  procedures  referenced  in  the  MEL  taking  into  account  the  operational  and  maintenance  procedures  referenced  in  the  MMEL.  These  procedures  shall  be  part  of  the  Operator’s  Manuals  or  the  MEL ;

h )   The  Operator  shall  amend  the  operational  and  maintenance  procedures  referenced  in  the  MEL  after  any  applicable  change  to  the  operational  and  maintenance  procedures  referenced  in  the  MMEL ;

i )   Unless  otherwise  specified  in  the  MEL,  the  operator  shall   complete :

1 )   the  operational  procedures  referenced  in  the  MEL  when  planning  for  and / or  operating  with  the  listed  item  inoperative ;                                                                  and

2 )   the  maintenance  procedures  referenced  in  the  MEL  prior  to  operating  with  the  listed  item  inoperative.

j )   Subject  to  a  specific  case - by - case  approval  by  the  GDCA,  the  Operator  may  operate  an  aircraft  with  inoperative  instruments,  items  of  equipment  or  functions  outside  the  constraints  of  the  MEL  but  within  the  constraints  of  the  MMEL,  provided  that :

1 )  the  concerned  instruments,  items  of  equipment  or  functions  are  within   the  scope  of  the MMEL  as  defined  in  point  ( a ) ;

2 )   the  approval  is  not  used  as  a  normal  means  of  conducting  operations  outside  the  constraints  of  the  approved  MEL  and  is  used  only  when  events  beyond  the  control  of  the  operator  have  precluded  the  MEL  compliance ;

3 )   a  description  of  specific  duties  and  responsibilities  for  controlling  the  operation  of  the  aircraft  under  such  approval  is  established  by  the  Operator ;                              and

4 )   a  plan  to  rectify  the  inoperative  instruments,  items  of  equipment  or  functions  or  to  return  operating  the  aircraft  under  the  MEL  constraints  at  the  earliest  opportunity  is  established.

ORO. MLR. 110   Journey   Log

Particulars  of  the  aircraft,  its  crew  and  each  journey  shall  be  retained  for  each  flight,  or  series  of  flights,  in  the  form  of  a  Journey  ( Flight )  Log,  or  equivalent.

ORO. MLR. 115   Record - Keeping

a )   The  following  records  shall  be  stored  for  at  least  5  years.

1 )  for  CAT  Operators,  records  of  the  activities  referred  to  in  ORO. GEN. 200 ;

2 )  for  declared  Operators,  a  copy  of  the  Operator’s  Declaration,  details  of  approvals  held  and Operations  Manual ;

3 )  for  SPO  authorization  holders,  in  addition  to  ( a )( 2 ),  records  related  to  the  risk  assessment  conducted  in  accordance  with  SPO. OP. 230  and  related  Standard  Operating Procedures.

b )   The  following  information  used  for  the  preparation  and  execution  of  a  flight,  and associated  reports,  shall  be  stored  for  3  ( three )  months :

1 )  the  Operational  Flight  Plan,  if  applicable ;

2 )  route-specific  Notice  to  Airmen  ( NOTAM )  and  Aeronautical  Information  Services ( AIS ) briefing  documentation,  if  edited  by  the  Operator ;

3 )  mass  and  balance  documentation ;

4 )  notification  of  special  loads,  including  written  information  to  the  Commander  about Dangerous  Goods,  if  applicable  ;

5 )  the  Journey  Log,  or  equivalent ;                                                                       and

6 )  Flight  Report(s)  for  recording  details  of  any  occurrence,  or  any  event  that  the  Commander  /  PIC  deems  necessary  to  report  or  record ;

c )    Personnel  records  shall  be  stored  for  the  periods  indicated  below :

| Flight Crew Licence and Cabin Crew Attestation | As long as the crew member is exercising the privileges of the licence or attestation for the aircraft operator |
| --- | --- |
| Crew member Training, Checking and Qualifications | 3 years |
| Records on crew member recent experience | 15 months |
| Crew member route and aerodrome / task and area competence, as appropriate | 3 years |
| Dangerous Goods Training, as appropriate | 3 years |
| Training / Qualification Records of other personnel for whom a Training Programme is required. | Last 2 Training Records |

d )    The  Operator  shall :

1 )  maintain  records  of  all  training,  checking  and  qualifications  of  each  crew  member,  as prescribed  in  Part - ORO ;                                                                          and

2 )  make  such  records  available,  on  request,  to  the  crew  member  concerned.

e )    The  Operator  shall  preserve  the  information  used  for  the  preparation  and  execution  of  a flight  and  personnel  training  records,  even  if  the  operator  ceases  to  be  the  Operator  of  that aircraft  or  the  employer  of  that  crew  member,  provided  this  is  within  the  timescales  prescribed in  ( c ).

f )    If  a  crew  member  becomes  a  crew  member  for  another  Operator,  the  operator  shall  make the  crew  member’s  records  available  to  the  new  Operator,  provided  this  is  within  the  timescales  prescribed  in  ( c ).

SUBPART   SEC.       SECURITY

ORO. SEC. 100. A   Flight   Crew   Compartment   Security  —  Aeroplanes

a )   In  an  aeroplane  which  is  equipped  with  a  flight  crew  compartment  door,  this  door  shall  be  capable  of  being  locked,  and  means  shall  be  provided  by  which  the  cabin  crew  can  notify  the  flight  crew  in  the  event  of  suspicious  activity  or  security  breaches  in  the  cabin ;

b )   All  passenger-carrying  aeroplanes  of  a  maximum  certificated  take-off  mass  exceeding 45 500 kg,  or  with  a  MOPSC  of  more  than  60  engaged  in  the  commercial  transportation  of  passengers,  shall  be  equipped  with   an  approved  flight  crew  compartment  door  that  is  capable  of  being  locked  and  unlocked  from  either  pilot’s  station  and  designed  to  meet  the  applicable  airworthiness  requirements ;

c )   In  all  aeroplanes  which  are  equipped  with  a  flight  crew  compartment  door  in accordance  with  point   ( b )  above :

1 )   this  door  shall  be  closed  prior  to  engine  start  for  Take - off  and  will  be  locked  when  required  by  security  procedures  or  by  the  Pilot - in - Command  until  engine  shut  down  after  landing,  except  when  deemed  necessary  for  authorized  persons  to  access  or  egress  in  compliance  with  national  civil  aviation  security  programmes ;                            and

2 )   means  shall  be  provided  for  monitoring  from  either  pilot’s  station  the  entire  door  area  outside  the  flight   crew  compartment  to  identify  persons  requesting  entry  and  to  detect  suspicious   behaviour  or  potential  threat.

ORO. SEC. 100. H    Flight   Crew   Compartment   Security  —  Helicopters

If  installed,  the  flight  crew  compartment  door  on  a  helicopter   operated  for  the  purpose  of  carrying  passengers  shall  be  capable  of  being  locked  from  within  the  flight  crew  compartment  in  order  to  prevent  unauthorized  access.

LEFT            INTENTIONALLY            BLANK

SUBPART   FC.     FLIGHT  CREW

ORO. FC. 005     Scope

This  Subpart  establishes  requirements  to  be  met  by  the  Operator  related  to  flight  crew  training,  experience  and  qualification  and  comprises :

a )   SECTION  1  specifying  common  requirements  applicable  to  both  Non-commercial  Operations  of  complex  motor-powered  aircraft  and  any  Commercial  Operation ;

b )   SECTION  2  specifying  additional  requirements  applicable  to  Commercial  Air  Transport Operations,  with  the  exception  of :

1 )  Commercial  Air  Transport  Operations  of  sailplanes  or  balloons ;         or

2 )  Commercial  Air  Transport  Operations  of  passengers  conducted  under  Visual  Flight  Rules   ( VFR )  by  Day,  starting  and  ending  at  the  same  aerodrome  or  operating  site  and  within  a  local  area  specified  by  the  competent  authority,  with

—  single-engined  propeller-driven  aeroplanes  having  a  maximum  certified  take-off  mass  of  5 700 kg  or  less  and  a  MOPSC  of  5 ;                               or

—   other-than-complex  motor-powered  helicopters,  single-engined,  with  a  MOPSC  of  5.

c )   SECTION  3  specifying  additional  requirements  for  Commercial  Specialized  Operations  and  for  those  referred  to  in   b ( 1 )  and  ( 2 ).

SECTION  1.       Common   Requirements

ORO. FC. 100   Composition   of   Flight   Crew

a )   The  composition  of  the  flight  crew  and  the  number  of  flight   crew  members  at  designated  crew  stations  shall  be  not  less  than  the  minimum  specified  in  the  Aircraft  Flight  Manual  or  operating  limitations  prescribed  for  the  aircraft ;

b )   The  flight  crew  shall  include  additional  flight  crew  members  when  required  by  the  type  of  operation  and  shall  not  be  reduced  below  the  number  specified  in  the  Operations  Manual ;

c )   All  flight  crew  members  shall  hold  a  licence  and  ratings  issued  or  accepted  in  accordance  with  ARM - AIR  CREW   Regulation  and  appropriate  to  the  duties  assigned  to  them ;

d )   The  flight  crew  member  may  be  relieved  in  flight  of  his / her  duties  at  the  controls  by  another  suitably  qualified  flight  crew  member ;

e )   When  engaging  the  services  of  flight  crew  members  who  are  working  on  a  freelance  or  part - time  basis,  the  Operator  shall  verify  that  all  applicable  requirements  of  this  Subpart  and  the  relevant  elements  of  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation ,  including  the  requirements  on  recent  experience,  are  complied  with,  taking  into  account  all  services  rendered  by  the  flight  crew  member  to  other   Operator(s)  to  determine  in  particular :

1 )   the  total  number  of  aircraft  types  or  variants  operated ;                             and

2 )   the  applicable  Flight   and  Duty  Time  Limitations  and   Rest  Requirements.

ORO. FC. 105   Designation  as  Pilot - in - Command  /  Commander

a )   In  accordance  with   8. e   of  Annex  IV  to  this  AIR  OPERATIONS  Regulation ,  one  pilot  amongst  the  flight  crew,  qualified  as  Pilot - in - Command  /  Commander  in  accordance   with  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation,  shall  be  designated  by  the  Operator  as  Pilot - in - Command   or,  for  Commercial  Air  Transport  Operations,  as  Commander ;

b )   The  Operator  shall  only  designate  a  flight  crew  member  to  act  as  Pilot - in - Command  /   Commander  if  he  /  she  has :

1 )   the  minimum  level  of  experience  specified  in  the  Operations  Manual ;

2 )   adequate  knowledge  of  the  route  or  area  to  be  flown  and  of  the  aerodromes,  including  alternate  aerodromes,  facilities  and  procedures  to  be  used ;

3 )   in  the  case  of  multi - crew  operations,  completed  an  Operator’s  Command  Course  if upgrading  from  First  Officer  ( Co - pilot )  to  Pilot - in - Command  /  Commander.

c )   In  the  case  of  commercial  operations  of  aeroplanes and  helicopters,  the  Pilot - in - Command  /  Commander  or  the  pilot,  to  whom  the  conduct  of  the  flight  may  be  delegated,  shall  have  had   initial  familiarization  training  of  the  route  or  area  to  be  flown  and  of  the  aerodromes,  facilities  and  procedures  to  be  used.  This  route / area  and  aerodrome  knowledge  shall  be  maintained  by  operating  at  least  1 ( once )  on  the  route  or  area  or  to  the  aerodrome  within  a  12 - month  period ;

d )   Point  ( c )  shall  not  apply  in  the  case  of :

1 )  Performance  Class  B  Aeroplanes  involved  in  Commercial  Air  Transport  Operations  under  VFR  by  Day ;                                                                                   and

2 )  Commercial  Air  Transport  Operations  of  passengers  conducted  under  VFR  by  Day,  starting  and  ending  at  the  same  aerodrome  or  operating  site  or  within  a  local  area  specified  by  the  GDCA,  with  other  than  complex  motor - powered  helicopters,  single - engined,  with   a  MOPSC  of   5.

ORO. FC. 110   Flight  Engineer

When  a  separate  flight  engineer  station  is  incorporated  in  the  design  of  an  aeroplane,  the  flight  crew  shall  include  one  crew  member  who  is  suitably  qualified  in  accordance  with applicable  national  rules.

ORO. FC. 115   Crew  Resource  Management  ( CRM )  Training

a )   Before  operating,  the  flight  crew  member  shall  have  received  CRM  Training,  appropriate  to  his / her  role,  as  specified  in  the  Operations  Manual ;

b )   Elements  of  CRM  Training  shall  be  included  in  the  aircraft  type  or  class  training   and recurrent  training  as  well  as  in  the  command   course.

ORO. FC. 120   Operator  Conversion  Training

a )   In  the  case  of  aeroplane  or  helicopter  operations,  the  flight  crew  member  shall  complete  the  Operator  Conversion  Training  Course  before  commencing  unsupervised  line  flying :

1 )   when  changing  to  an  aircraft  for  which  a  new  type  or  class  rating  is  required ;

2 )   when  joining  an  Operator.

b )   The  Operator  Conversion  Training  Course  shall   include  training  on  the  equipment  installed  on  the  aircraft  as  relevant  to   flight  crew  member’s  roles.

ORO. FC. 125   Differences  Training  and  Familiarization  Training

a )   Flight  crew  members  shall  complete  Differences  or  Familiarization  Training  when  required  by  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation  and  when  changing  equipment  or  procedures  requiring  additional  knowledge  on  types  or  variants  currently  operated ;

b )   The  Operations  Manual  shall  specify  when  such  Differences  or  Familiarization  Training  is  required.

ORO. FC. 130   Recurrent  Training  and  Checking

a )   Each  flight  crew  member  shall  complete  Annual  Recurrent  Flight  and  Ground  Training relevant  to  the  type  or  variant   of  aircraft  on  which  he / she  operates,  including  training  on  the  location  and  use  of  all  emergency  and  safety  equipment  carried ;

b )   Each  flight  crew  member  shall  be  periodically  Checked   to  demonstrate  competence  in carrying  out  normal,  abnormal  and  emergency  procedures.

ORO. FC. 135   Pilot  Qualification  to  Operate  in  Either  Pilot’s  Seat

Flight  crew  members  who  may  be  assigned  to  operate  in  either  pilot’s  seat  shall  complete  appropriate  Training  and  Checking  as  specified  in  the  Operations  Manual.  ORO. FC. 140  Operation  on  more  than one  type  or  variant

a )   Flight  crew  members  operating  more  than  one  type  or  variant  of  aircraft  shall  comply  with  the  requirements  prescribed  in  this  Subpart  for  each  type  or  variant,  unless  credits  related to  the  training,  checking,  and  recent  experience  requirements  are  defined  in  the  mandatory  part of  the  operational  suitability  data  established  in  accordance  with  CR - EU No 748 / 2012  for  the

relevant  types  or  variants ;

b )   Appropriate  procedures  and / or  operational  restrictions  shall  be  specified  in  the  Operations Manual  for  any  operation  on  more  than  one  type  or  variant.

ORO. FC. 140   Operation  on  More  than  One  Type  or  Variant

a )   Flight  crew  members  operating  more  than  One  Type  or  Variant  of  aircraft  shall  comply  with  the  requirements  prescribed  in  this  Subpart  for  each  type  or  variant,  unless  credits  related  to  the  training,  checking,  and  recent  experience  requirements  are  defined  in  the  mandatory  part  of  the  operational suitability  data  established  in  accordance  with   CR - EC No  748 / 2012  for  the  relevant  types  or  variants ;

b )   Appropriate  procedures  and / or  operational  restrictions  shall  be  specified  in  the  Operations  Manual  for  any  operation  on  more  than  one  type  or  variant.

ORO. FC. 145   Provision  of  Training

a )   All  the  training  required  in  this  Subpart  shall  be  conducted :

1 )   in  accordance  with  the  training  programmes  and  syllabi  established  by  the  Operator  in  the  Operations  Manual ;

2 )   by  appropriately  qualified  personnel.  In  the  case  of  flight  and  flight  simulation  training  and  checking,  the  personnel  providing  the  training  and  conducting  the  checks  shall   be  qualified  in  accordance  with  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation ;

b )   When  establishing  the  training  programmes  and  syllabi,  the  Operator  shall  include  the mandatory  elements  for  the  relevant  type  as  defined  in  the  data  established  in  accordance  with  CR - EC  No  748 / 2012 ;

c )   In  the  case  of  CAT  Operations,  training  and  checking  programmes,  including  syllabi  and use  of  individual   Flight  Simulation  Training  Devices  ( FSTD’s ),  shall  be  approved  by  the  GDCA  of  RA ;

d )   The  FSTD  shall  replicate  the  aircraft  used  by  the  Operator,  as  far  as  practicable. Differences  between  the  FSTD  and  the  aircraft  shall  be  described  and  addressed  through  a  briefing  or  training,  as  appropriate ;

e )   The  Operator  shall  establish  a  system  to  adequately  monitor  changes  to  the  FSTD  and  to  ensure  that  those  changes  do  not  affect  the  adequacy  of  the  training  programmes.

SECTION   2.     Additional  Requirements  for  Commercial  Air  Transport  Operations

ORO. FC. 200   Composition  of  Flight  Crew

a )   There  shall  not  be  more  than  one  inexperienced  flight   crew  member  in  any  flight  crew ;

b )   The  Pilot - in - Command  /  Commander  may  delegate  the  conduct  of  the  flight  to  another pilot  suitably  qualified  in  accordance  with  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation,   provided  that  the  requirements  of  ORO. FC. 105 ( b )( 1 ), ( b )( 2 )  and  ( c )  are  complied  with ;

c )   Specific  requirements  for  aeroplane  operations  under  Instrument  Flight  Rules  ( IFR )  or  at  night ;

1 )   the  minimum  flight  crew  shall  be  2  ( two )  pilots  for  all  turbo - propeller  aeroplanes  with  a  Maximum  Operational  Passenger  Seating  Configuration  ( MOPSC )  of  more  than  9  ( nine )  and  all  turbojet  aeroplanes ;

2 )   aeroplanes  other  than  those  covered  by  ( c )( 1 )  shall  be  operated  with  a  minimum  crew  of  2  ( two )  pilots,  unless  the  requirements  of  ORO. FC. 202  are  complied  with,  in  which  case  they  may  be  operated  by  a  single  pilot.

d )   Specific  requirements  for  Helicopter  operations :

1 )   for  all  operations  of  helicopters  with  an  MOPSC  of  more  than  19  and  for  operations  under  IFR  of  helicopters  with  an  MOPSC  of  more  than  more  than  9  ( nine ) :

( i )    the  minimum  flight  crew  shall  be  2  ( two )  pilots ;                                          and

( ii )   the  Commander   shall  be  the  holder  of  an  Airline  Transport  Pilot  Licence                            ( Helicopter )  ( ATPL ( H ) )  with  an  Instrument  Rating  issued  in  accordance  with Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation .

2 )   operations  not  covered  by  ( d )( 1 )  may  be  operated  by  a  single  pilot  under  IFR  or  at  night  provided  that  the  requirements  of  ORO. FC. 202  are  complied  with.

ORO. FC. A. 201   In - Flight  Relief  of  Flight  Crew  Members

a )   The  Commander  may  delegate  the  conduct  of  the  flight  to :

1 )   another  qualified  Commander ;                                  or

2 )   for  operations  only  above  Flight  Level  ( FL ) 200,  a  pilot  who  complies  with  the  following  minimum  qualifications :

( i )     ATPL ( A ) ;

( ii )   conversion  training  and  checking,  including  type  rating  training,  in  accordance  with  ORO. FC. 220 ;

( iii )  all  recurrent  training  and  checking  in  accordance  with  ORO. FC. 230  and                       ORO. FC. 240 ;

( iv )  route  /  area  and  aerodrome  competence  in  accordance  with   ORO. FC. 105.

b )   The  First  Officer  /  Co-pilot  may  be  relieved  by :

1 )   another  suitably  qualified  pilot ;

2 )   for  operations  only  above  FL 200,  a  cruise  relief  co - pilot  that  complies  with  the  following  minimum  qualifications :

( i )     valid  Commercial  Pilot  Licence  ( CPL )  with  an  Instrument  Rating ;

( ii )    conversion  training  and  checking,  including  type  rating  training,  in  accordance  with  ORO. FC. 220  except  the  requirement  for  take-off  and landing  training ;

( iii )   recurrent  training  and  checking  in  accordance  with  ORO. FC. 230  except  the  requirement  for  take-off  and  landing  training.

c )   A  Flight  Engineer  may  be  relieved  in  flight  by  a  crew  member  suitably  qualified  in accordance  with  applicable  national  rules.

ORO. FC. 202   Single - Pilot  Operations  under  IFR  or  at  Night

In  order  to  be  able  to  fly  under  IFR  or  at  Night  with  a  minimum  flight  crew  of  one  pilot,  as  foreseen  in  ORO. FC. 200 ( c )( 2 )  and  ( d )( 2 ),  the  following  shall  be  complied  with :

a )   The  Operator  shall  include  in  the  Operations  Manual  a  pilot’s  conversion  and  recurrent training  programme  that  includes  the  additional  requirements  for  a  single - pilot  operation.                        The  pilot  shall  have  undertaken  training  on  the  operator’s  procedures,  in  particular  regarding :

1 )   engine  management  and  emergency  handling ;

2 )   use  of  normal,  abnormal  and  emergency  Checklist ;

3 )   air  traffic  control  ( ATC )  communication ;

4 )   departure  and  approach  procedures ;

5 )   autopilot  management,    if  applicable ;

6 )   use  of  simplified  in - flight  documentation ;

7 )   single - pilot  crew  resource  management.

b )   The  Recurrent  Checks  required  by  ORO. FC. 230  shall  be  performed  in  the  single - pilot role  on  the  relevant  type  or  class of  aircraft  in  an   environment  representative  of  the  operation;

c )   For  aeroplane  operations  under  IFR  the  pilot  shall  have :

1 )   a  minimum  of  50  hours  flight  time  under  IFR  on  the  relevant  type  or  class  of  aeroplane,  of  which  10  hours  are  as  Commander ;                            and

2 )   completed  during  the  preceding  90  days  on  the  relevant  type  or  class  of aeroplane :

( i )     5 ( Five ) IFR  flights,  including  3  ( three )  instrument  approaches,  in  a  single - pilot   role ;            or

( ii )    an  IFR  instrument   approach  check.

d )   For  aeroplane  operations  at  night  the  pilot  shall  have :

1 )   A  minimum  of  15  hours  flight  time  at  night  which  may  be  included  in  the  50  hours  flight  time  under  IFR  in  ( c )( 1 ) ;                             and

2 )   completed  during  the  preceding  90  days  on  the  relevant  type  or  class  of  aeroplane :

( i )   3  ( three )  take-offs  and  landings  at  night  in  the  single  pilot  role ;              or

( ii )  a  night  take-off  and  landing  check.

e )   For  Helicopter  operations  under  IFR  the  pilot  shall  have :

1 )   25   hours   total   IFR  flight  experience  in  the  relevant  operating   environment ;    and

2 )   25  hours  flight  experience  as  a  single  pilot  on  the  specific  type  of  helicopter,  approved  for  single - pilot  IFR,  of  which  10  hours  may  be  flown  under  supervision,  including  five  sectors  of  IFR  line  flying  under  supervision  using  the  single - pilot  procedures ;    and

3 )   completed  during  the  preceding  90  days :

( i )    5  ( five )  IFR  flights  as  a  single  pilot,  including  3  ( three )  instrument  approaches,  carried  out  on  a  helicopter  approved  for  this  purpose ;                    or

( ii )   an  IFR  instrument  approach  check  as  a  single - pilot  on  the  relevant  type  of  Helicopter,  Flight  Training  Device  ( FTD )  or  Full  Flight  Simulator  ( FFS ).

ORO. FC. 205   Command  Course

a )   For  aeroplane  and  helicopter  operations,  the  command  course  shall  include  at  least  the following  elements :

1 )   training  in  an  FSTD,  which  includes  Line  Oriented  Flight  Training  ( LOFT )  and  /  or  flight  training ;

2 )   the  Operator  Proficiency  Check,  operating  as  Commander ;

3 )   Command  responsibilities  training ;

4 )   line  training  as  Commander  Under  Supervision,  for  a  minimum  of :

( i )    10  flight  sectors,  in  the  case  of  aeroplanes ;                        and

( ii )   10  hours,  including  at  least  10  flight  sectors,  in  the  case  of  Helicopters ;

5 )   Completion  of  a  Line  Check  as  Commander  and  demonstration  of  adequate  knowledge  of  the  route  or  area  to  be  flown  and  of  the  aerodromes,  including  alternate  aerodromes,  facilities  and  procedures  to  be  used ;                                       and

6 )   Crew  Resource  Management  Training.

ORO. FC. 215   Initial  Operator’s  Crew  Resource  Management  ( CRM )  Training

a )   The  flight  crew  member  shall  have  completed  an  initial  CRM  training  course  before commencing  unsupervised  line  flying ;

b )   Initial  CRM  training  shall  be  conducted  by  at  least  one  suitably  qualified  CRM  trainer  who  may  be  assisted  by  experts  in  order  to  address  specific  areas ;

c )   If  the  flight  crew  member  has  not  previously  received  theoretical  training  in  human  factors  to  the  ATPL  level,  he / she  shall  complete,  before  or  combined  with  the  initial  CRM  training,  a  theoretical  course  provided  by  the  Operator  and  based  on  the  human  performance  and  limitations  syllabus  for  the  ATPL  as  established  in  Annex  I  ( Part - FCL )  to  ARM - AIR  CREW  Regulation.

ORO. FC. 220   Operator  Conversion  Training  and  Checking

a )   CRM  training  shall  be  integrated  into  the  Operator  Conversion  Training  Course ;

b )   Once  an  Operator  conversion  course  has  been  commenced,  the  flight  crew  member  shall  not  be  assigned  to  flying  duties  on  another  type  or  class  of  aircraft  until  the course  is  completed  or  terminated.  Crew  members  operating  only  Performance  Class  B aeroplanes  may  be  assigned  to  flights  on  other  types  of  Performance  Class  B  aeroplanes  during   Conversion  Courses  to  the  extent  necessary  to  maintain  the operation ;

c )   The  amount  of  training  required  by  the  flight  crew  member  for  the  operator’s  conversion  course  shall  be  determined  in  accordance  with  the  standards  of  qualification  and  experience  specified  in  the  Operations  Manual,  taking  into  account  his  /  her  previous  training  and  experience ;

d )   The  flight  crew  member  shall  complete :

1 )   the  Operator  Proficiency  Check  and  the  Emergency  and  Safety  Equipment  Training  and  Checking  before  commencing  line  Flying  Under  Supervision  ( LIFUS ) ;           and

2 )   the  Line  Check  upon  completion  of  Line  Flying  Under  Supervision.  For  Performance  Class  B  aeroplanes,  LIFUS  may  be  performed  on  any  aeroplane  within  the  applicable  class.

e  )  In  the  case  of  aeroplanes,  pilots  that  have  been  issued  a  Type  Rating  based  on  a  Zero Flight-Time  Training  ( ZFTT )  course  shall :

1 )   commence  Line  Flying  Under  Supervision  not  later  than  21  days  after  the  completion  of  the  Skill  Test  or  after  appropriate  training  provided   by  the  Operator.  The  content  of  such  training  shall  be  described  in  the  Operations  Manual ;

2 )   complete  6  ( six )  take-offs  and  landings  in  a  FSTD  not  later  than  21  days  after  the completion  of  the  Skill  Test  under  the  supervision  of  a  type  rating  instructor  for  aeroplanes  ( TRI ( A ))  occupying  the  other  pilot  seat.  The  number  of  take-offs  and  landings  may  be  reduced  when  credits  are  defined  in  the  data  established  in  accordance  with  Regulation  ( EC ) No  1702 / 2003.                                                                                                                  If  these  take-offs  and  landings  have  not  been  performed  within  21  days,  the  Operator  shall  provide  Refresher  Training.  The  content  of  such  training  shall  be  described  in  the  Operations Manual ;

3 )   conduct  the  first  4  ( four )  take-offs  and  landings  of  the  LIFUS  in  the  aeroplane  under  the  supervision  of  a  TRI ( A )  occupying  the  other  pilot  seat.  The  number  of  take-offs  and  landings  may  be  reduced  when   credits  are  defined  in  the  data  established  in  accordance  with  Regulation   ( EC )  No  1702 / 2003.

ORO. FC. 230   Recurrent  Training  and  Checking

a )   Each  flight  crew  member  shall  complete  Recurrent  Training  and  Checking  relevant  to  the type  or  variant  of  aircraft  on  which  they  operate ;

b )   Operator  Proficiency  Check :

1 )   each  flight  crew  member  shall  complete  Operator  Proficiency  Checks  as  part  of  the normal  crew  complement  to  demonstrate  competence  in  carrying  out  normal,  abnormal  and  emergency  procedures ;

2 )   when  the  flight  crew  member  will  be  required  to  operate  under  IFR,  the  Operator Proficiency  Check   shall  be  conducted  without  external  visual  reference,  as  appropriate ;

3 )   the  validity  period  of  the  Operator  Proficiency  Check  shall  be  6  ( six )  calendar  months. For  operations  under  VFR  by  day  of  Performance  Class  B  aeroplanes  conducted  during  seasons  not  longer  than  eight  consecutive  months,  one  operator  proficiency check  shall  be  sufficient.  The  proficiency  check  shall  be  undertaken  before  commencing  Commercial  Air  Transport  operations ;

4 )   the  flight  crew  member  involved  in  operations  by  day  and  over  routes  navigated  by reference  to  visual  landmarks  with  an   other - than - complex  motor - powered  Helicopter  may  complete  the  operator  proficiency  check  in  only  one  of  the  relevant  types  held.  The  Operator  Proficiency  Check  shall  be  performed  each  time  on  the  type  least  recently  used  for  the  proficiency  check.  The  relevant  helicopter  types  that  may  be  grouped  for  the  purpose  of  the  operator   proficiency  check  shall  be  contained  in  the  Operations  Manual ;

5 )   notwithstanding  ORO. FC. 145 ( a )( 2 ),  for  operations  of  other - than - complex  motor -powered  Helicopters  by  day  and  over  routes  navigated  by  reference  to  visual  landmarks  and  Performance  Class  B  aeroplanes,  the  check  may  be  conducted  by  a  suitably  qualified  Commander  nominated  by  the  Operator,  trained  in  CRM  concepts  and  the  assessment  of  CRM  skills.   The  Operator  shall  inform  the  GDCA  about  the  persons  nominated.

c )   Line  Check :

1 )   Each  flight  crew  member  shall  complete  a  Line  Check  on  the  aircraft  to  demonstrate competence  in  carrying  out  normal  line  operations  described  in  the  Operations  Manual.                 The  validity  period  of  the  Line  Check  shall  be  12  calendar  months ;

2 )   notwithstanding  ORO. FC. 145 ( a )( 2 ),  Line  Checks  may  be  conducted  by  a  suitably qualified  Commander  nominated  by  the  Operator,  trained  in  CRM  concepts  and  the  assessment  of  CRM  skills.

d )   Emergency  and  Safety  Equipment  Training  and  Checking :

Each  flight  crew  member  shall  complete  training  and  checking  on  the  location  and  use  of  all  emergency  and  safety  equipment  carried.  The  validity  period  of  an  emergency  and  safety  equipment   check  shall  be  12  calendar  months.

e )   CRM  Training :

1 )  elements  of  CRM  shall  be  integrated  into  all  appropriate  phases  of  the  Recurrent  Training;

2 )  each  flight  crew  member  shall  undergo  specific  modular  CRM  training.                                                  All  major  topics  of  CRM  training  shall  be  covered  by  distributing  modular  training  sessions  as  evenly  as  possible  over  each  three - year  period.

f )   Each  flight  crew  member  shall  undergo  Ground  Training  and  Flight  Training  in  an  FSTD  or  an  aircraft,  or  a  combination  of  FSTD  and  aircraft  training,  at  least  every  12 calendar  months ;

g )   The  validity  periods  mentioned  in  ( b )( 3 ),  ( c )  and  ( d )  shall  be  counted  from  the  end of  the  month  when  the  check  was  taken.

h )   When  the  training  or  checks  required  above  are  undertaken  within  the  last  3  ( three )  months  of  the  validity  period,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date.

ORO. FC. 235   Pilot  Qualification  to  Operate  in  Either  Pilot’s  Seat

a )   Commanders  whose  duties  require  them  to  operate  in  either  pilot  seat  and  carry  out  the  duties  of  a  First  Officer / Co-pilot,  or  Commanders  required  to  conduct  Training  or  Checking duties,  shall  complete  additional  training  and  checking  as  specified  in  the  Operations  Manual.  The  check  may  be  conducted  together  with  the  Operator  Proficiency  Check  prescribed  in  ORO. FC. 230 ( b ) ;

b )  The  additional  training  and  checking  shall  include  at  least  the  following :

1 )   an  engine  failure  during  take-off ;

2 )   a  one-engine - inoperative  approach  and  Go - around ;              and

3 )   a  one - engine - inoperative  landing.

c )   In  the  case  of  Helicopters,  Commanders  shall  also  complete  their  proficiency  checks  from  left - and  right - hand  seats,  on  alternate  proficiency  checks,  provided  that  when  the  type  rating  proficiency  check   is  combined  with  the  operator  proficiency  check  the  Commander  completes  his  /  her  training  or  checking  from  the  normally  occupied   seat ;

d )   When  engine - out  manoeuvres  are  carried  out  in  an  aircraft,  the  engine  failure  shall  be  simulated ;

e )   When  operating  in  the  First  Officer  / Co-pilot’s  seat,  the  checks  required  by  ORO. FC. 230   for  operating  in  the  Commander’s  seat  shall,  in  addition,  be  valid  and  current ;

f )   The  pilot  relieving  the  Commander  shall  have  demonstrated,  concurrent  with  the  Operator  Proficiency  Checks  prescribed  in  ORO. FC. 230 ( b ),  practice  of  drills  and  procedures  that  would  not,  normally,  be  his / her  responsibility.                                                                                             Where  the  differences  between  left - and  right - hand  seats  are  not  significant,  practice  may  be  conducted   in  either  seat ;

g )   The  pilot  other  than  the  Commander  occupying  the  Commander’s  seat  shall  demonstrate  practice  of  drills  and  procedures,  concurrent  with  the  Operator  Proficiency  Checks  prescribed  in  ORO. FC. 230 ( b ),  which  are  the  Commander’s  responsibility  acting  as  pilot  monitoring.

Where  the  differences  between  left - and  right - hand  seats   are  not  significant,  practice  may  be  conducted   in  either  seat.

ORO. FC. 240     Operation  on  more  than  One  Type  or  Variant

a )   The  procedures  or  operational  restrictions  for  operation  on  more  than  one  type  or  variant  established   in  the  Operations  Manual  and  approved  by  the  GDCA  shall  cover :

1 )   the  flight  crew  members’  minimum  experience  level ;

2 )   the  minimum  experience  level  on  one  type  or  variant  before  beginning  training  for  and  operation  of  another  type  or  variant ;

3 )   the  process  whereby  flight   crew  qualified  on  one  type  or  variant  will  be  trained  and  qualified  on  another  type  or  variant ;                                             and

4 )   all  applicable  recent  experience  requirements  for  each  type  or  variant.

b )   When  a  flight  crew  member  operates  both  helicopters  and  aeroplanes,  that  flight  crew member  shall  be  limited  to  operations  on  only  one  type  of  aeroplane  and  one  type  of  helicopter ;

c )   Point  ( a )  shall  not  apply  to  operations  of  Performance  Class  B  aeroplane  if  they  are limited  to  single - pilot  classes  of  reciprocating  engine  aeroplanes  under  VFR  by  Day.                     Point  ( b )  shall  not  apply  to  operations  of  Performance  Class  B  aeroplane  if  they  are  limited  to  single - pilot  classes  of  reciprocating  engine  aeroplanes.

ORO. FC. A. 245   Alternative  Training  and  Qualification  Programme

a )   The  aeroplane  Operator  having  appropriate  experience  may  substitute  one  or  more  of  the following  training  and  checking  requirements  for  flight  crew  by  an  Alternative  Training  and  Qualification  Programme ( ATQP ),  approved  by  the  GDCA :

1 )   SPA. LVO. 120  on  flight  crew  Training  and  Qualifications ;

2 )   Conversion  Training  and  Checking ;

3 )   Differences  and  Familiarization  Training ;

4 )   Command  Course ;

5 )   Recurrent  Training  and  Checking ;                        and

6 )   Operation  on  more  than  One  Type  or  Variant.

b )   The  ATQP  shall  contain  training  and  checking  that  establishes  and  maintains  at  least  an  equivalent  level  of  proficiency  achieved  by  complying  with  the  provisions  of  ORO. FC. 220  and  ORO. FC. 230.   The  level  of  flight  crew  training  and  qualification  proficiency  shall  be  demonstrated  prior  to  being  granted  the  ATQP  approval  by  the  GDCA ;

c )   The  Operator  applying  for  an  ATQP  approval  shall  provide  the  GDCA  with  an  implementation  plan,  including  a  description  of  the  level  of  flight  crew  training  and  qualification  proficiency  to  be  achieved ;

d )   In  addition  to  the  Checks  required  by ORO. FC. 230  and  FCL. 060  of  Annex I  (Part - FCL)  to  ARM - AIR  CREW  Regulation,  each  flight  crew  member  shall  complete  a  Line  Oriented  Evaluation  ( LOE )  conducted  in  an  FSTD.    The  validity  period  of  an  LOE  shall  be  12  calendar  months.  The  validity  period  shall  be  counted  from  the  end  of  the  month  when  the  check  was  taken.  When  the  LOE  is  undertaken  within  the  last  3  ( three )  months  of  the  validity  period,  the  new  validity  period  shall  be counted  from  the  original  expiry  date ;

e )   After  2  ( two )  years  of  operating  with  an  approved  ATQP,  the  Operator  may,  with  the approval  of  the  GDCA,  extend  the  validity  periods  of  the  checks  in  ORO. FC. 230  as  follows :

1 )   Operator  Proficiency  Check  to  12  calendar  months.  The  validity  period  shall  be  counted  from  the  end  of  the  month  when  the  check  was  taken.                                                                            When  the  check  is  undertaken  within  the  last  3  ( three )  months  of  the  validity  period ,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date ;

2 )   Line  Check  to  24  calendar  months.  The  validity  period  shall  be  counted  from  the  end  of the  month  when  the  check  was  taken.  When  the  check  is  undertaken  within  the  last   6  ( six )  months  of  the  validity  period,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date ;

3 )   Emergency  and  Safety  Equipment  Checking  to  24  calendar  months.  The  validity  period  shall  be  counted  from  the  end  of  the  month  when  the  check  was  taken.  When  the  check  is  undertaken  within  the  last  6  ( six )  months  of  the  validity  period,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date.

ORO. FC. A. 250   Commanders  Holding  a  CPL ( A )

a )   The  holder  of  a  CPL ( A ) ( Aeroplane )  shall  only  act  as  Commander  in  Commercial  Air Transport  on  a  single - pilot  aeroplane  if :

1 )   When  carrying  passengers  under  VFR  outside  a  radius  of  50 nm  ( 90 km )  from  an  aerodrome  of  departure,  he / she  has  a  minimum  of  500  hours  of  flight  time  on  aeroplanes  or  holds  a  valid  Instrument   Rating ;                                                             or

2 )   when  operating  on  a  multi - engine  type  under  IFR,  he / she  has  a  minimum  of  700  hours  of  flight  time  on   aeroplanes,  including  400  hours  as  Pilot - in - Command.                                                   These  hours  shall   include  100  hours  under  IFR  and  40  hours  in  multi - engine  operations.  The   400  hours  as  Pilot - in - Command  may  be  substituted  by  hours  operating  as  Co-pilot  within  an  established  multi - pilot  crew  system  prescribed  in  the  Operations  Manual,  on  the  basis  of  2 ( two )  hours  of  flight  time   as  co-pilot  for  1 ( one )  hour  of  flight  time  as  Pilot - in - Command.

b )   For  operations  under  VFR  by  day  of  Performance  Class  B  aeroplanes  ( a )( 1 )  shall  not apply.

ORO. FC. H. 250   Commanders  Holding  a  CPL ( H )

a )   The  holder  of  a  CPL ( H ) ( Helicopter )  shall  only  act  as  Commander  in  Commercial  Air Transport  on  a  single - pilot  helicopter  if :

1 )   when  operating  under  IFR,  he / she  has  a  minimum  of  700  hours  total  flight  time  on  helicopters,  including  300  hours  as  Pilot - in - Command.                                                                           These  hours  shall  include  100  hours  under  IFR.   The  300  hours  as  Pilot - in - Command  may  be  substituted  by  hours  operating  as  Co - pilot  within  an  established  multi - pilot  crew  system  prescribed  in  the  Operations  Manual  on  the  basis  of  2  ( two )  hours  of  flight  time  as  Co-pilot  for  1  ( one )  hour  flight  time  as  Pilot - in - Command ;

2 )   when  operating  under  Visual  Meteorological  Conditions  ( VMC )  at  night,  he / she  has :

( i )    a  valid  Instrument  Rating ;                                       or

( ii )   300  hours  of  flight  time  on  Helicopters,  including  100  hours  as  Pilot - in - Command   and  10  hours  as  Pilot  Flying   ( PF )  at  night.

SECTION  3.    Additional  Requirements  for  Commercial  Specialized  Operations  and

CAT  Operations  referred  to  in   ORO. FC. 005 ( b )( 1 )  and  ( 2 )

ORO. FC. 330     Recurrent  Training  and  Checking  —  Operator  Proficiency  Check

a )   Each  flight  crew  member  shall  complete  Operator  Proficiency  Checks  to  demonstrate  his  / her  competence  in  carrying  out  normal,  abnormal  and  emergency  procedures,  covering  the relevant  aspects  associated  with  the  specialized  tasks  described  in  the  Operations  Manual ;

b )   Appropriate  consideration  shall  be  given  when  operations  are  undertaken  under  IFR  or  at night ;

c )   The  validity  period  of  the  Operator  Proficiency  Check  shall  be  12  calendar  months.  The validity  period  shall  be  counted  from  the  end  of  the  month  when  the  check  was  taken.  When  the  operator  proficiency  check  is  undertaken  within  the  last  3  ( three )  months  of  the validity  period,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date.

INTENTIONALLY            LEFT            BLANK

SUBPART   CC.     CABIN   CREW

ORO. CC. 005   Scope

This  Subpart  establishes  the  requirements  to  be  met  by  the  Operator  when  operating  an  aircraft  with  Cabin  Crew  and  comprises :

a )   Section  1  specifying  common  requirements  applicable  to  all  operations ;              and

b )   Section  2  specifying  additional  requirements  only  applicable  to  Commercial  Air  Transport Operations.

SECTION  I.     Common  Requirements

ORO. CC. 100   Number  and  Composition  of  Cabin  Crew

a )  The  number  and  composition  of  Cabin  Crew  shall  be  determined  in  accordance  with  7. a  of  Annex  IV  to  Air  Operations  CR - EC  No  216 / 2008,  taking  into   account  operational  factors  or  circumstances  of  the  particular  flight  to  be  operated.   Except  for  balloons ,  at  least  one  Cabin  Crew  member  shall  be  assigned  for  the  operation  of  aircraft  with  an  MOPSC  of  more  than  19  when  carrying  one  or  more  passenger(s) ;

b )   For  the  purpose  of  complying  with  ( a ),  the  minimum  number  of  Cabin  Crew  shall  be  the  greater  of  the  following :

1 )   the  number  of  cabin  crew  members  established   during  the  aircraft  certification  process  in  accordance  with  the  applicable  certification  specifications,  for  the  aircraft  cabin  configuration  used  by  the  operator ;                                          or

2 )   if  the  number  under  ( 1 )  has  not  been  established,  the  number  of  cabin  crew  established  during  the  aircraft  certification   process  for  the  maximum  certified  passenger  seating  configuration  reduced  by 1  for  every  whole  multiple  of  50  passenger  seats  of  the  aircraft  cabin  configuration  used  by  the  Operator  falling  below  the  maximum  certified  seating  capacity ;   or

3 )   1 ( one )  Cabin  Crew  Member  for  every  50 ,  or  fraction  of  50,  passenger  seats  installed  on  the  same  deck  of  the  aircraft  to  be  operated.

c )   For  operations  where  more   than  1 ( one )  cabin  crew  member  is  assigned,  the  Operator shall  nominate  1  ( one )  Cabin  Crew  Member  to  be  responsible  to  the  Pilot - in -Command / Commander.

ORO. CC. 110   Conditions  for  Assignment  to  Duties

a )   Cabin  crew  members  shall  only  be  assigned  to  duties  on  an  aircraft  if  they :

1 )   are  at  least  18  years  of  age ;

2 )   have  been  assessed,  in  accordance  with  the  applicable  requirements  of  Annex  IV  ( Part -MED )  to  ARM - AIR  CREW  Regulation,  as  physically  and  mentally  fit  to  perform  their  duties  and   discharge  their  responsibilities  safely ;                                        and

3 )   have  successfully  completed  all  applicable  training  and   checking  required  by  this  Subpart  and  are  competent  to  perform  the  assigned  duties  in  accordance  with  the  procedures  specified  in  the  Operations  Manual.

b )   Before  assigning  to  duties  cabin  crew  members  who  are  working  on  a  freelance  or  part -time  basis,  the  Operator  shall  verify  that  all  applicable  requirements  of  this  Subpart  are  complied  with,  taking  into  account  all  services  rendered  by  the  Cabin  Crew  member  to  any  other  operator(s),  to  determine  in  particular :

1 )   the  total  number  of  aircraft  types  and  variants  operated ;                               and

2 )   the  applicable  Flight  and  Duty  Time  Limitations  and  Rest  requirements.

c )   Operating  cabin  crew  members,  as  well  as  their  role  with   regard  to  the  safety  of passengers  and  flight,  shall   be  clearly  identified  to  the  passengers.

ORO. CC. 115   Conduct  of  Training  Courses  and  Associated  Checking

a )   A  detailed  programme  and  syllabus  shall  be  established  by  the  Operator  for  each  training  course  in  accordance  with  the  applicable  requirements  of  this  Subpart,  and  of  Annex V  ( Part -CC )  to  ARM - AIR  CREW  Regulation   where  applicable,  to  cover  the  duties  and responsibilities  to  be  discharged  by  the  Cabin  Crew  members ;

b )   Each  training  course  shall  include  theoretical  and  practical  instruction  together  with individual  or  collective  practice,  as  relevant  to  each  training  subject,  in  order  that  the  Cabin  Crew  member  achieves  and  maintains  the  adequate  level  of  proficiency  in  accordance  with  this  Subpart ;

c )   Each  training  course  shall  be :

1 )   conducted  in  a  structured  and  realistic  manner ;                       and

2 )   performed  by  personnel  appropriately  qualified  for  the  subject  to  be  covered.

d )   During  or  following  completion  of  all  training  required  by  this  Subpart,  each  Cabin  Crew  member  shall  undergo  a  check  covering  all  training  elements  of  the  relevant  training  programme,  except  for  Crew  Resource  Management  ( CRM )  Training.  Checks  shall  be  performed  by  personnel  appropriately  qualified  ( as  Instructor )  to  verify  that  the  Cabin  Crew  member  has  achieved  and  / or  maintains  the  required  level  of  proficiency ;

e )   CRM  training  courses  and  CRM  modules  where  applicable  shall  be  conducted  by  a  Cabin  Crew  CRM  Instructor.  When  CRM  elements  are  integrated  in  other  training,  a  Cabin  Crew  CRM  Instructor  shall  manage  the  definition  and  implementation  of  the  syllabus.

ORO. CC. 120   Initial  Training  Course

a )   Each  new  entrant  who  does  not  already  hold   a  valid  Cabin  Crew  Attestation  ( Licences ) issued  in  accordance  with  Annex V  ( Part - CC )  to  ARM - AIR  CREW  Regulation :

1 )   shall  be  provided  with  an  initial  training  course  as  specified  in  CC. TRA. 220   of  that  Annex ;                                                                          and

2 )   shall  successfully  undergo  the  associated   examination  before  undertaking  other  training  required  by  this  Subpart.

b )   Elements  of  the  Initial  Training  Programme  may  be  combined   with  the  first  Aircraft  Type  Specific  Training  and  Operator  Conversion  Training,  provided  that  the  requirements  of                       CC. TRA. 220  are  met  and  any  such  element(s)  are  recorded  as  elements  of  the  Initial   Training  Course  in  the  training  records  of  the  Cabin  Crew  members  concerned.

ORO. CC. 125   Aircraft  Type  Specific  Training  and  Operator  Conversion  Training

a )   Each  Cabin  Crew  member  shall  have  completed   appropriate  Aircraft   Type  Specific  Training  and  Operator  Conversion  Training,  as  well  as  the  associated  checks,  before  being :

1 )   first  assigned  by  the  Operator  to   operate  as  a  Cabin  Crew  member ;                 or

2 )   assigned  by  that  Operator  to  operate  on   another  aircraft  type.

b )   When  establishing  the  aircraft  type  Specific  and  the  Operator   Conversion  Training  Programmes  and  Syllabi,  the  Operator  shall  include,  where  available,  the  relevant  elements defined  in  the  mandatory  part  of  the  operational  suitability  data  established  in  accordance  with CR - EU No 748 / 2012.

c )   The  Aircraft  Type  Specific  Training  programme  shall :

1 )   involve  training  and   practice  on  a  representative  training  device  or  on  the  actual  aircraft;

and

2 )   cover   at  least  the  following  Aircraft  Type  Specific  Training  elements :

( i )     aircraft  description  as  relevant  to  cabin  crew  duties ;

( ii )    all  safety  equipment  and  systems  installed  relevant  to  Cabin  Crew  duties ;

( iii )   operation  and  actual  opening,  by  each  Cabin  Crew  member,  of  each  type  or  variant  of  normal  and  emergency  doors  and  exits  in  the  normal  and  emergency  modes ;

( iv )   demonstration  of  the  operation  of  the  other  exits  including  Flight  Crew  compartment  windows ;

( v )    fire  and  smoke  protection  equipment  where  installed ;

( vi )   evacuation  slide  training,  where  fitted ;

( vii )  operation  of  the  seat,  restraint  system  and  oxygen  system  equipment  relevant  to  pilot  incapacitation.

d )   The  operator  Conversion  Training  Programme  for  each  aircraft  type  to  be  operated  shall :

1 )   involve  training  and  practice  on  a  representative  training  device  or  on  the  actual  aircraft ;

2 )   include  training  in  the  Operator’s  Standard  Operating  Procedures  for  Cabin  Crew  members  to  be  first  assigned  to  duties  by  the  operator ;

3 )   cover  at  least  the  following  Operator  Specific  Training  elements  as  relevant  to  the  aircraft  type  to  be  operated :

( i )     description  of  the  cabin  configuration ;

( ii )    location,  removal  and  use  of  all  portable  safety  and  emergency  equipment  carried  on - board ;

( iii )   all  normal  and  emergency  procedures ;

( iv )   passenger  handling  and  crowd  control ;

( v )    fire  and  smoke  training  including  the  use  of  all  related  fire - fighting  and  protective  equipment  representative  of  that  carried  on - board ;

( vi )   Evacuation  procedures ;

( vii )   pilot  incapacitation  procedures ;

( viii )  applicable  security  requirements  and  procedures ;

( ix )    crew  resource  management.

ORO. CC. 130   Differences  Training

a )   In  addition  to  the  training  required  in  ORO. CC. 125,  the  Cabin  Crew  member  shall complete  appropriate  Training  and  Checking  covering  any  differences  before  being  assigned  on:

1 )   a  variant  of  an  aircraft  type  currently  operated ;                          or

2 )   a  currently  operated  aircraft  type  or  variant  with  different :

( i )    safety  equipment ;

( ii )   safety  and  emergency  equipment  location ;                             or

( iii )  normal  and  emergency  procedures.

b )   The  differences  training  programme  shall :

1 )   be  determined  as  necessary  on  the  basis  of  a  comparison  with  the  training  programme  completed  by  the  Cabin  Crew  member,  in  accordance  with  ORO. CC. 125 ( c ) and  ( d ),  for  the  relevant  aircraft  type ;                         and

2 )   involve  training  and  practice  in  a  representative  training  device  or  the  actual  aircraft  as  relevant  to  the  difference  training  element  to  be  covered.

c )    When  establishing  a  Differences  Training  Programme  and  Syllabus  for  a  variant  of  an aircraft  type  currently  operated,  the  Operator  shall  include,  where  available,  the  relevant elements  defined  in  the  mandatory  part  of  the  operational  suitability data  established  in accordance  with  CR - EU No 748 /2012.

ORO. CC. 135   Familiarization  -  Line  Training  under  Supervision

After  completion  of  Aircraft  Type  Specific  Training  and  Operator  Conversion  Training  on  an aircraft  type,  each  Cabin  Crew  member  shall  complete  appropriate  supervised  familiarization  on  the  type  before  being  assigned  to  operate  as  a  member  of  the  minimum  number  of  cabin  crew  required  in  accordance  with  ORO. CC. 100.

ORO. CC. 140   Recurrent  Training

a )   Each  Cabin  Crew  member  shall  complete  annually  Recurrent  Training  and  Checking ;

b )   Recurrent  Training  shall  cover  the  actions  assigned  to  each  member  of  the  Cabin  Crew in  normal  and  emergency  procedures  and  drills  relevant  to  each  aircraft  type  and / or  variant  to  be  operated ;

c )   Aircraft  Type  Specific  Training  elements :

1 )   recurrent  training  shall  include  annually  touch - drills  by  each  Cabin  Crew  member  for  simulating  the  operation  of  each  type  or  variant  of  normal  and  emergency  doors  and  exits  for  passenger  evacuation ;

2 )   recurrent  training  shall  also  include  at  intervals  not  exceeding  three  years :

( i )     operation  and  actual  opening  by  each  Cabin  Crew  member,  in  a  representative  training  device  or  in  the  actual  aircraft,  of  each  type  or  variant  of  normal  and  emergency  exits  in  the  normal  and  emergency  modes ;

( ii )    actual  operation  by  each  Cabin  Crew  member,  in  a  representative  training  device  or  in  the  actual  aircraft,  of  the  flight  crew  compartment  security  door,  in  both  normal  and  emergency  modes,  and  of  the  seat  and  restraint  system,  and  a  practical  demonstration  of  the  oxygen  system  equipment  relevant  to  pilot  incapacitation ;

( iii )   demonstration  of  the  operation  of  all  other  exits  including  the  flight  crew  compartment  windows ;                                                                                         and

( iv )   demonstration  of  the  use  of  the  life - raft,  or  slide  raft,  where  fitted.

d )   Operator  Specific  Training  elements :

1 )   Recurrent  Training  shall  include  annually :

( i )     by  each  cabin  crew  member :

( A )   location  and  handling  of  all  safety  and  emergency  equipment  installed  or  carried  on  board ;                          and

( B )   the  donning  of  life - jackets,  portable  oxygen  and  protective  breathing  equipment              ( PBE ) ;

( ii )    stowage  of  articles  in  the  passenger  compartment ;

( iii )   procedures  related  to  aircraft  surface  contamination ;

( iv )   emergency  procedures ;

( v )    evacuation  procedures ;

( vi )   incident  and  accident  review ;

( vii )  crew  resource  management ;

( viii ) aero-medical  aspects  and  first  aid  including  related  equipment ;

( ix )   security  procedures.

2 )   Recurrent  Training  shall  also  include  at  intervals  not  exceeding  3  ( three )  years :

( i )    use  of  pyrotechnics  ( actual  or  representative  devices ) ;

( ii )   practical  demonstration  of  the  use  of  Flight  Crew  Checklists ;

( iii )  realistic  and  practical  training  in  the  use  of  all  fire - fighting  equipment,  including  protective  clothing,  representative  of  that  carried  in  the  aircraft ;

( iv )  by  each  cabin  crew  member :

( A )   extinguishing  a  fire  characteristic  of  an  aircraft  interior  fire ;

( B )   donning  and  use  of  PBE  in  an  enclosed  simulated  smoke - filled  environment.

e )   Validity  periods :

1 )   the  annual  Recurrent  Training  validity  period  shall  be  12  calendar  months  counted  from  the  end  of  the  month  when  the  check  was  taken ;

2 )   if  the  Recurrent  Training  and  Checking  required  in  ( a )  are  undertaken  within  the  last  3  ( three )  calendar  months  of  the  validity  period,  the  new  validity  period  shall  be  counted  from  the  original  expiry  date ;

3 )   for  the  additional  triennial  training  elements  specified  in  ( c )( 2 )  and  ( d )( 2 ),  the  validity  period  shall  be  36  calendar  months  counted  from  the  end  of  the  month  when  the  Checks  were  taken.

ORO. CC. 145   Refresher  Training

a )   When  a  Cabin  Crew  member,  during  the  preceding  6  ( six )  months  within  the  validity period  of  the  last  relevant  Recurrent  Training  and  Checking :

1 )   has  not  performed  any  flying  duties,  he / she  shall,  before  being  reassigned  to  such  duties,  complete  Refresher  Training  and  Checking  for  each  aircraft  type  to  be  operated ;           or

2 )   has  not  performed  flying  duties  on  one  particular  aircraft  type,  he / she  shall,  before  being  reassigned  to  duties,  complete  on  that  aircraft  type :

( i )    Refresher  Training  and  Checking ;                                                          or

( ii )   2  ( two )  familiarization  flights  in  accordance  with  ORO. CC. 135.

b )   The  Refresher  Training  Programme  for  each  aircraft  type  shall  at  least  cover :

1 )   emergency  procedures ;

2 )   evacuation  procedures ;

3 )   operation  and  actual  opening,  by  each  Cabin  Crew  member,  of  each  type  or  variant  of  normal  and  emergency  exits  and  of  the  flight  crew  compartment  security  door  in  the  normal  and  emergency  modes ;

4 )   demonstration  of  the  operation  of  all  other  exits  including  the  flight   crew  compartment  windows ;

5 )   Location  and   handling  of  all  relevant  safety  and  emergency  equipment  installed  or  carried  on - board.

c )     The  Operator  may  elect  to  replace  Refresher  Training   by  Recurrent  Training  if  the reinstatement  of  the  Cabin  Crew  member’s  flying  duties  commences  within  the  validity  period  of  the  last  Recurrent  Training  and  Checking.                                                                                                                               If  that  validity  period  has  expired,  Refresher  Training  may  only  be  replaced  by  aircraft  type  specific  and  Operator  Conversion  Training  as  specified  in  ORO. CC. 125.

SECTION  2.    Additional  Requirements  for  Commercial  Air  Transport  Operations

ORO. CC. 200   Senior  Cabin  Crew  Member

a )   When  more  than  one  Cabin  Crew  member  is  required,  the  composition   of  the  Cabin  Crew  shall  include  a  Senior  Cabin  Crew  member  nominated  by  the  Operator ;

b )   The  Operator  shall  nominate  Cabin  Crew  members  to  the  position  of  Senior  Cabin  Crew  member  only  if  they :

1 )   have  at  least  1  ( one )  year  of  experience  as  operating  Cabin  Crew  member ;    and

2 )   have  successfully  completed  a  Senior  Cabin  Crew  Training  Course  and  the  associated  Check.

c )   The  Senior  Cabin  Crew  Training  Course  shall  cover  all  duties  and  responsibilities  of  Senior  Cabin  Crew  members  and  shall  include  at  least  the  following  elements :

1 )   pre - flight  briefing ;

2 )   cooperation  with  the  crew ;

3 )   review  of  Operator  requirements  and  legal  requirements ;

4 )   accident  and  incident  reporting ;

5 )   human  factors  and  Crew  Resource  Management  ( CRM ) ;       and

6 )   flight  and  duty  time  limitations  and  rest  requirements.

d )   The  Senior  Cabin  Crew  member  shall  be  responsible  to  the  Commander  for  the  conduct  and  coordination  of  normal  and  emergency  procedures  specified  in  the  Operations  Manual,  including  for  discontinuing  non - safety - related  duties  for  safety  or  security  purposes ;

e )   The  Operator  shall  establish  procedures  to  select  the  most  appropriately  qualified  Cabin  Crew  member  to  act  as  Senior  Cabin  Crew  member  if  the  nominated  Senior  Cabin  Crew  member  becomes  unable  to  operate.                                                                                                              Changes  to  these  procedures  shall  be  notified  to  the  GDCA .

ORO. CC. 201     Cabin  Crew  Instructor

a )   The  Operator  shall  nominate  Senior  Cabin  Crew  members  to  the  position  of  Cabin  Crew  Instructor  only  if  they :

1 )   have  at  least  3  ( tree )  year  of  experience  as  operating  Cabin  Crew  member ;

2 )   have  at  least  1  ( one )  year  of  experience  as  operating  Senior   Cabin  Crew  member ;    and

3 )   have  successfully  completed  a  Cabin  Crew  Instructor  Training  Course  and  the  associated  Check.

b )   The  Cabin  Crew  Instructor  Training  Course  shall  cover  all  duties  and  responsibilities  of   Cabin  Crew  Instructor’s   and  shall  include  at  least  the  following  elements :

1 )   pre - flight  briefing ;

2 )   cooperation  with  the  crew ;

3 )   review  of  Operator  requirements  and  legal  requirements ;

4 )   accident  and  incident  reporting ;

5 )   human  factors  and  crew  resource  management  ( CRM ) ;       and

6 )   flight  and  duty  time  limitations  and  rest  requirements.

c )   The  Cabin  Crew  Instructor  shall  be  responsible  to  the  Commander  for  the  conduct  and  coordination  of  normal  and  emergency  procedures  specified  in  the  Operations  Manual,  including  for  discontinuing  non - safety - related  duties  for  safety  or  security  purposes ;

d )   The  Operator  shall  establish  procedures  to  select  the  most  appropriately  qualified  Senior Cabin  Crew  member  to  act  as   Cabin  Crew  Instructor.                                                                                                                Changes  to  these  procedures  shall  be  notified  to  the  GDCA .

ORO. CC. 205   Reduction  of  the  number  of  Cabin  Crew  during  Ground  Operations

and  in  unforeseen  circumstances

a )   Whenever  any  passengers  are  on  board  an  aircraft,  the  minimum  number  of  Cabin  Crew  required  in  accordance  with  ORO. CC. 100  shall  be  present  in  the  passenger  compartment ;

b )   Subject  to  the  conditions  specified  in  ( c ),  this  number  may  be  reduced :

1 )   at  its  parking  station ;              or

2 )   during  normal  ground  operations  not  involving  refuelling / defuelling  when  the  aircraft  is

in  unforeseen  circumstances  if  the  number  of  passengers  carried  on  the  flight  is  reduced.  In  this  case  a  report  shall  be  submitted  to  the  GDCA  after  completion  of  the  flight.

c )   Conditions :

1 )   procedures  ensuring  that  an  equivalent  level  of  safety  is  achieved  with  the  reduced  number  of  Cabin  Crew,  in  particular  for  evacuation  of  passengers,  are  established  in  the  Operations  Manual ;

2 )   the  reduced  Cabin  Crew  includes   a  Senior  Cabin  Crew  member  as  specified  in  ORO. CC. 200 ;

3 )   at  least  one  Cabin  Crew  member  is  required  for  every  50,  or  fraction  of  50,  passengers  present  on  the  same  deck  of  the  aircraft ;

4 )   in  the  case  of  normal  ground  operations  with  aircraft  requiring  more  than  one  Cabin  Crew  member,  the  number  determined  in  accordance  with  ( c )( 3 )  shall  be  increased   to  include  one  Cabin  Crew  member  per  pair  of  floor  level  emergency  exits.

ORO. CC. 210   Additional  Conditions  for  Assignment  to  Duties

Cabin  Crew  members  shall  only  be  assigned  to  duties,  and  operate,  on  a  particular  aircraft  type  or  variant  if  they :

a )   Hold  a  valid  Attestation  ( Licence )  issued  in  accordance  with  Annex  V  ( Part - CC)  to  ARM - AIR  CREW  Regulation ;

b )   Are  qualified  on  the  type  or  variant  in  accordance  with  this  Subpart ;

c )   Comply  with  the  other  applicable  requirements  of  this  Subpart   and  Annex IV (Part - CAT);

d )   Wear  the  Operator’s  Cabin  Crew  uniform.

ORO. CC. 215   Training  and  Checking  Programs  and  related  Documentation

a )   Training  and  Checking  programmes  including  syllabi  required  by  this  Subpart  shall  be approved  by  the  GDCA  and  specified  in  the  Operations  Manual ;

b )   After  a  Cabin  Crew  member  has  successfully  completed  a  training  course  and  the associated  Check,  the  operator  shall :

1 )   update  the  Cabin  Crew  member’s  Training  Records  in  accordance  with  ORO. MLR. 115 ;

and

2 )   provide  him / her  with  a  list  showing  updated  validity  periods  as  relevant  to  the  aircraft  type(s)  and  variant(s)  on  which   the  Cabin  Crew  member  is  qualified  to  operate.

ORO. CC. 250   Operation  on  more  than  One  Aircraft  Type  or  Variant

a )   A  Cabin  Crew  member  shall  not  be  assigned  to  operate  on  more  than  3  ( three )  aircraft  types,  except  that,  with  the  approval  of  the  GDCA,  the  Cabin  Crew  member  may  be  assigned  to  operate  on  4  ( four )  aircraft  types  if  for  at  least  2  ( two )  of  the  types :

1 )   safety  and  emergency  equipment  and  type - specific  normal  and  emergency  procedures  are  similar ;                                                                                                           and

2 )   non - type - specific  normal  and  emergency  procedures  are  identical.

b )   For  the  purpose  of  ( a )  and  for  Cabin  Crew  Training  and  Qualifications,  the  Operator  shall  determine :

1 )   each  aircraft  as  a  type  or  a  variant  taking  into   account,  where  available,  the  relevant  data  established   in  accordance  with  CR - EC No  748 / 2012   for  the  relevant  aircraft   type  or  variant ;                                                                                               and

2 )   variants  of  an  aircraft  type  to  be  different  types  if  they  are  not  similar  in  the  following  aspects :

( i )    emergency  exit  operation ;

( ii )   location  and  type  of  portable  safety  and  emergency  equipment ;

( iii )  type - specific  emergency  procedures.

ORO. CC. 255   Single  Cabin  Crew  Member  Operations

a )   The  Operator  shall  select,  recruit,  train  and  check  the  proficiency  of  Cabin  Crew  members  to  be  assigned   to  single  cabin  crew  member  operations  according  to  criteria  appropriate  to  this  type  of  operation ;

b )   Cabin  Crew  members  who  have  no  previous  operating  experience  as  single  cabin  crew  member  shall  only  be  assigned   to  such  type  of  operation  after  they  have :

1 )   completed  training  as  required  in  ( c )  in  addition  to  other  applicable  Training  and  Checking  required  by  this  Subpart ;

2 )   Successfully  passed  the  Checks  verifying  their  proficiency  in  discharging  their  duties  and  responsibilities  in  accordance  with  the  procedures  specified  in  the  Operations  Manual ;                                                                                                                            and

3 )   undertaken  familiarization  flying  of  at  least  20  hours  and  15  sectors  on  the  relevant  aircraft  type  under  the  supervision  of  an  appropriately  experienced  cabin  crew  member.

c )   The  following  additional  Training  elements  shall  be  covered  with  particular  emphasis  to  reflect  single  cabin  crew  operations :

1 )   responsibility  to  the  Commander  for  the  conduct  of  normal  and  emergency  procedures ;

2 )   importance  of  coordination  and  communication  with  the  flight   crew,  in  particular  when  managing  unruly  or  disruptive  passengers ;

3 )   review  of  Operator  requirements  and  legal  requirements ;

4 )   documentation ;

5 )   accident  and  incident  reporting ;                                         and

6 )    flight  and  duty  time  limitations  and  rest  requirements.

SUBPART    FTL.    FLIGHT   &   DUTY   TIME   LIMITATIONS   &   REST

REQUIREMENTS    ОГРАНИЧЕНИЯ   ПОЛЕТНОГО   и   РАБОЧЕГО

ВРЕМЕНИ   и  ТРЕБОВАНИЯ   по   ВРЕМЕНИ   ОТДЫХА.

SECTION  I.    General

ORO. FTL. 005     General   &   Applicability -  Общие  Положения  и  Применимость

The  Armenia   Civil  Aviation  Authority  has  established  the  following  regulations  specifying  the  Limitations   applicable  to  the  Flight  Time  and   Flight  Duty  Periods  for  crew  members.  This  Subpart  is  applicable  to  Republic  of  Armenia  registered  airplanes,  as  well  as  foreign  registered  airplanes  operated   under  a  Republic  of  Armenia   Air  Operators  Certificate.                                                                             In   particular  :                                                                                                                                                                 ( a )   Commercial  Air Transport  Operations  with  aeroplanes ,  or  operations  operated   by  an  air  transport  undertaking  ;                                                                                                                                                     ( b )   Private  use  operations  of  turbo-jet  and  turbo-propeller  airplanes ;

( c )    Air  Taxi,  Emergency Medical  Service  and  Single  pilot  CAT  Operations  by  aeroplanes  and   recreational   aircraft   operations  to  related  national  exemptions  based  on  safety  risk  assessments ;

( d )    CAT  Operations  with  Helicopters  shall  comply  with  national  requirements.

This  Subpart  is  not  applicable  to  flying  schools,  to  single  pilot  operations .

Each  Operator  will  have  to  demonstrate  to  the  GD CA  of  RA,  using  operational  experience  and  taking  into  account  other   relevant  factors  such  as  current  scientific  knowledge,  that  its  request  for  a  variation  produces  an  equivalent  level  of   safety.

Such  variations  will  be  accompanied  with  suitable  mitigation  measures  where  appropriate.

Any  additional  provisions  that  are  applied  by  the  GD CA  of  RA  in  accordance  with  the  provisions  of   AIR - OPERATIONS,  ANNEX  III   Part - ORO,  Subpart  FTL   for  the  purpose  of  maintaining   safety.

ORO. FTL. 010     General  Principles  -   Общие  Принципы

The  prime  objective  of  a  Flight  and  Duty  Time  Limitation  scheme  shall  ensure  that  crew  members  are  adequately  rested  at  the  beginning  of  each  Flying  Duty  Period  ( FDP ),  and  whilst  flying  is  sufficiently  free  from  fatigue  so  that  they  can  operate  to  a  satisfactory  level  of  efficiency  and  safety  in  all  normal  and  abnormal  situations.  Aircraft  operators  are  expected  to  appreciate  the  relationship  between   the  frequency  and  pattern  of  scheduled  FDP’s  and  Rest  Periods  and  time  off,  and  give  due  consideration  to  the cumulative  effects  of  long  working  hours  interspersed  with  minimum  rest.  Factors  to  be  considered  when  planning  duty  periods  shall  include :

( 1 )   The  allocation  of  work  patterns,  which  avoid  such  undesirable  practices  as :

-    alternating   day / night  duties ;                                                                                                                                             -    the  positioning  of  crews  so  that  a  serious  disruption  of  established  sleep / work  patterns

occur  ;

-    the  scheduling  of  rest  periods,  especially  after  long   flights,  crossing  multiple  time  zones.                                       ( 2 )   Planning   days  off   and   notifying   crews   well   in   advance,                                                                            ( 3 )   Consultation  between  operators  and  crews  ( Airline  Pilot  Association )  to  agree  basic  roster  concepts,  which  ensure  adequate  rest  prior  to  flight  but,  within  that  constraint.

ORO. FTL. 100     Scope

This  Subpart  establishes  the  requirements  to  be  met  by  an  Operator  and  its  crew  members with  regard  to  Flight  and  Duty  Time  Limitations  and  Rest  Requirements  for  crew  members.

ORO. FTL. 105     Definitions

For  the  purpose  of  this  Subpart,  the  following  Definitions  shall  apply :

1 )   “ Acclimatized ”   means  a  state  in  which  a  crew  member’s  circadian  biological  clock  is synchronized  to  the  Time  Zone  where  the  crew  member  is.                                                                                        A  crew  member  is  considered  to  be  acclimatized  to  a  2 - hour  wide  time  zone  surrounding  the local  time  at  the  point  of  departure.  When  the  local  time  at  the  place  where  a  duty commences  differs  by  more  than  2  hours  from  the  local  time  at  the  place  where  the  next  duty  starts,  the  crew  member,  for  the  calculation  of  the  maximum  daily  flight  duty  period,  is considered  to  be  acclimatized  in  accordance  with  the  values  in  the   Table  1.

Table  1.

| Time difference ( h ) between reference time and local time where the crew member starts the next duty | Time elapsed since reporting at reference time |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | < 48 | 48 – 71:59 | 72 – 95:59 | 96 – 119:59 | ≥ 120 |
| < 4 | B | D | D | D | D |
| ≤ 6 | B | X | D | D | D |
| ≤ 9 | B | X | X | D | D |
| ≤ 12 | B | X | X | X | D |

“ B ”   means   acclimatized  to  the  local  time  of  the  Departure  Time  Zone ;

“ D ”   means   acclimatized  to  the  local  time  where  the  crew  member  starts  his / her  next  duty ;

and

“ X ”    means   that  a  crew  member  is  in  an  unknown  state  of  acclimatization ;

2 )    “ Reference  Time ”   means   the  local  time  at  the  reporting  point  situated  in  a  2 - hour wide  time  zone  band  around  the  local  time  where  a  crew  member  is  acclimatized ;

3 )    “ Accommodation ”    means,  for  the  purpose  of  standby  and  split  duty,  a  quiet  and comfortable  place  not  open  to  the  public  with  the  ability  to  control  light  and  temperature, equipped  with  adequate  furniture  that  provides  a  crew  member  with  the  possibility  to  sleep, with  enough  capacity  to  accommodate  all  crew  members  present  at  the  same  time  and  with access  to  food  and  drink ;

4 )    “ Suitable  Accommodation ”   means,  for  the  purpose  of  standby,  split  duty  and  rest,  a separate  room  for  each  crew  member  located  in  a  quiet  environment  and  equipped  with  a  bed, which  is  sufficiently  ventilated,  has  a  device  for  regulating  temperature  and  light  intensity,  and access  to  food  and  drink ;

5 )    “ Augmented  Flight  Crew ”   means  a  flight  crew  which  comprises  more  than  the  minimum  number  required  to  operate  the  aircraft,  allowing  each  flight  crew  member  to  leave the  assigned  post,  for  the  purpose  of  in - flight  rest,  and  to  be  replaced  by  another appropriately  qualified  flight  crew  member ;

6 )    “ Break ”   means  a  period  of  time  within  a  Flight  Duty  Period  ( FDP ),  shorter  than  a  rest  period,  counting  as  duty  and  during  which  a  crew  member  is  free  of  all  tasks ;

“ Cabin   Crew “   -  a  crew  member,  other  than  a  flight  crew  member,  assigned  to  duty  in

a  passenger  carrying   aircraft  during  flight  time ;

“ Commander “   means   the  pilot  in  Command  ;

“ Crew  member “   means  a  person  assigned  by  an  Operator  to  duty  on  an  aircraft  a

Flight  Duty  Period ;

“ Flight  Crew “  means    Pilot's,  Navigator's   and   Flight  Engineer's  ;

7 )    “ Delayed  Reporting ”   means  the  postponement  of  a  scheduled  FDP  by  the  Operator before  a  crew  member  has  left  the  place  of  rest ;

“ Dispatch  Crew “  / выпускающая  команда /   a  fully  qualified  and  current  flight / cabin  crew  member  authorized  to  carry  out  pre-flight  duties  as  defined  by  an  Operator ;

8 )    “ Disruptive  Schedule ”   means  a  crew  member’s  roster  which  disrupts  the  sleep opportunity  during  the  optimal  sleep  time  window  by  comprising  an  FDP  or  a  combination  of FDP’s  which  encroach,  start  or  finish  during  any  portion  of  the  day  or  of  the  night  where  a crew  member  is  acclimatized.  A  schedule  may  be  disruptive  due  to  early  starts,  late  finishes  or  night  duties ;

( a )   “ Early  Type ”   of  disruptive  schedule  means :

( i )   for  “ Early  Start ”  a  duty  period  starting  in  the  period  between  05 : 00  and  05 : 59  in  the  time  zone  to  which  a  crew  member  is  acclimatized ;                             and

( ii )  for  “ Late  Finish ”  a  duty  period  finishing  in  the  period  between  23 : 00  and  01 : 59  in the  time  zone  to  which  a  crew  member  is  acclimatized ;

( b )   “ Late  Type ”   of  disruptive  schedule  means :

( i )   for  “ Early  Start ”   a  duty  period  starting  in  the  period  between  05 : 00  and  06 : 59  in  the  time  zone  to  which  a  crew  member  is  acclimatized ;                               and

( ii )  for  “ Late  Finish ”   a  duty  period  finishing  in  the  period  between  00 : 00  and  01 : 59  in the  time  zone  to  which  a  crew  member  is  acclimatized ;

9 )    “ Night  Duty ”    means  a  duty  period  encroaching   any   portion   of   the   period   between  02 : 00  and  04 : 59  in  the  time  zone  to  which  the  crew  is  acclimatized ;

10 )   “ Duty ”   means  any  task  that  a  crew  member  performs  for  the  Operator,  including  flight duty,  administrative  work,  giving  or  receiving  training  and  checking,  positioning,  and  some elements  of  standby ;

11 )    “ Duty  Period ”   means  a  period  which  starts  when  a  crew  member  is  required  by  an Operator  to  report  for  or  to  commence  a  duty  and  ends  when  that  person  is  free  of  all  duties,  including  post - flight  duty ;

12 )    “ Flight  Duty  Period “  ( FDP )   means  a  period  that  commences  when  a  crew  member  is  required  to  report  for  duty,  which  includes  a  sector  or  a  series  of  sectors,  and  finishes when  the  aircraft  finally  comes  to  rest  and  the  engines  are  shut  down,  at  the  end  of  the  last sector  on  which  the  crew  member  acts  as  an  operating  crew  member.

FDP   is  any  time  during  which  a  person  operates  in  an  aircraft  as  a  member  of  its  crew ;

13 )    “ Flight  Time ”  / летное время /   means,  for  aeroplanes  and  touring  motor  gliders,  -  the  time  between  an aircraft  first  moving  for  the  purpose  of  Taking - off  from  Runway  until  it  comes  to  stop,  or  on  the  taxing  speed  leaving  the  Runway  for  the  purpose  to  finish  current  flight.

“Flight  Block  Time “ / полетное  время / :   The  time  between  an  airplane  first  moving       ( on  owner  power )  using  its  engines  (  from  its  parking  place  or  star - up  point )  for  the  purpose  of  Taking - off  until  it  comes  to  rest  on  the  designated   parking   position   and   all  engines  or  propellers  are  shut  down  / stopped  / ;

14 )    “ Home  Base ”  means  the  location,  assigned  by  the  Operator  to  the  crew  member,  from where  the  crew  member  normally  starts  and  ends  a  duty  period  or  a  series  of  duty  periods and  where,  under  normal  circumstances,  the  Operator  is  not  responsible  for  the  accommodation of  the  crew  member  concerned ;

15 )    “ Local  Day ”  means  a  24 - hour  period  commencing  at  00 : 00  local  time ;

16 )    “ Local  Night ”   means  a  period  of   8  hours  falling  between  22 : 00  and  08 : 00  local

time ;

17 )    “ Operating  Crew  Member ”   means  a  crew  member  carrying  out  duties  in  an  aircraft

during  a  sector ;

18 )    “ Positioning ”   means  the  transferring  of  a  non - operating  crew  member  from  one  place to  another,  at  the  behest  of  the  Operator,  excluding :

—   the  time  of  travel  from  a  private  place  of  rest  to  the  designated  reporting  place  at  home base  and  vice  versa ;                     and

—   the  time  for  local  transfer  from  a  place  of  rest  to  the  commencement  of  duty  and  vice versa ;

“ Dead  Heading  Crew “   means   the  practice  of  transferring  a  crews  during  flight  or  part  of  flight  out  of  duties  ( non - operating  crew ),  for  purpose  spent  to  duties   part  of   flight  or  next  sector  of  flight,  at  the  behest  of  the  Operator ;

“ Reporting Time “  means  the  time  at  which  a  crew  member  is  required  by  an

Operator  to  report  for  any  duty ;

19 )    “ Rest  Facility ”   means  a  bunk  or  seat  with  leg  and  foot  support  suitable  for  crew members’  sleeping  on  board  an  aircraft ;

20 )    “ Reserve ”   means  a  period  of  time  during  which  a  crew  member  is  required  by  the Operator  to  be  available  to  receive  an  assignment  for  an  FDP,  positioning  or  other  duty notified  at  least  10  hours  in  advance ;

21 )    “ Rest  Period ”   means  a  continuous,  uninterrupted  and  defined  period  of  time,  following duty  or  prior  to  duty,  during  which  a  crew  member  is  free  of  all  duties,  standby  and  reserve ;

22 )    “ Rotation ”   is  a  duty  or  a  series  of  duties,  including  at  least  one  flight  duty,  and  rest periods  out  of  home  base,  starting  at  home  base  and  ending  when  returning  to  home  base  for a  rest  period  where  the  Operator  is  no  longer  responsible  for  the  accommodation  of  the  crew member ;

“ Rostered  /  Planned   Duty “   means  a  duty  period,  or  series  of  duty  periods,  with

stipulated  start  and  finish  times,  notified  by  the  Operator

to  crews  in  advance ;

“ Rostering  Period “   means  a  number  of  consecutive  weeks,  usually  4,  but  defined  by

the  Operator ;

“ Scheduled   Duty “  means  the  allocation  of  a  specific  flight  or  flights  or  other  duties

to  a  crew  member  within  the  pre- notified rostered / planned  series

of  duty  periods ;

“ Split  Duty “   means  a  flying  duty  period,  which  consists  of  two  or  more  sectors,

separated   by  less  than  a  minimum  rest  period ;

23 )    “ Single  Day  Free  Of  Duty ”   means,  for  the  purpose  of  complying  with  the  provisions of  Council  Directive  2000 / 79 / EC,  a  time  free  of  all  duties  and  standby  consisting  of  one  day  and  two  local  nights,  which  is  notified  in  advance.                                                                                           A  rest  period  may  be  included  as part  of  the  single  day  free  of  duty ;

24 )    “ Sector ” ( Leg )  means  the  segment  of  an  FDP  between  an  aircraft  first  moving  for  the purpose  of  Taking - off  until  it  comes  to  rest  after  landing  on  the  designated  parking  position ;

25 )    “ Standby ”   means  a  pre - notified  and  defined  period  of  time  during  which  a  crew member  is  required  by  the  Operator  to  be  available  to  receive  an  assignment  for  a  flight, positioning  or  other  duty  without  an  intervening  rest  period ;

26 )    “ Airport  Standby ”   means  a  standby  performed  at  the  airport ;

27 )    “ Other  Standby ”   means  a  standby  either  at  home  or  in  a  suitable  accommodation ;

“ Standby  Duty “   means  a  defined  period  of  time  during  which  a  crew  member  is

required  by  the  Operator  to  be  available  to  receive  an  assignment  for  a  Flight,

Positioning   or  other  Duty   without   an  intervening  Rest  Period.  However,  it  shall

not  include  any  time  during  which  an  Operator  requires  a  crew  member  to  be

contactable  for  the  purpose  of  giving  notification  of  a  duty,  which  is  due  to  start

10   hours  or  more  ahead ;

“ Travelling “   means   all  time  spent  by  a  crew  member  transiting  between  the  place  of  rest,  and  the  place  of  reporting   for   duty.

Travelling   Time  is  defined  as :

— time  from  home  to  a  designated  reporting  place  and  vice  versa ;

— time  for  local  transfer  from  a  place  of  rest  to  the  commencement  of  duty  and  vice  versa ;

“ Time  Zone  Difference “   means  defined  sector  of  the  flight,  which  covers  more  of                ( 3 )  three  Time  Zones.  An  Operator  will  ensure  that  effects  on  crew  members  of  time  zone  differences  will  be compensated  by  additional  rest                                   -   это  сектор  полета,  который  охватывает  диапазон  более  ( 3 )  трех  временных  поясов.  Эксплуатант  обязан  компенсировать  эффект  разницы  во  времени  предоставлением  дополнительного  отдыха  экипажу ;

“ Ultra  Long  Range  Operation “ ( ULR ) -  augmented   long  range  operations  involving

any  sector  between  a  specific  city  pair  in  which  the  planned  flight

time  exceeds  16   hours,  taking  into  account  mean  wind  conditions

and  seasonal  changes ;

“ Week “   means   period  of   7  ( seven )  consecutive  days  starting  at  any  set  time  and

on  set  day  as  specified  and  stated  by  the  Operator ;

28 )    “ Window  of  Circadian  Low “ ( WOCL )   means  the  period  between  02 : 00  and  05 : 59 hours  in  the  time  zone  to  which  a  crew  member  is  acclimatized.

Циркадный  Период  -  это  время  между  02 : 00  и  05 : 59  той  временной  зоны,  где

экипаж  находится  на  аклиматизации ;

ORO. FTL. 109     Responsibilities  -  General

A  crew  member  shall  not  fly,  and  an  Operator  shall  not  require  that  crew  member  to  fly,  if  either  has  reason  to  believe  that  he / she  is  suffering,  or  is  likely  to  suffer  while  flying,  from  such  fatigue  as  may  endanger  the  safety  of  the  aircraft  or  of  its  occupants  -  члены  экипажа  не  должны выполнять  полеты,  а  эксплуатант  не  может  требовать  от  члена  экипажа  выполнять  полеты, если  по  любой  из  причин  он / она  уже  подвержен  или  возможно  будут  подвержены  во  время  полета  такой  усталости, которая  может  подвергнуть  опасности  как  сам  самолет,  так  и  всех  тех,  кто  находится  внутри  этого  самолета ;

Comprehensive  guidance  and  instructions  shall  be  included  in  the  Operations  Manual  for  the  benefit  of  all  crew  members  and  the  staff  concerned  with  the  preparation   and   day  to  day  management  of  rostering  and  scheduling   -   всеобъемлющие   и  полные  требования  и  инструкции   должны  быть  внесены  в  Руководство  по  Производству  Полетов  с  целью  ознакомления  с  ним  членов  экипажей  и  персонала  причастного  к  ежедневному  составлению  графика  полетов  и  планирования ;

Although  Operators  must  plan  their  schemes  in  accordance  with  the  requirements,  it  is  recognized  that  the  standard  provisions  will  not  necessarily  satisfy  every  type  of  operation  -  несмотря  на  сказанное  выше,  эксплуатант  должен  выполнять  планирование  согласно  своей  схемы,  в  соответствии  с  установленными требованиями, он  должен  признавать  то,  что,  стандартные положения  не  обязательно  удовлетворяют  любому  виду  деятельности  эксплуатанта ;

In  these  circumstances  Operators  may  apply  for  a  change  to  the  standard  provisions  -   в  данных  обстоятельствах  эксплуатант  может  обратиться  с  предложением  по  изменению  стандартных  положений ;

Consideration  will  only  be  given  where  an  Operator  can  show  that   any  proposal  will  ensure  a  better  or  equivalent  level  of  protection  against  fatigue  than  the  basic  requirements  -  к  рассмотрению  можно  приступать  в  случае,  когда  эксплуатант  сможет  продемонстри- ровать  какое-либо  предложение  по  обеспечению  лучшего  или  соответствующего  уровня  защиты  от  усталости, чем  это  предписано  в  основных  положениях.

ORO. FTL. 110     Operator   Responsibilities

An   Operator   shall :

a )    to  prepare  and  publish  duty  rosters  sufficiently  in  advance  to  provide  the  opportunity  for  crew  members to  plan  adequate  rest   -  подготовить  и  опубликовать  наряд  на  полеты,  как  можно  в  достаточной  мере  на   большее   количество   дней,  что  даст    возможности  экипажам  планировать  соответствующий  предполетный   отдых ;

b )    ensure  that  flight  duty  periods  are  planned  in  a  way  that  enables  crew  members  to  remain  sufficiently  free  from  fatigue  so  that  they  can  operate  to  a  satisfactory  level  of  safety under  all  circumstances    -  должен  обеспечивать  планирование  периода  летной  деятельнос-эти  таким  образом,  чтобы  позволять  членам  экипажа  оставаться  в  достаточной  мере  свободными  от  усталости, что  позволяет  поддерживать  достаточный  уровень  Безопасности  Полетов  в  любых  обстоятельствах ;

Flights  are  planned  to  be  completed  within  the  allowable  Flight  Duty  Period  taking  into  account  the  time  necessary  for  pre-flight  duties,  the  flight,  turn-around    and   post-flight  duties   times  -  полеты  планируются  так, чтобы  можно  было  завершить  их  в  допустимое  Полетное  Рабочее  Время  так,  чтобы  эти  полеты  включали  предполетную  подготовку, полетное  время,  время  оборачиваемости  ВС / стоянки  и  послеполетное  время ;

Shall  plan  local  days  free  of  duty  and  notify  crew  members  in  advance  -  должен  планировать  и  предоставлять   заблаговременно   информацию  о  планируемом   отдыхе  по  месту  постоянного  проживания  членов  экипажа ;

c )    specify  reporting  times  that  allow  sufficient  time  for  ground  duties ;

d )    take  into  account  the  relationship  between  the  frequency  and  pattern  of  flight  duty  periods  and  rest  periods  and  give  consideration  to  the  cumulative  effects  of  undertaking  long duty  hours  combined  with  minimum  rest  periods    -   должен  ожидать  оценки  за  соотно-       шение  между  частотой  и   периодами  Летной  Деятельности  и  периодами  Отдыха  и  надлежащим  образом  рассматривать  эффект  накопления  усталости  от  имевшегося  долгого  периода  летной  деятельности,  распределенного  между  минимальными   временами  отдыха .

Shall  ensure  that  rest  periods  provide  sufficient  time  to  enable  crew  to  overcome  the  effects  of  the  previous  duties  and  to  be  well  rested  by  the  start  of  the  following  flight  duty  period   -   должен  гарантировать,  что  период  отдыха  обеспечивает  достаточное  время,  которое  дает  возможность  экипажу  преодолеть  эффект  усталости  от  предыдущей  Летной   Деятельности  и  дает  достаточный  отдых  ко  времени  следующего  периода  Летной  Деятельности ;

e )    allocate  duty  patterns  which  avoid  practices  that  cause  a  serious  disruption  of  an established  sleep  /  work  pattern,  such  as  alternating  day  /  night  duties   -  должен  распреде-         лять  период  Летной  Деятельности  так,  чтобы  избегать  такой  практики,  как  варианта  деятельности  день / ночь  или  практики  перемещения  членов  экипажей,  что  приводит  к  серьезным  нарушениям  привычных  ритмов  сон / работа ;

f )    comply  with  the  provisions  concerning  disruptive  schedules  in  accordance  with  ARO. OPS. 230 ;

g )    provide  rest  periods  of  sufficient  time  to  enable  crew  members  to  overcome  the  effects  of  the  previous  duties  and  to  be  rested  by  the  start  of  the  following  flight  duty  period ;

h )    plan  recurrent  extended  recovery  rest  periods  and  notify  crew  members  sufficiently  in advance ;

i )    plan  flight  duties  in  order  to  be  completed  within  the  allowable  flight  duty  period  taking into  account  the  time  necessary  for  pre - flight  duties,  the  sector  and  turnaround  times.

Shall  establish  minimum  periods  of  notification  of  Duty  for  operating  crews  ( for  at  least  3  day\s ),  or  where  this  not  practicable  due  to  the  nature  of  the  operation,  shall  establish  in  advance  minimum  periods  of  notification  of  days  off,  during  which  a  crew  member  will  not  be  required  for  any  duties    -  должен  установить  минимальное  время  уведомления  для  планирующихся  членов  экипажей ( как  минимум  за  3  дня ),  или  если  это  практически  не  возможно  по  роду  деятельности,  должен  установить  минимальное  время  уведомления о  выходных днях,  в  течении  которых  эти  члены  экипажей  не  могут  быть  запланированы  для  работы ;

j )    training  of  rostering  staff  shall  include  guidance  on  the  effects  of  disturbing  circadian  rhythms  and  sleep  deprivation  -  тренировка  лиц,  ответственных  за  планирование  членов  экипажей,  должна  включать  руководство  по  последствиям  и  расстройствам  связанных  с  циркадными  ритмами  и   влиянию  проблем  с  засыпанием  и  с  недосыпанием ;

k )    change  a  schedule  and  / or  crew  arrangements  if  the  actual  operation  exceeds  the  Maximum  Flight  Duty  Period  on  more  than  33 %  of  the  flight  duties  in  that  schedule  during  a  scheduled  seasonal  period ;

l )      away  from  Base  the  Operator  shall  provide  for  crew  members  delivery  to  airport,  and  from  airport  to  place  of  suitable  accommodation  for  pre-flight  rest   -   при  нахождении  вне  места  базирования эксплуатант  обязан  организовать  доставку  членов экипажа в  аэропорт и из аэропорта  к месту    проживания,  в  месте  их  временного  базирования  с  целью  получения  полноценного  отдыха ;

m )    duration  of  the  annual  payment  vacation  must  be  not  less  than  28  working  days.  This  vacation  may  be  available partially  in  2  ( two ) part's   -  продолжительность  ежегодного  оплачиваемого  отпуска,  должна  быть  не  менее  28   рабочих  дней.  Допускается  его  разделение  на  две  част и;

n )     when  an  Operator  employs  a  crew  member  on  an  irregular  basis,  then  that  employer   shall  ensure  that  the  crew  member  satisfies  the  provision  of  the  approved  scheme.  Furthermore,  Operators  shall  satisfy  themselves  that  crew  members,  who  undertake  other  employment,  if  allowed  by  the  Operator,  still  have  the  opportunity  to  enjoy  adequate  pre-flight  rest  -  когда  эксплуатает  нанимает  члена  экипажа с  целью временного  выполнения  своих  обязанностей  и  за  это  время   работодатель  убеждается,  что  этот  член  экипажа  удовлетворяет  и  обеспечивает  предъявляемые  требования.  Более  того,  эксплуатант    удовлетворит,  то  что,  член  экипажа,  который  переходит  к  другому  работодателю,  если  позволяет  эксплуатант,  по - прежнему может  иметь  возможность  получения  адекватного  предполетного  отдыха ;

o )    when  the  crew  rest  has  been  in  aircraft  on  ground,  in  this  case,  crew  must  be  carry  out  procedure  for  aircraft  ventilation  and  adjusting  the  comfortable  temperature, using  external  or  internal  source  for  this  purpose.  This  crew  rest  must  be  carry  out  without  of  passenger  -  если отдых имеет место в самолете на земле, экипаж должен осуществлять соответствующий контроль  температуры  и  вентиляции  внутри  воздушного  судна,  посредством  использования  наземной или  бортовой  силовой  установки.  Пассажиры  не  должны  находиться  на  борту.

ORO. FTL. 115     Crew   Member   Responsibilities

Crew   members   shall :

a )    Comply  with  point  CAT. GEN. MPA. 100 ( b )  of  Annex  IV  ( Part - CAT ) ;

b )    Make  optimum  use  of  the  opportunities  and  facilities  for  rest  provided  and  plan  and  use their  rest  periods  properly.

c )    Responsibility  for  preventing  the  onset  of  fatigue  cannot  rest  on   the  operator  alone  -   ответственность  за  предотвращение  накопления  усталости  из-за  отсутствия  отдыха  не  ложится  на  эксплуатанта  в  одиночку  ;

d )    Crew  members  shall  ensure  that  they  are  not  in  breach  of  the  operator’s  scheme   -   члены  экипажа  гарантированы,  что  они  не  являются  брешью  в  схеме  эксплуатанта ;

e )    They  shall  make  optimum  use  of  the  opportunities  and  facilities  for  rest  provided,  and  plan  and  use  their  rest  periods  properly   -   они  должны  оптимально  использовать  благопри- ятные  возможности  и  условия  для  отдыха,  а  также  планировать  и  использовать  период  отдыха  должным  образом ;

f )    Crew  members  are  reminded   that  they  are  not  entitled  to  act  as  a  member  of  the  crew  of  an  aircraft  if  they  know  or  suspect  that  their  physical  or  mental  condition  renders  them  temporarily  unfit  so  to  act  -   членам  экипажа  напоминают  о  том ,  что  они  не  уполномочены  для  деятельности, как  член  экипажа,  если  они  знают  или  подозревают,  что  их  физическое  или  психологическое  состояние  не  позволяет  им  временно  исполнять  эти  обязанности ;

g )    A  crew  member  shall  not  operate  an  airplane  if   he / she  knows  that  he / she  is  suffering  from  or  is  likely  to  suffer  from  fatigue  or  feels  unfit,  to  the  extent  that  the  flight  may  be endangered   -  член  экипажа  не  может  осуществлять  деятельность  в  самолете,  если  он / она  знают,  что  он / она  находится  под  воздействием  усталости  или  вероятно,  что  находится  под  воздействием  усталости  или  ощущает  неспособность  продолжать  полет,  так  как  продолжение  полета  может  быть  небезопасно.

ORO. FTL. 116     Pilot - in - Command   Responsibilities

a  )   A  Commander   mast  avoid  operation  an  airplane  ( not  operate  an  airplane ) if  he / she  knows  that  crew  member  ( member's )   is  suffering  from  or  is  likely  to  suffer  from  fatigue  or  feels  unfit,  to  the  extent  that  the  flight  may  be  endangered   -   Командир  Воздушного  Судна  обязан  избегать  осуществлять  деятельность  в  самолете,  если  он / она  знает,  что  член   ( члены )  экипажа  находится  под  воздействием  усталости  или  вероятно, что  находится  под  воздействием  усталости  или  ощущает  неспособность  продолжать  полет,  так  как  продолжение  полета  может  быть  небезопасно ;

b )  In  extraordinary  cases,  when  flight  is  operated  with  too  late  notification  received  by  the  Operator  within  a  very  short  time  period  not  providing  the  crew  with  proper  accommodation  and  without  representative ,  the  Commander  will  take  the  authority  for  crew  accommodation  -   в  экстренных  случаях,  если  полет  выполнен  с  уведомлением,  полученным  эксплуатантом   за  очень  короткий  период, который  не  позволяет  обеспечить  надлежащего  размещения  экипажа  и  при  условии  отсутствия  представителя  в  данном  месте,  то  полномочия  по  размещению  экипажа  возлагаются  на  Командира  ВС.

ORO. FTL. 120     Fatigue   Risk   Management   ( FRM )

a )    When  FRM  is  required  by  this  Subpart  or  an  applicable   certification  specification,  the Operator  shall  establish,  implement  and  maintain  a  FRM  as  an  integral  part  of  its  management system.  The  FRM  shall  ensure  compliance  with  the  essential  requirements  in  points  7. f, 7. g  and   8. f  of  Annex  IV  to  CR - EC  No  216 / 2008.                                                                                            The  FRM  shall  be  described  in  the Operations  Manual ;

b )    The  FRM  established,  implemented  and  maintained  shall  provide  for  continuous improvement  to  the  overall  performance  of  the  FRM  and  shall  include :

1 )   a  description  of  the  philosophy  and  principles  of  the  Operator   with  regard  to  FRM, referred  to  as  the  FRM  policy ;

2 )   documentation  of  the  FRM  processes,  including  a  process  for  making  personnel  aware  of their  responsibilities  and  the  procedure  for  amending  this  documentation ;

3 )   scientific  principles  and   knowledge ;

4 )   a  hazard  identification  and  risk  assessment  process  that  allows  managing  the  operational risk(s)  of  the  Operator  arising  from  crew  member  fatigue  on  a  continuous  basis ;

5 )   a  risk  mitigation  process  that  provides  for  remedial  actions  to  be  implemented  promptly, which  are  necessary  to  effectively mitigate  the  Operator’s  risk(s)  arising  from  crew  member  fatigue  and  for  continuous  monitoring  and  regular  assessment  of  the  mitigation  of  fatigue  risks  achieved  by  such  actions ;

6 )   FRM  safety  assurance  processes ;

7 )   FRM  promotion  processes.

c )    The  FRM  shall  correspond  to  the  flight  time  specification  scheme,  the  size  of  the  Operator  and  the  nature  and  complexity  of  its  activities,  taking  into  account  the  hazards  and associated  risks  inherent  in  those  activities  and  the  applicable  flight  time  specification  scheme ;

d )    The  Operator  shall  take  mitigating  actions  when  the  FRM  safety  assurance  process  shows that  the  required  safety  performance  is  not  maintained.

ORO. FTL. 125     Flight   Time   Specification   Schemes

a )    Operators  shall  establish,  implement  and  maintain  flight  time  specification  schemes  that  are  appropriate  for  the  type(s)  of  operation  performed  and  that  comply  with  Regulation  ( EC ) No  216 / 2008  and  this  Subpart ;

b )    Before  being  implemented,  flight  time  specification  schemes,  including  any  related  FRM where  required,  shall  be  approved  by  the  Competent  Authority ;

c )    To  demonstrate  compliance  with  CR - EC  No  216 / 2008  and  this  Subpart,  the  Operator   shall  apply  the  applicable  certification  specifications  adopted  by  the  GDCA  of  RA. Alternatively,  if  the  Operator  wants  to  deviate  from  those  certification  specifications  in accordance  with  Article  22 ( 2 )  of  CR - EC  No  216 / 2008,  it  shall  provide  the  Competent  Authority  with  a  full  description  of  the  intended  deviation  prior  to  implementing  it. The  description  shall  include  any  revisions  to  manuals  or  procedures  that  may  be  relevant,  as well  as  an  assessment  demonstrating  that  the  requirements  of  CR - EC  No  216 / 2008 and  of  this  Subpart  are  met ;

d )    For  the  purpose  of  point  ARO. OPS. 235 ( d ),  within  2  years  of  the  implementation  of  a deviation  or  derogation,  the  Operator  shall  collect  data  concerning  the  granted  deviation  or derogation  and  analyze  that  data  using  scientific  principles  with  a  view  to  assessing  the  effects of  the  deviation  or  derogation  on  aircrew  fatigue.  Such  analysis  shall  be  provided  in  the  Form of  a  report  to  the  Competent  Authority.

INTENTIONALLY            LEFT            BLANK

SECTION   II.     Commercial  Air  Transport  (  CAT )  Operators

ORO. FTL. 200     Home   Base

An  Operator  shall  assign  a  home  base  to  each  crew  member.

ORO. FTL. 205     Maximum  Daily  Flight   Duty   Period  ( FDP )

Ежедневное  Максимальное  Полетное  Рабочее   Время

a )    The  Operator  shall :

1 )   define  reporting  times  appropriate  to  each  individual  operation,   that  realistically  reflects  the  time  for  safety  related  ground  duties  as  approved  by  the  GD CA  of  RA,  but  not  less  than  1  hour  before  scheduled  departure  time , depending  on  aircraft  type  and  flight  log,  taking  into  account  ORO. FTL. 110 ( c )   -   эксплуатант  должен  определить  Время  явки,  одобренное  со  сторопы  ГУГА РА,  которое  реалистично  отражает  время  пред- полетной  подготовки, необходимое  из  условий  обеспечения  безопасности  полета  и  которое  не  может  быть менее  1 ч.  до  запланированного  времени  вылета  в  зависимо-с ти  от  типа  ВС  и  задания  на  полет  принимая во внимание  ORO. FTL. 110 ( c ) ;

2 )   establish  procedures  specifying  how  the  Commander  shall,  in  case  of  special  circumstances  which  could  lead  to  severe  fatigue,  and  after  consultation  with  the  crew  members  concerned,  reduce  the  actual  FDP  and / or  increase  the  Rest  Period  in  order  to  eliminate  any  detrimental  effect  on  flight  safety.

b )   Flight Crew  Duty  Time  terminates  after  engine  shut  down,  but  not  early  than  30 minutes  after  engine  shut  down  time.  Полетное  рабочее  время  членов  экипажа  заканчивается  не  ранее  чем  через  30  мин.  после  выключения  двигателей ;

c )   The  maximum  basic  daily  FDT  is  specified  as  13  hours  -   Максимальная  продолжительность  суточного  полетного  рабочего  времени  установлена  в  13 часов ;

d )   These  13  hours  will  be  reduced  for  each  sector  from  the  3  ( third )  sector  onwards  providing  a  maximum  total  reduction  does  not  exceed   3  ( tree )  hours  -  указанное  ограничение  в  13 ч.  должно  быть  уменьшено  для  каждого  последующего  сектора,  начиная  с  3-го  сектора,  но  с  условием  максимального  уменьшения  общего  рабочего  времени  не  более  3  часов ;

e )    Basic  maximum  daily  FDP.

FDT  limitations  established  in  this  subparagraph   taking  into  account  the  Reporting  Times  and  planed  sectors  shall  not  be  exceed  values  specified  in  Table  2  below :  -  Установленная  в  данном   пункте   максимальная   продолжительность  суточного  полетного  рабочего  времени  с  учетом  времени  начала  работы  и  количества  посадок  не  должна  превышать  значений,  приведенных   в   Таблице  2.      Start   of   FDP   at  reference  time :

1 )   the  maximum  daily  FDP  without  the  use  of  extensions  for  acclimatized  crew  members  shall  be  in  accordance  with  the  following  Table :

Table   2.         Maximum   Daily   FDP   —   Acclimatized  Crew  Members

| Start of FDP at reference time | Sector ( s ) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 – 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 06 : 00 - 13 : 29 | 13 : 00 | 12 : 30 | 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 |
| 13 : 30 - 13 : 59 | 12 : 45 | 12 : 15 | 11 : 45 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 |
| 14 : 00 - 14 : 29 | 12 : 30 | 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 |
| 14 : 30 - 14 : 59 | 12 : 15 | 11 : 45 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 | 09 : 00 |
| 15 : 00 - 15 : 29 | 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 | 09 : 00 |
| 15 : 30 - 15 : 59 | 11 : 45 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 | 09 : 00 | 09 : 00 |
| 16 : 00 - 16 : 29 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 | 09 : 00 | 09 : 00 |
| 16 : 30 - 16 : 59 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 | 09 : 00 | 09 : 00 | 09 : 00 |
| 17 : 00 - 04 : 59 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 | 09 : 00 | 09 : 00 | 09 : 00 |
| 05 : 00 - 05 : 14 | 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 | 09 : 00 |
| 05 : 15 - 05 : 29 | 12 : 15 | 11 : 45 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 | 09 : 00 |
| 05 : 30 - 05 : 44 | 12 : 30 | 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 |
| 05 : 45 - 05 : 59 | 12 : 45 | 12 : 15 | 11 : 45 | 11 : 15 | 10 : 45 | 10 : 15 | 09 : 45 | 09 : 15 | 09 : 00 |

Примечание        For  Training  /  Base  Flight  the  following  limitations  are  specified :

-  При  выполнении  учебных / тренировочных  полетов  в  зоне  аэродрома

устанавливаются  следующие  ограничения :

1.  Flight  Time  shall  not  exceed  6 hrs   -  Полетное  время  не  более  6 ч.

2.  Flight  Duty  Time  shall  not  exceed 10 hrs  -  Полетное  рабочее  время  не  более  10 ч.

3.  Number  of  performed  Approaches  shall  not  exceed 25  - Количество  заходов  на  посадку  не   более  25.

2)   the  maximum  daily  FDP  when  crew  members  are  in  an  unknown  state  of  acclimatization shall  be  in  accordance  with  the  following  Table :

Table  3.    Crew  Members  in  an  unknown   State  of  acclimatization

| Maximum Daily FDP according to sectors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 – 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 | 09 : 00 | 09 : 00 |

3 )   the  maximum  daily  FDP  when  crew  members  are  in  an  unknown  state  of  acclimatization and  the  Operator  has  implemented  a  FRM,  shall  be  in  accordance  with  the  following  Table :

Table  4.    Crew  Members  in  an  unknown  State  of  acclimatization  under  FRM

The  values  in  the  following  Table  may  apply  provided  the  Operator’s  FRM  continuously monitors  that  the  required  safety  performance  is  maintained.

| Maximum Daily FDP according to sectors |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 – 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 12 : 00 | 11 : 30 | 11 : 00 | 10 : 30 | 10 : 00 | 09 : 30 | 09 : 00 |

f )    Maximum  daily  FDP  for  acclimatized  crew  members  with  the  use  of  extensions  without  in - flight  rest.

1 )   the  maximum  daily  FDP  may  be  extended  by  up  to  1 hour  not  more  than  twice  in  any                  7  consecutive  days.   In  that  case :

( i )   the  minimum  pre-flight  and  post-flight  rest  periods  shall  be  increased  by  2  hours ;   or

( ii )  the  post - flight  rest  period  shall  be  increased  by  4  hours.

2 )   when  extensions  are  used  for  consecutive  FDP’s,  the  additional  pre - and  post - flight  rest between  the  two  extended  FDP’s  required  under  Subparagraph  1  shall  be  provided consecutively ;

3 )   the  use  of  the  extension  shall  be  planned  in  advance,  and  shall  be  limited  to  a  maximum of :

( i )    5  sectors  when  the  WOCL  is  not  encroached ;                                       or

( ii )   4  sectors,  when  the  WOCL  is  encroached  by  2  hours  or  less ;          or

( iii )  2  sectors,  when  the  WOCL  is  encroached  by  more  than  2  hours.

4 )   extension  of  the  maximum  basic  daily  FDP  without  in-flight  rest  shall  not  be  combined with  extensions  due  to  in - flight  rest  or  split  duty  in  the  same  duty  period ;

5 )   flight  time  specification  schemes  shall  specify  the  limits  for  extensions  of  the  maximum basic  daily  FDP  in  accordance  with  the  certification  specifications  applicable  to  the  type  of  operation,  taking  into  account :

( i )   the  number  of  sectors  flown ;                       and

( ii )  WOCL  encroachment.

g )    Maximum  daily  FDP  with  the  use  of  extensions  due  to  in - flight  rest  -  Максимальное   Полетное   Рабочее   Временя   в   случае   отдыха   в   полете.

Flight  time  specification  schemes  shall  specify  the  conditions  for  extensions  of  the  maximum basic  daily  FDP  with  in - flight  rest  in  accordance  with  the  certification  specifications applicable  to  the  type  of  operation,  taking  into  account :

1 )   the  number  of  sectors  flown ;

2 )   the  minimum  in - flight  rest  allocated  to  each  crew  member ;

3 )   the  type  of  in - flight  rest  facilities ;                                                  and

4 )   the  augmentation  of  the  basic  flight  crew.

The  GD CA  of  RA  approved  the  requirements  related  the  augmentation  of  basic  Flight  Crew  for  the  purpose  of  extending  the  Flight  Duty  Period  up  to  16  hours,   providing  this  approval  covers  only  Flight  Duty  Period  involving  not  more  that  2  ( two )  sectors / landings   -   ГУГА  РА  разрешает  применение  продленного  рабочего  времени  до  16  часов,  только  при  условии  выполнения  полета  в  увеличенном  от  базового  составе  экипаже  и  при  условии,  что  за  один  период  Полетного  Рабочего  Времени  производится  не  более  2 посадок.

h )    Unforeseen  circumstances  in  flight  operations  -  Commanders  discretion  -  непредвиденные  обстоятельства  во  время  фактической  летной  деятельности  -  Решение  Командира  ВС.

Any  such  modifications  must  be  acceptable  to  the  Commander   after  conformation  with  all  other  crew  members.

1 )   the  conditions  to  modify  the  limits  on  flight  duty,  duty  and  rest  periods  by  the Commander  in  the  case  of  unforeseen  circumstances  in  flight  operations,  which  start  at  or  after  the  reporting  time,  shall  comply  with  the  following :

( i )    the  maximum  daily  FDP  which  results  after  applying  points  ( b )  and  ( e )  of  point  ORO. FTL. 205  or  point  ORO. FTL. 220  may  not  be  increased  by  more  than  2  hours  unless  the  flight  crew  has  been  augmented,  in  which  case  the  maximum  flight  duty  period  may  be  increased  by  not  more  than  3  hours.  An  extension  of  3  hours  is  the  absolute  maximum  permitted,  except  in  the  cases  of  emergency.  Commander’s  discretion  to  extend   a  FDP  may  only  be  exercised  once  the  FDP  has  commenced ;

( ii )   if  on  the  final  sector  within  an  FDP  the  allowed  increase  is  exceeded  because  of unforeseen  circumstances  after  Take-off,  the  flight  may  continue  to  the  planned  destination  or  alternate  aerodrome ;                                  and

( iii )  the  rest  period  following  the  FDP  may  be  reduced  but  can  never  be  less  than  10  hours.

2 )   in  case  of  unforeseen  circumstances  which  could  lead  to  severe  fatigue,  the  Commander shall  reduce  the  actual  Flight  Duty  Period  and  /  or  increase  the  Rest  Period  in  order  to eliminate  any  detrimental  effect  on  flight  safety.

3 )   the  Commander  shall  consult  all  crew  members  on  their  alertness  levels  before  deciding the  modifications  under   Subparagraphs  1  and  2 ;

4 )   the  Commander  shall  submit  a  Captain   Report   Form  acceptable  to  the  GDCA  of  RA,  to  the  Operator  when  an  FDP  is  increased  or  a  Rest  Period  is  reduced  at  his  or  her  discretion ;

5 )   where  the  increase  of  an  FDP  or  reduction  of  a  rest  period  exceeds  1 ( one ) hour,  a  copy  of  the  report,  to  which  the  Operator  shall  add  its  comments,  shall  be  sent  by  the  Operator  to  the  Competent  Authority  not  later  than  28  days  after  the  event,  or  within  14  days  of  the  aircraft’s  return  to  base ;

6 )   the  Operator  shall  implement  a  non - punitive  process  for  the  use  of  the  discretion described  under  this  provision  and  shall  describe  it  in  the  Operations  Manual ;

7 )   an  Operator  shall   not  plan   a  FDP  on  the  basis  of  the  use  of  Commander’s   discretion.

i )    Unforeseen  circumstances  in  flight  operations  —  delayed  reporting.

The  Operator  shall  establish  procedures,  in  the  Operations  Manual,  for  delayed  reporting  in  the event  of  unforeseen  circumstances,  in  accordance  with  the  certification  specifications  applicable to  the  type  of  operation.

ORO. FTL. 210     Flight  Block  Times  and  Duty  Time  Limitations

Ограничения  Полетного  и  Рабочего  Времени

No  person  shall  act  as  a  member  of  the  flight  crew  of  the  aircraft  registered  in  the  Republic  of  Armenia,  if  at  the  beginning  of  the  flight  he/ she  has  exceeded  all  his / her  previous  Flight   and   Duty  Times   -  ни  одно  лицо,  в  качестве  члена  летного  экипажа  воздушного   судна,  зарегистрированного  в  Республике  Армения , не  может  начинать  полет  если  он / она  превысили  за  предыдущий  период  следующее  Летное  и  Рабочее  Полетное  Время.

a )    An  Operator  shall  ensure  that  the  total  Duty  Periods  to  which  a  crew  member  is  assigned  during  the  flight  and  on  ground  shall  not  exceed   -  эксплуатант  должен  обеспечить, чтобы  общий  период  рабочего  времени, назначаемого  члену экипажа,  при  выполнении  полетов  и  на  земле  не  превышал :

1 )  55  Duty  Hours  within  any  7  consecutive  days,  but  may  be  increased  upto  60  hours,  when  rostered  duty  covering  a  series  of  duty  periods,  once  commenced,  is  subject  to  unforeseen  delays  -  55  рабочих  часов  в  течение  7-и  последовательных  дней,  которые  могут   быть  увеличены  до  60  рабочих  часов,  когда  запланированая  серия  из  нескольких  рабочих  периодов, непредвиденно  задерживается ;

2 )  95  hours  within  any  2  consecutive  weeks,  but  may  be  increased  upto  110   hours,  when  is  subject  to  unforeseen  circumstances  -  95  часов  в  течение 2-х  последовательных  недель, которые  могут   быть  увеличены  до  110  рабочих  часов,  когда  это  необходимо  по  непредвиденным  обстоятельствам ;

3 )   as  normal  rules  apply  158  Duty  Hours  in  any  28  consecutive  days,  spread  as  evenly  as  practicable  throughout   this   period,  but  taking  into  consideration  irregular  nature  for  working  hours   calculation  for  flight  crew  duties,  as  an  exception  the  extended  Duty  Time  of  190  cumulative  hours  can  be  applied  in  any  28  consecutive  days  spread  as  evenly  as  practicable  throughout   this   period  -  как  правило  применяется  158  ч.  за  период  равный  28  последовательным  дням,  распределенным  как  можно  равномерно,

но  учитывая  ненормированный  характер  работы  членов  летных  экипажей, как  исключение  разрешается  применять  норму  в  190 ч.  суммированного  рабочего  времени  за  период  равный  28  последовательным  календарным  дням,  распределенным  максимально  равномерно  на  протяжении  всего  этого периода.

b )  Limit   on   Total   Flights  Block   Times  -  Ограничение  Общего  Полетного  Времени

An  Operator  shall  ensure  that  the  Total  Flights  Block  Times  of  the  flights  on  which  an  individual  crew  member  is  assigned  as  an  operating  crew  member  does  not  exceed  limitation  stated  in  Table  5  below -  эксплуатант  должен  обеспечить, чтобы  общее  полетное  время  при выполнении  полетов, на  которые  член  экипажа  запланирован, как  член  рабочего экипа- жа,  не  превышало  следующих ограничений  полетного  времени, указанных в  Таблице. 5 :

1 )   maximum  90  Flights  Block  Hours  in  any  28  ( twenty  eight )  consecutive  days  -  максимум  90  часов  общего  полетного  времени  за  период  равный  28  последова- тельным  дням ;

2 )   maximum  900  ( nine  hundred )  Flights  Block  Hours  in  a  calendar  year  -  максимум  900  часов  общего  полетного  времени  за  календарный  год ;

3 )   in  case  operational  conditions  require,  only  3  times  per  year,  having  mandatory  consent  of  the  crew  member,  maximum  Flights  Block  Hours  in  any  28  consecutive  days  stated  in  Table  5  may  be  extended  up  to  100  hours,  providing  the  mandatory  condition  requiring  to  prohibit  such  an  extension  during  no  more  than  2 ( two ) time,  in  consecutive  period’s  -  в  случае  необходимости, только  3  раза  в  год,  при  обязательном   согласии  члена  летного  экипажа,   полетное  время  за  период  28  последовательных  дней,  указанное   в  Табл. 5,   может  быть  увеличено  до  100  часов,.  Обязательным  условием  при  этом  является  запрет  на  применение  вышеуказанного  продления   в  течении   не  более  двух   ( 2 )  последовательных   28-и  дневных  периодах   подряд ;                            and

4 )   maximum  Block  hours  in  a  Calendar  year,  indicated  in  Table 5,  cannot   be  extended  -   годовое  полетное   время,   указанное   в   Табл. 5   не  может   быть  увеличено.

Table   5  /  Таблица  5.

| Aircraft type / Типы ВС | Total Flight Block Times ( h ) Полетное время ( ч. ) |  |
| --- | --- | --- |
|  | 28 consecutive days за 28 последова - тельных дней | a calendar year эа календарный год |
| A - 318 / 319 / 320 / 321 / 330 / 340 /350 / 380 , RRJ 95, B -737 / 757 / 767 / 777 / 787 / 747, CRJ - 100 / 200 / 700 / 900 / 1000, ATR 42 / 72, Тu - 204 / 214, Jetstream - 32 | 90 | 900 |
| Il - 76, Yak - 42 / 40, Аn - 12 / 32 / 26 / 24 , | 80 | 800 |

Примечание  1.   Для  вновь  вводимых  в  эксплуатацию  ВС  предельные  величины  полетного времени  устанавливаются  на  уровне  аналогичного  типа  и  класса  самолета  из  числа   указанных  в  Табл .5.

Примечание  2.    В  исключительных  случаях,  разрешается  превышение  полетного   времени  за  28  дней  и  за  год  не  более  чем  на   1ч. 30 мин.,  при  сохранении  требования обязательного  донесения  в  ГУГА  РА,  в  срок  не  более  чем  5  рабочих  дней  после  случая  данного  превышения.

Примечание  3.    В  случаях  производственной  необходимости,  только  для  самолетов советского  производства,  с  согласия  членов  летного  экипажа  и  по  согласованию  с  врачом – членом  Врачебно  Летной  Экспертной  Комиссии  (  ВЛЭК  ),  полетное  время  за  28  последова тельных  дней,  указанное  в  Табл. 5,   может  быть  увеличено   на  20 % ,  только   3  раза  в  год,  но  не  более  2  раз  подряд.  Только  при  соблюдении   этого  условия,   годовое   полетное   время, указанное  в   Табл. 5  может  быть  увеличено  до  860  часов.

Примечание  4.:    Рабочее  время  КЛС  формируется  как  сумма  офисного  времени  ( время  работы  КЛС  на  земле,  согласно  требованиям   контракта  и  должностных  инструкций )  и полетного   рабочего   времени.  В  любом  случае,  суммарное   рабочее   время   КЛС  должно  соответствовать  значениям,  указанным   в   Табл.2

c )    Post - flight  duty  shall  count  as  duty  period.                                                                                            The  Operator  shall  specify  in  its  Operations  Manual  the  minimum  time  period  for  post - flight duties.

Define  post - flight  duties  times  appropriate  to  each  individual  operation,   that  realistically  reflects  the  time  for  safety  related  ground  duties  as  approved  by  the  GD CA  of  RA,  but  not  less  than  30  minutes  after  actual  arrival  time , depending  on  type   of  aircraft   and   flight   -  эксплуатант  должен  определить  время  на  послеполетную  деятельность,  одобренную  со  сторопы  ГУГА  РА,  которое  реалистично  отражает  время  послеполетной  работы, необходимое  из  условий  обеспечения  безопасности  полета  и  которое  не  может  быть  менее  30  минут  от  реального  времени  прибытия,  в  зависимости  от  типа  ВС  и  задания  на  полет.

ORO. FTL. 212     Cabin  Crew  Requirements  -  Требования  к  Кабинному  Экипажу

The  limitations,  which  shall  be  applied  to  Cabin  Crew,  are  those  applicable  to  flight  crew  members,  but  with  the  following  differences  -   ограничения,  которые  применяются  для  Кабинного  Экипажа   такие  же,  которые  применяются  для  Летного  Экипажа,  только  с  последующими  отличиями :

a )    FDP  with  different  reporting  time  for  Flight  Crew  and  Cabin  Crew.

Whenever  Cabin  Crew  requires  more  time  than  the  flight  crew  for  their  pre - flight  briefing  for  the  same  sector  or  series  of  sectors,  the  FDP  of  the  Cabin  Crew  may  be  extended  by  the difference  in  reporting  time  between  the  cabin  crew  and  the  flight  crew  -  когда  кабинному  экипажу  требуется  больше  времени  на  предполетную  подготовку  для  выполнения  полета  или   полетов,  длительность   полетного   рабочего  времени  членов  кабинного  экипажа  может  быть  увеличена  на  разницу  между  временем  явки  летного  и  кабинного  экипажей.                                                                                                                                                                     The  difference  shall  not  exceed 1  hour.   The  maximum  daily  FDP  for  Cabin  Crew  shall  be  based  on  the  time  at  which  the  flight  crew report  for  their  FDP,  but  the  FDP  shall  start  at  the  reporting  time  of  the  cabin  crew  -   максимальная   длительность   полетного   рабочего  времени  членов  кабинного  экипажа  базируется  на  Времени  явки  на  вылет летного  экипажа, но  полетное   рабочее  время  членов  кабинного  экипажа  должно  начинаться  по  времени  явки  членов  кабинного  экипажа.  Разница  во  времени  явки   членов  кабинного  экипажа   и  летного  экипажа назначенных  для  выполнения  полета  или   полетов,  не  должна  превышать   1 часа ;

b )   The  limits  relating  to  the  ( 2 )  two  pilot  flight  crew  for  Long  Range  operations  do  not  apply  for  Cabin  crew -  ограничения  применяемые  для  экипажа  из  ( 2 )  двух  пилотов   при  выполнении  длительных  полетов  для  Кабинного  экипажа  не  применимы.

ORO. FTL. 215     Positioning   -  Перемещение  Экипажа

If  an  Operator  positions  a  crew  member,  the  following  shall  apply :

a )    positioning  ( as  a  Dead  Heading  Crew )  after  reporting  but  prior  to  operating  shall  be  counted  as  FDP  but  shall  not count  as  a  sector   -   если  перемещение члена  экипажа  осуществляется  после  времени  явки  перед  выполнением  полета,  то  в  общее  Полетное  Рабочее  Время  члену  нерабочего  экипажа  оно  учитывается  в  объеме  100 %  но  не  зачитывается  как  сектор ;

b )    all  time  spent  on  positioning  shall  count  as  duty  period   -  все  время,  затраченное  на  перемещение ( передвижение ),  рассматривается  как  рабочее  время ;

c  )  in  exceptional  cases,  maximum  time  not  exceeding  4  hours  allowed  for  positioning    of  Dead  Heading  Crew  without  rest  conditions  in  flight   -   в  исключительных  случаях допускается  максимальное  время  перемещения членов  нерабочего  экипажа  ( DHC )  без  предоставления  условий  для  отдыха  на борту  ВС  только  до  4  часов ;

d )   A  positioning  sector  immediately  following  the  operation  sector  will  be  taken  into  account  for  the  minimum  rest  time  calculation   -  сектор перемещения, следующий немедленно после сектора  выполнения  полета,  должен  учитываться  для  расчета  минимального  времени  отдыха ;

ORO. FTL. 220     Split  Duty

The  conditions  for  extending  the  basic  maximum  daily  FDP  due  to  a  break  on  the  ground shall  be  in  accordance  with  the  following :

a )    Flight  time  specification  schemes  shall  specify  the  following  elements  for  Split  Duty  in accordance  with  the  certification  specifications  applicable  to  the  type  of  operation :

1 )   the  minimum  duration  of  a  break  on  the  ground ;                                                 and

2 )   the  possibility  to  extend  the  FDP  prescribed  under  point  ORO. FTL. 205 ( b )  taking  into account  the  duration  of  the  break  on  the  ground,  the  facilities  provided  to  the  crew  member  to  rest  and  other  relevant  factors ;

b )    The  break  on  the  ground  shall  count  in  full  as  FDP ;

c )    Split  Duty  shall  not  follow  a  reduced  rest.

ORO. FTL. 225     Standby  and  Duties  at  the  Airport

If  an  Operator  assigns  crew  members  to  standby  or  to  any  duty  at  the  airport,  the  following shall  apply  in  accordance   with  the  certification  specifications  applicable  to  the  type  of operation :

a )    Standby  and  any  duty  at  the  airport  shall  be  in  the  roster  and  the  start  and  end  time  of standby  shall  be  defined  and  notified  in  advance  to  the  crew  members  concerned  to  provide them  with  the  opportunity  to  plan  adequate  rest    -  резерв  или  иная  деятельность  в  аэропорту   должна  быть  запланирована   по  наряду  и  время  начала  и  окончания  резерва должно  быть  определено  и  своевременно  доведено  до  членов  экипажа  с  целью  обеспечения  ими  возможности  планирования  соответствующего  отдыха.

b )    A  crew  member  is  considered  on  airport  standby  from  reporting  at  the  reporting  point until  the  end  of  the  notified  airport  standby  period   -  члена  экипажа  считается  в  резерве  в  аэропорту  со  Времени  явки  в  месте  доклада  вплоть  до  конца  времени  определенного  для  резерва ;

c )    Airport  standby  shall  count  in  full  as  duty  period  for  the  purpose  of  points  ORO. FTL. 210   and   ORO. FTL. 235  -  время  нахождения  в  резерве  в  аэропорту  включается   в  общее  месячное  Рабочее  Время  согласно  пунктов   ORO. FTL. 210   and   ORO. FTL. 235 ;

d )    Any  duty  at  the  airport  shall  count  in  full  as  duty  period  and  the  FDP  shall  count  in  full  from  the  airport  duty  reporting  time   -  любая  деятельность  в  аэропорту   должна  быть  включена  в  общее  месячное  Рабочее  Время,  отсчитываемое  со  Времени  явки  в  месте  доклада ;

e )    The  Operator  will  provide  the  crew  member  with  accommodation ,  a  quiet  and  comfortable  place  not  open  to  the  public    -   Эксплуатант  должен  обеспечить членам  экипажа  тихое,  спокойное,  удобное  и  изолированное  от  посторонних  помещение ;

f )    Flight  time  specification  schemes  shall  specify  the  following  elements :

1 )    the  maximum  duration  of  any  standby ;

2 )    the  impact  of  the  time  spent  on  standby  on  the  maximum  FDP  that  may  be  assigned, taking  into  account  facilities  provided  to  the  crew  member  to  rest,  and  other  relevant  factors  such  as :

—   the  need  for  immediate  readiness  of  the  crew  member ;

—   the  interference  of  standby  with  sleep ;                                      and

—   sufficient  notification  to  protect  a  sleep  opportunity  between  the  call  for  duty  and  the assigned  FDP ;

3 )   the  minimum  rest  period  following  standby  which  does  not  lead  to  assignment  of  an  FDP ;

4 )   how  time  spent  on  standby  other  than  airport  standby  shall  be  counted  for  the  purpose  of  cumulative  duty  periods.

g )    Where  airport  standby  is  immediately  followed  by  a  flight  duty,  the  relationship  between  such  airport  standby  and  the  assigned  flight  duty  will  make  full  FDP.  In  such  a  case,  airport  standby  shall  be  added  to  the  Duty  Period  for  the  purposes  of  calculating  minimum  rest               -  в  случае  заступления  в  резерв после  завершения  Полетного  Рабочего  Времени, взаимо- связь между  резервом  в  аэропорту  и  установленным  временем  Полетного   Рабочего  Времени  составляет  полную  сумму  Общего  Рабочего  Времени.

h )   Where  the  airport  standby  does  not  lead  to  assignment  on  a  flight  duty,  it  shall  be  followed  at  least  by  a  rest  period    -   когда  резерв  в  аэропорту  не  сопровождается выполнением  полетного  задания,  после  нахождения  в  резерве  требуется,  как  минимум  время  для  полноценного  отдыха.

ORO. FTL. 230     Reserve

If  an  Operator  assigns  crew  members  to  reserve,  the  following  requirements  shall  apply  in accordance  with  the  certification  specifications  applicable  to  the  type  of  operation :

a )    reserve  shall  be  in  the  roster  and / or  notified  in  advance   -  резерв  должен  быть  запланирован   и  или  заранее  должно  быть  уведомление  о  планах ;

b )    flight  time  specification  schemes  shall  specify  the  following  elements :

1 )    the  maximum  duration  of  any  single  reserve  period ;

2 )    the  number  of  consecutive  reserve  days  that  may  be  assigned  to  a  crew  member.

c )   тhe  maximum  length  and  place  of  any  standby  time  at  a  place  other  than  a  specified  reporting  point  shall  be  unambiguously  determined  -  максимальная  продолжительность  и  место  нахождения  любого  резерва,  отличного  от  места  явки  должно  быть  однозначно  определено ;

d  )   тhe  counting  of  standby  time  is  made  in  order  to  be  taken  into  account  for  the  cumulative  Monthly  Duty  Hours  -  учет  времени  нахождения  в  резерве  производится  с  целью  включения  его  в  общее  месячное  Рабочее  Время .

ORO. FTL. 235     Rest  Periods

a )    Minimum  Rest  Period  at  Home  Base :

1 )   the  minimum  rest  period  provided  before  undertaking  an  FDP  starting  at  Home  Base  shall be  at  least  as  long  as  the  preceding  duty  period,  or  12  hours,  whichever  is  greater ;

2 )   by  way  of  derogation  from  point  ( 1 ),  the  minimum  rest  provided  under  point  ( b )  applies  if  the  Operator  provides  suitable  accommodation  to  the  crew  member  at  Home  Base.

b )    Operator  shall  grant  the  rest  period  at  the  home  base  not  less  than  48  hours  in  case  during   any  7  consecutive  days  crew  member  has  operated  3  and  more  FDP  encroaching  on  night  time   -   эксплуатант  обязан  предоставить   отдых  по  месту  постоянного  проживания  в  течение  48 ч,  если  за  7  суток  в  любой   последовательности   выполнены  3  и  более   периодов   полетного   рабочего  времени,  захватывающих  ночное  время ;

c )    Minimum  Rest  Period  away  from  Home  Base :

The  minimum  rest  period  provided  before  undertaking  an  FDP  starting  away  from  home  base shall  be  at  least  as  long  as  the  preceding  duty  period,  or  10  hours,  whichever  is  greater  -  минимальное  время  отдыха, предоставляемое  членам  экипажа  перед  началом  выполнения  служебных  обязанностей  начинающееся  вне  базового  аэропорта,  должно  быть,  как  минимум  равно  объему  предшествующего  рабочего  времени,  или  10 ч., в  зависимости  от  того,  которое  из  них  больше ;

This  period  shall  include  an  8 - hour  sleep  opportunity  in  addition  to  the  time  for  travelling  and  physiological  needs   -  этот  отдых  включает  время  продолжительностью  8 ч. для  сна  дополнительно  ко  времени  в  пути  до  места  отдыха  и  на  другие  физиологические  потребности.

d )    Reduced   Rest :

By  derogation  from  points  ( a )  and  ( b ),  flight  time  specification  schemes  may  reduce  the minimum  rest  periods  in  accordance  with  the  certification  specifications  applicable  to  the  type of  operation  and  taking  into  account  the  following  elements :

1 )   the  minimum  reduced  rest  period ;

2 )   the  increase  of  the  subsequent  rest  period ;                        and

3 )   the  reduction  of  the  FDP  following  the  reduced  rest.

e )    Recurrent   Extended   Recovery   Rest   Periods :

Flight  time  specification  schemes  shall  specify  recurrent  extended  recovery  rest  periods  to compensate  for  cumulative  fatigue.  The  minimum  recurrent  extended  recovery  rest  period  shall be  36  hours,  including  2  local  nights,  and  in  any  case  the  time  between  the  end  of  one recurrent  extended  recovery  rest  period  and  the  start  of  the  next  extended  recovery  rest  period shall  not  be  more  than  168  hours.  The  recurrent  extended  recovery  rest  period  shall  be increased  to  2  local  days  twice  every  month ;

f )    Flight  time  specification  schemes  shall  specify  additional  rest  periods  in  accordance  with the  applicable  certification  specifications  to  compensate  for :

1 )   the  effects  of  time  zone  differences  and  extensions  of  the  FDP ;

2 )   additional  cumulative  fatigue  due  to  disruptive  schedules ;                        and

3 )   a  change  of  home  base.

g )    Crew   members   shall   -   Члены  экипажа  должны :

1  )   not  be  on  duty  more  than  7  ( seven )  consecutive  days  between  days  off,  but  may  be  positioned  to  the  usual  operating  base  on  the  8  ( eighth ) day,  provided  they  are  then  allocated  at  least  2  ( two )  consecutive  days  off,  and  have  2  ( two )  consecutive  days  off  in  any  consecutive  14  ( fourteen )  days  following  the  previous  2  ( two )  consecutive  days  off     -                  не  должны  выполнять  служебные обязанности  более  чем  в  течении  7  последовательных  дней  между  выходными  днями,  но  при  перебазировании  на  базовый  а / м,  на  следующий              8-ой   день,  им  должно  быть  предоставлено  2  последовательных   выходных  дня  и  они  должны  иметь  2  последовательных  выходных  дня   в  любые  последующие  14  дней,  следующих  после  предшествующих  2-х  последовательных  выходных  дней  ;              and

2  )   have  a  minimum  of  7  ( seven )  days  off  in  any  consecutive  4  ( four )  weeks   -  должны  иметь  минимум  7  дней  отдыха  в  течение  любых  последующих 4-х  недель ;              and

3  )   have  an  average  of  at  least  8  days  off  in  each  consecutive  4  week  period,  averaged  over  ( 3 ) three  such  periods    -   должны  иметь  в  среднем  не  менее  8  выходных  в  течение  каждых  последующих 4  недель,  усреднено  на  протяжении  3  таких  периодов ;

4  )  The  minimum  rest  provided  before  undertaking  a  Flight  Duty  Period  with  term  of  preceding  FDP  accomplished  during  the  time (  from  06 : 00   to  20 : 00 ),  may  be  granted  by  reduced  period  and  take  into  account  the  following  criteria   -  минимальное  время  отдыха, предоставляемое  членам  экипажа  перед  началом  выполнения  служебных  обязанностей,  при  условии,  что   предшествующее  рабочее  время  осуществлялось  в  дневной  период  времени  (  с  06 : 00   до 20 : 00 )  может  быть  предоставлено  в  сокращенном  объеме  и  рассчитывается   по   следующим  критериям :

- If  preceding  FDP  does not  exceed  6  hours, in  this  case  minimum  rest  period  shall  make 8  hours  -  если  предшествующее  рабочее  время  не  превышает  6 ч.,  то  минимальное  время  предоставляемое  для  отдыха  составит  8  часов ;

- If  preceding  FDP  does  not  exceed   8  hours,  in  this  case  minimum   rest  period  shall  make  10  hours -  если  предшествующее  рабочее  время  не  превышает  8 ч.  то  минимальное  время  предоставляемое  для  отдыха  составит  10  часов ;

- If  preceding  FDP  does  not  exceed  10  hours,  in  this case  minimum   rest  period  shall  make  12  hours -  если  предшествующее  рабочее  время  не  превышает  10 ч.  то  минимальное  время  предоставляемое  для  отдыха  составит  12  часов.

h )   After  flight  with  capture  night  FDP  from  23 : 00  to  06 : 00  the  rest  period  calculation  must  take  into  account  the  following :   -  после  полетов  с  захватом  ночного  периода  Полетного Рабочего Времени  с  23 : 00  до  06 : 00 ч.  расчет   времени   отдыха   производится   по   следующей   схеме :

-   if  FDP  completed  before  WOCL, in  this  case, 50 %  from  captured  night  FDP  should  be  added  to  the  fixed  rest  period  -  если   период  полетного  рабочего  времени  завершен  до  начала  времени  Циркадного  Периода  ( WOCL ),  то  ко  времени  установленного  предполетного  отдыха  должно  быть  добавлено  50 %  от  захваченного  рабочего  ночного  времени ;

-   if  FDP  encroaches  on  the  WOCL,  in  this  case, 100 %   from  captured  night  FDP should  be  added  to  the  fixed  rest  period,  but  should  not  exceed  16  hours -  если   период  Полетного Рабочего Времени  завершен с захватом  времени Циркадного Периода ( WOCL ),  то  ко  времени  установленного  предполетного  отдыха  должно  быть  добавлено  100 %  от  захваченного  рабочего  ночного  времени,  но  не  более  чем  16 часов ;

j )   Operator  shall  increase  minimum  rest  period  by  time  for  traveling  to  the  place  for  accommodation  and  back  to  airport  if  positioning  time  exceeds  1  hour    -    эксплуатант  увеличивает  минимальный   период   отдыха   на   величину  продолжительности  общего  времени  переезда  до  места  размещения  на  отдых  и  обратно,  если  время  переезда  превышает   1 час ;

m )    Rest  period  following  several  successive  occurrences  of  rest  period  reducing  and  increasing  FDP,  cannot  be  reduced  -   время  отдыха,  следующее  после  нескольких  последовательных  случаев  сокращения  времени  отдыха  и  продления  Полетного  Рабочего  Времени,  не  может  быть  сокращено ;

n )    For  purpose  of  medical  check  every  crew  member  is granted  with   2  ( two )  days  off  before  commencing  medical  check   -   с  целью  прохождения  медицинского  освидетельствования,  до  ее  начала, каждому  члену  экипажа  по  его  требованию   предоставляется  2  выходных  дня   ( пр. ГУГА РА  от  25. 04. 2007  № 65-Ն   гл .2,  п. 19  ).

ORO. FTL. 240     Nutrition  -  Питание

a )    During  the  FDP  there  shall  be  the  opportunity  for  a  meal  and  drink  in  order  to  avoid  any  detriment  to  a  crew  member’s  performance,  especially  when  the  FDP  exceeds  6  hours   -   в  течении  летного  рабочего  периода,  для  поддержания  должного  уровня  работоспособ- ности   и  здоровья  члены  экипажа  должны  быть  обеспечены  полноценным  питанием  и  напитками  с  периодичностью  каждые  6  часов ;

b )    An  Operator  shall  specify  in  its  Operations  Manual  how  the  crew  member’s  nutrition during  FDP  is  ensured.

ORO. FTL. 242     FDP,  and  Simulator  Flight  Time  Limitation's  during   Simulator

Training  activities  -  Ограничения  продолжительности  рабочего  и

летного   времени  при  прохождении  тренажерной   подготовки.

a )  Maximum  Briefing  time  before  Simulator  Training  Session is  limited  to  1  hour  30  minutes  -  максимальная  продолжительность  предварительной  подготовки  перед  проведением тренировки  на тренажере   не  более  1 часа  30 минут ;

b )  Maximum  Debriefing  time  after  Simulator  Training  Session is limited  to  1  hour  -    продолжительность  разбора  после  проведения  тренажерной  тренировки  не  более  1 часа ;

c )   Maximum  duration  of  FFS  Training  session  for  the  training  crew  shall  not  exceed  4  hours  ( 1  training  session only ),  during  the  consecutive  24  hours   -   продолжительность  времени  тренировки  на  тренажере,  тренируемого  экипажа,  не  должна   быть  более  4  час   в  сутки  ( 1  тренировочная  сессия ) ;

d )    Maximum  duration  of  FFS  session  for  training  staff  (  TRI / TRE / SFI / SFE )  is  limited  to  8  hours  ( 2  training  session ),  during  the  consecutive  24  hours,  with  an  interruption  between  two  sessions  being  not  less  than  2  hours   -    продолжительность  полетного времени на тренажере  лиц  инструкторского  состава,  проводящих  тренажерную  подготовку  не  должна  превышать   8 часов    ( 2 сессии )  за  последовательные 24  часа,  с  перерывом  между  ними  не  менее  2 часов ;

e )   For  training  staff   carrying  out  training  duties  on  Simulator  it  is  not  allowed  to  perform    2  consecutive  FFS  training  operations,  if  one  of  those  sessions  encroaches  on  WOCL  -  лицам  инструкторского  состава,  проводящим  тренажерную  подготовку,  запрещается проводить  2  последовательные  тренажерные  сессии ,  если  одна  из  них  по  времени  совпадает  с  Циркадным  периодом  времени  суток ;

f )   Total  Duty  Time  for  training  staff  (  TRI / TRE / SFI / SFE ) shall  not  exceed  14  hours  during  one  working  day -  общая  продолжительность  рабочего  времени  лиц,  проводящих  тренажерную подготовку  не  должна  превышать  14  часов   в  сутки ;

g )   The Total  FFS  Time  for  training  staff   shall  not exceed  90  hours  in  any  28 ( twenty  eight )  consecutive  days  -  общее  полетное  временя  на  тренажере, лиц  инструкторского  состава, проводящих  тренажерную  подготовку  не  должна  превышать  90 часов  за  период  равный  28  последовательным  дням ;

h )  The  Total  Flight  Duty  Time  for  training  staff  ( TRE,  TRI  ) having  the  rating  to  perform  FFS  training  and  allowance  for  aircraft  operation,  shall  not  exceed  90 hours  in  total  in  any  28  ( twenty  eight )  consecutive  days  for   both  type  activities   -  общее  полетное  время, лиц  инструкторского  состава, имеющих  право  проводить  тренажерную  подготовку  и  допуск  к  выполнению полетов  в  сумме по обоим  видам  деятельности, не  должна  превышать  90  часов  за  период  равный  28  последовательным  дням.

ORO. FTL. 245     Records  of  Home  Base,  FDP,  Flight  Times,  Flight  Block  Times,

and   Rest   Periods  -  хранениe  записей  Полетного  Рабочего,  Полетного

Летного,  Рабочего  Времени,  Времени  Отдыха  и Времени на  Базе  членов

экипажей.

a )    An  Operator  shall  maintain,  for  a  period  of   24  months  -   эксплуатант  должен   обеспечить  сохранение,  как  минимум  на  период  24  календарных  месяца :

1 )   individual  records  for  each  crew  member  including  -  индивидуальные  записи  для  каждого  члена  экипажа,  включая :

( i )     Flight  Times  -  летное  время ;

( ii )    Block  Тimes   -  полетное  время ;

( iii )   start,  duration  and  end  of  each  Duty  Period  and  FDP   -  начало, продолжитель-

ность  и  конец  выполнения  Рабочего  и  Полетного  Рабочего  Времени ;

( iv )   rest  periods  and  days  free  of  all  duties   -  продолжительность   периодов

отдыха   и  дней свободных  от  любого  вида  работы ;                                     and

( v )    assigned  home  base   -  нахождение  на  Базе ;

2 )   reports  on  extended  flight  duty  periods  and  reduced  rest  periods  -  донесения  КВС  о  продлении  Полетного  Рабочего  Времени  и  сокращении  Времени  Отдыха  экипажа ;

b )    Upon  request,  the  Operator  shall  provide  copies  of  individual  records  of  flight  times,  duty periods  and  rest  periods  to   -   по  запросу,  эксплуатант  должен  предоставить  записи  по любому  члену  экипажа  о  Полетном,  Рабочем  Времени  и  Времени  Отдыха :

1 )   the  crew  member  concerned  -  заинтересованному  члену  экипажа ;                  and

2 )   to  another  Operator,  in  relation  to  a  crew  member  who  is  or  becomes  a  crew  member  of  the  Operator  concerned  -   другому  эксплуатанту,  который  нанимает  члена  экипажа   для  выполнения  полетных  служебных  обязанностей  как  члена  экипажа ;

c )    Records  referred  to  in  point  CAT. GEN. MPA. 100 ( b ) ( 5 )  in  relation  to  crew  members who  undertake  duties  for  more  than  one  Operator  shall  be  kept  for  a  period  of   24  months  -

упомянутые  в  пункте  CAT. GEN. MPA. 100 ( b ) ( 5 )  записи,  связанные  с  деятельностью члена  экипажа,  выполняющего свои  служебные  обязательства  более,  чем  у  одного  эксплуатанта,  должны  сохраняться  на  период  24  календарных  месяца.

ORO. FTL. 250     Fatigue  Management  Training

a )    The  Operator  shall  provide  Initial  and  Recurrent  Fatigue  Management  Training  to  crew members,  personnel  responsible  for  preparation  and  maintenance  of  crew  rosters  and management  personnel  concerned :

b )    This  training  shall  follow  a  training  programme  established  by  the  operator  and  described in  the  Operations  Manual.

The  training  syllabus  shall  cover  the  possible  causes  and  effects  of  Fatigue  and  fatigue countermeasure.

INTENTIONALLY        LEFT         BLANK

INTENTIONALLY        LEFT         BLANK

SUBPART   TC.    Technical   Crew   in   HEMS,   HHO   or   NVIS   Operations

ORO.TC.100   Scope

This  Subpart  establishes  the  requirements  to  be  met  by  the  Operator  when  operating  an  aircraft  with  technical  crew  members  in  Commercial  Air  Transport  Helicopter  :

- Emergency   Medical   Service  (  HEMS  ) ;

- Night   Vision   Imaging   System   (  NVIS  )  operations ;      or

- Helicopter   Hoist   Operations   (  HHO  ).

ORO. TC. 105   Conditions  for  Assignment  to  Duties

a )   Technical  crew  members  in  commercial   air  transport  HEMS,  HHO  or  NVIS  operations  shall  only  be  assigned  duties  if  they :

1 )   are  at  least  18  years  of  age ;

2 )   are  physically  and  mentally  fit  to  safely  discharge  assigned  duties  and  responsibilities ;

3 )   have  completed  all  applicable  training  required  by  this  Subpart  to  perform  the  assigned  duties ;

4 )   have  been  Checked  as  proficient  to  perform  all  assigned  duties  in  accordance  with  the  procedures  specified  in  the  Operations  Manual.

b )   Before  assigning  to  duties  technical  crew  members  who  are  self - employed  and / or working  on  a  freelance  or  part - time  basis,  the  Operator  shall  verify  that  all  applicable requirements  of  this  Subpart  are  complied  with,  taking  into  account  all  services  rendered  by  the  technical  crew  member  to  other  operator(s)  to  determine  in  particular :

1 )   the  total  number  of  aircraft  types  and  variants  operated ;

2 )   the  applicable  flight  and  duty  time  limitations  and  rest  requirements.

ORO. TC. 110   Training  and  Checking

a )   The  Operator  shall  establish  a  training  programme  in  accordance  with  the  applicable requirements  of  this  Subpart  to  cover  the  duties  and  responsibilities  to  be  performed  by  technical  crew  members ;

b )   Following  the  completion  of  initial,  operator  conversion,  differences  and  recurrent  training,  each  technical  crew  member  shall  undergo  a  check  to  demonstrate  their  proficiency  in  carrying  out  normal  and  emergency  procedures ;

c )     Training  and  Checking  shall  be  conducted  for  each  training  course  by  personnel  suitably  qualified  and  experienced  in  the  subject  to  be  covered.  The  Operator  shall  inform  the  GDCA  of  RA  about  the  personnel  conducting   the  checks.

ORO. TC. 115   Initial  Training

Before  undertaking  the  Operator  Conversion  Training ,  each  technical  crew  member  shall complete  initial  training,  including :

a )   General  theoretical  knowledge  on  aviation  and  aviation  regulations  covering  all  elements  relevant  to  the  duties  and  responsibilities  required  of  technical  crew ;

b )   Fire  and  smoke  training ;

c )   Survival  training  on  ground  and  in  water,  appropriate  to  the  type  and  area  of  operation ;

d )   Aero - medical  aspects  and  first - aid ;

e )   Communication  and  relevant  CRM  elements  of  ORO. FC. 115  and  ORO. FC. 215.

ORO. TC. 120    Operator  Conversion  Training

Each  technical  crew  member  shall  complete :

a )   operator  conversion  training,  including  relevant  CRM  elements :

1 )   before  being  first  assigned   by  the  Operator  as  a  technical  crew  member ;     or

2 )   When  changing  to  a  different  aircraft  type  or  class,  if  any  of  the  equipment  or  procedures  mentioned  in  ( b )  are  different.

b )   Operator  Conversion  Training  shall  include :

1 )   the  location  and  use  of  all  safety  and  survival  equipment  carried  on  the  aircraft ;

2 )   all  normal  and  emergency  procedures ;

3 )   on - board  equipment  used  to  carry  out  duties  in  the  aircraft  or  on  the  ground  for  the  purpose  of  assisting  the  pilot  during  HEMS,  HHO  or  NVIS  operations.

ORO. TC. 125   Differences  Training

a )   Each  technical  crew  member  shall  complete  Differences  Training  when  changing  equipment  or  procedures  on  types  or  variants  currently  operated ;

b )   The  Operator  shall  specify  in  the  Operations  Manual  when  such  differences  training  is required.

ORO. TC. 130   Familiarization  Flights

Following  completion  of  the  Operator  Conversion  Training,  each  technical  crew  member  shall  undertake  familiarization  flights   prior  to  operating  as  a  required  technical  crew  member  in  HEMS,  HHO  or  NVIS  operations.

ORO. TC. 135   Recurrent  Training

a )   Within  every  12 - month  period,  each  technical  crew  member  shall  undergo  Recurrent Training  relevant  to  the  type  or  class  of  aircraft  and  equipment  that  the  technical  crew member  operates.  Elements  of  CRM  shall  be  integrated  into  all  appropriate  phases  of  the  Recurrent  Training ;

b )  Recurrent  Training  shall  include  theoretical  and  practical  instruction  and  practice.

ORO. TC. 140   Refresher  Training

a )   Each  technical  crew  member  who  has  not  undertaken  duties  in  the  previous  6  ( six )  months  shall  complete  the  Refresher  Training   specified  in  the  Operations  Manual ;

b )   The  technical  crew  member  who  has  not  performed  flying  duties  on  one  particular  aircraft  type  or  class  during  the  preceding  6  ( six )  months  shall,  before  being  assigned  on  that  type  or  class,  complete  either :

1 )   Refresher  Training  on  the  type  or  class ;              or

2 )   2  ( two )  familiarization  sectors  on  the  aircraft  type  or  class.

LEFT            INTENTIONALLY            BLANK

Appendix

| DECLARATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| A | Operator |  |  |  |
| 1 | Name : Place in which the operator is established or residing and place from which the operations are directed : |  |  |  |
| 2 | Name and contact details of the Accountable Manager : |  |  |  |
| B | Aircraft Operation |  |  |  |
| 1 | Starting date of operation / applicability date of the change : |  |  |  |
| 2 | Type(s) of operation : □ Part - NCC : ( specify if passenger and / or cargo ) |  |  |  |
| 3 | Type(s) of aircraft, registration(s) and main base : |  |  |  |
| 4 | Details of approvals held ( attach list of specific approvals to the declaration, if applicable ) |  |  |  |
| 5 | List of alternative means of compliance with references to the AMC’s they replace ( attach to the declaration ) |  |  |  |
| C | Statements |  |  |  |
| 1 | □ The management system documentation including the Operations Manual reflect the applicable requirements set out in Part - ORO, Part - NCC and Part - SPA. All flights will be carried out in accordance with the procedures and instructions specified in the Operations Manual. |  |  |  |
| 2 | □ All aircraft operated hold a valid certificate of airworthiness : |  |  |  |
| 3 | □ All flight crew and cabin crew members, as applicable, are trained in accordance with the applicable requirements. |  |  |  |
| 4 | □ ( if applicable ) The Operator has implemented and demonstrated conformance to an officially recognized industry standard. Reference of the standard : Certification body : Date of the last conformance audit : |  |  |  |
| 5 | □ Any change in the operation that affects the information disclosed in this declaration will be notified to the GDCA of RA. |  |  |  |
| 6 | □ The Operator confirms that the information disclosed in this declaration is correct. |  |  |  |
| 7 | Date & Name of the Accountable Manager : |  |  |  |
| 8 | Accountable Manager signature |  |  |  |
|  | ( in accordance with CR - EU No 965 / 2012 on Air Operations ) |  |  |  |

LEFT            INTENTIONALLY            BLANK