Appendix   7.     IR   Skill   Test

GENERAL

1.  An  applicant  for  an  IR  shall  have  received  instruction  on  the  same  class  or  type  of  aircraft  to  be  used  in  the  test.

2.  An  applicant  shall  pass  all  the  relevant  sections  of  the  Skill  Test.  If  any  item  in  a  section is  failed,  that  section   is   failed.

Failure  in  more  than  one  section  will  require  the  applicant  to  take  the  entire  test  again.  An  applicant  failing  only  one  section  shall  only  repeat  the  failed  section.

Failure  in  any  section  of  the  retest,  including   those  sections  that  have  been  passed  on  a  previous  attempt,  will  require  the  applicant  to  take  the  entire  test  again.

All  relevant  sections  of  the  Skill  Test  shall  be  completed  within   6 months.

Failure  to  achieve  a  pass  in  all  relevant  sections  of  the  test  in  2 ( two ) attempts  will  require  further  training.

3.  Further   training  may  be  required  following   a  failed  Skill Test.

There  is  no  limit  to  the  number  of  Skill  Tests  that  may  be  attempted.

CONDUCT    of    the     IR    SKILL     TEST

4.  The  test  is  intended  to  simulate  a  practical  flight.   The  route  to  be  flown  shall be  chosen  by  the  examiner.  An  essential  element  is  the ability of  the  applicant  to  plan  and  conduct  the flight  from  routine  briefing   material.  The  applicant  shall  undertake  the  flight  planning  and  shall  ensure  that  all  equipment  and  documentation  for  the execution  of  the  flight  are  on  board.  The  duration  of  the  flight  shall  be at  least 1 hour.

5.  Should  the  applicant  choose  to  terminate  a  Skill  Test  for  reasons  considered  inadequate  by  the  examiner,  the  applicant  shall  retake  the  entire  Skill  Test.   If  the  test  is  terminated  for  reasons  considered   adequate  by  the  examiner,  only  those  sections  not  completed  shall  be  tested   in  a  further  flight.

6.  At  the  discretion  of  the  examiner,  any  maneuver  or  procedure  of  the  test  may  be  repeated  once  by  the  applicant.

The  examiner  may  stop  the  test  at  any  stage  if  it  is  considered  that  the  applicant’s   demonstration  of  flying  skill  requires  a  complete  retest.

7.  An  applicant  shall  fly  the  aircraft  from  a  position  where  the  PIC  functions  can  be  performed  and  to  carry  out  the  test  as  if  there  is  no  other  crew  member.  The  examiner  shall  take  no  part  in  the operation  of  the  aircraft,  except  when  intervention  is  necessary  in  the  interests  of  safety  or  to  avoid  unacceptable  delay  to other  traffic.  Responsibility for  the  flight  shall  be  allocated  in  accordance  with  national  regulations.

8.  Decision  heights / altitude ( DH / DA ),  minimum  descent  heights / altitudes  ( MDH / MDA ) and  missed  approach  point  shall   be  determined  by  the  applicant  and  agreed  by  the  examiner.

9.  An  applicant  for  an  IR  shall  indicate  to  the  examiner the  checks  and  duties  carried  out,  including  the  identification  of  radio  facilities.  Checks  shall  be  completed  in  accordance  with  the  authorized  Checklist  for  the  aircraft  on  which  the  test  is  being  taken.   During   pre-flight  preparation  for  the  test  the  applicant  is  required  to  determine  power  settings  and  speeds.  Performance  data  for  take-off,  approach  and  landing  shall  be  calculated  by  the  applicant  in  compliance  with  the  operations  manual  or  flight  manual  for  the  aircraft  used.

FLIGHT    TEST   TOLERANCES

10.   The  applicant  shall  demonstrate  the  ability  to :

-  operate  the  aircraft  within  its  limitations ;

-  complete  all  maneuvers  with  smoothness  and  accuracy ;

-  exercise  good   judgment  and  airmanship ;

-  apply  aeronautical  knowledge ;                                                     and

-  maintain  control  of  the  aircraft  at  all  times  in  such a manner  that  the successful  outcome  of  a  procedure  or  maneuver  is  never  seriously  in  doubt.

11.   The  following  limits  shall  apply,  corrected  to  make  allowance  for  turbulent  conditions  and  the  handling  qualities  and  performance  of  the aircraft  used :

Height :

Generally	                                                                               ±  100 feet

Starting  a Go - around  at Decision  Height / Altitude	        +  50 feet / –  0 feet

Minimum  Descent  Height  / Altitude  /  MAP	                    +  50 feet / –  0 feet

Tracking :

On  radio  aids	                                                                 ±  5 °

Precision  approach	                                                      -  half  scale  deflection,

azimuth  and  glide  path

Heading :

all  engines  operating	                                                      ±  5 °

with  simulated  engine  failure	                                          ±  10 °

Speed :

all  engines  operating	                                                      ±  5 knots

with  simulated  engine  failure	                                          +  10  knots  /  –  5  knots

A.     CONTENT    of    the    IR    SKILL   TEST    for    the    AEROPLANE

| SECTION 1 — PRE - FLIGHT OPERATIONS and DEPARTURE |  |
| --- | --- |
| Use of Checklist, airmanship, anti - icing / de - icing procedures, etc., apply in all sections |  |
| a | Use of Flight Manual ( or equivalent ) especially A / C performance calculation, mass and balance |
| b | Use of Air Traffic Services document, weather document |
| c | Preparation of ATC Flight Plan, IFR Flight Plan / log |
| d | Pre-flight inspection |
| e | Weather Minima |
| f | Taxiing |
| g | Pre - Take-off briefing, Take-off |
| h ( o ) | Transition to instrument flight |
| i ( o ) | Instrument departure procedures, altimeter setting |
| j ( o ) | ATC liaison — compliance, R / T procedures |
| SECTION 2 — GENERAL HANDLING ( o ) |  |
| a | Control of the aeroplane by reference solely to instruments, including : level flight at various speeds, trim |
| b | Climbing and descending turns with sustained Rate 1 turn |
| c | Recoveries from unusual attitudes, including sustained 45° bank turns and steep descending turns |
| d ( o ) | Recovery from approach to stall in level flight, climbing/ descending turns and in landing configuration — only applicable to aeroplanes |
| e | Limited panel : stabilised climb or descent, level turns at Rate 1 onto given headings, recovery from unusual attitudes — only applicable to aeroplanes |
| SECTION 3 — EN - ROUTE IFR PROCEDURES ( o ) |  |
| a | Tracking, including interception, e. g. NDB, VOR, RNAV |
| b | Use of radio aids |
| c | Level flight, control of heading, altitude and airspeed, power setting, trim technique |
| d | Altimeter settings |
| e | Timing and revision of ETAs ( en - route hold, if required ) |
| f | Monitoring of flight progress, flight log, fuel usage, systems’ management |
| g | Ice protection procedures, simulated if necessary |
| h | ATC liaison — compliance, R / T procedures |
| SECTION 4 — PRECISION APPROACH PROCEDURES ( o ) |  |
| a | Setting and checking of navigational aids, identification of facilities |
| b | Arrival procedures, altimeter checks |
| c | Approach and landing briefing, including descent / approach / landing checks |
| d ( + ) | Holding procedure |
| e | Compce with published approach procedure |
| f | Approach timing |
| g | Altitude, speed heading control ( stabilised approach ) |
| h ( + ) | Go - around action |
| i ( + ) | Missed approach procedure / landing |
| j | ATC liaison — compliance, R / T procedures |

Aeroplanes   (  cont’d  )

| SECTION 5 — NON-PRECISION APPROACH PROCEDURES ( o ) |  |
| --- | --- |
| a | Setting and checking of navigational aids, identification of facilities |
| b | Arrival procedures, altimeter settings |
| c | Approach and landing briefing, including descent / approach / landing checks |
| d ( + ) | Holding procedure |
| e | Compliance with published approach procedure |
| f | Approach timing |
| g | Altitude, speed heading control ( stabilised approach ) |
| h ( + ) | Go - around action |
| i ( + ) | Missed approach procedure / landing |
| j ( o ) | ATC liaison — compliance, R / T procedures |
| SECTION 6 — FLIGHT with ONE ENGINE INOPERATIVE ( multi - engine aeroplanes only ) ( o ) |  |
| a | Simulated engine failure after Take-off or on Go-around |
| b | Approach, Go-around and procedural missed approach with one engine inoperative |
| c | Approach and landing with one engine inoperative |
| d | ATC liaison — compliance, R / T procedures |
| ( * ) May be performed in an FFS, FTD 2 / 3 or FNPT II. |  |
| ( + ) May be performed in either section 4 or section 5. |  |
| ( o ) Must be performed by sole reference to instruments. |  |

B.     CONTENT    of    the    IR    SKILL   TEST    for    the     HELICOPTER

| SECTION 1 — DEPARTURE |  |
| --- | --- |
| Use of Checklist, airmanship, anti - icing / de - icing procedures, etc., apply in all sections |  |
| a | Use of Flight Manual ( or equivalent ) especially A / C performance calculation, mass and balance |
| b | Use of Air Traffic Services document, weather document |
| c | Preparation of ATC Flight Plan, IFR Flight Plan / log |
| d | Pre-flight inspection |
| e | Weather Minima |
| f | Taxiing / Air taxy in compliance with ATC or instructions of instructor |
| g | Pre - Take-off briefing, procedures and checks |
| h | Transition to instrument flight |
| i | Instrument departure procedures. |
| SECTION 2 — GENERAL HANDLING |  |
| a | Control of the helicopter by reference solely to instruments, including : level flight at various speeds, trim |
| b | Climbing and descending turns with sustained Rate 1 turn |
| c | Recoveries from unusual attitudes, including sustained 30 ° bank turns and steep descending turns |
| d ( o ) | Recovery from approach to stall in level flight, climbing/ descending turns and in landing configuration — only applicable to aeroplanes |
| e | Limited panel : stabilised climb or descent, level turns at Rate 1 onto given headings, recovery from unusual attitudes — only applicable to aeroplanes |
| SECTION 3 — EN - ROUTE IFR PROCEDURES |  |
| a | Tracking, including interception, e. g. NDB, VOR, RNAV |
| b | Use of radio aids |
| c | Level flight, control of heading, altitude and airspeed, power setting. |
| d | Altimeter settings |
| e | Timing and revision of ETAs. |
| f | Monitoring of flight progress, flight log, fuel usage, systems’ management |
| g | Ice protection procedures, simulated if necessary and if applicable |
| h | ATC liaison — compliance, R / T procedures |
| SECTION 4 — PRECISION APPROACH PROCEDURES |  |
| a | Setting and checking of navigational aids, identification of facilities |
| b | Arrival procedures, altimeter checks |
| c | Approach and landing briefing, including descent / approach / landing checks |
| d ( * ) | Holding procedure |
| e | Compce with published approach procedure |
| f | Approach timing |
| g | Altitude, speed, heading control ( stabilised approach ) |
| h ( * ) | Go - around action |
| i ( * ) | Missed approach procedure / landing |
| j | ATC liaison — compliance, R / T procedures |

Helicopters         (  cont’d  )

| SECTION 5 — NON - PRECISION APPROACH PROCEDURES ( o ) |  |
| --- | --- |
| a | Setting and checking of navigational aids, identification of facilities |
| b | Arrival procedures, altimeter settings and check’s |
| c | Approach and landing briefing, including descent / approach / landing checks |
| d ( * ) | Holding procedure |
| e | Compliance with published approach procedure |
| f | Approach timing |
| g | Altitude, speed heading control ( stabilised approach ) |
| h ( * ) | Go - around action |
| i ( * ) | Missed approach procedure ( * ) / landing |
| j | ATC liaison — compliance, R / T procedures |
| SECTION 6 — ABNORMAL and EMERGENCY PROCEDURES |  |
| This section may be combined with sections 1 through 5. The test shall have regard to control of the helicopter, identification of the failed engine, immediate actions ( touch drills ), follow-up actions and checks and flying accuracy, in the following situations : |  |
| a | Simulated engine failure after Take-off and on /during approach ( ** ) ( at a safe altitude unless carried out in an FFS or FNPT II / III, FTD 2, 3 ) |
| b | Failure of stability augmentation devices / hydraulic system ( if applicable ) |
| c | Limited panel Approach and landing with one engine inoperative |
| d | Autorotation and recovery to a pre-set altitude |
| e | Precision approach manually without ( FD ) flight director ( *** ) Precision approach manually with ( FD ) flight director ( *** ) |
| ( * ) To be performed in section 4 or section 5. |  |
| ( ** ) Multi - engine helicopter only. |  |
| ( *** ) Only one item to be tested. |  |

C.     CONTENT    of    the    IR    SKILL   TEST    for    the     AIRSHIP

(   reserved  )

INTENTIONALLY            LEFT            BLANK