Appendix   9

Training,   Skill   Test  and   Proficiency  Check  for

MPL, ATPL,   Type  and   Class  Ratings,  and  Proficiency  Check   for  IRs

A.    GENERAL

1.  An  applicant  for  a  Skill  Test  shall  have  received  instruction  on  the  same  class  or  type  of  aircraft  to  be  used   in  the  test.

2.  Failure  to  achieve  a  pass  in  all  sections  of  the  test  in  two  attempts  will  require  further  training.

3.  There  is  no  limit  to  the  number  of  Skill  Tests  that  may  be  attempted.

CONTENT   of   the   TRAINING,    SKILL   TEST  /  PROFICIENCY    CHECK

4.   Unless  otherwise  determined  in  the  operational  suitability  data  established  in  accordance  with  Part - 21,  the  syllabus  of  flight  instruction  shall  comply  with  this  Appendix.  The  syllabus  may  be reduced  to  give  credit  for  previous  experience  on  similar  aircraft  types,  as  determined  in  the operational  suitability  data  established  in  accordance  with  Part - 21.

5.   Except  in   the  case  of  Skill  Tests  for  the   issue  of  an   ATPL,   when  so  defined   in   the operational suitability  data  established  in accordance  with  Part - 21  for  the  specific  type,  credit  may  be  given  for  Skill  Test  items  common  to  other  types  or  variants  where  the  pilot  is  qualified.

CONDUCT   of   the   TEST  /  CHECK

6.  The  examiner  may  choose  between  different  Skill  Test  or  Proficiency  Check  scenarios  containing simulated  relevant  operations  developed  and  approved  by  the  competent  authority.  Full  flight ( FFS ) simulators  and  other  training  devices  ( OTD ),  when  available,  shall  be  used,  as  established   in  this  Part.

7.  During  the  Proficiency  Check,  the  examiner  shall  verify  that  the  holder  of  the  Class  or  Type Rating  maintains  an   adequate  level  of  theoretical  knowledge.

8.  Should  the  applicant  choose  to  terminate  a  Skill  Test  for  reasons  considered  inadequate  by  the Examiner,  the  applicant shall  retake  the  entire  Skill  Test.  If   the  test  is  terminated  for  reasons considered   adequate  by  the  Examiner,  only  those  sections  not  completed  shall  be  tested  in  a  further flight.

9.  At  the  discretion  of  the  Examiner,  any  maneuver  or  procedure  of  the  test  may  be  repeated  once by  the  applicant.

The  Examiner  may  stop  the  test  at  any  stage  if  it  is  considered  that  the  applicant’s  demonstration of  flying  skill  requires  a  complete  re-test.

10.  An  applicant  shall  be  required  to  fly  the  aircraft  from  a  position  where  the  PIC  or  co-pilot functions,  as  relevant,  can  be  performed  and  to  carry  out  the  test  as  if  there  is  no  other  crew member  if  taking  the  test / check  under  single - pilot  conditions.  Responsibility for  the  flight  shall  be allocated  in  accordance  with  national  regulations.

11.  During  pre-flight  preparation  for  the  test  the  applicant  is  required  to  determine  power  settings and  speeds.  The  applicant  shall  indicate  to   the  examiner  the  checks  and  duties  carried  out, including  the  identification  of  radio  facilities.  Checks  shall  be  completed  in  accordance  with  the Check - List  for  the aircraft  on  which  the  test  is  being  taken  and,  if  applicable,  with  the   MCC concept.  Performance  data  for  Take-off,  approach  and   landing  shall  be  calculated  by   the   applicant in   compliance  with   the   operations  manual (OM) or   flight  manual ( FM ) for   the   aircraft  used.  Decision  heights / altitude  ( DH / A ),  minimum  descent  heights / altitudes  ( MDH / A )  and  missed approach  point  ( MAP ) shall  be  agreed  upon  with  the  Examiner.

12.  The  Examiner  shall  take  no  part  in  the  operation  of  the  aircraft  except  where  intervention  is necessary  in  the  interests  of  safety  or  to  avoid  unacceptable  delay  to  other  traffic.

SPECIFIC   REQUIREMENTS   for  the  SKILL  TEST /  PROFICIENCY  CHECK   for  MULTI - PILOT  AIRCRAFT   TYPE   RATINGS,  for   SINGLE - PILOT  AEROPLANE   TYPE   RATINGS,    when   OPERATED    in   MULTI - PILOT   OPERATIONS,    for    MPL   and   ATPL

13.  The  Skill  Test  for  a  multi - pilot  aircraft ( MPA )  or  a  single - pilot  aeroplane  ( SPA )  when operated  in  multi - pilot  operations  shall  be  performed  in  a  multi - crew  environment. Another  applicant  or  another  type  rated  qualified   pilot  may  function  as  second  pilot.  If   an  aircraft  is  used, the  second  pilot  shall  be  the  examiner  or  an  instructor.

14.  The  applicant  shall  operate  as  PF  during  all  sections  of  the  Skill  Test,  except  for  abnormal  and emergency  procedures,  which  may  be  conducted  as  PF  or  PNF  in  accordance  with  MCC.  The  applicant  for  the  initial  issue  of  a  multi - pilot  aircraft  Type  Rating  or  ATPL  shall  also  demonstrate  the  ability  to  act  as  PNF.  The  applicant  may  choose  either  the  left  hand  or  the  right  hand  seat  for the  Skill  Test  if  all  items  can  be  executed  from  the  selected  seat.

15.  The  following  matters  shall  be  specifically  checked   by  the  examiner  for  applicants  for  the  ATPL or  a  Type  Rating  for  multi - pilot aircraft or  for  multi - pilot operations  in  a  single - pilot  aeroplane extending  to   the  duties  of  a  PIC,  irrespective  of  whether  the  applicant  acts  as  PF  or  PNF :

a )   management  of  crew  cooperation ;

b )   maintaining  a  general  survey  of  the  aircraft  operation by appropriate  supervision ;  and

c )    setting  priorities  and  making  decisions  in  accordance  with  safety  aspects  and  relevant  rules and   regulations  appropriate  to  the  operational  situation,  including   emergencies.

16.  The  test / check  should  be  accomplished  under  IFR,  if  the  IR  rating  is  included,  and  as  far  as possible  be  accomplished  in  a  simulated  commercial  air  transport  environment.  An  essential  element  to  be  checked   is  the  ability  to  plan  and  conduct   the  flight  from  routine  briefing   material.

17.  When  the  Type  Rating  Course  has  included  less  than  2  hours  flight  training on the aircraft,  the Skill  Test  may  be  conducted  in  an  FFS   and  may  be  completed  before  the  flight  training  on  the aircraft.  In  that  case,  a  certificate  of  completion  of  the  type  rating  course  including  the  flight  training on  the  aircraft  shall  be  forwarded  to  the  competent  Authority  before  the  new  Type  Rating  is  entered  in  the  applicant’s  licence.

B.    Specific   requirements   for   the   aeroplane   category

PASS    MARKS

1.  In  the  case  of  single - pilot  aeroplanes,  with  the  exception  of  for  single - pilot  high  performance complex  aeroplanes,  the  applicant  shall  pass  all  sections  of  the  Skill  Test  or  Proficiency  Check.  If   any  item  in  a  section  is  failed,  that  section  is  failed.                                                                                       Failure  in  more  than  1 ( one )  section  will  require  the  applicant  to  take  the  entire  test  or  check  again.                                                                                                                                                                                                                                Any  applicant  failing  only  1 ( one ) section  shall  take  the  failed   section  again.                                                 Failure  in  any  section  of  the  re-test  or  re-check  including  those  sections  that  have  been  passed  at  a previous  attempt  will  require  the  applicant  to  take  the  entire  test  or  check  again.                                              For  single - pilot  multi - engine  aeroplanes,  section  6  of  the  relevant  test  or  check,  addressing asymmetric  flight,  shall  be  passed.

2.  In  the  case  of  multi - pilot  and  single - pilot  high  performance  complex  aeroplanes, the  applicant shall  pass  all  sections  of  the  Skill  Test  or  Proficiency  Check.                                                                            Failure  of  more  than  5 ( five )  items  will  require  the  applicant  to  take  the  entire  test  or  check  again. Any  applicant  failing  5 ( five )  or  less  items  shall  take  the  failed  items  again.                                                    Failure  in  any  item  on  the  re-test  or  re-check  including  those  items  that  have  been  passed  at  a previous  attempt  will  require  the  applicant  to  take  the  entire  check  or  test  again.                                         Section  6  is  not  part  of  the  ATPL  or  MPL  Skill  Test.                                                                                       If   the  applicant  only  fails  or  does  not  take  section  6,  the  Type  Rating  will  be  issued  without  CAT II  or   CAT  III  privileges.                                                                                                                                                             To  extend  the  Type  Rating  privileges  to  CAT  II   or   CAT  III,  the  applicant  shall  pass  the  section  6  on  the  appropriate  type  of  aircraft.

FLIGHT    TEST   TOLERANCE

3.  The  applicant  shall  demonstrate  the  ability  to :

a )  operate  the  aeroplane  within  its  limitations ;

b )  complete  all  maneuvers  with  smoothness  and  accuracy ;

c )   exercise  good  judgement  and  airmanship ;

d )  apply  aeronautical  knowledge ;

e )   maintain  control  of  the  aeroplane  at  all  times  in  such  a  manner  that  the  successful  outcome  of  a  procedure  or  maneuver  is  always  assured ;

f )   understand  and  apply  crew  coordination  and  incapacitation  procedures,  if  applicable ;  and

g )  communicate  effectively with  the  other  crew  members,  if  applicable.

4.  The  following  limits  shall  apply,  corrected  to  make  allowance  for  turbulent  conditions  and  the  handling  qualities  and  performance  of  the  aeroplane  used :

Height

Generally	                                                                       ± 100 feet

Starting  a  Go-around  at  DH ( decision  height ) 	+  50  feet /  – 0 feet

MDH / A  ( minimum  descent  height / altitude ) 	             +  50  feet /  – 0 feet

Tracking

on  radio  aids	                                                                       ±  5 °

Precision  approach	                                     half  scale  deflection, azimuth  and  glide  path

Heading

all  engines  operating	                                                          ±  5 °

with  simulated  engine  failure	                                             ±  10 °

Speed

all  engines  operating	                                                          ±  5  knots

with  simulated  engine  failure	                                              +  10 knots  /  –  5  knots

CONTENT   of   the   TRAINING   /   SKILL   TEST   /   PROFICIENCY   CHECK   (  SPL  )

5.    Single - pilot  aeroplanes,  except  for  high  performance complex  aeroplanes :

a )   The  following  symbols  mean :

P    = Trained  as  PIC  or  Co-pilot  and  as  Pilot  Flying  ( PF )  and  Pilot  Not  Flying  ( PNF ) ;

X    =  Flight  simulators  shall  be   used  for   this   exercise,  if  available,  otherwise  an   aeroplane  shall be   used  if   appropriate  for   the  maneuver  or  procedure ;

P #  =  The  training  shall  be  complemented  by  supervised   aeroplane   inspection ;

b )  The  practical  training  shall  be  conducted  at   least  at   the   training  equipment  level  shown    as  ( P ),   or   may  be  conducted  on  any  higher  level  of  equipment  shown  by  the  arrow  ( ——> )

The  following  abbreviations  are  used  to  indicate  the  training  equipment  used :

A	=   Aeroplane ;

FFS       =   Full   Flight   Simulator ;

FTD      =   Flight  Training  Device   (  including    FNPT  II   for  ME  Class  Rating  ) ;

c )   The  starred  ( * )  items  of   section 3 B  and,  for   multi - engine,  section 6,   shall  be   flown  solely  by  reference  to  instruments  if  revalidation / renewal  of  an  IR  is  included  in  the  Skill  Test  or Proficiency  Check.  If   the  starred  ( * )  items  are  not  flown  solely  by  reference  to  instruments  during  the  Skill  Test  or  Proficiency  Check,  and  when  there  is  no  crediting  of  IR  privileges, the  Class  or  Type  Rating  will  be  restricted  to  VFR  only ;

d )  Section 3 A  shall  be  completed  to  revalidate  a  Type  or  multi - engine  Class  rating,  VFR   only, where  the  required  experience  of  10  route  sectors  within  the  previous  12  months  has  not  been  completed.       Section  3 A   is  not  required  if  section  3 B   is  completed.

e )   Where  the  letter “ M  “  appears  in  the  Skill  Test  or  Proficiency  Check  column  this  will  indicate  the mandatory  exercise  or  a  choice  where  more  than  one  exercise  appears.

f  )    An  FFS  or  an  FNPT  II   shall  be  used  for  practical  training  for  Type  or  multi - engine  Class  Ratings   if  they  form  part  of  an  approved  Class  or  Type  Rating  Course.  The  following  considerations  will  apply  to  the  approval  of  the  course :

( i )       the  qualification of  the  FFS  or  FNPT  II   as  set  out  in  the  relevant  requirements  of               Part - ARA  and  Part - ORA ;

( ii )      the  qualifications of  the  instructors ;

(iii)       the  amount  of  FFS  or  FNPT  II  training  provided  on  the  course ;                  and

(iv)       the  qualifications and  previous  experience  on  similar  types  of  the  pilot  under

training.

g  )   When  a  Skill  Test  or  Proficiency  Check  is  performed  in  multi - pilot  operations,  the  Type  Rating  shall  be  restricted  to  multi - pilot  operations.

| Applicant’s Name | PRACTICAL TRAINING |  |  |  | Class or Type Rating, Skill Test , Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- |
| Single - Pilot Aeroplanes, except for High Performance Complex Aeroplanes |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | A |  | FFS A |  |
| SECTION 1 |  |  |  |  |  |  |
| 1 Departure 1.1 Pre-flight including : Documentation Mass and Balance, Weather briefing, NOTAM |  |  |  |  |  |  |
| 1.2 Pre-start checks : |  |  |  |  |  |  |
| 1.2.1 External | P # |  | P |  |  |  |
| 1.2.2 Internal |  |  | P |  | M |  |
| 1.3 Engine starting : Normal ; Malfunctions | P-----> | ----> | ----> |  | M |  |
| 1.4 Taxiing |  | P-----> | ----> |  | M |  |
| 1.5 Pre-departure Checks : Engine Run-up ( if applicable ) | P-----> | -----> | -----> |  | M |  |
| 1.6 Take-Off Procedure : Normal with Flight Manual flap setting’s ; Crosswind ( if conditions available ) |  | P-----> | -----> |  |  |  |
| 1.7 Climbing : Vx / Vy ; Turns onto headings ; Level Off |  | P-----> | -----> |  | M |  |
| 1.8 ATC liaison – Compliance, R / T procedure |  |  |  |  |  |  |
| SECTION 2 |  | P-----> | -----> |  |  |  |
| 2. Airwork [ VMC ] 2.1 Straight and level flight at various airspeeds including flight at critically low airspeed with and without flaps ( including approach to VMCA when applicable ) |  |  |  |  |  |  |
| 2.2 Steep turns ( 360° left and right, at 45° bank ) |  | P-----> | -----> |  | M |  |
| 2.3 Stalls and recovery : i. clean stall ; ii. approach to Stall in descending turn with bank with approach configuration and power ; iii. approach to Stall in Landing configuration and power ; iv. approach to Stall, climbing turn with Take- Off flap and climb power ( single engine aeroplane only ) |  | P-----> | -----> |  | M |  |
| 2.4 Handling using autopilot and flight director ( may be conducted in Section 3 ) if applicable |  | P-----> | -----> |  | M |  |
| 2.5. ATC liaison – Compliance, R / T :Procedure |  |  |  |  |  |  |
| SECTION 3A |  |  |  |  |  |  |
| 3A. En Route procedures VFR ( see B. 5 ( c ) and ( d ) ) 3A.1 Flight plan, dead reckoning and map reading |  |  |  |  |  |  |
| 3A. 2 Maintenance of Altitude, Heading and Speed |  |  |  |  |  |  |
| 3A. 3 Orientation, timing and revision of ETA’s |  |  |  |  |  |  |
| 3A. 4 Use of radio navigation aids ( if applicable ) |  |  |  |  |  |  |
| 3A. 5 Flight management ( flight log, routine checks including fuel, systems and icing ) |  |  |  |  |  |  |
| 3A. 6 ATC liaison - Compliance, R/T procedure |  |  |  |  |  |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  | Class or Type Rating, Skill Test , Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- |
| Single - Pilot Aeroplanes, except for High Performance Complex Aeroplanes |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | A |  | FFS A |  |
| SECTION 3 B |  | P-----> | ----> |  | M |  |
| 3 B. Instrument flight 3.B.1* Departure : IFR |  |  |  |  |  |  |
| 3.B.2* En - route IFR |  | P-----> | ----> |  | M |  |
| 3.B.3* Holding Procedures |  | P-----> | ----> |  | M |  |
| 3.B.4* ILS to DH / A of 200’ ( 60 m) or to procedure minima ( autopilot may be used to glideslope interception ) |  | P-----> | ----> |  | M |  |
| 3.B.5* Non - precision Approach to MDH / A and MAP |  | P-----> | ----> |  | M |  |
| 3.B.6* Flight exercises including simulated failure of the compass and altitude indicator : rate 1 turns ; recoveries from unusual altitudes. | P-----> | -----> | ----> |  | M |  |
| 3.B.7* Failure of Localiser or Glideslope | P-----> | -----> | -----> |  |  |  |
| 3.B.8* ATC liaison – Compliance, R / T procedure |  |  |  |  |  |  |
| Intentionally left blank |  |  |  |  |  |  |
| SECTION 4 |  | P-----> | -----> |  | M |  |
| 4. Arrival and Landings 4.1 Aerodrome arrival procedure |  |  |  |  |  |  |
| 4.2 Normal landing |  | P-----> | -----> |  | M |  |
| 4.3 Flapless landing |  | P-----> | -----> |  | M |  |
| 4.4 Crosswind landing ( if suitable conditions ) |  | P-----> | -----> |  |  |  |
| 4.5 Approach and landing with idle power from up to 2 000' above the runway ( single - engine aeroplane only ) |  | P-----> | -----> |  |  |  |
| 4.6 Go - around from minimum height |  | P-----> | -----> |  | M |  |
| 4.7 Night Go - around and landing ( if applicable ) | P-----> | -----> | -----> |  |  |  |
| 4.8 ATC liaison - Compliance, R/T procedure |  |  |  |  |  |  |
| SECTION 5 |  |  |  |  |  |  |
| 5. Abnormal and Emergency procedures ( this section may be combined with sections 1 through 4 ) |  |  |  |  |  |  |
| 5.1 Rejected Take-off at a reasonable speed |  | P-----> | -----> |  | M |  |
| 5.2 Simulated engine failure after Take- off ( single - engine aeroplanes only ) |  |  | P |  | M |  |
| 5.3 Simulated forced landing without power ( single - engine aeroplanes only ) |  |  | P |  | M |  |
| 5.4 Simulated emergencies : ( i ) fire or smoke in flight ; ( ii ) systems’ malfunctions as appropriate | P-----> | -----> | -----> |  |  |  |
| 5.5 Engine shutdown and restart ( ME Skill Test only ) ( at a safe altitude if performed in the aircraft ) | P-----> | -----> | -----> |  |  |  |
| 5.6 ATC liaison - Compliance, R/T procedure |  |  |  |  |  |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  | Class or Type Rating, Skill Test , Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- |
| Single - Pilot Aeroplanes, except for High Performance Complex Aeroplanes |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | A |  | FFS A |  |
| SECTION 6 | P-----> | -----> | ----> X |  | M |  |
| 6. Simulated asymmetric flight 6.1* ( this section may be combined with sections 1 through 5 ) Simulated engine failure during Take-off ( at a safe altitude unless carried out in FFS or FNPT II ) |  |  |  |  |  |  |
| 6.2* Asymmetric approach and Go - around | P-----> | -----> | ----> |  | M |  |
| 6.3* Asymmetric approach and full stop landing | P-----> | -----> | ----> |  | M |  |
| 6.4 ATC liaison - Compliance, R/T procedure |  |  |  |  |  |  |

-      cont’d        -

CONTENT   of   the   TRAINING  /  SKILL   TEST  /  PROFICIENCY   CHECK    (  MPL  )

6.   Multi - pilot  aeroplanes  and  single - pilot  high  performance  complex  aeroplanes :

( a )  The  following   symbols   mean :

P    =  Trained  as  PIC   or  Co-pilot  and  as  PF  and  PNF  for  the  issue  of  a  Type  Rating  as  applicable ;

X    =  Simulators  shall  be  used  for  this  exercise,  if  available ;  otherwise  an  aircraft  shall  be  used  if appropriate  for   the  manoeuvre  or  procedure ;

P #   =  The  training  shall  be  complemented  by  supervised   aeroplane  inspection ;

( b )   The  practical  training shall be conducted  at  least  at  the  training equipment level shown  as  ( P ),  or  may be  conducted   up  to  any  higher  equipment  level  shown  by  the  arrow  ( ———> ).

The  following  abbreviations  are  used  to  indicate   the  training  equipment  used :

A	 =   Aeroplane ;

FFS        =   Full  Flight  Simulator ;

FTD     =   Flight  Training  Device ;

OTD       =   Other  Training  Devices

( c )   The  starred  items  ( * )  shall  be  flown  solely  by  reference  to  instruments.  If   this  condition  is  not met  during  the  Skill  Test  or  Proficiency  Check,   the  Type  Rating  will  be  restricted  to  VFR  only.

( d )  Where  the  letter  “ M “  appears  in  the  Skill  Test  or  Proficiency  Check  column  this  will  indicate  the  mandatory  exercise.

( e )   An  FFS  shall  be  used  for  practical  training  and  testing  if  the  FFS  forms  part  of  an  approved Type   Rating  Course.   The  following  considerations will  apply  to  the  approval  of  the  course :

( i )   the  qualification of  the  FFS  or  FNPT  II ;

( ii )  the  qualifications of  the  instructors ;

( iii )  the  amount  of  FFS  or  FNPT  II  training  provided  on  the  course ;           and

( iv )  the  qualifications  and  previous  experience  on  similar  types  of  the  pilot  under  training.

( f )   Maneuvers  and  procedures shall include  MCC  for  multi - pilot  aeroplane  and  for  single - pilot high  performance  complex  aeroplanes  in  multi - pilot  operations ;

( g )  Maneuvers  and  procedures  shall  be  conducted  in single - pilot  role for single - pilot  high performance complex aeroplanes in  single - pilot   operations ;

( h )  In  the  case of  single - pilot  high  performance complex aeroplanes,  when  a  Skill  Test   or  Proficiency  Check  is  performed  in multi - pilot  operations,  the  Type  Rating  shall  be  restricted  to  multi - pilot  operations.  If   privileges  of  single - pilot  are  sought,  the   maneuvers / procedures  in  2.5,  3. 9.  3. 4,   4. 3,   5. 5  and  at  least   one   maneuver  / procedure  from  section  3. 4  have  to  be  completed  in  addition  as  single - pilot.

( i )   In  case  of  a  restricted  Type  Rating  issued  in  accordance  with  FCL. 720. A ( e ),   the  applicants shall  fulfill  the same  requirements  as  other  applicants  for  the  Type  Rating  except  for  the  practical exercises  relating  to  the  Take-off   and   landing  phases.

| Applicant’s Name | PRACTICAL TRAINING |  |  |  |  | ATPL / MPL Type Rating, Skill Test or Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi - Pilot Aeroplanes and Single - Pilot high performance complex Aeroplanes |  |  |  |  |  |  |  |
|  |  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | OTD | FTD | FFS | A |  | FFS A |  |
| SECTION 1 | P |  |  |  |  |  |  |
| 1. Flight preparation 1.1 Performance calculation |  |  |  |  |  |  |  |
| 1.2 Aeroplane external visual inspection ; location of each item and purpose of inspection | P # |  |  | P |  |  |  |
| 1.3 Cockpit inspection |  | P ---> | ---> | ---> |  |  |  |
| 1.4 Use of Checklist prior to starting engines, starting procedures, radio and navigation equipment check, selection and setting of navigation and communication frequencies | P ---> | ---> | ---> | ---> |  | M |  |
| 1.5 Taxiing in compliance with air traffic control or instructions of instructor |  |  | P ---> | ----> |  |  |  |
| 1.6 Before Take-off checks |  | P ---> | ---> | ----> |  | M |  |
| SECTION 2 |  |  | P----> | -----> |  |  |  |
| 2. Take-Off’s : 2.1 Normal T.O. with different flap setting’s, including expedited Take-off |  |  |  |  |  |  |  |
| 2.2* Instrument T. O.; transition to instrument flight is required during rotation or immediately after becoming airborne |  |  | P----> | -----> |  |  |  |
| 2.3 Crosswind Take - off |  |  | P----> | ----> |  |  |  |
| 2.4 Take-off at maximum T. O. mass ( actual or simulated maximum T.O. mass ) |  |  | P----> | -----> |  |  |  |
| 2.5 Take - offs with simulated engine failure : 2.5.1 * shortly after reaching V2 ( in aeroplanes which are not certificated as transport category or commuter category aeroplanes, the engine failure shall not be simulated until reaching a minimum height of 500 ft above runway end. In aeroplanes having the same performance as a transport category aeroplane regarding T. O. mass and density altitude, the instructor may simulate the engine failure shortly after reaching V2 ) |  |  | P----> | -----> |  |  |  |
| 2.5.2 * between V1 and V2 |  |  | P | X |  | M FFS only |  |
| 2.6 Rejected Take-off at a reasonable speed before reaching V1 |  |  | P----> | --->X |  | M |  |
| SECTION 3 |  |  | P----> | -----> |  |  |  |
| 3. Flight Manoeuvres and Procedures 3.1 Turns with and without spoilers |  |  |  |  |  |  |  |
| 3.2 Tuck under and Mach buffets after reaching the critical Mach number, and other specific flight characteristics of the aeroplane ( e. g. Dutch Roll ) |  |  | P----> | --->X An aircraft may not be used for this exercise |  |  |  |
| 3.3 Normal operation of systems and controls engineer’s panel | P----> | ----> | ----> | ----> |  |  |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  |  | ATPL / MPL Type Rating, Skill Test or Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi - Pilot Aeroplanes and Single - Pilot high performance complex Aeroplanes |  |  |  |  |  |  |  |
|  |  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | OTD | FTD | FFS | A |  | FFS A |  |
| 3.4 Normal and abnormal operations of following systems : |  |  |  |  |  | M | A mandatory minimum of 3 abnormal shall be selected from 3.4.0 to 3.4.14 inclusive |
| 3.4.0. Engine ( if necessary propeller ) | P ---> | -----> | -----> | -----> |  |  |  |
| 3.4.1. Pressurisation and air-conditioning | P ---> | ---> | ---> | ---> |  |  |  |
| 3.4.2. Pitot / static system | P ---> | ---> | ---> | ---> |  |  |  |
| 3.4.3. Fuel system | P ---> | -----> | ----> | ----> |  |  |  |
| 3.4.4. Electrical system | P ---> | ----> | ----> | ----> |  |  |  |
| 3.4.5. Hydraulic system | P ---> | -----> | ----> | -----> |  |  |  |
| 3.4.6. Flight control and Trim - system |  | -----> | ----> | -----> |  |  |  |
| 3.4.7 Anti - icing / de - icing system, Glare shield heating |  | -----> | ----> | ----> |  |  |  |
| 3.4.8. Autopilot / Flight Director |  | -----> | ----> | -----> |  | M ( single pilot only ) |  |
| 3.4.9. Stall warning devices or stall avoidance devices, and stability augmentation devices | P----> | -----> | -----> | -----> |  |  |  |
| 3.4.10. Ground proximity warning system, weather radar, radio altimeter, transponder | P----> | P----> | -----> | -----> |  |  |  |
| 3.4.11. Radios, navigation equipment, instruments, flight management system | P----> | -----> | -----> | ----> |  |  |  |
| 3.4.12. Landing gear and brake | P----> | -----> | ----> | -----> |  |  |  |
| 3.4.13. Slat and flap system | P----> | -----> | ----> | ----> |  |  |  |
| 3.4.14. Auxiliary power unit | P----> | ----> | ----> | ----> |  |  |  |
| Intentionally left blank |  |  |  |  |  |  |  |
| 3.6. Abnormal and Emergency procedures |  |  |  |  |  | M | A mandatory minimum of three items shall be selected from 3.6.1 to 3.6.9 inclusive |
| 3.6.1. Fire drills, e. g. engine, APU, cabin, cargo compartment, flight deck, wing and electrical fires including evacuation |  | P----> | -----> | ----> |  |  |  |
| 3.6.2. Smoke control and removal |  | P----> | -----> | ----> |  |  |  |
| 3.6.3. Engine failures, shutdown and restart at a safe height |  | P----> | -----> | ----> |  |  |  |
| 3.6.4. Fuel dumping ( simulated ) |  | P----> | -----> | -----> |  |  |  |
| 3.6.5. Wind shear at Take-off / landing |  |  | P | X |  | FFS only |  |
| 3.6.6. Simulated cabin pressure failure / Emergency descent |  |  | P----> | ----> |  |  |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  |  | ATPL / MPL Type Rating, Skill Test or Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi - Pilot Aeroplanes and Single - Pilot high performance complex Aeroplanes |  |  |  |  |  |  |  |
|  |  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | OTD | FTD | FFS | A |  | FFS A |  |
| 3.6.7. Incapacitation of flight crew member |  | P----> | ----> | ----> |  |  |  |
| 3.6.8. Other emergency procedures as outlined in the appropriate Aeroplane Flight Manual |  | P----> | -----> | -----> |  |  |  |
| 3.6.9. ACAS event | P ---> | ---> | ---> | An aircraft may not be used |  | FFS only |  |
| 3.7. Steep turns with 45° bank, 180° to 360° left and right |  | P ---> | ----> | ----> |  |  |  |
| 3.8. Early recognition and counter measures on approaching stall ( up to activation of stall warning device ) in Take-off configuration ( flaps in Take-off position ), in cruising flight configuration and in landing configuration ( flaps in landing position, gear extended ) |  |  | P----> | ----> |  |  |  |
| 3.8.1. Recovery from full stall or after activation of stall warning device in climb, cruise and approach configuration |  |  | P | X |  |  |  |
| 3.9. Instrument flight procedures |  |  |  |  |  |  |  |
| 3.9.1* Adherence to departure and arrival routes and ATC instructions |  | P----> | ----> | -----> |  | M |  |
| 3.9.2* Holding procedures |  | P----> | ----> | ----> |  |  |  |
| 3.9.3* Precision approaches down to a decision height ( DH ) not less than 60 m ( 200 ft ) |  |  |  |  |  |  |  |
| 3.9.3.1* manually, without FD ( flight director ) |  |  | P----> | -----> |  | M ( skill test only ) |  |
| 3.9.3.2* manually, with FD ( flight director ) |  |  | P----> | -----> |  |  |  |
| 3.9.3.3* with autopilot |  |  | P----> | ----> |  |  |  |
| 3.9.3.4* manually, with one engine simulated inoperative ; engine failure has to be simulated during final approach before passing the outer marker ( OM ) until touchdown or through the complete missed approach procedure In aeroplanes which are not certificated as transport category aeroplanes ( JAR / FAR 25 ) or as commuter category aeroplanes ( SFAR 23 ), the approach with simulated engine failure and the ensuing Go-around shall be initiated in conjunction with the non-precision approach as described in 3.9.4. The Go-around shall be initiated when reaching the published obstacle clearance height ( OCH / A ), however not later than reaching a MDH / A of 500 ft above runway threshold elevation. In aeroplanes having the same performance as a transport category aeroplane regarding take-off mass and density altitude, the instructor may simulate the engine failure in accordance with 3.9.3.4. |  |  | P---> | -----> |  | M |  |
| 3.9.4* Non - precision approach down to the MDH / A |  |  | P*--> | ----> |  | M |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  |  | ATPL / MPL Type Rating, Skill Test or Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi - Pilot Aeroplanes and Single - Pilot high performance complex Aeroplanes |  |  |  |  |  |  |  |
|  |  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | OTD | FTD | FFS | A |  | FFS A |  |
| 3.9.5 Circling approach under following conditions : ( a )* approach to the authorised minimum circling approach altitude at the aerodrome in question in accordance with the local instrument approach facilities in simulated instrument flight conditions ; followed by : ( b ) circling approach to another runway at least 90° off centerline from final approach used in item ( a ), at the authorized minimum circling approach altitude. Remark : if ( a ) and ( b ) are not possible due to ATC reasons, a simulated low visibility pattern may be performed. |  |  | P*---> | ----> |  |  |  |
| SECTION 4 |  |  |  |  |  |  |  |
| 4. Missed Approach Procedures |  |  |  |  |  |  |  |
| 4.1. Go - around with all engines operating * after an ILS approach on reaching DH |  |  | P*---> | ---> |  |  |  |
| 4.2. Other missed approach procedures |  |  | P*---> | ----> |  |  |  |
| 4.3* Manual Go-around with the critical engine simulated inoperative after an instrument approach on reaching DH, MDH or MAP’t |  |  | P*---> | ----> |  | M |  |
| 4.4 Rejected landing at 15 m ( 50 ft ) above runway threshold and Go-around |  |  | P----> | -----> |  |  |  |
| SECTION 5 |  |  |  |  |  |  |  |
| 5. Landing’s |  |  |  |  |  |  |  |
| 5.1. Normal landings* also after an ILS approach with transition to visual flight on reaching DH |  |  | P |  |  |  |  |
| 5.2. Landing with simulated jammed horizontal stabilizer in any out - of - trim position |  |  | P----> | An aircraft may not be used for this exercise |  |  |  |
| 5.3. Crosswind landings ( a / c, if practicable ) |  |  | P----> | -----> |  |  |  |
| 5.4. Traffic pattern and landing without extended or with partly extended flaps and slats |  |  | P----> | ----> |  |  |  |
| 5.5. Landing with critical engine simulated inoperative |  |  | P----> | -----> |  | M |  |
| 5.6. Landing with two engines inoperative : — aeroplanes with 3 engines : the centre engine and 1 outboard engine as far as practicable according to data of the AFM ; — aeroplanes with 4 engines : 2 engines at one side. |  |  | P | X |  | M FFS only ( skill test only ) |  |
| General remarks : Special requirements for extension of a Type Rating for instrument approaches down to a DH ( decision height ) of less than 200 feet ( 60 m ), i. e. Cat II / III operations. |  |  |  |  |  |  |  |

| Applicant’s Name | PRACTICAL TRAINING |  |  |  |  | ATPL / MPL Type Rating, Skill Test or Proficiency Check |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Multi - Pilot Aeroplanes and Single - Pilot high performance complex Aeroplanes |  |  |  |  |  |  |  |
|  |  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | OTD | FTD | FFS | A |  | FFS A |  |
| SECTION 6 |  |  |  |  |  |  |  |
| Additional authorization on a Type Rating for instrument approaches down to a DH decision height of less than 60 m ( 200 ft )( CAT II / III ). The following manoeuvres and procedures are the minimum training requirements to permit instrument approaches down to a DH of less than 60 m ( 200 ft ). During the following instrument approaches and missed approach procedures all aeroplane equipment required for type certification of instrument approaches down to a DH of less than 60 m ( 200 ft ) shall be used. |  |  |  |  |  |  |  |
| 6.1* Rejected Take - off at minimum authorised RVR |  |  | P*---> | --->X an aircraft may not be used for this exercise |  | M* |  |
| 6.2. ILS approaches : in simulated instrument flight conditions down to the applicable DH, using flight guidance system. Standard Procedures of crew coordination ( task sharing, call out procedures, mutual surveillance, information exchange and support ) shall be observed |  |  | P---> | ----> |  | M |  |
| 6.3* Go-around : after approaches as indicated in 6. 2 on reaching DH. The training shall also include a Go- around due to ( simulated ) insufficient RVR, wind shear, aeroplane deviation in excess of approach limits for a successful approach, and ground / airborne equipment failure prior to reaching DH and, Go-around with simulated airborne equipment failure. |  |  | P---> | ----> |  | M* |  |
| 6.4* Landing (s) : with visual reference established at DH following an instrument approach. Depending on the specific flight guidance system, an automatic landing shall be performed |  |  | P ---> | ----> |  | M |  |
| Intentionally left blank |  |  |  |  |  |  |  |
| Intentionally left blank |  |  |  |  |  |  |  |

Note :   CAT  II / III  operations  shall be accomplished  in accordance  with  the applicable air operations  requirements.

7.     Class   Ratings  —  sea.

(  reserved  )

Training,   SKILL   TEST  and   PROFICIENCY  CHECK    for   MPL,  ATPL,

Type  and   Class  Ratings,    and    PROFICIENCY   CHECK   for   IR

(  continued -   Helicopter  )

C.   Specific   Requirements   for   the   Helicopter   Category

1.    In case of  Skill Test or Proficiency Check for Type Ratings and the ATPL the  applicant shall pass sections 1 to 4 and 6 ( as  applicable ) of the Skill Test or Proficiency Check. Failure in more than five  items will require the applicant to take the entire test or check again. An applicant failing not more than five items shall take the failed items again. Failure in any item of the re-test or re-check or failure in any other items already passed will require the applicant to take the entire test or check again. All sections of the Skill Test or Proficiency Check shall be completed within 6 months.

2.  In case of Proficiency Check for an IR the applicant shall pass section 5 of the Proficiency Check. Failure in more than three items will require the applicant to take the entire section 5 again. An applicant failing not more than three items shall take the failed items again. Failure in any item of the re-check or failure in any other items of section 5 already passed will require the applicant to take the entire check again.

FLIGHT   TEST   TOLERANCE

3.  The applicant shall demonstrate the ability to :

a )  operate the helicopter within its limitations ;

b )  complete all maneuvers with smoothness and accuracy ;

c )   exercise  good  judgement  and  airmanship ;

d )  apply aeronautical knowledge ;

e )   maintain control of the helicopter at all times in such a manner that the successful outcome of a procedure  or  maneuver  is  never  in doubt ;

f )   understand  and apply crew coordination and incapacitation procedures, if  applicable ;    and

g )  communicate effectively with the other crew members, if applicable.

4.  The  following  limits  shall  apply,  corrected  to  make  allowance  for  turbulent  conditions  and  the  handling  qualities  and  performance  of  the  helicopter  used.

( a )   IFR   flight   limits

Height :

Generally                                                                                 ± 100 feet

Starting  a  Go - around  at  decision  height / altitude	       +  50 feet / – 0 feet

Minimum  descent   height / altitude	                               +  50 feet / – 0 feet

Tracking :

On  radio  aids :                                                                      ± 5 °

Precision approach :                                               half  scale deflection, azimuth and glide path

Heading :

Normal  operations :	                                                                                           ± 5 °

Abnormal  operations / emergencies :	                               ± 10 °

Speed :

Generally :                                                                               ± 10 knots

With  simulated  engine  failure	                                          +  10 knots  /  – 5 knots

( b )     VFR   flight    limits

Height :

Generally	                                                                             ± 100 feet

Heading:

Normal operations :	                                                                 ± 5 °

Abnormal  operations  /  emergencies	                                         ± 10 °

Speed :

Generally :                                                                             	    ± 10 knots

With simulated  engine  failure :	                                        + 10 knots  /  – 5 knots

Ground  drift :

T.O.  hover  I. G. E.	                                                               ± 3 feet

Landing	                                                                                       ± 2 feet

(  with  0  feet  rearward  or  lateral  flight  )

CONTENT  of   the   TRAINING  /  SKILL    TEST  /  PROFICIENCY   CHECK   for    SPL ( H )  /  MPL  ( H )

GENERAL

5.  The following  symbols mean :

P  =  Trained as PIC for the issue of a Type Rating for SPH or trained as PIC or Co-pilot and  as

PF  and  PNF for the issue of a Type Rating for MPH.

6.  The practical training shall be conducted at least at the training equipment level shown as ( P ), or may be conducted  up to any higher equipment level shown by the arrow ( ——> ).

The following abbreviations are used to indicate the training equipment used :

FFS     =  Full  Flight  Simulator ;

FTD    =  Flight Training Device ;

H         =  Helicopter

7.  The starred items ( * ) shall be flown in actual or simulated IMC, only by applicants wishing to renew or revalidate an  IR ( H ),  or extend  the  privileges of that rating to another type.

8.  Instrument flight procedures ( section 5 ) shall be performed only by applicants wishing to renew or revalidate  an  IR ( H )  or extend  the  privileges of that rating to another type. An FFS  or FTD 2 / 3  may be used for this purpose.

9.  Where the letter “ M “  appears in the Skill Test or Proficiency Check column this will  indicate  the mandatory exercise.

10.  An FSTD shall be used for practical training and testing if the  FSTD forms part of a Type Rating Course. The following considerations will apply to the course :

( i )    the qualification of  the  FSTD  as set out  in  the  relevant  requirements  of  Part - ARA  and Part - ORA ;

( ii )   the qualifications of  the instructor and  examiner ;

( iii )  the amount of  FSTD  training provided on the course;

( iv )  the  qualifications and previous experience in similar types of the pilot under training ;   and

( v )   the amount of supervised flying experience provided after the issue of the new Type Rating.

MULTI - PILOT   HELICOPTERS

11.  Applicants for the Skill Test for the issue of the multi-pilot helicopter type rating and ATPL ( H ) shall take only sections  1  to 4  and,  if  applicable, section 6.

12.  Applicants for the revalidation or renewal of the multi-pilot helicopter Type Rating / Proficiency Check shall take only sections 1  to  4 and,  if  applicable,  section 6.

| Applicant’s Name | Practical Training |  |  |  | SKILL TEST or PROFICIENCY CHECK |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINGLE / MULTI - PILOT HELICOPTERS |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | H |  | FFS H |  |
| SECTION 1 --- Pre - flight Preparations and Checks |  |  |  |  |  |  |
| - Helicopter exterior visual inspection ; location of each item and purpose of inspection |  |  | P |  | M ( if performed in the helicopter ) |  |
| 1.2 Cockpit inspection |  | P | -----> |  | M |  |
| 1.3. Starting procedures, radio and navigation equipmen check, selection and setting of navi ­ gation and communication frequencies | P | -----> | -----> |  | M |  |
| 1.4. Taxiing / air taxiing in compliance with air traffic control instructions or with instructions of an instructor |  | P | -----> |  | M |  |
| 1.5. Pre - Take-off procedures and checks | P | -----> | -----> |  | M |  |
| SECTION 2 --- Flight Manoeuvres and Procedures |  |  |  |  |  |  |
| 2.1. Take - offs ( various profiles ) |  | P | -----> |  | M |  |
| 2.2. Sloping ground or cross wind Take - offs & landings |  | P | -----> |  |  |  |
| 2.3. Take-off at maximum Ttake-off mass ( actual or simulated maximum Take-off mass ) | P | ------> | ------> |  |  |  |
| 2.4. Take-off with simulated engine failure shortly before reaching TDP or DPATO |  | P | ------> |  | M |  |
| 2.4.1. Take - off with simulated engine failure shortly after reaching TDP or DPATO |  | P | -----> |  | M |  |
| 2.5. Climbing and descending turns to specified headings | P | -----> | -----> |  | M |  |
| 2.5.1. Turns with 30 ° bank, 180 ° to 360 ° left and right, by sole reference to instruments | P | -----> | -----> |  | M |  |
| 2.6. Autorotative descent | P | -----> | -----> |  | M |  |
| 2.6.1. Autorotative landing ( SEH only) or power recovery |  | P | -----> |  | M |  |
| 2.7. Landings, various profiles |  | P | -----> |  | M |  |
| 2.7.1. Go - around or landing following simulated engine failure before LDP or DPBL |  | P | -----> |  | M |  |
| 2.7.2. Landing following simulated engine failure after LDP or DPBL |  | P | -----> |  | M |  |
| SECTION 3 --- Normal and abnormal operations of the following systems and procedures |  |  |  |  |  |  |
| 3. Normal and abnormal operations of the following systems and procedures : |  |  |  |  | M | A mandatory minimum of three items shall be selected from this section |
| 3.1. Engine | P | -----> | -----> |  |  |  |
| 3.2. Air conditioning ( heating, ventilation ) | P | -----> | -----> |  |  |  |
| 3.3. Pitot / static system | P | -----> | -----> |  |  |  |
| 3.4. Fuel system | P | -----> | -----> |  |  |  |
| 3.5. Electrical system | P | -----> | -----> |  |  |  |
| 3.6. Hydraulic system | P | -----> | -----> |  |  |  |
| 3.7. Flight Control and Trim system | P | -----> | -----> |  |  |  |

| Applicant’s Name | Practical Training |  |  |  | SKILL TEST or PROFICIENCY CHECK |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINGLE / MULTI - PILOT HELICOPTERS |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | H |  | FFS H |  |
| 3.8. Anti - icing and de - icing system | P | -----> | -----> |  |  |  |
| 3.9. Autopilot / Flight Director | P | -----> | -----> |  |  |  |
| 3.10. Stability augmentation devices | P | -----> | -----> |  |  |  |
| 3.11. Weather radar, radio altimeter, transponder | P | ------> | -----> |  |  |  |
| 3.12. Area Navigation System | P | ------> | -----> |  |  |  |
| 3.13. Landing gear system | P | ------> | -----> |  |  |  |
| 3.14. Auxiliary Power Unit | P | ------> | -----> |  |  |  |
| 3.15. Radio, navigation equipment, instruments flight management system | P | ------> | ------> |  |  |  |
| SECTION 4 --- Abnormal and Emergency Procedures |  |  |  |  |  |  |
| 4. Abnormal and emergency procedures |  |  |  |  | M | A mandatory minimum of three items shall be selected from this section |
| 4.1. Fire drills ( including evacuation, if applicable ) | P | -----> | -----> |  |  |  |
| 4.2. Smoke control and removal | P | -----> | -----> |  |  |  |
| 4.3. Engine failures, shutdown and restart at a safe height | P | -----> | -----> |  |  |  |
| 4.4. Fuel dumping ( simulated ) | P | -----> | -----> |  |  |  |
| 4.5. Tail rotor control failure ( if applicable ) | P | -----> | -----> |  |  |  |
| 4.5.1. Tail rotor loss ( if applicable ) | P | -----> | Helicopter may not beused for this exercise |  |  |  |
| 4.6. Incapacitation of crew member - MPH only | P | -----> | -----> |  |  |  |
| 4.7. Transmission malfunctions | P | -----> | -----> |  |  |  |
| 4.8. Other emergency procedures as outlined in the appropriate Flight Manual | P | -----> | -----> |  |  |  |
| SECTION 5 --- Instrument flight procedures ( to be performed in IMC or simulated IMC ) |  |  |  |  |  |  |
| 5.1. Instrument Take - off : transition to instrument flight is required as soon as possible after becoming airborne | P* | ----->* | ----->* |  |  |  |
| 5.1.1. Simulated engine failure during departure | P* | ----->* | ----->* |  | M* |  |
| 5.2. Adherence to departure and arrival routes and ATC instructions | P* | ----->* | ----->* |  | M* |  |
| 5.3. Holding procedures | P* | ----->* | ----->* |  |  |  |
| 5.4. ILS approaches down to CAT I DH ( Decision Height ) | P* | ----->* | ----->* |  |  |  |
| 5.4.1. Manually, without FD ( flight director ) | P* | ----->* | ----->* |  | M* |  |
| 5.4.2. Precision approach manually, with or without FD ( flight director ) | P* | ----->* | ----->* |  | M* |  |
| 5.4.3. With coupled autopilot | P* | ----->* | ----->* |  |  |  |
| 5.4.4. Manually, with one engine simulated inoperative. ( Engine failure has to be simulated during final approach before passing the outer marker ( OM ) until touchdown or until comple- tion of the missed approach procedure ) | P* | ----->* | ----->* |  | M* |  |

| Applicant’s Name | Practical Training |  |  |  | SKILL TEST or PROFICIENCY CHECK |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINGLE / MULTI - PILOT HELICOPTERS |  |  |  |  |  |  |
|  |  |  |  | Instructors initials when training completed | Checked in | Examiners initials when test completed |
| Manoeuvres / Procedures | FTD | FFS | H |  | FFS H |  |
| Left Intentionally Blank |  |  |  |  |  |  |
| 5.5. Non - precision approach down to the MDA / H ( minimum descent altitude ) | P* | ----->* | ----->* |  | M* |  |
| 5.6. Go - around with all engines operating on reaching DA / DH or MDA / MDH | P* | ----->* | ----->* |  |  |  |
| 5.6.1. Other missed approach procedures | P* | ----->* | ----->* |  |  |  |
| 5.6.2. Go - around with one engine simulated inoperative on reaching DA / DH or MDA / MDH | P* |  |  |  | M* |  |
| 5.7. IMC autorotation with power recovery | P* | ----->* | ----->* |  | M* |  |
| 5.8. Recovery from unusual attitudes | P* | ----->* | ----->* |  | M* |  |
| SECTION 6 --- Use of optional equipment |  |  |  |  |  |  |
| 6. Use of optional equipment | P | ------> | -----> |  |  |  |
| Left Intentionally Blank |  |  |  |  |  |  |
| Left Intentionally Blank |  |  |  |  |  |  |
| Left Intentionally Blank |  |  |  |  |  |  |
| Left Intentionally Blank |  |  |  |  |  |  |
| Left Intentionally Blank |  |  |  |  |  |  |

D.   Specific  requirements  for  the  powered - lift  aircraft   category

(  reserved  )

E.     Specific  requirements  for  the  airship   category

(  reserved  )