Appendix   3

Training    Courses    for    the    issue    of    a    CPL    and    an    ATPL

1.  This  Appendix  describes  the  requirements  for  the  different  types  of  Training  Courses  for  the  issue   of  a  CPL  and  an  ATPL,  with  and   without   an  IR.

2.  An  applicant  wishing  to  transfer  to  another  ATO  during  a  training  course  shall  apply  to  the  competent  Authority  for  a  formal  assessment  of  the  further  hours  of  training   required

A.    ATP   Integrated   Course  —   Aeroplanes

GENERAL

1. The  aim  of  the  ATP ( A )  Integrated  Course  is  to  train  pilots  to  the  level  of  proficiency          necessary   to  enable  them  to  operate  as  co-pilot  on  multi - pilot   multi - engine   aeroplanes   in      commercial   air   transport   and   to  obtain  the  CPL ( A ) / IR.

2. An  applicant  wishing  to  undertake  an  ATP ( A )   Integrated  Course  shall   complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3. An  applicant  may  be  admitted  to  training  either  as  an   ab - initio  entrant,  or  as  a  holder  of   a   PPL ( A )  or  PPL ( H )  issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.              In  the  case  of  a  PPL( A )  or  PPL( H )  entrant,  50 %  of  the  hours  flown  prior  to  the  course  shall  be  credited,  up to  a  maximum  of  40  hours  flying  experience,  or  45  hours  if  an  aeroplane  night  rating  has  been obtained,  of  which  up  to  20  hours  may  count  towards  the  requirement  for  dual  instruction  flight  time.

4. The  course  shall  comprise :

( a )  theoretical   knowledge  instruction   to  the  ATPL ( A )  knowledge  level ;

( b )  visual   and   instrument   flying    training ;                     and

( c )   training   in  MCC  for  the  operation  of  multi - pilot  aeroplanes.

5.  An  applicant  failing  or  unable  to  complete  the  entire  ATP ( A )  Course  may  apply  to  the competent  Authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with lower  privileges   and  an  IR   if  the  applicable   requirements  are  met.

THEORETICAL    KNOWLEDGE

6. An  ATP( A )  theoretical  knowledge  course  shall  comprise  at  least  750  hrs  of  instruction.

7.  The  MCC  course  shall  comprise  at  least  25  hrs  of  theoretical  knowledge  instruction  and exercises.

THEORETICAL    KNOWLEDGE      EXAMINATION

8.  An  applicant  shall  demonstrate  the  level  of  knowledge  appropriate  to  the  privileges granted  to  the  holder  of  an  ATPL ( A ).

FLYING   TRAINING

9.  The  flying  training,  not  including  Type  Rating  Training,  shall  comprise  a  total  of  at  least  195  hours,  to  include  all  progress  tests,  of  which   up  to  55  hours  for  the  entire course  may  be  Instrument  Ground  Time.  Within   the  total  of  195 hours,  applicants  shall complete  at  least :

a )  95  hours  of  dual  instruction,  of  which  up  to  55  hours  may  be  Instrument  Ground Time ;

b )  70  hours  as  PIC,  including  VFR   flight  and  instrument  flight  time  as  Student  Pilot-in-Command  ( SPIC ).   The  instrument  flight  time  as  SPIC  shall  only  be  counted  as  PIC  flight time  up  to  a  maximum  of   20  hours ;

c )   50  hours  of  cross - country   flight  as  PIC,  including  a  VFR  cross-country  flight  of  at  least  540  km ( 300  nm ),   in  the  course  of  which  full  stop  landings  at  two  aerodromes  different  from  the  aerodrome  of  departure  shall  be  made ;

d )  5  hours  flight  time  shall  be  completed  at  night,  comprising  3  hours  of  dual instruction,  which  will  include  at  least  1 hour  of  cross-country  navigation   and  5  solo  Take-offs    and  5  solo  full  stop  landings ;                                                    and

e )   115  hours  of  instrument   time   comprising,   at  least:

1 )   20  hours  as  SPIC ;

2 )   15  hours  MCC,  for  which  an  FFS  or  FNPT  II  may  be  used ;

3 )    50  hours  of   instrument  flight   instruction,  of  which  up  to :

( i )    25  hours  may  be  instrument  ground  time  in  a  FNPT  I;          or

( ii )   40  hours  may  be  Instrument  Ground  Time  in  a  FNPT  II,  FTD  2  or  FFS,  of

which  up  to  10  hours  may  be  conducted  in  an  FNPT  I.                                                                                                               An  applicant  holding  a  course  completion  certificate  for  the  Basic  Instrument  Flight  Module  shall   be  credited  with  up  to  10  hours  towards  the  required  instrument  instruction  time.  Hours  done  in  a  BITD shall  not  be  credited ;

f  )   5  hours  to  be  carried  out  in  an  aeroplane  certificated  for  the  carriage  of  at  least  4  persons  that has  a  variable  pitch  propeller  and  retractable  landing  gear.

SKILL    TEST

10.   Upon  completion  of  the  related  flying  training,  the  applicant  shall  take  the  CPL ( A )  Skill   Test  on  either  a  single-engine  or  a  multi-engine   aeroplane  and  the  IR  Skill  Test  on  a  multi-engine aeroplane.

B.     ATP    Modular    Course   —   Aeroplanes

1. Applicants  for  an  ATPL ( A )  who  complete  their  theoretical  knowledge  instruction  at  a modular  course  shall :

a )  hold  at  least  a  PPL ( A )  issued  in  accordance   with  Annex 1 to  the  Chicago  Convention ;                       and   complete  at  least  the  following   hours of  theoretical  knowledge  instruction :

1 )   for  applicants  holding  a  PPL ( A ) :   650  hours ;

2 )   for  applicants  holding  a  CPL ( A ) :   400  hours ;

3 )   for  applicants  holding  an  IR ( A ) :    500  hours ;

4 )   for  applicants  holding  a  CPL ( A )   and   an  IR ( A ) :    250  hours.

The  theoretical  knowledge  instruction  shall  be  completed  before  the  Skill  Test  for  the  ATPL ( A )  is taken.

C.      CPL  /  IR   Integrated    Course   —   Aeroplanes

GENERAL

1.  The  aim  of  the  CPL ( A )  and  IR ( A )  Integrated  Course  is  to  train  pilots  to  the  level  of proficiency  necessary  to  operate  single - pilot  single - engine  or  multi - engine  aeroplanes  in  commercial air  transport  and  to  obtain   the  CPL ( A )  /  IR.

2.  An  applicant  wishing  to  undertake  a  CPL ( A )  /  IR  Integrated  Course  shall  complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3.  An  applicant   may  be  admitted  to  training  either  as  an   ab - initio  entrant,  or  as  a  holder  of  a   PPL ( A )  or  PPL ( H )  issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.    In  the  case  of  a  PPL ( A )  or  PPL( H )  entrant, 50 %  of  the  hours  flown  prior to  the  course  shall  be  credited,  up to  a  maximum  of  40   hours  flying  experience,  or  45  hours  if  an  aeroplane   night  rating  has  been obtained,  of  which  up  to  20  hours  may  count  towards  the  requirement  for  dual  instruction  flight time.

4.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( A )  and  IR  knowledge  level ;        and

b )  visual  and  instrument   flying   training.

5.  An  applicant  failing  or  unable  to  complete  the  entire  CPL  /  IR ( A )  course  may  apply  to  the competent  Authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with lower  privileges  and  an  IR  if  the  applicable   requirements  are  met.

THEORETICAL     KNOWLEDGE

6.  A  CPL ( A )  / IR  theoretical  knowledge  course  shall  comprise  at  least  500  hours  of  instruction.

THEORETICAL     KNOWLEDGE     EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of  knowledge  appropriate  to  the  privileges  granted  to  the holder  of  a  CPL ( A )  and  an  IR.

FLYING    TRAINING

8.  The  flying  training,  not  including  Type  Rating  Training,  shall  comprise  a  total  of  at  least  180  hours,  to  include  all  progress  tests,  of  which  up  to  40  hours  for  the  entire  course  may  be Instrument  Ground  Time.   Within  the  total  of  180  hours,  applicants  shall  complete  at  least :

a )  80 hrs  of  dual  instruction,  of  which  up  to  40 hours  may  be  instrument  ground  time ;

b )  70  hours  as  PIC, including  VFR  flight  and  instrument  flight  time  which  may  be  flown  as  SPIC.  The  instrument  flight  time  as  SPIC  shall  only  be  counted  as  PIC  flight  time  up  to a  maximum  of  20  hrs ;

c )   50  hours  of  cross - country  flight  as  PIC,  including  a  VFR  cross-country  flight  of  at  least               270  km  ( 150 nm ),  in  the  course  of  which  full  stop  landings  at  two  aerodromes  different   from  the  aerodrome  of  departure  shall  be  made ;

d )  5  hours  flight  time  shall  be  completed  at  night,  comprising  3  hours  of  dual  instruction,  which shall  include  at  least  1 hour  of  cross-country  navigation  and  5 solo  Take-offs   and  5 solo  full  stop landings ;                                          and

e )   100  hours  of  instrument   time  comprising,  at  least :

1 )  20  hours  as  SPIC ;                     and

2 )  50  hours  of  instrument  flight  instruction,  of  which  up  to :

( i )     25  hours  may  be  Instrument  Ground   Time   in   an   FNPT  I ;            or

( ii )    40  hours  may  be  instrument  ground  time  in  an  FNPT II,  FTD 2  or  FFS,  of  which  up  to  10  hours  may  be  conducted   in  an  FNPT  I.                                                                                                                An  applicant  holding  a  course  completion  certificate  for  the  Basic  Instrument   Flight  Module  shall  be  credited  with  up  to  10  hours  towards   the  required  instrument  instruction  time.  Hours  done  in  a  BITD  shall  not  be  credited ;

f  )   5  hours  to  be  carried  out  in  an  aeroplane  certificated  for  the  carriage  of  at  least  4  persons  that has  a  variable  pitch  propeller  and  retractable  landing  gear.

SKILL    TEST

9.  Upon  completion  of  the  related  flying   training   the  applicant  shall  take  the  CPL ( A )  Skill  Test  and  the  IR  Skill  Test  on  either  a  multi - engine   aeroplane  or  a  single - engine aeroplane.

D.      CPL   Integrated    Course   —   Aeroplanes

GENERAL

1.  The  aim  of  the  CPL ( A )   Integrated  Course  is  to  train  pilots  to  the  level  of  proficiency necessary   for  the  issue  of  a  CPL ( A ).

2.  An  applicant  wishing  to  undertake  a  CPL ( A )   Integrated  Course  shall  complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged   by  an  ATO.

3.  An  applicant  may  be  admitted  to  training  either  as  an  ab - initio  entrant,  or  as  a  holder  of  a   PPL ( A )  or  PPL ( H )  issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.               In  the  case  of  a  PPL ( A )  or  PPL ( H )  entrant,  50 %  of  the  hours  flown  prior  to  the  course  shall  be  credited,  up to  a  maximum  of  40  hours  flying  experience,  or  45 hours  if  an  aeroplane  night  rating  has  been obtained,  of  which  up  to  20  hours  may  count  towards  the  requirement  for  dual  instruction  flight time.

4.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( A ) knowledge  level ;           and

b )  visual  and  instrument  flying   training.

5.  An  applicant  failing  or  unable  to  complete  the  entire  CPL ( A )  course  may  apply  to  the competent  Authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with  lower privileges,  if  the  applicable  requirements  are  met.

THEORETICAL     KNOWLEDGE

6.  A  CPL ( A )  theoretical  knowledge  course  shall  comprise  at  least  350  hours  of  instruction.

THEORETICAL     KNOWLEDGE     EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of  knowledge  appropriate  to  the  privileges  granted  to  the holder  of  a  CPL ( A ).

FLYING    TRAINING

8.  The  flying  training,  not  including  Type  Rating  Training,  shall  comprise  a  total  of  at  least  150  hours,  to  include  all  progress  tests,  of  which  up  to  5  hours  for  the  entire  course  may  be Instrument  Ground  Time.  Within  the  total  of   150  hours,  applicants  shall  complete  at  least :

a )  80  hours  of  dual  instruction,  of  which  up  to  5 hrs  may  be  Instrument  Ground  Time ;

b )  70  hours  as  PIC ;

c )   20  hours  of  cross - country  flight  as  PIC,  including  a  VFR  cross-country  flight  of  at  least  270 km ( 150 nm ),  in  the  course  of  which  full  stop  landings  at  two  aerodromes  different  from  the  aerodrome  of  departure  shall  be  made ;

d )  5  hours  flight  time  shall  be  completed  at  night,  comprising  3  hours  of  dual  instruction,  which shall  include  at  least  1 hour  of  cross-country  navigation  and  5  solo  Take-offs  and  5  solo  full  stop landings ;

e )   10  hours  of  instrument  flight   instruction,  of  which  up  to  5  hours  may  be   Instrument Ground  Time  in  an  FNPT I,  FTD 2,  FNPT  II   or   FFS.    An  applicant   holding  a  course completion  certificate  for  the  Basic  Instrument  Flight  Module  shall  be  credited   with  up  to 10  hours  towards  the  required   instrument   instruction   time.  Hours   done  in  a  BITD  shall  not be  credited ;

f  )   5  hours  to  be  carried  out  in  an  aeroplane  certificated  for  the  carriage  of  at  least  ( 4 ) four  persons  that  has  a  variable  pitch  propeller  and  retractable  landing  gear.

SKILL    TEST

9.  Upon  completion  of  the  flying   training  the  applicant  shall  take  the  CPL ( A )  Skill   Test  on  a  single - engine  or  a  multi - engine   aeroplane.

E.       CPL     Modular     Course   —   Aeroplanes

GENERAL

1.  The  aim  of  the  CPL ( A )  Modular  Course  is  to  train  PPL ( A )  holders  to  the  level  of  proficiency necessary  for  the  issue  of  a  CPL ( A ).

2.  Before  commencing  a  CPL ( A )  modular  course  an  applicant  shall  be  the  holder  of  a  PPL ( A ) issued  in  accordance  with  Annex  1  to  the  Chicago   Convention.

3.  Before  commencing  the flight  training  the  applicant  shall :

a )   have  completed   150  hours  flight  time ;

b )   have  complied  with  the  prerequisites  for  the  issue  of  a  Class  or   Type  Rating  for  multi- engine  aeroplanes  in  accordance   with  Subpart  H,  if  a  multi-engine   aeroplane  is  to  be  used  on  the  Skill  Test.

4.  An  applicant  wishing  to  undertake  a  modular  CPL ( A )  course  shall  complete  all  the  flight instructional  stages  in  one  continuous  course  of  training  as  arranged   by  an  ATO.  The  theoretical knowledge  instruction  may  be  given  at  an  ATO  conducting  theoretical  knowledge  instruction  only.

5.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( A )   knowledge  level ;                  and

b )  visual  and  instrument  flying   training.

THEORETICAL     KNOWLEDGE

6.  An  approved  CPL ( A )  theoretical  knowledge  course  shall  comprise  at  least  250  hours  of instruction.

THEORETICAL     KNOWLEDGE      EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of  knowledge  appropriate  to  the  privileges  granted  to  the holder  of  a  CPL ( A ).

FLYING    TRAINING

8.  Applicants  without  an  IR  shall  be  given  at  least  25  hours  dual  flight  instruction,  including  10  hours  of  instrument   instruction  of  which  up  to  5  hours   may  be  Instrument  Ground   Time  in  a BITD,  an  FNPT I  or  II,  an  FTD 2  or  an   FFS.

9.  Applicants  holding  a  valid   IR ( A )   shall  be  fully  credited  towards  the  dual  instrument instruction  time.  Applicants  holding  a  valid   IR ( H )  shall  be  credited   up  to  5  hours  of  the  dual instrument   instruction  time,  in  which  case  at  least  5  hours  dual  instrument  instruction  time  shall  be given  in  an  aeroplane.   An  applicant  holding  a  Course  Completion  Certificate  for  the  Basic  Instrument Flight  Module  shall  be  credited  with  up  to  10  hours  towards  the  required   instrument   instruction time.

10.

a )  Applicants  with  a  valid  IR  shall  be  given  at  least  15  hrs  dual  visual  flight  instruction.

b )  Applicants  without  a  night  rating  aeroplane  shall  be  given  additionally  at  least  5  hrs night  flight  instruction,  comprising   3  hours  of  dual  instruction,  which  shall  include  at  least                     1  hr  of  cross-country  navigation  and   5 solo  Take-offs   and  5 solo  full  stop  landings.

11.  At  least   5  hours  of  the  flight  instruction  shall  be  carried  out  in  an  aeroplane  certificated  for  the  carriage  of  at  least  ( 4 ) four  persons  and  have  a  variable   pitch  propeller  and  retractable  landing  gear.

EXPERIENCE

12.  The  applicant  for  a  CPL ( A )  shall  have  completed  at  least  200  hours  flight  time,  including  at least :

a )  100  hours  as  PIC,   of  which  20  hours  of  cross-country  flight  as  PIC,  which  shall  include  a VFR  cross-country  flight  of  at  least  270 km ( 150 nm ),  in  the  course  of  which  full  stop  landings  at two  aerodromes  different  from  the  aerodrome  of  departure  shall  be  made ;

b )   5  hours  of  flight  time  shall  be  completed   at  night,  comprising  3  hours  of  dual  instruction, which  shall  include  at  least  1 hour  of  cross-country  navigation  and  5 solo  Take-offs  and  5 solo  full stop  landings ;                  and

c )   10  hours  of  instrument  flight  instruction,  of  which  up  to  5  hours  may  be  Instrument  Ground Time  in  an  FNPT I,  or  FNPT  II  or  FFS.  An  applicant  holding  a  course  completion  certificate  for  the Basic  Instrument  Flight  Module  shall  be  credited  with  up  to  10  hours  towards  the  required  instrument  instruction  time.  Hours  done  in  a  BITD  shall  not  be  credited ;

d )   6  hours  of  flight  time  shall  be  completed   in  a  multi-engine  aeroplane, if  a  multi-engine aeroplane  is  used  for  the  skill  test ;

e )    Hours  as  PIC  of  other  categories  of  aircraft   may  count   towards   the  200  hours  flight   time,  in  the  following  cases :

( i )    30  hours  in  helicopter,  if  the  applicant  holds  a  PPL ( H ) ;       or

( ii )   100  hours  in  helicopters,  if  the  applicant  holds  a  CPL ( H ) ;    or

( iii )  30  hours  in  TMGs  or  sailplanes ;                                                         or

( iv )  30  hours  in  airships,   if  the  applicant  holds  a  PPL ( As ) ;     or

( v )    60  hours  in  airships,  if  the  applicant  holds  a  CPL ( As ).

SKILL    TEST

13.   Upon  completion  of  the  flying  training  and  relevant  experience  requirements  the  applicant shall  take  the  CPL ( A )  Skill   Test  on  either  a  single - engine  or  a   multi - engine   aeroplane.

F.       ATP  /  IR    Integrated    Course   —   Helicopters

GENERAL

1.  The  aim  of  the  ATP ( H ) / IR  Integrated  Course  is  to  train  pilots  to  the  level  of  proficiency necessary  to  enable  them  to  operate  as  co-pilot  on  multi-pilot  multi-engine   helicopters   in  commercial air  transport  and  to  obtain   the  CPL ( H ) / IR.

2.  An  applicant  wishing   to  undertake  an  ATP ( H ) / IR  Integrated  Course  shall  complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3.  An  applicant  may  be  admitted  to   training  either  as  an  ab-initio   entrant,  or   as  a  holder  of  a PPL ( H )   issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.  In  the  case  of  a  PPL ( H ) entrant,  50 %   of  the  relevant  experience  shall  be  credited,  up  to  a  maximum  of :

a )   40  hours,  of  which  up  to  20  hours  may  be  dual   instruction ;                   or

b )   50   hours,  of  which  up  to  25  hours  may  be  dual  instruction,  if  a  helicopter  night  rating  has  been  obtained.

4.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  the  ATPL ( H )  and  IR  knowledge  level ;

b )  visual  and  instrument  flying   training ;                                 and

c )   training  in  MCC  for  the  operation   of   multi - pilot   helicopters.

5.  An  applicant  failing  or  unable  to  complete  the  entire  ATP ( H ) / IR  course  may  apply  to  the competent  Authority  for  the  theoretical  knowledge  examination  and  Skill   Test  for  a  licence  with lower  privileges   and  an  IR,  if  the  applicable  requirements  are  met.

THEORETICAL     KNOWLEDGE

6.  An  ATP ( H ) / IR  theoretical  knowledge  course  shall  comprise  at  least  750  hours  of  instruction.

7.  The  MCC  course  shall  comprise  at  least  25  hours  of  theoretical  knowledge  instruction  exercises.

THEORETICAL     KNOWLEDGE      EXAMINATION

8.  An  applicant  shall  demonstrate  the  level  of  knowledge  appropriate  to  the  privileges  granted  to the  holder  of  an   ATPL ( H )  and  an  IR.

FLYING    TRAINING

9.  The  flying  training  shall  comprise  a  total  of  at  least  195  hours,  to  include  all  Progress  Tests. Within  the  total  of  195  hours,  applicants  shall  complete  at  least :

a )  140  hours  of  dual  instruction,  of  which :

1 )  75  hours  visual  instruction  may  include :

( i )     30  hours  in  a  helicopter  FFS,  level  C / D ;                or

( ii )    25  hours  in  a  FTD  2, 3 ;                                            or

( iii )   20  hours  in  a  helicopter  FNPT  II / III ;                         or

( iv )  20  hours  in  an  aeroplane  or  TMG ;

2 )  50  hours  instrument  instruction  may  include :

(  i )    up  to  20  hours   in  a  helicopter  FFS  or  FTD  2, 3  or  FNPT  II / III ;         or

(  ii )   10  hours  in  at  least  a  helicopter  FNPT  1  or  an  aeroplane ;

3 )  15  hours  MCC,  for  which  a  helicopter  FFS  or  helicopter  FTD  2, 3 ( MCC )  or  FNPT  II / III   ( MCC )  may  be  used.  If  the  helicopter  used  for  the  flying  training  is  of  a  different  type  from  the  helicopter  FFS  used  for  the  visual  training,  the  maximum  credit  shall  be  limited  to  that  allocated  for the  helicopter  FNPT  II / III ;

b )  55  hrs  as  PIC,   of  which  40  hours  may  be  as  SPIC.  At  least  14  hrs  solo  day  and  1 hr  solo  night  shall  be  made ;

c )   50   hrs  of  cross - country  flight,  including  at  least  10  hrs  of  cross - country  flight  as  SPIC  including  a  VFR  cross - country  flight  of  at  least  270  km ( 150 nm )  in  the  course  of  which  landings  at  two  different  aerodromes  from  the  aerodrome  of  departure  shall  be  made ;

d )  5  hours  flight   time  in  helicopters  shall  be completed  at  night  comprising   3  hours  of  dual instruction  including   at  least  1 hr  of  cross - country  navigation  and  5  solo  night  circuits.  Each  circuit  shall  include  a  Take - off   and  a  landing ;

e )   50  hours  of  dual   instrument   time  comprising :

(  i )    10  hours  basic   instrument   instruction  time ;                      and

(  ii )   40   hours  IR  Training,  which  shall  include  at  least  10  hours  in  a  multi-engine  IFR - certificated  helicopter.

SKILL    TEST

10.   Upon  completion  of  the  related  flying  training,  the  applicant  shall  take  the  CPL ( H )  Skill Test  on  a  multi-engine  helicopter  and  the  IR  Skill  Test  on  an  IFR  certificated  multi-engine   helicopter  and  shall  comply  with  the  requirements  for  MCC  training.

G.    ATP    Integrated     Course   —   Helicopter

GENERAL

1.  The  aim  of  the  ATP ( H )  Integrated  Course  is  to  train  pilots  to  the  level  of proficiency  necessary   to  enable  them  to  operate  as  co-pilot  on  multi - pilot   multi - engine helicopters    limited    to   VFR   privileges    in   commercial    air   transport   and   to  obtain   the  CPL ( H ).

2.  An  applicant  wishing  to  undertake  an  ATP ( H )  Integrated  Course  shall  complete  all  the  instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3. An  applicant  may  be  admitted  to   training  either  as  an   ab-initio   entrant,  or   as  a holder  of  a  PPL ( H )   issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.  In  the case  of  a  PPL ( H )  entrant,  50 %  of  the  relevant  experience  shall  be  credited,  up  to  a maximum  of :

a )  40  hours,  of  which   up  to  20  hours  may  be  dual  instruction ;         or

b )  50   hours,  of  which  up  to  25  hours  may  be  dual  instruction,   if  a  helicopter  night rating  has  been  obtained.

4.  The  course  shall  comprise :

a )  theoretical  knowledge   instruction  to  the  ATPL ( H )   knowledge  level ;

b )  visual  and  basic  instrument  flying   training ;                                                  and

c )   training   in  MCC  for  the  operation   of  multi - pilot   helicopters.

5.  An  applicant  failing   or  unable  to  complete  the  entire  ATP ( H )  course  may  apply  to  the competent  authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with  lower  privileges,  if  the  applicable  requirements  are  met.

THEORETICAL     KNOWLEDGE

6.  An  ATP ( H )  theoretical  knowledge  course  shall  comprise  at  least  650  hours  of instruction.

7.  The  MCC  course  shall  comprise  at  least  20  hours  of  theoretical  knowledge  instruction exercises.

THEORETICAL     KNOWLEDGE        EXAMINATION

8.  An  applicant  shall  demonstrate  the  level  of  knowledge  appropriate  to  the  privileges granted  to  the  holder  of  an   ATPL  ( H ).

FLYING    TRAINING

9.  The  flying  training  shall  comprise  a  total  of  at  least  150  hours,  to  include  all  progress  tests.   Within  the  total  of  150  hours,  applicants   shall  complete  at  least :

a )  95  hours  of  dual  instruction,  of  which :

1 )    75  hours  visual  instruction  may  include :

( i )     30  hours  in  a  helicopter  FFS  level  C / D ;                or

( ii )   25  hours  in  a  helicopter  FTD  2, 3 ;                        or

( iii )  20   hours  in  a  helicopter   FNPT  II / III ;                or

( iv )  20  hours  in  an  aeroplane  or  TMG ;

2 )  10  hrs  basic  instrument  instruction   may  include  5  hrs  in  at  least  a  helicopter  FNPT  I  or  an  aeroplane ;

3 )   10  hrs  MCC,  for  which  a  helicopter :   helicopter  FFS  or  FTD  2, 3  ( MCC )  or  FNPT  II  /  III  ( MCC )  may  be  used.  If   the  helicopter  used  for  the  flying  training  is  of  a  different   type  from  the  helicopter  FFS  used  for  the  visual  training,  the  maximum  credit  shall  be  limited  to  that  allocated   for the  helicopter   FNPT  II / III ;

b )  55  hrs  as  PIC,  of  which   40  hrs  may  be  as  SPIC.  At  least  14  hrs  solo  day  and  1 hr solo  night  shall  be  made ;

c )   50  hrs  of  cross - country  flight,  including   at  least  10  hrs  of  cross-country  flight  as  SPIC,  including  a VFR  cross - country  flight  of  at  least  270 km ( 150 nm )  in  the  course  of  which  landings  at  two  different  aerodromes  from  the  aerodrome  of  departure  shall  be  made ;

d )   5  hrs  flight  time  in  helicopters  shall   be  completed   at  night  comprising   3  hrs  of  dual instruction  including   at  least  1 hr  of  cross-country  navigation  and  5  solo  night  circuits.  Each  circuit  shall  include   a  Take-off   and  a  landing.

SKILL    TEST

10.  Upon  completion  of  the  related  flying  training  the  applicant  shall  take  the  CPL ( H )  Skill Test  on  a  multi-engine  helicopter  and  comply  with  MCC  requirements.

H.    ATP    Modular    Course   —   Helicopters

GENERAL

1.  Applicants  for  an  ATPL ( H )  who  complete  their  theoretical  knowledge  instruction  at  a  modular Course  shall  hold  at  least  a  PPL ( H )  and  complete  at  least  the  following  hours  of  instruction  within  a  period  of  18  months :

a )  for  applicants  holding  a  PPL ( H )  issued  in  accordance  with  Annex  1  to  the  Chicago Convention :    -   550  hours ;

b )  for  applicants  holding  a  CPL ( H ) :   -  300  hours.

2.  Applicants  for  an  ATPL ( H ) / IR  who  complete  their  theoretical  knowledge  instruction  at  a  Modular  Course   shall   hold  at  least  a  PPL ( H )  and  complete  at  least  the  following   hours of  instruction :

( a )  for  applicants  holding  a  PPL ( H ) :                         -  650  hours ;

( b )   for  applicants  holding  a  CPL ( H ) :                            -   400  hours ;

( c )   for  applicants  holding  an  IR ( H ) :                     -  500  hours ;

( d )  for  applicants  holding  a  CPL ( H )  and  an  IR ( H ) :  -  250  hours.

I.     CPL  /  IR      Integrated      Course   —   Helicopters

GENERAL

1.  The  aim  of  the  CPL ( H ) / IR  integrated  course  is  to  train  pilots  to  the  level  of  proficiency  necessary  to  operate  single- pilot  multi-engine  helicopters  and  to  obtain  the  CPL ( H ) / IR  multi-engine  helicopter.

2.  An  applicant  wishing  to  undertake  a  CPL ( H ) / IR  integrated  course  shall  complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3.  An  applicant  may  be  admitted  to  training  either  as  an  ab-initio   entrant,  or   as  a  holder  of  a PPL ( H )   issued  in  accordance  with  Annex  1  to  the  Chicago  Convention.  In  the  case  of  an entrant  holding  a  PPL( H ),  50 %  of  the  relevant  experience  shall  be  credited,  up  to  a  maximum  of :

a )  40  hours,  of  which  up  to  20  hours  may  be  dual  instruction ;     or

b )  50   hours,  of  which  up  to  25  hours  may  be  dual  instruction,  if  a  helicopter  night  rating has  been  obtained.

4.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( H )  and  IR  knowledge  level,  and  the  initial  multi-engine  helicopter  type  rating ;               and

b )  visual  and  instrument  flying   training.

5.  An  applicant  failing  or  unable  to  complete  the  entire  CPL ( H ) / IR  course  may  apply  to  the competent  authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with lower  privileges  and  an  IR,  if  the  applicable  requirements  are   met.

THEORETICAL     KNOWLEDGE

6.  A  CPL ( H ) / IR  theoretical  knowledge  course  shall  comprise  at  least  500  hours  of instruction.

THEORETICAL     KNOWLEDGE       EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of  knowledge  appropriate  to  the  privileges granted  to  the holder  of  a  CPL ( H )  and  an  IR.

FLYING    TRAINING

8. The flying training shall comprise a total of at least 180 hours including all progress tests. Within the 180 hours, applicants shall complete at least:

a )  125  hours  of  dual   instruction,  of  which :

1 )   75  hours  visual  instruction,  which  may  include :

( i )   30  hours  in  a  helicopter   FFS  level  C / D ;              or

( ii )   25  hours  in  a  helicopter  FTD  2, 3 ;                     or

( iii ) 20  hours  in  a  helicopter  FNPT  II / III ;           or

( iv ) 20  hours  in  an  aeroplane  or  TMG ;

2 )   50  hours  instrument  instruction  which  may  include :

( i )   up  to  20  hours  in  a  helicopter  FFS  or  FTD  2, 3,  or  FNPT   II,  III ;           or

( ii )  10  hours  in  at  least  a  helicopter  FNPT  I  or  an  aeroplane.

If  the  helicopter  used  for  the  flying  training  is  of  a  different  type  from  the  FFS  used  for  the  visual training,  the  maximum  credit  shall  be  limited  to  that  allocated  for  the  FNPT  II / III ;

b )  55  hrs  as  PIC,  of  which  40  hrs  may  be  as  SPIC.  At  least  14  hrs  solo  day  and  1 hr  solo  night  shall  be  made ;

c )   10  hrs  dual  cross - country  flying ;

d )  10  hrs  of  cross - country  flight  as  PIC,  including  a  VFR  cross - country  flight  of  at  least              270  km ( 150 nm )  in  the  course  of  which  full  stop  landings  at  two  different  aerodromes  from  the  aerodrome  of  departure  shall  be  made ;

e )   5  hours  of  flight  time  in  helicopters  shall  be  completed  at  night  comprising  3  hours  of  dual instruction  including  at  least  1 hour  of  cross-country  navigation  and  5  solo  night  circuits.   Each  circuit  shall  include   a  take-off  and  a  landing ;

f  )    50  hours  of  dual  instrument  time  comprising :

(  i )   10  hours  basic  instrument  instruction   time ;                 and

(  ii )  40   hours  IR  Training,  which  shall  include  at  least  10  hours  in  a  multi-engine  IFR-certificated  helicopter.

SKILL    TEST

9.  Upon  completion  of  the  related   flying  training,  the  applicant  shall  take   the  CPL ( H )  Skill  Test  on  either  a  multi-engine  or  a  single-engine  helicopter  and  the  IR  Skill  Test  on  an  IFR - certificated multi - engine   helicopter.

J.    CPL   integrated     course   —   Helicopters

GENERAL

1.  The  aim  of  the  CPL ( H )  Integrated  Course  is  to  train  pilots  to  the  level  of  proficiency  necessary  for  the  issue  of  a  CPL ( H ).

2.  An  applicant  wishing  to  undertake  a  CPL ( H )   Integrated  Course  shall  complete  all  the instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.

3.  An  applicant  may  be  admitted  to   training  either  as  an   ab-initio   entrant,  or   as  a holder  of  a PPL ( H )  issued  in  accordance  with  Annex 1  to  the  Chicago  Convention. In  the  case  of  an  entrant  holding  a  PPL ( H ),  50 %   of  the  relevant  experience  shall  be  credited,  up  to  a  maximum  of :

a )  40  hours,  of  which  up  to  20  hours  may  be  dual  instruction ;      or

b )  50   hours,  of  which  up  to  25  hours  may  be  dual  instruction  if  a  helicopter  night  rating  has  been  obtained.

4.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( H )  knowledge  level ;          and

b )  visual  and  instrument  flying   training.

5.  An  applicant  failing  or  unable  to  complete  the  entire  CPL ( H )  course  may  apply  to  the competent   Authority  for  the  theoretical  knowledge  examination  and  Skill  Test  for  a  licence  with  lower privileges,  if  the  applicable  requirements  are  met.

THEORETICAL     KNOWLEDGE

6.  An  approved  CPL ( H )  theoretical  knowledge  course  shall  comprise  at  least  350  hours  of instruction  or  200  hours  if  the  applicant  is  the  holder  of  a  PPL.

THEORETICAL     KNOWLEDGE      EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of  knowledge  appropriate  to  the  privileges  granted  to  the  holder  of  a  CPL ( H ).

FLYING    TRAINING

8.  The  flying  training  shall  comprise  a  total  of  at  least  135  hours,  to  include  all  Progress Tests,  of  which  up  to  5  hours  may  be  instrument  ground  time.  Within  the  135  hours  total, applicants  shall  complete  at  least :

a )  85  hours  of  dual  instruction,  of  which :

1 )   up  to  75  hours  may  be  visual  instruction,  and  may  include :

( i )    30  hours  in  a  helicopter  FFS  level  C / D ;      or

( ii )   25  hours  in  a  helicopter  FTD  2, 3 ;                 or

( iii )  20  hours  in  a  helicopter  FNPT  II / III;           or

( iv )  20  hours  in  an  aeroplane   or  TMG ;

2 )   up  to  10  hours  may  be  instrument  instruction,  and  may  include  5  hours  in  at  least  a  helicopter  FNPT  I  or  an  aeroplane.

If  the  helicopter  used  for  the  flying  training  is  of  a  different  type  from  the  FFS  used  for  the visual  training,  the  maximum  credit  shall  be  limited  to  that  allocated  for  the  FNPT  II / III ;

b )  50  hours  as  PIC,  of  which  35  hours   may  be  as  SPIC.   At  least  14  hours  solo  day  and            1 hour  solo  night  shall  be  made ;

c )   10  hours  dual  cross - country  flying ;

d )  10  hours  of  cross - country  flight   as  PIC,  including  a  VFR  cross - country  flight  of  at  least  270  km ( 150 nm )  in  the  course  of  which  full  stop  landings  at  two  different  aerodromes  from  the  aerodrome  of  departure  shall  be  made ;

e )   5  hours  flight  time  in  helicopters  shall  be  completed   at  night  comprising   3  hours  of  dual instruction  including   at  least  1 hour  of cross-country  navigation  and  5 solo  night  circuits.  Each  circuit  shall  include  a  take-off   and  a  landing;

f  )   10 hours  of  instrument  dual  instruction  time,  including  at  least   5  hours  in  a  helicopter.

SKILL    TEST

9.  Upon  completion  of  the  related  flying   training,  the  applicant  shall  take  the  CPL ( H )  Skill  Test.

K.     CPL    Modular    Course  —  Helicopters

GENERAL

1.  The  aim  of  the  CPL ( H )  Modular  Course  is  to  train  PPL ( H )  holders  to  the  level  of  proficiency necessary  for  the  issue  of  a  CPL ( H ).

2.  Before  commencing  a  CPL ( H )  Modular  Course  an  applicant  shall  be  the  holder  of  a  PPL ( H ) issued  in  accordance  with   Annex  1  to  the   Chicago  Convention.

3.  Before  commencing  the  flight  training  the  applicant  shall :

a )  have  completed  155  hours  flight  time,  including  50  hours  as  PIC  in  helicopters  of  which  10  hours  shall  be  cross-country.  Hours  as  PIC  of  other  categories  of  aircraft  may  count  towards  the  155  hours  flight  time  as  prescribed  in  paragraph  11  of  Section  K ;

b )  have  complied  with  FCL. 725  and   FCL. 720. H   if  a  multi - engine  helicopter  is  to  be  used  on the  Skill  Test.

4.  An  applicant  wishing  to  undertake  a  modular  CPL ( H )  course  shall  complete  all  the  flight instructional  stages  in  one  continuous  course  of  training  as  arranged  by  an  ATO.  The  theoretical knowledge  instruction  may  be  given  at  an  ATO  that  conducts  theoretical  knowledge  instruction  only.

5.  The  course  shall  comprise :

a )  theoretical  knowledge  instruction  to  CPL ( H )  knowledge  level ;         and

b )  visual  and  instrument  flying   training.

THEORETICAL     KNOWLEDGE

6.  An  approved   CPL ( H )  theoretical  knowledge  course  shall  comprise  at  least   250  hours  of instruction.

THEORETICAL     KNOWLEDGE       EXAMINATION

7.  An  applicant  shall  demonstrate  a  level  of   knowledge  appropriate  to  the  privileges  granted  to  the holder  of  a  CPL ( H ).

FLYING    TRAINING

8.  Applicants  without  an  IR  shall  be  given  at  least  30  hours  dual  flight  instruction,  of  which :

a )  20  hours visual  instruction,  which  may  include  5  hours  in  a  helicopter  FFS  or  FTD  2, 3  or  FNPT  II,  III ;                                                                       and

b )  10  hours  instrument  instruction,  which  may  include  5  hours  in  at  least  a  helicopter  FTD 1  or  FNPT  I   or  aeroplane.

9.  Applicants  holding  a  valid  IR ( H )  shall  be  fully  credited  towards  the  dual  instrument  instruction time.  Applicants  holding  a  valid  IR ( A )  shall  complete  at  least  5  hours  of  the  dual  instrument instruction  time  in  a  helicopter.

10.   Applicants  without  a  night  rating  helicopter  shall  be  given  additionally  at  least  5  hours  night flight  instruction  comprising  3  hrs  of  dual  instruction  including   at  least  1  hr  of  cross - country navigation  and  5  solo  night   circuits.  Each  circuit   shall  include  a  Take-off   and  a  landing.

EXPERIENCE

11.   The  applicant  for  a  CPL ( H )  shall  have  completed  at  least  185  hours  flight  time,  including  50 hours  as  PIC,  of  which  10  hours  of  cross - country  flight  as  PIC,  including  a VFR  cross - country flight  of  at  least  185 km ( 100 nm ),  in  the  course  of  which  full  stop  landings  at  two  aerodromes different  from  the  aerodrome  of  departure  shall  be  made.

Hours  as  pilot-in-command  of  other  categories  of  aircraft  may  count  towards  the  185  hours  flight time,  in  the  following  cases :

a )  20  hours  in  aeroplanes,  if  the  applicant   holds  a  PPL ( A ) ;   or

b )  50  hours  in  aeroplanes,  if  the  applicant   holds  a  CPL ( A ) ;   or

c )   10  hours  in  TMGs   or  sailplanes ;                                                       or

d )  20  hours  in  airships,  if  the  applicant   holds  a  PPL ( As ) ;     or

e )  50  hours  in  airships,  if  the  applicant  holds  a  CPL ( As ).

SKILL    TEST

12.   Upon  completion  of  the  related  flying   training  and  relevant   experience,  the  applicant  shall  take the  CPL ( H )  Skill   Test.

L.     CPL  /  IR     Integrated     Course  —  Airships

(  reserved  )

M.    CPL    Integrated     Course  —  Airships

(  reserved  )

N.     CPL    Modular     Course  —  Airships

(  reserved  )

INTENTIONALLY            LEFT            BLANK