SUBPART   C.    PRIVATE   PILOT   LICENCE  ( PPL ),

SAILPLANE   PILOT   LICENCE  ( SPL )   and

BALLOON   PILOT   LICENCE  ( BPL ).

AMC  1.    FCL. 210 ;    FCL. 215

SYLLABUS   of   THEORETICAL   KNOWLEDGE   for   the   PPL ( A )   and   PPL ( H )

The  following  Tables  contain  the  syllabi  for  the  courses  of  theoretical  knowledge,  as  well  as for  the  theoretical  knowledge  examinations  for  the  PPL ( A )  and  PPL ( H ).  The  training  and examination  should  cover  aspects  related  to  non-technical  skills  in  an  integrated  manner,  taking into  account  the  particular  risks  associated  to  the  licence  and  the  activity.  An  approved  course shall  comprise  at  least  100  hours  of  theoretical  knowledge  instruction.  This  theoretical knowledge  instruction  provided  by  the  ATO  should  include  a  certain  element  of  formal classroom  work  but  may  include  also  such  facilities  as  interactive  video,  slide  or  tape presentation,  computer-based  training  and  other  media  distance  learning  courses.  The  training organization  responsible  for  the  training  has  to  check  if  all  the  appropriate  elements  of  the training  course  of  theoretical  knowledge  instruction  have  been  completed  to  a  satisfactory standard  before  recommending  the  applicant  for  the  examination.

The  applicable  items  for  each  licence  are  marked  with  “ x “.  An  “ x “  on  the  main  title  of  a subject  means  that  all  the  sub-divisions  are  applicable.

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 1. | AIR LAW and ATC PROCEDURES | PPL | Bridge Course | PPL | Bridge Course |
|  | International Law : Conventions, Agreements and Organizations |  |  |  |  |
|  | The Convention on International Civil Aviation ( Chicago ) Doc. 7300 / 6 |  |  |  |  |
|  | Part I. Air Navigation : relevant parts of the following Chapters : a) general principles and application of the convention ; b) flight over territory of Contracting States ; c) nationality of aircraft ; d) measures to facilitate air navigation ; e) conditions to be fulfilled on aircraft ; f) international standards and recommended practices ; g) validity of endorsed certificates and licences ; h) notification of differences. | x |  | x |  |
|  | Part II. The International Civil Aviation Organization ( ICAO ) : Objectives and Composition | x |  | x |  |
|  | Annex 8. Airworthiness of Aircraft |  |  |  |  |
|  | Foreword and Definitions | x |  | x |  |
|  | Certificate of Airworthiness | x |  | x |  |
|  | Annex 7. Aircraft Nationality & Registration Marks |  |  |  |  |
|  | Foreword and Definitions | x |  | x |  |
|  | Common - and Registration Marks | x |  | x |  |
|  | Certificate of Registration and Aircraft Nationality | x |  | x |  |
|  | Annex 1. Personnel Licensing |  |  |  |  |
|  | Definitions |  |  |  |  |
|  | Relevant parts of Annex 1 connected to Part- FCL and Part-MED |  |  |  |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 1. | AIR LAW and ATC PROCEDURES | PPL | Bridge Course | PPL | Bridge Course |
|  | Annex 2. Rules of the Air |  |  |  |  |
|  | Essential definitions, applicability of the rules of the air, general rules ( except water operations ), visual flight rules, signals and interception of civil aircraft | x |  | x |  |
|  | Procedures for Air Navigation : aircraft operations Doc. 8168 - OPS / 611, Volume 1 |  |  |  |  |
|  | Altimeter Setting Procedures ( including ICAO Doc. 7030 – regional supplementary procedures ) |  |  |  |  |
|  | Basic requirements ( except tables ), procedures applicable to operators and pilots ( except tables ) | x |  | x |  |
|  | Secondary Surveillance Radar Transponder Operating Procedures ( including ICAO Doc. 7030 ) |  |  |  |  |
|  | Operation of Transponders | x |  | x |  |
|  | Phraseology | x |  | x |  |
|  | Annex 11, Doc. 4444 Air Traffic Management |  |  |  |  |
|  | Definitions | x |  | x |  |
|  | General provisions for air traffic services | x |  | x |  |
|  | Visual separation in the vicinity of aerodromes | x |  | x |  |
|  | Procedures for aerodrome control services | x |  | x |  |
|  | Radar services | x |  | x |  |
|  | Flight information service and alerting service | x |  | x |  |
|  | Phraseologies | x |  | x |  |
|  | Procedures related to emergencies, communication failure and contingencies | x |  | x |  |
|  | Annex 15. Aeronautical Information Service |  |  |  |  |
|  | Introduction, Essential Definitions | x |  | x |  |
|  | AIP, NOTAM, AIRAC and AIC | x |  | x |  |
|  | Annex 14. Volume 1 an& 2 : Aerodromes | x |  | x |  |
|  | Definitions | x |  | x |  |
|  | Aerodrome Data : conditions of the movement area and related facilities | x |  | x |  |
|  | Visual aids for navigation : a) indicators and signaling devices ; b) markings; c) lights ; d) signs ; e) markers. | x |  | x |  |
|  | Visual aids for denoting obstacles : a) marking of objects ; b) lighting of objects. | x |  | x |  |
|  | Visual aids for denoting restricted use of areas | x |  | x |  |
|  | Emergency and other services : a) rescue and fire fighting ; b) apron management service. | x |  | x |  |
|  | Annex 12. Search and Rescue |  |  |  |  |
|  | Essential Definitions | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 1. | AIR LAW and ATC PROCEDURES | PPL | Bridge Course | PPL | Bridge Course |
|  | Annex 12. Search and Rescue - [ continued ] |  |  |  |  |
|  | Operating Procedures : a) procedures for PIC at the scene of an accident ; b) procedures for PIC intercepting a distress transmission ; c) search and rescue signals. | x |  | x |  |
|  | Search and Rescue Signals : a) signals with surface craft ; b) ground or air visual signal code ; c) air or ground signals. | x |  | x |  |
|  | Annex 17. Security |  |  |  |  |
|  | General : aims and objectives | x |  | x |  |
|  | Annex 13. Aircraft Accident Investigation |  |  |  |  |
|  | Essential Definitions | x |  | x |  |
|  | Applicability | x |  | x |  |
|  | National Law |  |  |  |  |
|  | National law and differences to relevant ICAO Annexes and relevant EU regulations. | x |  | x |  |
| 2. | HUMAN PERFORMANCE |  |  |  |  |
|  | Human Factors : Basic Concepts |  |  |  |  |
|  | Human Factors in Aviation |  |  |  |  |
|  | Becoming a competent pilot | x |  | x |  |
|  | Basic Aviation Physiology and Health Maintenance |  |  |  |  |
|  | The atmosphere : a ) composition ; b ) gas laws. | x |  | x |  |
|  | Respiratory and circulatory systems : a ) oxygen requirement of tissues ; b ) functional anatomy ; c ) main forms of hypoxia ( hypoxic and anaemic ) : 1) sources, effects and counter- measures of carbon monoxide ; 2) counter measures and hypoxia ; 3) symptoms of hypoxia. d ) hyperventilation ; e ) the effects of accelerations on the circulatory system ; f ) hypertension and coronary heart disease. | x |  | x |  |
|  | Man and Environment |  |  |  |  |
|  | Central, peripheral and autonomic nervous systems | x |  | x |  |
|  | Vision : a ) functional anatomy ; b ) visual field, foveal and peripheral vision ; c ) binocular and monocular vision ; d ) monocular vision cues ; e ) night vision ; f ) visual scanning and detection techniques and importance of “ look - out “ ; g ) defective vision. | x |  | x |  |
|  |  | x |  | x |  |
|  |  | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 2. | HUMAN PERFORMANCE | PPL | Bridge Course | PPL | Bridge Course |
|  | Man and Environment - [ continued ] |  |  |  |  |
|  | Hearing : a ) descriptive and functional anatomy ; b ) flight related hazards to hearing ; c ) hearing loss. | x |  | x |  |
|  | Equilibrium : a ) functional anatomy ; b ) motion and acceleration ; c ) motion sickness. | x |  | x |  |
|  | Integration of sensory inputs : a ) spatial disorientation : forms, recognition and avoidance ; b ) illusions : forms, recognition and avoidance : 1 ) physical origin ; 2 ) physiological origin ; c ) approach and landing problems. | x |  | x |  |
|  | Health and Hygiene |  |  |  |  |
|  | Personal Hygiene : personal fitness | x |  | x |  |
|  | Body rhythm and sleep : a ) rhythm disturbances ; b ) symptoms, effects and management. | x |  | x |  |
|  | Problem areas for pilots : a ) common minor ailments including cold, influenza and gastro-intestinal upset ; b ) entrapped gases and barotrauma, ( scuba diving ) ; c ) obesity; d ) food hygiene ; e ) infectious diseases ; f ) nutrition ; g ) various toxic gases and materials. | x |  | x |  |
|  | Intoxication : a ) prescribed medication ; b ) tobacco ; c ) alcohol and drugs ; d ) caffeine ; e ) self - medication. | x |  | x |  |
|  | Basic Aviation Psychology |  |  |  |  |
|  | Human Information Processing |  |  |  |  |
|  | Attention and vigilance : a ) selectivity of attention ; b ) divided attention. | x |  | x |  |
|  | Perception : a ) perceptual illusions ; b ) subjectivity of perception ; c ) processes of perception. | x |  | x |  |
|  | Memory : a ) sensory memory ; b ) working or short term memory ; c ) long term memory to include motor memory ( skills ). | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 2. | HUMAN PERFORMANCE | PPL | Bridge Course | PPL | Bridge Course |
|  | Human Error and Reliability |  |  |  |  |
|  | Reliability of human behaviour | x |  | x |  |
|  | Error generation : social environment ( group, organization ) | x |  | x |  |
|  | Decision Making |  |  |  |  |
|  | Decision - making concepts : a ) structure ( phases ) ; b ) limits ; c ) risk assessment ; d ) practical application. | x |  | x |  |
|  | Avoiding and Managing Errors : Cockpit Management |  |  |  |  |
|  | Safety awareness : a ) risk area awareness ; b ) situational awareness. | x |  | x |  |
|  | Communication : verbal and non - verbal communication | x |  | x |  |
|  | Human Behaviour |  |  |  |  |
|  | Personality and attitudes : a ) development ; b ) environmental influences. | x |  | x |  |
|  | Identification of hazardous attitudes ( error proneness ) | x |  | x |  |
|  | Human Overload and Under-load |  |  |  |  |
|  | Arousal | x |  | x |  |
|  | Stress : a ) definition(s) ; b ) anxiety and stress ; c ) effects of stress. | x |  | x |  |
|  | Fatigue and stress management : a ) types, causes and symptoms of fatigue ; b ) effects of fatigue ; c ) coping strategies ; d ) management techniques ; e ) health and fitness programmes | x |  | x |  |
| 3. | METEOROLOGY |  |  |  |  |
|  | The Atmosphere |  |  |  |  |
|  | Composition, Extent and Vertical Division |  |  |  |  |
|  | Structure of the atmosphere | x |  | x |  |
|  | Troposphere | x |  | x |  |
|  | Air Temperature | x |  | x |  |
|  | Definition and units | x |  | x |  |
|  | Vertical distribution of temperature | x |  | x |  |
|  | Transfer of heat | x |  | x |  |
|  | Lapse rates, stability and instability | x |  | x |  |
|  | Development of inversions and types of inversions | x |  | x |  |
|  | Temperature near the earth’s surface, surface effects, diurnal and seasonal variation, effect of clouds and effect of wind | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 3. | METEOROLOGY | PPL | Bridge Course | PPL | Bridge Course |
|  | Atmospheric Pressure |  |  |  |  |
|  | Barometric pressure and isobars | x |  | x |  |
|  | Pressure variation with height | x |  | x |  |
|  | Reduction of pressure to mean sea level | x |  | x |  |
|  | Relationship between surface pressure centres and pressure centres aloft | x |  | x |  |
|  | Air Density |  |  |  |  |
|  | Relationship between pressure, temperature and density | x |  | x |  |
|  | ISA | x |  | x |  |
|  | ICAO Standard Atmosphere | x |  | x |  |
|  | Altimetry |  |  |  |  |
|  | Terminology and Definitions | x |  | x |  |
|  | Altimeter and altimeter settings | x |  | x |  |
|  | Calculations | x |  | x |  |
|  | Effect of accelerated airflow due to topography | x |  | x |  |
|  | Wind |  |  |  |  |
|  | Definition and measurement of wind |  |  |  |  |
|  | Definition and measurement | x |  | x |  |
|  | Primary cause of wind |  |  |  |  |
|  | Primary cause of wind, pressure gradient, coriolis force and gradient wind | x |  | x |  |
|  | Variation of wind in the friction layer | x |  | x |  |
|  | Effects of convergence and divergence | x |  | x |  |
| 4. | COMMUNICATIONS |  |  |  |  |
|  | VFR Communications |  |  |  |  |
|  | Definitions |  |  |  |  |
|  | Meanings and significance of associated terms | x |  | x |  |
|  | ATS abbreviations | x |  | x |  |
|  | Q - code groups commonly used in RTF air - ground communications | x |  | x |  |
|  | Categories of messages | x |  | x |  |
|  | General Operating Procedures |  |  |  |  |
|  | Transmission of letters | x |  | x |  |
|  | Transmission of numbers ( including level information ) | x |  | x |  |
|  | Transmission of time | x |  | x |  |
|  | Transmission technique | x |  | x |  |
|  | Standard words and phrases ( relevant RTF phraseology included ) | x |  | x |  |
|  | R / T call signs for aeronautical stations including use of abbreviated call signs | x |  | x |  |
|  | R / T call signs for aircraft including use of abbreviated call signs | x |  | x |  |
|  | Transfer of communication | x |  | x |  |
|  | Test procedures including readability scale | x |  | x |  |
|  | Read back and acknowledgement requirements | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 4. | COMMUNICATIONS | PPL | Bridge Course | PPL | Bridge Course |
|  | Relevant Weather Information Terms ( VFR ) |  |  |  |  |
|  | Aerodrome weather | x |  | x |  |
|  | Weather broadcast | x |  | x |  |
|  | Action required to be taken in case of communication failure | x |  | x |  |
|  | Distress and Urgency Procedures |  |  |  |  |
|  | Distress ( definition, frequencies, watch of distress frequencies, distress signal and distress message ) | x |  | x |  |
|  | Urgency ( definition, frequencies, urgency signal and urgency message ) | x |  | x |  |
|  | General principles of VHF propagation and allocation of frequencies | x |  | x |  |
| 5. | PRINCIPLES of FLIGHT |  |  |  |  |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane |  |  |  |  |
|  | Subsonic Aerodynamics |  |  |  |  |
|  | Basics Concepts, Laws and Definitions |  |  |  |  |
|  | Laws and definitions : a ) conversion of units ; b ) Newton´s laws ; c ) Bernoulli’s equation and venture ; d ) static pressure, dynamic pressure and total pressure ; e ) density ; f ) IAS and TAS. | x |  | x |  |
|  | Basics about airflow : a ) streamline ; b ) two - dimensional airflow ; c ) three - dimensional airflow. | x |  | x |  |
|  | Aerodynamic forces on surfaces : a ) resulting airforce ; b ) lift ; c ) drag ; d ) angle of attack. | x |  | x |  |
|  | Shape of an aerofoil section: a ) thickness to chord ratio ; b ) chord line ; c ) camber line ; d ) camber ; e ) angle of attack. | x |  | x |  |
|  | The wing shape : a ) aspect ratio ; b ) root chord ; c ) tip chord ; d ) tapered wings ; e ) wing plan-form. | x |  | x |  |
|  | The two-dimensional airflow about an aerofoil |  |  |  |  |
|  | Streamline pattern | x |  | x |  |
|  | Stagnation point | x |  | x |  |
|  | Pressure distribution | x |  | x |  |
|  | Centre of pressure | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane | PPL | Bridge Course | PPL | Bridge Course |
|  | Influence of angle of attack | x |  | x |  |
|  | Flow separation at high angles of attack | x |  | x |  |
|  | The lift – α graph | x |  | x |  |
|  | The Coefficients |  |  |  |  |
|  | The lift coefficient Cl : the lift formula |  |  |  |  |
|  | The drag coefficient Cd : the drag formula | x |  | x |  |
|  | The three-dimensional airflow round a wing and a fuselage |  |  |  |  |
|  | Streamline pattern : a ) span - wise flow and causes ; b ) tip vortices and angle of attack ; c ) upwash and downwash due to tip vortices ; d ) wake turbulence behind an aeroplane ( causes, distribution and duration of the phenomenon ). | x |  | x |  |
|  | Induced drag : a ) influence of tip vortices on the angle of attack ; b ) the induced local  ; c ) influence of induced angle of attack on the direction of the lift vector ; d ) induced drag and angle of attack. | x |  | x |  |
|  | Drag |  |  |  |  |
|  | The parasite drag : a ) pressure drag ; b ) interference drag ; c ) friction drag. | x |  | x |  |
|  | The parasite drag and speed | x |  | x |  |
|  | The induced drag and speed | x |  | x |  |
|  | The total drag | x |  | x |  |
|  | The ground effect |  |  |  |  |
|  | Effect on take-off and landing characteristics of an aeroplane | x |  | x |  |
|  | The stall |  |  |  |  |
|  | Flow separation at increasing angles of attack : a ) the boundary layer : 1 ) laminar layer ; 2 ) turbulent layer ; 3 ) transition. b ) separation point ; c ) influence of angle of attack ; d ) influence on : 1 ) pressure distribution ; 2 ) location of centre of pressure ; 3 ) CL ; 4 ) CD ; 5 ) pitch moments. e ) buffet ; f ) use of controls. | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane | PPL | Bridge Course | PPL | Bridge Course |
|  | The stall speed : a ) in the lift formula ; b ) 1 g stall speed ; c ) influence of : 1 ) the centre of gravity ; 2 ) power setting ; 3 ) altitude ( IAS ) ; 4 ) wing loading ; 5 ) load factor n : ( i ) definition ; ( ii ) turns ; ( iii ) forces. | x |  | x |  |
|  | The initial stall in span - wise direction: a ) influence of plan-form ; b ) geometric twist ( wash out ) ; c ) use of ailerons. | x |  | x |  |
|  | Stall warning : a ) importance of stall warning ; b ) speed margin ; c ) buffet ; d ) stall strip ; e ) flapper switch ; f ) recovery from stall. | x |  | x |  |
|  | Special phenomena of stall : a ) the power - on stall ; b ) climbing and descending turns ; c ) T - tailed aeroplane ; d ) avoidance of spins : 1 ) spin development ; 2 ) spin recognition ; 3 ) spin recovery. e ) ice ( in stagnation point and on surface ) : 1 ) absence of stall warning ; 2 ) abnormal behaviour of the aircraft during stall. | x |  | x |  |
|  | C L augmentation |  |  |  |  |
|  | Trailing edge flaps and the reasons for use in take-off and landing : a ) influence on CL - α-graph ; b ) different types of flaps ; c ) flap asymmetry ; d ) influence on pitch movement. | x |  | x |  |
|  | Leading edge devices and the reasons for use in take-off and landing | x |  | x |  |
|  | The boundary layer |  |  |  |  |
|  | Different types : a ) laminar ; b ) turbulent. | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane | PPL | Bridge Course | PPL | Bridge Course |
|  | Special circumstances |  |  |  |  |
|  | Ice and other contamination : a ) ice in stagnation point ; b ) ice on the surface ( frost, snow and clear ice ) ; c ) rain ; d ) contamination of the leading edge ; e ) effects on stall ; f ) effects on loss of controllability ; g ) effects on control surface moment ; h ) influence on high lift devices during take- off, landing and low speeds. | x |  | x |  |
|  | Stability |  |  |  |  |
|  | Condition of equilibrium in steady horizontal flight |  |  |  |  |
|  | Precondition for static stability | x |  | x |  |
|  | Equilibrium : a ) lift and weight ; b ) drag and thrust. | x |  | x |  |
|  | Methods of achieving balance |  |  |  |  |
|  | Wing and empennage ( tail and canard ) | x |  | x |  |
|  | Control surfaces | x |  | x |  |
|  | Ballast or weight trim | x |  | x |  |
|  | Static and dynamic longitudinal stability |  |  |  |  |
|  | Basics and definitions : a ) static stability, positive, neutral and negative ; b ) precondition for dynamic stability ; c ) dynamic stability, positive, neutral and negative. | x |  | x |  |
|  | Location of centre of gravity : a ) aft limit and minimum stability margin ; b ) forward position ; c ) effects on static and dynamic stability. | x |  | x |  |
|  | Dynamic lateral or directional stability |  |  |  |  |
|  | Spiral dive and corrective actions | x |  | x |  |
|  | Control |  |  |  |  |
|  | General |  |  |  |  |
|  | Basics, the three planes and three axis | x |  | x |  |
|  | Angle of attack change | x |  | x |  |
|  | Pitch control |  |  |  |  |
|  | Elevator | x |  | x |  |
|  | Downwash effects | x |  | x |  |
|  | Location of centre of gravity | x |  | x |  |
|  | Yaw control |  |  |  |  |
|  | Pedal or rudder | x |  | x |  |
|  | Roll control |  |  |  |  |
|  | Ailerons : function in different phases of flight | x |  | x |  |
|  | Adverse yaw | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane | PPL | Bridge Course | PPL | Bridge Course |
|  | Means to avoid adverse yaw : a ) frieze ailerons ; b ) differential ailerons deflection. | x |  | x |  |
|  | Means to reduce control forces |  |  |  |  |
|  | Aerodynamic balance : a ) balance tab and anti - balance tab ; b ) servo tab. | x |  | x |  |
|  | Mass balance |  |  |  |  |
|  | Reasons to balance : means | x |  | x |  |
|  | Trimming |  |  |  |  |
|  | Reasons to trim | x |  | x |  |
|  | Trim tabs | x |  | x |  |
|  | Limitations |  |  |  |  |
|  | Operating limitations |  |  |  |  |
|  | Flutter | x |  | x |  |
|  | v f e | x |  | x |  |
|  | v n o , v n e | x |  | x |  |
|  | Manoeuvring envelope |  |  |  |  |
|  | Manoeuvring load diagram : a ) load factor ; b ) accelerated stall speed ; c ) v a ; d ) manoeuvring limit load factor or certification category. | x |  | x |  |
|  | Contribution of mass | x |  | x |  |
|  | Gust envelope |  |  |  |  |
|  | Gust load diagram | x |  | x |  |
|  | Factors contributing to gust loads | x |  | x |  |
|  | Propellers |  |  |  |  |
|  | Conversion of engine torque to thrust |  |  |  |  |
|  | Meaning of pitch | x |  | x |  |
|  | Blade twist | x |  | x |  |
|  | Effects of ice on propeller | x |  | x |  |
|  | Engine failure or engine stop |  |  |  |  |
|  | Windmilling drag | x |  | x |  |
|  | Moments due to propeller operation |  |  |  |  |
|  | Torque reaction | x |  | x |  |
|  | Asymmetric slipstream effect | x |  | x |  |
|  | Asymmetric blade effect | x |  | x |  |
|  | Flight mechanics |  |  |  |  |
|  | Forces acting on an aeroplane |  |  |  |  |
|  | Straight horizontal steady flight | x |  | x |  |
|  | Straight steady climb | x |  | x |  |
|  | Straight steady descent | x |  | x |  |
|  | Straight steady glide | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 1 | PRINCIPLES OF FLIGHT : Aeroplane | PPL | Bridge Course | PPL | Bridge Course |
|  | Steady coordinated turn : a ) bank angle ; b ) load factor ; c ) turn radius ; d ) rate one turn. | x |  | x |  |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter |  |  |  |  |
|  | Subsonic Aerodynamics |  |  |  |  |
|  | Basic concepts, laws and definitions |  |  | x | x |
|  | Conversion of units |  |  | x | x |
|  | Definitions and basic concepts about air : a ) the atmosphere and International Standard Atmosphere ; b ) density ; c ) influence of pressure and temperature on density. |  |  | x | x |
|  | Newton’s laws : a ) Newton’s second law : Momentum equation ; b ) Newton’s third law : action and reaction. |  |  | x | x |
|  | Basic concepts about airflow : a ) steady airflow and unsteady airflow ; b ) Bernoulli’s equation ; c ) static pressure, dynamic pressure, total pressure and stagnation point; d ) TAS and IAS ; e ) two - dimensional airflow and three - dimensional airflow ; f ) viscosity and boundary layer. |  |  | x | x |
|  | Two - dimensional airflow |  |  | x | x |
|  | Aerofoil section geometry : a ) aerofoil section ; b ) chord line, thickness and thickness to chord ratio of a section ; c ) camber line and camber ; d ) symmetrical and asymmetrical aerofoil’s sections. |  |  | x | x |
|  | Aerodynamic forces on aerofoil elements : a ) angle of attack ; b ) pressure distribution ; c ) lift and lift coefficient ; d ) relation lift coefficient : angle of attack ; e ) profile drag and drag coefficient ; f ) relation drag coefficient : angle of attack ; g ) resulting force, centre of pressure and pitching moment. |  |  | x | x |
|  | Stall : a ) boundary layer and reasons for stalling ; b ) variation of lift and drag as a function of angle of attack ; c ) displacement of the centre of pressure and pitching moment. |  |  | x | x |
|  | Disturbances due to profile contamination : a ) ice contamination ; b ) ice on the surface ( frost, snow and clear ice ). |  |  | x | x |
|  | The three-dimensional airflow round a wing and a fuselage |  |  | x | x |
|  | The wing : a ) plan-form, rectangular and tapered wings ; b ) wing twist. |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter | PPL | Bridge Course | PPL | Bridge Course |
|  | Airflow pattern and influence on lift : a ) span wise flow on upper and lower surface ; b ) tip vortices ; c ) span-wise lift distribution. |  |  | x | x |
|  | Induced drag : causes and vortices |  |  | x | x |
|  | The airflow round a fuselage : a ) components of a fuselage ; b ) parasite drag ; c ) variation with speed. |  |  | x | x |
|  | Transonic aerodynamics and compressibility effects |  |  |  |  |
|  | Airflow velocities |  |  | x | x |
|  | Airflow speeds : a ) speed of sound ; b ) subsonic, high subsonic and supersonic flows. |  |  | x | x |
|  | Shock waves : a ) compressibility and shock waves ; b ) the reasons for their formation at upstream high subsonic airflow ; c ) their effect on lift and drag. |  |  | x | x |
|  | Influence of wing plan-form : sweep - angle |  |  | x | x |
|  | Rotorcraft types |  |  | x | x |
|  | Rotorcraft |  |  | x | x |
|  | Rotorcraft types : a ) autogyro ; b ) helicopter. |  |  | x | x |
|  | Helicopters |  |  | x | x |
|  | Helicopters configurations : the single main rotor helicopter |  |  | x | x |
|  | The helicopter, characteristics and associated terminology : a ) general lay-out, fuselage, engine and gearbox ; b ) tail rotor, fenestron and NOTAR ; c ) engines ( reciprocating and turbo shaft engines ) ; d ) power transmission ; e ) rotor shaft axis, rotor hub and rotor blades ; f ) rotor disc and rotor disc area ; g ) teetering rotor ( two blades ) and rotors with more than two blades ; h ) skids and wheels ; i ) helicopter axes and fuselage centre line ; j ) roll axis, pitch axis and normal or yaw axis ; k ) gross mass, gross weight and disc loading. |  |  | x | x |
|  | Main rotor aerodynamics |  |  | x | x |
|  | Hover flight outside ground effect |  |  | x | x |
|  | Airflow through the rotor discs and round the blades : a ) circumferential velocity of the blade sections ; b ) induced airflow, through the disc and downstream ; c ) downward fuselage drag ; d ) equilibrium of rotor thrust, weight and fuselage drag ; e ) rotor disc induced power ; f ) relative airflow to the blade ; g ) pitch angle and angle of attack of a blade section ; |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter | PPL | Bridge Course | PPL | Bridge Course |
|  | h ) lift and profile drag on the blade element ; i ) resulting lift and thrust on the blade and rotor thrust ; j ) collective pitch angle changes and necessity of blade feathering ; k ) required total main rotor-torque and rotor-power ; l ) influence of the air density. |  |  | x | x |
|  | Anti - torque force and tail rotor : a ) force of tail rotor as a function of main rotor - torque ; b ) anti - torque rotor power ; c ) necessity of blade feathering of tail rotor blades and yaw pedals. |  |  | x | x |
|  | Maximum hover altitude OGE: a ) total power required and power available ; b ) maximum hover altitude as a function of pressure altitude and OAT. |  |  | x | x |
|  | Vertical climb |  |  | x | x |
|  | Relative airflow and Angles of Attack ( AoA ) : a ) climb velocity VC, induced and relative velocity and AoA ; b ) collective pitch angle and blade feathering. |  |  | x | x |
|  | Power and vertical speed: a ) induced power, climb power and profile power ; b ) total main rotor power and main rotor torque ; c ) tail rotor power ; d ) total power requirement in vertical flight. |  |  | x | x |
|  | Forward flight |  |  | x | x |
|  | Airflow and forces in uniform inflow distribution : a ) assumption of uniform inflow distribution on rotor disc ; b ) advancing blade ( 90° ) and retreating blade ( 270° ) ; c ) airflow velocity relative to the blade sections, area of reverse flow ; d ) lift on the advancing and retreating blades at constant pitch angles; e ) necessity of cyclic pitch changes ; f ) compressibility effects on the advancing blade tip and speed limitations; g ) high AoA on the retreating blade, blade stall and speed limitations; h ) thrust on rotor disc and tilt of thrust vector ; i ) vertical component of the thrust vector and gross weight equilibrium; (j) horizontal component of the thrust vector and drag equilibrium. |  |  | x | x |
|  | The flare ( power flight ) : a ) thrust reversal and increase in rotor thrust ; b ) increase of rotor RPM on non governed rotor. |  |  | x | x |
|  | Power and maximum speed : a ) induced power as a function of helicopter speed ; b ) rotor profile power as a function of helicopter speed ; c ) fuselage drag and parasite power as a function of forward speed ; d ) tail rotor power and power ancillary equipment ; e ) total power requirement as a function of forward speed ; f ) influence of helicopter mass, air density and drag of additional external equipment ; g ) translational lift and influence on power required. |  |  | x | x |
|  | Hover and forward flight in ground effect |  |  | x | x |
|  | Airflow in ground effect and downwash : rotor power decrease as a function of rotor height above the ground at constant helicopter mass |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter | PPL | Bridge Course | PPL | Bridge Course |
|  | Vertical descent |  |  | x | x |
|  | Vertical descent, power on : a ) airflow through the rotor, low and moderate descent speeds ; b ) vortex ring state, settling with power and consequences. |  |  | x | x |
|  | Autorotation : a ) collective lever position after failure ; b ) up flow through the rotor, auto-rotation and anti-autorotation rings; c ) tail rotor thrust and yaw control ; d ) control of rotor RPM with collective lever ; e ) landing after increase of rotor thrust by pulling collective and reduction in vertical speed. |  |  | x | x |
|  | Forward flight : Autorotation |  |  | x | x |
|  | Airflow through the rotor disc : a ) descent speed and up flow through the disc ; b ) the flare, increase in rotor thrust, reduction of vertical speed and ground speed. |  |  | x | x |
|  | Flight and landing : a ) turning ; b ) flare ; c ) autorotative landing ; d ) height or velocity avoidance graph and dead man’s curve. |  |  | x | x |
|  | Main rotor mechanics |  |  | x | x |
|  | Flapping of the blade in hover |  |  | x | x |
|  | Forces and stresses on the blade : a ) centrifugal force on the blade and attachments ; b ) limits of rotor RPM ; c ) lift on the blade and bending stresses on a rigid attachment ; d ) the flapping hinge of the articulated rotor and flapping hinge offset; e) the flapping of the hinge less rotor and flexible element. |  |  | x | x |
|  | Coning angle in hover: a ) lift and centrifugal force in hover and blade weight negligible ; b ) flapping, tip path plane and disc area. |  |  | x | x |
|  | Flapping angles of the blade in forward flight |  |  | x | x |
|  | Forces on the blade in forward flight without cyclic feathering : a ) aerodynamic forces on the advancing and retreating blades without cyclic feathering ; b ) periodic forces and stresses, fatigue and flapping hinge ; c ) phase lag between the force and the flapping angle ( about 90° ) ; d ) flapping motion of the hinged blades and tilting of the cone and flap back of rotor ; e ) rotor disc attitude and thrust vector tilt. |  |  | x | x |
|  | Cyclic pitch ( feathering ) in helicopter mode, forward flight : a ) necessity of forward rotor disc tilt and thrust vector tilt ; b ) flapping and tip path plane, virtual rotation axis or no flapping axis and plane of rotation ; c ) shaft axis and hub plane ; d ) cyclic pitch change ( feathering ) and rotor thrust vector tilt ; |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter | PPL | Bridge Course | PPL | Bridge Course |
|  | e ) collective pitch change, collective lever, swash plate, pitch link and pitch horn ; f ) cyclic stick, rotating swash plate and pitch link movement and phase angle. |  |  |  |  |
|  | Blade lag motion |  |  | x | x |
|  | Forces on the blade in the disc plane ( tip path plane ) in forward flight : a ) forces due to the Coriolis effect because of the flapping ; b ) alternating stresses and the need of the drag or lag hinge. |  |  | x | x |
|  | The drag or lag hinge: a ) the drag hinge in the fully articulated rotor ; b ) the lag flexure in the hinge less rotor ; c ) drag dampers. |  |  | x | x |
|  | Ground resonance : a) blade lag motion and movement of the centre of gravity of the blades and the rotor ; b ) oscillating force on the fuselage ; c) fuselage, undercarriage and resonance. |  |  | x | x |
|  | Rotor systems |  |  | x | x |
|  | See - saw or teetering rotor |  |  | x | x |
|  | Fully articulated rotor : a ) three hinges arrangement ; b ) bearings and elastomeric hinges. |  |  | x | x |
|  | Hinge less rotor and bearing less rotor |  |  | x | x |
|  | Blade sailing : a ) low rotor RPM and effect of adverse wind ; b ) minimizing the danger ; c ) droop stops. |  |  | x | x |
|  | Vibrations due to main rotor : a ) origins of the vibrations : in plane and vertical ; b ) blade tracking and balancing. |  |  | x | x |
|  | Tail rotors |  |  | x | x |
|  | Conventional tail rotor |  |  | x | x |
|  | Rotor description : a ) two-blades tail rotors with teetering hinge ; b ) rotors with more than two blades ; c ) feathering bearings and flapping hinges ; d ) dangers to people and to the tail rotor, rotor height and safety |  |  | x | x |
|  | Aerodynamics : a ) induced airflow and tail rotor thrust ; b ) thrust control by feathering, tail rotor drift and roll ; c ) effect of tail rotor failure and vortex ring. |  |  | x | x |
|  | The fenestron : technical lay - out |  |  | x | x |
|  | The NOTAR : technical lay - out |  |  | x | x |
|  | Vibrations : high frequency vibrations due to the tail rotors |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 5. 2 | PRINCIPLES OF FLIGHT : Helicopter | PPL | Bridge Course | PPL | Bridge Course |
|  | Equilibrium, stability and control |  |  | x | x |
|  | Equilibrium and helicopter attitudes |  |  | x | x |
|  | Hover : a ) forces and equilibrium conditions ; b ) helicopter pitching moment and pitch angle ; c ) helicopter rolling moment and roll angle. |  |  | x | x |
|  | Forward flight : a ) forces and equilibrium conditions ; b ) helicopter moments and angles ; c ) effect of speed on fuselage attitude. |  |  | x | x |
|  | Control |  |  | x | x |
|  | Control power a ) fully articulated rotor ; b ) hinge less rotor ; c ) teetering rotor. |  |  | x | x |
|  | Static and dynamic roll over |  |  | x | x |
|  | Helicopter performances |  |  |  |  |
|  | Engine performances |  |  | x | x |
|  | Piston engines : a ) power available ; b ) effects of density altitude. |  |  | x | x |
|  | Turbine engines : a ) power available ; b ) effects of ambient pressure and temperature. |  |  | x | x |
|  | Helicopter performances |  |  | x | x |
|  | Hover and vertical flight : a ) power required and power available ; b ) OGE and IGE maximum hover height ; c ) influence of AUM, pressure, temperature and density. |  |  | x | x |
|  | Forward flight : a ) maximum speed ; b ) maximum rate of climb speed ; c ) maximum angle of climb speed ; d ) range and endurance ; e ) influence of AUM, pressure, temperature and density. |  |  | x | x |
|  | Manoeuvring : a ) load factor ; b ) bank angle and number of g’s ; c ) manoeuvring limit load factor. |  |  | x | x |
|  | Special conditions : a ) operating with limited power ; b ) over pitch and over torque. |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 6. | OPERATIONAL PROCEDURES | PPL | Bridge Course | PPL | Bridge Course |
|  | General |  |  |  |  |
|  | Operation of Aircraft : ICAO Annex 6, General Requirements |  |  |  |  |
|  | Definitions | x | x | x | x |
|  | Applicability | x | x | x | x |
|  | Special operational procedures and hazards ( general aspects ) | x | x | x | x |
|  | Noise Abatement |  |  |  |  |
|  | Noise abatement procedures | x | x | x | x |
|  | Influence of the flight procedure ( departure, cruise and approach ) | x | x | x | x |
|  | Runway incursion awareness ( meaning of surface markings and signals ) | x | x | x | x |
|  | Fire or Smoke |  |  |  |  |
|  | Carburettor fire | x | x | x | x |
|  | Engine fire | x | x | x | x |
|  | Fire in the cabin and cockpit, ( choice of extinguishing agents according to fire classification and use of the extinguishers ) | x | x | x | x |
|  | Smoke in the cockpit ( effects and action to be taken ) and smoke in the cockpit and cabin ( effects and actions taken ) | x | x | x | x |
|  | Windshear and Microburst |  |  |  |  |
|  | Effects and recognition during departure and approach | x | x | x | x |
|  | Actions to avoid and actions taken during encounter | x | x | x | x |
|  | Wake Turbulence |  |  |  |  |
|  | Cause | x | x | x | x |
|  | List of relevant parameters | x | x | x | x |
|  | Actions taken when crossing traffic, during take-off and landing | x | x | x | x |
|  | Emergency and Precautionary Landings |  |  |  |  |
|  | Definition | x | x | x | x |
|  | Cause | x | x | x | x |
|  | Passenger information | x | x | x | x |
|  | Evacuation | x | x | x | x |
|  | Action after landing | x | x | x | x |
|  | Contaminated Runways |  |  |  |  |
|  | Kinds of contamination | x | x |  |  |
|  | Estimated surface friction and friction coefficient | x | x |  |  |
|  | Rotor downwash |  |  | x | x |
|  | Operation influence by meteorological conditions (Hel-pter) |  |  |  |  |
|  | White out, sand or dust |  |  | x | x |
|  | Strong winds |  |  | x | x |
|  | Mountain environment |  |  | x | x |
|  | Emergency Procedures |  |  |  |  |
|  | Influence by technical problems |  |  |  |  |
|  | Engine failure |  |  | x | x |
|  | Fire in cabin, cockpit or engine |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 6. | OPERATIONAL PROCEDURES | PPL | Bridge Course | PPL | Bridge Course |
|  | Tail, rotor or directional control failure |  |  | x | x |
|  | Ground resonance |  |  | x | x |
|  | Blade stall |  |  | x | x |
|  | Settling with power ( vortex ring ) |  |  | x | x |
|  | Overpitch |  |  | x | x |
|  | Overspeed : rotor or engine |  |  | x | x |
|  | Dynamic rollover |  |  | x | x |
|  | Mast bumping |  |  | x | x |
| 7. | FLIGHT PERFORMANCE and PLANNING |  |  |  |  |
| 7. 1 | MASS and BALANCE : Aeroplanes or Helicopters |  |  |  |  |
|  | Purpose of Mass and Balance Considerations |  |  |  |  |
|  | Mass Limitations |  |  |  |  |
|  | Importance in regard to structural limitations | x | x | x | x |
|  | Importance in regard to performance limitations | x | x | x | x |
|  | CG Limitations |  |  |  |  |
|  | Importance in regard to stability and controllability | x | x | x | x |
|  | Importance in regard to performance | x | x | x | x |
|  | Loading |  |  |  |  |
|  | Terminology |  |  |  |  |
|  | Mass terms | x | x | x | x |
|  | Load terms ( including fuel terms ) | x | x | x | x |
|  | Mass Limits |  |  |  |  |
|  | Structural limitations | x | x | x | x |
|  | Performance limitations | x | x | x | x |
|  | Baggage compartment limitations | x | x | x | x |
|  | Mass Calculations |  |  |  |  |
|  | Maximum masses for take-off and landing | x | x | x | x |
|  | Use of standard masses for passengers, baggage and crew | x | x | x | x |
|  | Fundamentals of CG Calculations |  |  |  |  |
|  | Definition of centre of gravity | x | x | x | x |
|  | Conditions of equilibrium ( balance of forces and balance of moments ) | x | x | x | x |
|  | Basic calculations of CG | x | x | x | x |
|  | Mass and Balance details of Aircraft |  |  |  |  |
|  | Contents of Mass and Balance Documentation |  |  |  |  |
|  | Datum and moment arm | x | x | x | x |
|  | CG position as distance from datum | x | x | x | x |
|  | Extraction of basic Mass and Balance Data from Aircraft Documentation |  |  |  |  |
|  | BEM | x | x | x | x |
|  | CG position or moment at BEM | x | x | x | x |
|  | Deviations from standard configuration | x | x | x | x |
|  | Determination of CG Position |  |  |  |  |
|  | Methods |  |  |  |  |
|  | Arithmetic method | x | x | x | x |
|  | Graphic method | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 7. 1 | MASS and BALANCE : Aeroplanes or Helicopters | PPL | Bridge Course | PPL | Bridge Course |
|  | Load and Trim Sheet |  |  |  |  |
|  | General considerations | x | x | x | x |
|  | Load Sheet and CG envelope for light aeroplanes and for helicopters | x | x | x | x |
| 7. 2 | PERFORMANCE : Aeroplanes |  |  |  |  |
|  | Introduction |  |  |  |  |
|  | Performance classes | x | x |  |  |
|  | Stages of flight | x | x |  |  |
|  | Effect of aeroplane mass, wind, altitude, runway slope and runway conditions | x | x |  |  |
|  | Gradients | x | x |  |  |
|  | SE Aeroplanes |  |  |  |  |
|  | Definitions of terms and speeds | x | x |  |  |
|  | Take-off and Landing Performance |  |  |  |  |
|  | Use of aeroplane flight manual data | x | x |  |  |
|  | Climb and Cruise Performance |  |  |  |  |
|  | Use of aeroplane flight data | x | x |  |  |
|  | Effect of density altitude and aeroplane mass | x | x |  |  |
|  | Endurance and the effects of the different recommended power or thrust settings | x | x |  |  |
|  | Still air range with various power or thrust settings | x | x |  |  |
| 7. 3 | FLIGHT PLANNING and FLIGHT MONITORING |  |  |  |  |
|  | Flight Planning for VFR Flights |  |  |  |  |
|  | VFR Navigation Plan |  |  |  |  |
|  | Routes, Airfields, Heights and Altitudes from VFR Charts | x | x | x | x |
|  | Courses and Distances from VFR Charts |  |  |  |  |
|  | Aerodrome Charts and Aerodrome Directory | x | x | x | x |
|  | Communications and Radio Navigation Planning Data | x | x | x | x |
|  | Completion of Navigation Plan | x | x | x | x |
|  | Fuel Planning |  |  |  |  |
|  | General knowledge | x | x | x | x |
|  | Pre-flight Calculation of Fuel Required |  |  |  |  |
|  | Calculation of extra fuel | x | x | x | x |
|  | Completion of the fuel section of the navigation plan ( fuel log ) and calculation of total fuel | x | x | x | x |
|  | Pre - flight Preparation |  |  |  |  |
|  | AIP and NOTAM Briefing |  |  |  |  |
|  | Ground facilities and services | x | x | x | x |
|  | Departure, destination and alternate aerodromes | x | x | x | x |
|  | Airway routings and airspace structure | x | x | x | x |
|  | Meteorological Briefing |  |  |  |  |
|  | Extraction and analysis of relevant data from meteorological documents | x | x | x | x |
|  | ICAO Flight Plan ( ATS Flight Plan) |  |  |  |  |
|  | Individual Flight Plan |  |  |  |  |
|  | Format of Flight Plan | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 7. 3 | FLIGHT PLANNING and FLIGHT MONITORING | PPL | Bridge Course | PPL | Bridge Course |
|  | Completion of the Flight Plan | x | x | x | x |
|  | Submission of the Flight Plan | x | x | x | x |
|  | Flight Monitoring and in-flight Re- planning |  |  |  |  |
|  | Flight Monitoring |  |  |  |  |
|  | Monitoring of track and time | x | x | x | x |
|  | In-flight fuel management | x | x | x | x |
|  | In-flight re-planning in case of deviation from planned data | x | x | x | x |
| 7. 4 | PERFORMANCE : Helicopters |  |  |  |  |
|  | General |  |  |  |  |
|  | Introduction |  |  |  |  |
|  | Stages of flight |  |  | x | x |
|  | Effect on performance of atmospheric, airport or heliport and helicopter conditions |  |  | x | x |
|  | Applicability of Airworthiness Requirements |  |  | x | x |
|  | Definitions and Terminology |  |  | x | x |
|  | Performance : SE Helicopters |  |  | x | x |
|  | Definitions of Terms : a ) masses ; b ) velocities : v x, v y ; c ) velocity of best range and of maximum endurance ; d ) power limitations ; e ) altitudes. |  |  | x | x |
|  | Take-off, Cruise and Performance |  |  |  |  |
|  | Use and interpretation of diagrams and tables : a ) Take - off : 1 ) take-off run and distance available ; 2 ) take-off and initial climb ; 3 ) effects of mass, wind and density altitude ; 4 ) effects of ground surface and gradient. b ) Landing : 1 ) effects of mass, wind, density altitude and approach speed; 2 ) effects of ground surface and gradient. c ) In - flight : 1 ) relationship between power required and power available; 2 ) performance diagram ; 3 ) effects of configuration, mass, temperature and altitude ; 4 ) reduction of performance during climbing turns ; 5 ) autorotation ; 6 ) adverse effects ( icing, rain and condition of the airframe ) |  |  | x | x |
| 8. | AIRCRAFT GENERAL KNOWLEDGE |  |  |  |  |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT |  |  |  |  |
|  | System Design, Loads, Stresses, Maintenance |  |  |  |  |
|  | Loads and combination loadings applied to an aircraft’s structure | x | x | x | x |
|  | Airframe |  |  |  |  |
|  | Wings, Tail Surfaces and Control Surfaces |  |  |  |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT | PPL | Bridge Course | PPL | Bridge Course |
|  | Design and Constructions | x | x |  |  |
|  | Structural components and materials | x | x |  |  |
|  | Stresses | x | x |  |  |
|  | Structural limitations | x | x |  |  |
|  | Fuselage, Doors, Floor, Wind-screen and Windows |  |  |  |  |
|  | Design and Constructions | x | x | x | x |
|  | Structural components and materials | x | x | x | x |
|  | Stresses | x | x | x | x |
|  | Structural limitations | x | x | x | x |
|  | Flight and Control Surfaces |  |  |  |  |
|  | Design and Constructions |  |  | x | x |
|  | Structural components and materials |  |  | x | x |
|  | Stresses and aero elastic vibrations |  |  | x | x |
|  | Structural limitations |  |  | x | x |
|  | Hydraulics |  |  |  |  |
|  | Hydromechanics : basic principles | x | x | x | x |
|  | Hydraulic Systems | x | x | x | x |
|  | Hydraulic fluids : types and characteristics, limitations | x | x | x | x |
|  | System components : design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Landing Gear, Wheels, Tyres and Brakes |  |  |  |  |
|  | Landing Gear |  |  |  |  |
|  | Types and materials | x | x | x | x |
|  | Nose Wheel Steering : design and operation | x | x |  |  |
|  | Brakes |  |  |  |  |
|  | Types and materials | x | x | x | x |
|  | System components: design, operation, indications and warnings | x | x | x | x |
|  | Wheels and Tyres |  |  |  |  |
|  | Types and operational limitations | x | x | x | x |
|  | Helicopter Equipments |  |  | x | x |
|  | Flight Controls |  |  |  |  |
|  | Mechanical or powered | x | x | x | x |
|  | Control systems and mechanical | x | x | x | x |
|  | System components : design, operation, indications and warnings, degraded modes of operation and jamming | x | x | x | x |
|  | Secondary Flight Controls |  |  |  |  |
|  | System components : design, operation, degraded modes of operation, indications and warnings | x | x |  |  |
|  | Anti-icing Systems |  |  |  |  |
|  | Types and operation ( pitot and windshield ) | x | x | x | x |
|  | Fuel System |  |  |  |  |
|  | Piston Engine |  |  |  |  |
|  | System components : design, operation, degraded modes of operation, indications and warnings | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT | PPL | Bridge Course | PPL | Bridge Course |
|  | Turbine Engine |  |  |  |  |
|  | System components : design, operation, indications and warnings, degraded modes of operation, indications and warnings |  |  | x | x |
|  | Electrics |  |  |  |  |
|  | Electrics : general and definitions |  |  |  |  |
|  | Direct current : voltage, current, resistance, conductivity, Ohm’s law, power and work | x | x | x | x |
|  | Alternating current : voltage, current, amplitude, phase, frequency and resistance | x | x | x | x |
|  | Circuits : series and parallel | x | x | x | x |
|  | Magnetic field : effects in an electrical circuit | x | x | x | x |
|  | Batteries |  |  |  |  |
|  | Types, characteristics and limitations | x | x | x | x |
|  | Battery chargers, characteristics and limitations | x | x | x | x |
|  | Static Electricity : general |  |  |  |  |
|  | Basic principles | x | x | x | x |
|  | Static dischargers | x | x | x | x |
|  | Protection against interference | x | x | x | x |
|  | Lightning effects | x | x | x | x |
|  | Generation : production, distribution and use |  |  |  |  |
|  | DC Generation : types, design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | AC Generation : types, design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Electric Components |  |  |  |  |
|  | Basic elements : basic principles of switches, circuit - - breakers and relays |  |  |  |  |
|  | Distribution |  |  |  |  |
|  | General : a ) bus bar, common earth and priority ; b ) AC and DC comparison. | x | x | x | x |
|  | Piston Engines |  |  |  |  |
|  | General |  |  |  |  |
|  | Types of internal combustion engine : basic principles and definitions | x | x | x | x |
|  | Engine : design, operation, components and materials | x | x | x | x |
|  | Fuel |  |  |  |  |
|  | Types, grades, characteristics and limitations | x | x | x | x |
|  | Alternate fuel : characteristics and limitations | x | x | x | x |
|  | Carburettor or Injection System |  |  |  |  |
|  | Carburettor : design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Injection : design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Icing | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT | PPL | Bridge Course | PPL | Bridge Course |
|  | Air Cooling Systems |  |  |  |  |
|  | Design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Lubrication Systems |  |  |  |  |
|  | Lubricants : types, characteristics and limitations | x | x | x | x |
|  | Design, operation, degraded modes of operation, indications and warnings | x | x | x | x |
|  | Ignition Circuits |  |  |  |  |
|  | Design, operation, degraded modes of operation | x | x | x | x |
|  | Mixture |  |  |  |  |
|  | Definition, characteristic mixtures, control instruments, associated control levers and indications | x | x | x | x |
|  | Propellers |  |  |  |  |
|  | Definitions and general : a ) aerodynamic parameters ; b ) types ; c ) operating modes. | x | x |  |  |
|  | Constant speed propeller : design, operation and system components | x | x |  |  |
|  | Propeller handling : associated control levers, degraded modes of operation, indications and warnings | x | x |  |  |
|  | Performance and Engine Handling |  |  |  |  |
|  | Performance : influence of engine parameters, influence of atmospheric conditions, limitations and power augmentation systems | x | x | x | x |
|  | Engine handling : power and mixture settings during various flight phases and operational limitations | x | x | x | x |
|  | Turbine Engines |  |  |  |  |
|  | Definitions |  |  | x | x |
|  | Coupled turbine engine : design, operation, components and materials |  |  | x | x |
|  | Free turbine engine : design, operation, components and materials |  |  | x | x |
|  | Fuel |  |  |  |  |
|  | Types, characteristics and limitations |  |  | x | x |
|  | Main Engine Components |  |  |  |  |
|  | Compressor : a ) types, design, operation, components and materials ; b ) stresses and limitations ; c ) stall, surge and means of prevention. |  |  | x | x |
|  | Combustion chamber : a ) types, design, operation, components and materials ; b ) stresses and limitations ; c ) emission problems |  |  | x | x |
|  | Turbine : a ) types, design, operation, components and materials ; b ) stresses, creep and limitations. |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT | PPL | Bridge Course | PPL | Bridge Course |
|  | Exhaust : a ) design, operation and materials ; b ) noise reduction. |  |  | x | x |
|  | Fuel control units : types, operation and sensors. |  |  | x | x |
|  | Helicopter air intake : different types, design, operation, materials and optional equipments |  |  | x | x |
|  | Additional Components and Systems |  |  |  |  |
|  | Helicopter additional components and systems : lubrication system, ignition circuit, starter, accessory gearbox, free wheel units : design, operation and components |  |  | x | x |
|  | Performance Aspects |  |  |  |  |
|  | Torque, performance aspects, engine handling and limitations: a ) engine ratings ; b ) engine performance and limitations ; c ) engine handling. |  |  | x | x |
|  | Protection and Detection Systems |  |  |  |  |
|  | Fire Detection Systems |  |  |  |  |
|  | Operation and indications |  |  | x | x |
|  | Miscellaneous Systems |  |  |  |  |
|  | Rotor Design |  |  | x | x |
|  | Rotor Heads |  |  |  |  |
|  | Main Rotor |  |  | x | x |
|  | Types |  |  | x | x |
|  | Structural components and materials, stresses and structural limitations |  |  | x | x |
|  | Design and construction |  |  | x | x |
|  | Adjustment |  |  | x | x |
|  | Tail Rotor |  |  |  |  |
|  | Types |  |  | x | x |
|  | Structural components and materials, stresses and structural limitations |  |  |  |  |
|  | Design and construction |  |  | x | x |
|  | Adjustment |  |  | x | x |
|  | Transmission |  |  |  |  |
|  | Main Gear Box |  |  |  |  |
|  | Different types, design, operation and limitations |  |  | x | x |
|  | Rotor Brake |  |  |  |  |
|  | Different types, design, operation and limitations |  |  | x | x |
|  | Auxiliary Systems |  |  | x | x |
|  | Drive Shaft and Associated Installation |  |  | x | x |
|  | Intermediate and Tail Gear Box |  |  |  |  |
|  | Different types, design, operation and limitations |  |  | x | x |
|  | Blades |  |  |  |  |
|  | Main Rotor Blade |  |  |  |  |
|  | Design and construction |  |  | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 1 | AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT | PPL | Bridge Course | PPL | Bridge Course |
|  | Structural components and materials |  |  | x | x |
|  | Stresses |  |  | x | x |
|  | Structural Limitations |  |  | x | x |
|  | Adjustment |  |  | x | x |
|  | Tip shape |  |  | x | x |
|  | Tail Rotor Blade |  |  |  |  |
|  | Design and Construction |  |  | x | x |
|  | Structural components and materials |  |  | x | x |
|  | Stresses |  |  | x | x |
|  | Structural limitations |  |  | x | x |
|  | Adjustment |  |  | x | x |
| 8. 2 | INSTRUMENTATION |  |  |  |  |
|  | Instrument and Indication Systems |  |  |  |  |
|  | Pressure Gauge |  |  |  |  |
|  | Different types, design, operation, characteristics and accuracy | x | x | x | x |
|  | Temperature Sensing |  |  |  |  |
|  | Different types, design, operation, characteristics and accuracy | x | x | x | x |
|  | Fuel Gauge |  |  |  |  |
|  | Different types, design, operation, characteristics and accuracy | x | x | x | x |
|  | Flow Meter |  |  |  |  |
|  | Different types, design, operation, characteristics and accuracy | x | x | x | x |
|  | Position Transmitter |  |  |  |  |
|  | Different types, design, operation, characteristics and accuracy | x | x | x | x |
|  | Torque Meter |  |  |  |  |
|  | Design, operation, characteristics and accuracy | x | x | x | x |
|  | Tachometer |  |  |  |  |
|  | Design, operation, characteristics and accuracy | x | x | x | x |
|  | Measurement of Aerodynamic Parameters |  |  |  |  |
|  | Pressure Measurement |  |  |  |  |
|  | Static pressure, dynamic pressure, density and definitions | x | x | x | x |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Temperature Measurement : Aeroplane |  |  |  |  |
|  | Design, operation, errors and accuracy | x | x |  |  |
|  | Displays | x | x |  |  |
|  | Temperature Measurement : Helicopter |  |  |  |  |
|  | Design, operation, errors and accuracy |  |  | x | x |
|  | Displays |  |  | x | x |
|  | Altimeter |  |  |  |  |
|  | Standard atmosphere | x | x | x | x |
|  | The different barometric references ( QNH, QFE and 1013.25 ) | x | x | x | x |
|  | Height, indicated altitude, true altitude, pressure altitude and density altitude | x | x | x | x |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Displays | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 8. 2 | INSTRUMENTATION | PPL | Bridge Course | PPL | Bridge Course |
|  | Vertical Speed Indicator |  |  |  |  |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Displays | x | x | x | x |
|  | Air Speed Indicator |  |  |  |  |
|  | The different speeds IAS, CAS, TAS : definition, usage and relationships | x | x | x | x |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Displays | x | x | x | x |
|  | Magnetism : Direct Reading Compass |  |  |  |  |
|  | Earth Magnetic Field | x | x | x | x |
|  | Direct Reading Compass |  |  |  |  |
|  | Design, operation, data processing, accuracy and deviation | x | x | x | x |
|  | Turning and acceleration errors | x | x | x | x |
|  | Gyroscopic Instruments |  |  |  |  |
|  | Gyroscope : Basic Principles |  |  |  |  |
|  | Definitions and design | x | x | x | x |
|  | Fundamental properties | x | x | x | x |
|  | Drifts | x | x | x | x |
|  | Turn and Bank Indicator |  |  |  |  |
|  | Design, operation and errors | x | x | x | x |
|  | Attitude Indicator |  |  |  |  |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Directional Gyroscope |  |  |  |  |
|  | Design, operation, errors and accuracy | x | x | x | x |
|  | Communication Systems |  |  |  |  |
|  | Transmission modes : VHF, HF and SATCOM |  |  |  |  |
|  | Principles, bandwidth, operational limitations and use | x | x | x | x |
|  | Voice Communication |  |  |  |  |
|  | Definitions, general and applications | x | x | x | x |
|  | Alerting Systems and Proximity Systems |  |  |  |  |
|  | Flight Warning Systems |  |  |  |  |
|  | Design, operation, indications and alarms | x | x | x | x |
|  | Stall Warning |  |  |  |  |
|  | Design, operation, indications and alarms | x | x |  |  |
|  | Radio - Altimeter |  |  |  |  |
|  | Design, operation, errors, accuracy and indications |  |  | x | x |
|  | Rotor or Engine over speed Alert System |  |  |  |  |
|  | Design, operation, displays and alarms |  |  | x | x |
|  | Integrated Instruments : Electronic Displays |  |  |  |  |
|  | Display Units |  |  |  |  |
|  | Design, different technologies and limitations | x | x | x | x |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 9. | NAVIGATION | PPL | Bridge Course | PPL | Bridge Course |
| 9. 1 | GENERAL NAVIGATION |  |  |  |  |
|  | Basics of Navigation |  |  |  |  |
|  | The Solar System |  |  |  |  |
|  | Seasonal and apparent movements of the sun | x |  | x |  |
|  | The Earth |  |  |  |  |
|  | Great circle, small circle and rhumb line | x |  | x |  |
|  | Latitude and difference of latitude | x |  | x |  |
|  | Longitude and difference of longitude | x |  | x |  |
|  | Use of latitude and longitude co-ordinates to locate any specific position | x |  | x |  |
|  | Time and Time Conversions |  |  |  |  |
|  | Apparent time | x |  | x |  |
|  | UTC | x |  | x |  |
|  | LMT | x |  | x |  |
|  | Standard times | x |  | x |  |
|  | Dateline | x |  | x |  |
|  | Definition of sunrise, sunset and civil twilight | x |  | x |  |
|  | Directions |  |  |  |  |
|  | True north, magnetic north and compass north | x |  | x |  |
|  | Compass deviation | x |  | x |  |
|  | Magnetic poles, isogonals, relationship between true and magnetic | x |  | x |  |
|  | Distance |  |  |  |  |
|  | Units of distance and height used in navigation : nautical miles, statute miles, kilometres, metres and ft. | x |  | x |  |
|  | Conversion from one unit to another | x |  | x |  |
|  | Relationship between nautical miles and minutes of latitude and minutes of longitude |  |  |  |  |
|  | Magnetism and Compasses |  |  |  |  |
|  | General Principles |  |  |  |  |
|  | Terrestrial magnetism | x |  | x |  |
|  | Resolution of the earth’s total magnetic force into vertical and horizontal components | x |  | x |  |
|  | Variation - annual change | x |  | x |  |
|  | Aircraft Magnetism |  |  |  |  |
|  | The resulting magnetic fields | x |  | x |  |
|  | Keeping magnetic materials clear of the compass | x |  | x |  |
|  | Charts |  |  |  |  |
|  | General Properties of Miscellaneous Types of Projections |  |  |  |  |
|  | Direct Mercator | x |  | x |  |
|  | Lambert conformal conic | x |  | x |  |
|  | The Representation of Meridians, Parallels, Great Circles and Rhumb Lines |  |  |  |  |
|  | Direct Mercator | x |  | x |  |
|  | Lambert conformal conic | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 9. 1 | GENERAL NAVIGATION | PPL | Bridge Course | PPL | Bridge Course |
|  | The use of Current Aeronautical Charts |  |  |  |  |
|  | Plotting positions | x |  | x |  |
|  | Methods of indicating scale and relief ( ICAO topographical chart ) | x |  | x |  |
|  | Conventional signs | x |  | x |  |
|  | Measuring tracks and distances | x |  | x |  |
|  | Plotting bearings and distances | x |  | x |  |
|  | DR Navigation |  |  |  |  |
|  | Basis of DR |  |  |  |  |
|  | Track | x |  | x |  |
|  | Heading ( compass, magnetic and true ) | x |  | x |  |
|  | Wind velocity | x |  | x |  |
|  | Air speed ( IAS, CAS and TAS ) | x |  | x |  |
|  | Ground speed | x |  | x |  |
|  | ETA | x |  | x |  |
|  | Drift and wind correction angle | x |  | x |  |
|  | DR position fix | x |  | x |  |
|  | Use of the Navigational Computer |  |  |  |  |
|  | Speed | x |  | x |  |
|  | Time | x |  | x |  |
|  | Distance | x |  | x |  |
|  | Fuel consumption | x |  | x |  |
|  | Conversions | x |  | x |  |
|  | Air speed | x |  | x |  |
|  | Wind velocity | x |  | x |  |
|  | True altitude | x |  | x |  |
|  | The Triangle of Velocities |  |  |  |  |
|  | Heading | x |  | x |  |
|  | Ground speed | x |  | x |  |
|  | Wind velocity | x |  | x |  |
|  | Track and drift angle | x |  | x |  |
|  | Measurement of DR Elements |  |  |  |  |
|  | Calculation of altitude | x |  | x |  |
|  | Determination of appropriate speed | x |  | x |  |
|  | In - Flight Navigation |  |  |  |  |
|  | Use of visual observations and application to in-flight navigation | x |  | x |  |
|  | Navigation in Cruising Flight, use of Fixes to revise Navigation Data |  |  |  |  |
|  | Ground speed revision | x |  | x |  |
|  | Off - track corrections | x |  | x |  |
|  | Calculation of wind speed and direction | x |  | x |  |
|  | ETA revisions | x |  | x |  |
|  | Flight Log | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 9. 2 | RADIO NAVIGATION | PPL | Bridge Course | PPL | Bridge Course |
|  | Basic Radio Propagation Theory |  |  |  |  |
|  | Antennas |  |  |  |  |
|  | Characteristics | x |  | x |  |
|  | Wave Propagation |  |  |  |  |
|  | Propagation with the frequency bands | x |  | x |  |
|  | Radio Aids |  |  |  |  |
|  | Ground DF |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Coverage | x |  | x |  |
|  | Range | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |
|  | NDB / ADF |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Coverage | x |  | x |  |
|  | Range | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |
|  | VOR |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Coverage | x |  | x |  |
|  | Range | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |
|  | DME |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Coverage | x |  | x |  |
|  | Range | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |
|  | Radar |  |  |  |  |
|  | Ground Radar |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Coverage | x |  | x |  |
|  | Range | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |

| N 0 |  | Aeroplane |  | Helicopter |  |
| --- | --- | --- | --- | --- | --- |
| 9. 2 | RADIO NAVIGATION | PPL | Bridge Course | PPL | Bridge Course |
|  | Secondary Surveillance Radar and Transponder |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Presentation and interpretation | x |  | x |  |
|  | Modes and codes | x |  | x |  |
|  | GNSS |  |  |  |  |
|  | GPS, GLONASS or GALILEO |  |  |  |  |
|  | Principles | x |  | x |  |
|  | Operation | x |  | x |  |
|  | Errors and accuracy | x |  | x |  |
|  | Factors affecting range and accuracy | x |  | x |  |

AMC  2.    FCL. 210 ;    FCL. 215

SYLLABUS    of    THEORETICAL    KNOWLEDGE    for    the    PPL ( AS )

The  following  Table  contains  the  syllabi  for  the  courses  of  theoretical  knowledge,  as  well  as  for  the  theoretical  knowledge  examinations  for  the  PPL(As).  The  training  and  examination should  cover  aspects  related  to  non-technical  skills  in  an  integrated  manner,  taking  into  account  the  particular  risks  associated  to  the  licence  and  the  activity.

| N0 |  | PPL |
| --- | --- | --- |
| 1. | AIR LAW and ATC PROCEDURES |  |
|  | International Law : conventions, agreements and organizations | x |
|  | Airworthiness of aircraft | x |
|  | Aircraft nationality and registration marks | x |
|  | Personnel licensing | x |
|  | Rules of the Air | x |
|  | Procedures for air navigation services : aircraft operations | x |
|  | Air Traffic Services and Air Traffic Management | x |
|  | Aeronautical Information Service | x |
|  | Aerodromes | x |
|  | Search and Rescue | x |
|  | Security | x |
|  | Aircraft Accident and Incident Investigation | x |
|  | National Law | x |
| 2. | HUMAN PERFORMANCE |  |
|  | Human factors : basic concepts | x |
|  | Basic aviation physiology and health maintenance | x |
|  | Basic aviation psychology | x |

| N0 |  | PPL |
| --- | --- | --- |
| 3. | METEOROLOGY |  |
|  | The atmosphere | x |
|  | Wind | x |
|  | Thermodynamics | x |
|  | Clouds and fog | x |
|  | Precipitation | x |
|  | Air masses and fronts | x |
|  | Pressure systems | x |
|  | Climatology | x |
|  | Flight hazards | x |
|  | Meteorological information | x |
| 4. | COMMUNICATIONS |  |
|  | VFR Communications |  |
|  | Definitions | x |
|  | General operating procedures | x |
|  | Relevant weather information terms ( VFR ) | x |
|  | Action required to be taken in case of communication failure | x |
|  | Distress and urgency procedures | x |
|  | General principles of VHF propagation and allocation of frequencies | x |
| 5. | PRINCIPLES of FLIGHT |  |
|  | Basics of aerostatics | x |
|  | Basics of subsonic aerodynamics | x |
|  | Aerodynamics of airships | x |
|  | Stability | x |
|  | Controllability | x |
|  | Limitations | x |
|  | Propellers | x |
|  | Basics of airship flight mechanics | x |
| 6. | OPERATIONAL PROCEDURES |  |
|  | General requirements | x |
|  | Special operational procedures and hazards ( general aspects ) | x |
|  | Emergency procedures | x |
| 7. | FLIGHT PERFORMANCE and PLANNING |  |
| 7. 1 | MASS and BALANCE |  |
|  | Purpose of mass and balance considerations | x |
|  | Loading | x |
|  | Fundamentals of CG calculations | x |
|  | Mass and balance details of aircraft | x |
|  | Determination of CG position | x |
|  | Passenger, cargo and ballast handling | x |
| 7. 2 | PERFORMANCE |  |
|  | Airworthiness requirements | x |
|  | Basics of airship performance | x |
|  | Definitions and terms | x |
|  | Stages of flight | x |
|  | Use of flight manual | x |
| 7. 2 | FLIGHT PLANNING and FLIGHT MONITORING |  |
|  | Flight planning for VFR flights | x |
|  | Fuel planning | x |
|  | Pre - flight preparation | x |
|  | ATS Flight Plan | x |
|  | Flight monitoring and in - flight re - planning | x |

| N0 |  | PPL |
| --- | --- | --- |
| 8. | AIRCRAFT GENERAL KNOWLEDGE |  |
| 8. 1 | ENVELOPE, AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT |  |
|  | Design, materials, loads and stresses | x |
|  | Envelope and airbags | x |
|  | Framework | x |
|  | Gondola | x |
|  | Flight controls | x |
|  | Landing gear | x |
|  | Hydraulics and pneumatics | x |
|  | Heating and air conditioning | x |
|  | Fuel system | x |
|  | Piston engines ( propellers ) | x |
|  | Turbine engines ( basics ) | x |
|  | Electrics | x |
|  | Fire protection and detection systems | x |
|  | Maintenance | x |
| 8. 2 | INSTRUMENTATION |  |
|  | Sensors and instruments | x |
|  | Measurement of air data and gas parameters | x |
|  | Magnetism : direct reading compass and flux valve | x |
|  | Gyroscopic instruments | x |
|  | Communication systems | x |
|  | Alerting systems | x |
|  | Integrated instruments : electronic displays | x |
|  | Flight management system ( general basics ) | x |
|  | Digital circuits and computers | x |
| 9. | NAVIGATION |  |
| 9. 1 | GENERAL NAVIGATION |  |
|  | Basics of navigation | x |
|  | Magnetism and compasses | x |
|  | Charts | x |
|  | DR navigation | x |
|  | In - flight navigation | x |
| 9. 2 | RADIO NAVIGATION |  |
|  | Basic radio propagation theory | x |
|  | Radio aids | x |
|  | Radar | x |
|  | GNSS | x |

AMC  3.    FCL. 210 ;    FCL. 215

SYLLABUS    of    THEORETICAL    KNOWLEDGE    for    the    BPL    and    SPL

The  syllabi  for  the  theoretical  knowledge  instruction  and  examination  for  the  LAPL ( B )  and LAPL ( S )  in  AMC 1.  FCL. 115  and   FCL. 120  should  be  used  for  the  BPL  and   SPL, respectively.

AMC  1.    FCL. 215 ;    FCL. 235

THEORETICAL   KNOWLEDGE   EXAMINATION   and   SKILL   TEST   for  the   PPL ( A )

a )    Theoretical  Knowledge  Examination :

1 )  The  examinations  should  comprise  a  total  of  120  multiple-choice  questions  covering  all  the  subjects ;

2 )  Communication  practical  classroom  testing  may  be  conducted ;

3 )  The  period  of  18  months  mentioned  in  FCL. 025 ( b )( 2 )  should  be  counted  from  the  end of  the  calendar  month  when  the  applicant  first  attempted  an  examination.

b )     Skill   Test :

Further  training  may  be  required  following  any  failed  skill  test  or  part  thereof.  There  should be  no  limit  to  the  number  of  skill  tests  that  may  be  attempted ;

c )     Conduct   of   the   Test :

1 )  If  the  applicant  chooses  to  terminate  a  skill  test  for  reasons  considered  inadequate  by  the FE,  the  applicant  should  retake  the  entire  skill  test.  If  the  test  is  terminated  for  reasons considered  adequate  by  the  FE,  only  those  sections  not  completed  should  be  tested  in  a  further  flight ;

2 )  Any  maneuver  or  procedure  of  the  test  may  be  repeated  once  by  the  applicant.                         The  FE  may  stop  the  test  at  any  stage  if  it  is  considered  that  the  applicant’s  demonstration  of  flying  skill  requires  a  complete  retest ;

3 )  An  applicant  should  be  required  to  fly  the  aircraft  from  a  position  where  the  PIC  functions  can  be  performed  and  to  carry  out  the  test  as  if  there  is  no  other  crew  member. Responsibility  for  the  flight  should  be  allocated  in  accordance  with  national  regulations.

AMC  1.    FCL. 235     Skill   Test

CONTENTS   of   the   SKILL   TEST   for   the   ISSUE   of   a   PPL ( A )

a )   The  route  to  be  flown  for  the  navigation  test  should  be  chosen  by  the  FE.  The  route  may end  at  the aerodrome  of  departure  or  at  another  aerodrome.                                                                   The  applicant  should  be  responsible  for  the  flight  planning  and  should  ensure  that  all  equipment  and  documentation  for  the  execution  of  the  flight  are  on  board.  The  navigation section  of  the  test  should  have  a  duration  that  allows  the  pilot  to  demonstrate  his  / her  ability to  complete  a  route  with  at  least  3  ( three )  identified  waypoints  and  may,  as  agreed  between  the  applicant  and  FE,  be  flown  as  a  separate  test ;

b )   An  applicant  should  indicate  to  the  FE  the  checks  and  duties  carried  out,  including  the identification  of  radio  facilities.  Checks  should  be  completed  in  accordance  with  the  authorized checklist  for  the  aeroplane  on  which  the  test  is  being  taken.  During  pre-flight  preparation  for the  test  the  applicant  should  be  required  to  determine  power  settings  and  speeds.  Performance data  for  take-off,  approach  and  landing  should  be  calculated  by  the  applicant  in  compliance with  the  operations  manual  or  flight  manual  for  the  aeroplane  used.

FLIGHT    TEST    TOLERANCE

c )  The  applicant  should  demonstrate  the  ability  to :

1 )  operate  the  aeroplane  within  its  limitations ;

2 )  complete  all  maneuvers  with  smoothness  and  accuracy ;

3 )  exercise  good  judgment  and  airmanship ;

4 )  apply  aeronautical  knowledge ;

5 )  maintain  control  of  the  aeroplane  at  all  times  in  such  a  manner  that  the  successful  outcome  of  a  procedure  or  maneuver  is  never  seriously  in  doubt.

d )   The  following  limits  are  for  general  guidance.  The   FE  should  make  allowance  for turbulent  conditions  and  the  handling  qualities  and  performance  of  the  aeroplane  used :

1 )   Height :

( i )   normal  flight                                   ±  150  ft

( ii )  with  simulated  engine  failure       ±  200  ft  ( if  ME  aeroplane  is  used ) ;

2 )   Heading   or   Tracking   of   Radio   Aids :

( i )   normal  flight                                   ±  10 °

( ii )  with  simulated  engine  failure       ±  15 °  ( if  ME  aeroplane  is  used ) ;

3 )    Speed :

( i )   take-off   and   approach                 +  15  /  – 5  knots

( ii )  all  other  flight  regimes                 ±  15  knots.

CONTENT    of    the    SKILL    TEST

e )   The  skill  test  contents  and  sections  set  out  in  this  AMC  should  be  used  for  the  skill  test for  the  issue  of  a  PPL ( A )  on  SE  and  ME  aeroplanes  or  on  TMGs.

| SECTION 1. PRE - FLIGHT OPERATIONS and DEPARTURE |  |
| --- | --- |
| Use of Checklist, Airmanship, control of aeroplane by external visual reference, anti / de-icing procedures, etc.. , apply in all sections. |  |
| a | Pre-flight documentation, NOTAM and weather briefing |
| b | Mass and balance and performance calculation |
| c | Aeroplane inspection and servicing |
| d | Engine starting and after starting procedures |
| e | Taxiing and aerodrome procedures, pre-take-off procedures |
| f | Take-off and after take-off checks |
| g | Aerodrome departure procedures |
| h | ATC compliance and R / T procedures |
| SECTION 2. GENERAL AIRWORK |  |
| a | ATC compliance and R / T procedures |
| b | Straight and level flight, with speed changes |
| c | Climbing : i. best rate of climb ; ii. climbing turns ; iii. leveling off. |
| d | Medium ( 30 ° bank ) turns |
| e | Steep ( 45 ° bank ) turns ( including recognition and recovery from a spiral dive ) |
| f | Flight at critically low air speed with and without flaps |
| g | Stalling : i. clean stall and recover with power ; ii. approach to stall descending turn with bank angle 200, approach configuration ; iii. approach to stall in landing configuration. |
| h | Descending : i. with and without power ; ii. descending turns (steep gliding turns); iii. leveling off. |

| SECTION 3. EN - ROUTE PROCEDURES |  |
| --- | --- |
| a | Flight plan, dead reckoning and map reading |
| b | Maintenance of altitude, heading and speed |
| c | Orientation, timing and revision of ETAs and log keeping |
| d | Diversion to alternate aerodrome ( planning and implementation ) |
| e | Use of radio navigation aids |
| f | Basic instrument flying check ( 180 ° turn in simulated IMC ) |
| g | Flight management ( checks, fuel systems and carburettor icing, etc.. ) |
| h | ATC compliance and R / T procedures |
| SECTION 4. APPROACH and LANDING PROCEDURES |  |
| a | Aerodrome arrival procedures |
| b | * Precision landing ( short field landing ), crosswind, if suitable conditions available |
| c | * Flapless landing |
| d | * Approach to landing with idle power ( SE only ) |
| e | Touch and Go |
| f | Go - around from low height |
| g | ATC compliance and R / T procedures |
| h | Actions after flight |
| SECTION 5. ABNORMAL and EMERGENCY PROCEDURES |  |
| This section may be combined with Sections 1 through 4 |  |
| a | Simulated engine failure after Take - off ( SE only ) |
| b | * Simulated forced landing ( SE only ) |
| c | Simulated precautionary landing ( SE only ) |
| d | Simulated emergencies |
| e | Oral questions |
| SECTION 6. SIMULATED ASYMMETRIC FLIGHT and RELEVANT CLASS or TYPE ITEMS |  |
| This section may be combined with Sections 1 through 5 |  |
| a | Simulated engine failure during Take-off ( at a safe altitude unless carried out in an FFS ) |
| b | Asymmetric approach and go - around |
| c | Asymmetric approach and full stop landing |
| d | Engine shutdown and restart |
| e | ATC compliance, R / T procedures or airmanship |
| f | As determined by the FE : any relevant items of the Class or Type Rating Skill Test to include, if applicable : i. aeroplane systems including handling of auto pilot ; ii. operation of pressurization system ; iii. use of de-icing and anti-icing system. |
| g | Oral questions |

*   These  items  may  be  combined,  at  the  discretion  of  the  FE.

AMC  2.    FCL. 235     Skill   Test

CONTENTS   of   the   SKILL   TEST   for   the   ISSUE   of   a   PPL ( H )

a )   The  area  and  route  to  be  flown  should  be  chosen  by  the  FE  and  all  low  level  and  hover  work  should  be  at  an  adequate  aerodrome  or  site.  Routes  used  for  Section  3  may  end  at  the aerodrome  of  departure  or  at  another  aerodrome.

The  applicant  should  be  responsible  for  the  flight  planning  and  should  ensure  that  all equipment  and  documentation  for  the  execution  of  the  flight  are  on  board.                                              The  navigation  section  of  the  test,  as  set  out  in  this  AMC  should  consist  of at  least  3 ( three ) legs,  each  leg  of  a  minimum  duration  of  10  minutes.

The  Skill  Test  may  be  conducted  in  two  flights ;

b )   An  applicant  should  indicate  to  the  FE  the  checks  and  duties  carried  out,  including  the identification  of  radio  facilities.  Checks  should  be  completed  in  accordance  with  the  authorized checklist  or  pilot  operating  handbook  for  the  helicopter  on  which  the  test  is  being  taken.  During  pre-flight  preparation  for the  test  the  applicant  is  required  to  determine  power  settings  and  speeds.  Performance data  for  take-off,  approach  and  landing  should  be  calculated  by  the  applicant  in  compliance with  the  operations  manual  or  flight  manual  for  the  helicopter  used.

FLIGHT    TEST    TOLERANCE

c )  The  applicant  should  demonstrate  the  ability  to :

1 )  operate  the  helicopter  within  its  limitations ;

2 )  complete  all  manoeuvres  with  smoothness  and  accuracy ;

3 )  exercise  good  judgement  and  airmanship ;

4 )  apply  aeronautical  knowledge ;

5 )  maintain  control  of  the  helicopter  at  all  times  in  such  a  manner  that  the  successful  outcome  of  a  procedure  or  manoeuvre  is  never  seriously  in  doubt ;

d )   The  following  limits  are  for  general  guidance.  The   FE  should  make  allowance  for turbulent  conditions  and  the  handling  qualities  and  performance  of  the  helicopter  used.

1 )   Height :

( i )    normal  flight                                           ±  150  ft

( ii )   with  simulated  major  emergency          ±  200  ft

( iii )  hovering  IGE  flight                                ±  2  ft

2 )   Heading   or   Tracking   of   Radio   Aids :

( i )   normal  flight                                             ±  10 °

( ii )  with  simulated  major  emergency           ±  15 °

3 )    Speed :

( i )   take-off   and   approach                 +  15  /  – 10  knots

( ii )  all  other  flight  regimes                 ±  15  knots.

4 )     Ground   Drift :

( i )   Take - off   hover   IGE                   ±  3  ft

( ii )   Landing                                        no  sideways  or  backwards  movement

CONTENT    of    the    SKILL    TEST

e )   The  skill  test  contents  and  sections  set  out  in  this  AMC  should  be  used  for  the  skill  test for  the  issue  of  a  PPL ( H )  on  SE  or  ME  helicopters.

| SECTION 1. PRE - FLIGHT or POST - FLIGHT CHECKS and PROCEDURES |  |  |
| --- | --- | --- |
| Use of Checklist, Airmanship, control of helicopter by external visual reference, anti - icing procedures, etc.. , apply in all sections. |  |  |
| a | Helicopter knowledge, ( for example technical log, fuel, mass and balance, performance ), flight planning, NOTAM and weather briefing |  |
| b | Pre - flight inspection or action, location of parts and purpose |  |
| c | Cockpit inspection and starting procedure |  |
| d | Communication and navigation equipment checks, selecting and setting frequencies |  |
| e | Pre - Take - off procedure, R / T procedure and ATC compliance |  |
| f | Parking, shutdown and post - flight procedure |  |
| SECTION 3. HOVER MANOEUVRES, ADVANCED HANDLING and CONFINED AREAS |  |  |
| a | Take - off and landing ( lift - off and touch down ) |  |
| b | Taxi and hover taxi |  |
| c | Stationary hover with head, cross or tail wind |  |
| d | Stationary hover turns, 360 ° left and right ( spot turns ) |  |
| e | Forward, sideways and backwards hover manoeuvring |  |
| f | Simulated engine failure from the hover |  |
| g | Quick stops into and downwin |  |
| h | Sloping ground or unprepared sites landings and take - offs |  |
| i | Take - offs ( various profiles ) |  |
| j | Crosswind and downwind Take - off ( if practicable ) |  |
| k | Take-off at maximum take - off mass ( actual or simulated ) |  |
| l | Approaches ( various profiles ) |  |
| m | Limited power take - off and landing |  |
| n | Autorotations, ( FE to select two items from : basic, range, low speed and 360 ° turns ) |  |
| o | Autorotative landing |  |
| p | Practice forced landing with power recovery |  |
| q | Power checks, reconnaissance technique, approach and departure technique |  |
| SECTION 2. NAVIGATION - EN - ROUTE PROCEDURES |  |  |
| a |  | Navigation and orientation at various altitudes or heights and map reading |
| b |  | Altitude or height, speed, heading control, observation of airspace and altimeter setting |
| c |  | Monitoring of flight progress, flight log, fuel usage, endurance, ETA, assessment of track error and re-establishment of correct track and instrument monitoring |
| d |  | Observation of weather conditions and diversion planning |
| e |  | Use of navigation aids ( where available ) |
| f |  | ATC liaison with due observance of regulations, etc.. . |
| SECTION 4. FLIGHT PROCEDURES and MANOEUVRES |  |  |
| a |  | Level flight, control of heading, altitude or height and speed |
| b |  | Climbing and descending turns to specified headings |
| c |  | Level turns with up to 30 ° bank, 180 ° to 360 ° left and right |
| d |  | Level turns 180 ° left and right by sole reference to instruments |

| SECTION 5. ABNORMAL and EMERGENCY PROCEDURES ( Simulated where Appropriate ) |  |
| --- | --- |
| Note (1) where the test is conducted on an ME helicopter, a simulated engine failure drill, including an SE approach and landing should be included in the test. Note ( 2 ) the FE should select four items from the following : |  |
| a | Engine malfunctions, including governor failure, carburettor or engine icing and oil system, as appropriate |
| b | Fuel system malfunction |
| c | Electrical system malfunction |
| d | Hydraulic system malfunction, including approach and landing without hydraulics, as applicable |
| e | Main rotor or anti-torque system malfunction ( FFS or discussion only ) |
| f | Fire drills, including smoke control and removal, as applicable |
| g | Other abnormal and emergency procedures as outlined in an appropriate flight manual and with reference to Appendix 9 C to Part - FCL, Sections 3 and 4, including for ME helicopters : a ) Simulated engine failure at Take - off : 1 ) rejected take-off at or before TDP or safe forced landing at or before DPATO ; 2 ) shortly after TDP or DPATO. b ) Landing with simulated engine failure : 1 ) landing or go-around following engine failure before LDP or DPBL ; 2 ) following engine failure after LDP or safe forced landing after DPBL. |

AMC  3.    FCL. 235     Skill   Test

CONTENT   of   the   SKILL   TEST   for   the   ISSUE   of   the   PPL ( AS )

a )  The  area  and  route  to  be  flown  is  chosen  by  the  FE.  Routes  used  for  section  3  may  end  at  the  aerodrome  of  departure  or  at  another  aerodrome  and  one  destination  should  be  a controlled  aerodrome.  The  skill  test  may  be  conducted  in  two  flights.  The  total  duration  of  the flight(s)  should  be  at  least  60 minutes ;

b )  The  applicant  should  demonstrate  the  ability  to :

1 )  operate  the  airship  within  its  limitations ;

2 )  complete  all  manoeuvres  with  smoothness  and  accuracy ;

3 )  exercise  good  judgement  and  airmanship ;

4 )  apply  aeronautical  knowledge ;

5 )  maintain  control  of  the  airship  at  all  times  in  such  a  manner  that  the  successful  outcome of  a  procedure  or  manoeuvre  is  never  seriously  in  doubt.

FLIGHT    TEST    TOLERANCES

c )  The  following  limits  should  apply,  corrected  to  make  allowance  for  turbulent  conditions  and  the  handling  qualities  and  performance  of  the  airship  used :

1 )   Height :          ( i )     normal  flight                             ± 200  ft

( ii )   simulated  major  emergency    ±  300  ft

2 )   Tracking  on  Radio  Aids :                                         ±  15 °

3 )    Heading :       ( i )     normal  flight                            ±  15 °

( ii )    simulated  major  emergency    ±  20 °

CONTENT    of    the    TEST

d )   The  skill  test  contents  and  sections  set  out  in  this  AMC  should  be  used  for  the  skill  test for  the  issue  of  a  PPL ( As ) ;

e )   Items  in  sections  5  and  6  may  be  performed  in  an  FNPT ( As )  or  a  FS ( As ).

| SECTION 1. PRE - FLIGHT OPERATIONS and DEPARTURE |  |
| --- | --- |
| Use of airship Checklists, Airmanship, control of airship by external visual reference, anti-icing procedures, and principles of threat and error management, etc.. apply in all Sections |  |
| a | Pre - flight, including : flight planning, documentation, mass and balance, NOTAM and weather briefing |
| b | Airship inspection and servicing |
| c | Off-mast procedure, ground manoeuvring and take-off |
| d | Performance considerations and trim |
| e | Aerodrome and traffic pattern operations |
| f | Departure procedure, altimeter setting, collision avoidance ( look-out ) |
| g | ATC compliance and R / T procedures |
| SECTION 2. GENERAL AIRWORK |  |
| a | Control of the airship by external visual reference, including straight and level, climb, descent and look-out |
| b | Flight close to pressure height |
| c | Turns |
| d | Steep descents and climbs |
| e | Flight by reference solely to instruments, including : i. Level flight, control of heading, altitude and air speed ; ii. Climbing and descending turns ; iii. Recoveries from unusual attitudes. |
| f | ATC compliance and R / T procedures |
| SECTION 3. EN - ROUTE PROCEDURES |  |
| a | Flight Plan, dead reckoning and map reading |
| b | Maintenance of altitude, heading and speed and collision avoidance ( look-out procedures ) |
| c | Orientation, timing and revision of ETAs and log keeping |
| d | Observation of weather conditions and diversion to alternate aerodrome ( planning and implementation ) |
| e | Use of radio navigation aids |
| f | Flight management ( checks, fuel systems, etc.. ) |
| g | ATC compliance and R / T procedures |
| SECTION 4. APPROACH and LANDING PROCEDURES |  |
| a | Aerodrome arrival procedures, altimeter setting, checks and look-out |
| b | ATC compliance and R / T procedures |
| c | Go - around action |
| d | Normal landing |
| e | Short field landing |
| f | Post - flight actions |
| SECTION 5. ABNORMAL and EMERGENCY PROCEDURES |  |
| This Section may be combined with Sections 1 through 4 |  |
| a | Simulated engine failure after take-off ( at a safe altitude ) and fire drill |
| b | Equipment malfunctions |
| c | Forced landing ( simulated ) |
| d | ATC compliance and R / T procedures |
| e | Oral questions |

| SECTION 6. RELEVANT TYPE ITEMS |  |
| --- | --- |
| This Section may be combined with Sections 1 through 5 |  |
| a | Simulated engine failure during take-off ( at a safe altitude unless carried out in a FFS ) |
| b | Approach and go-around with failed engine(s) |
| c | Approach and full stop landing with failed engine(s) |
| d | Malfunctions in the envelope pressure system |
| e | ATC compliance and R / T procedures and Airmanship |
| f | As determined by the FE : any relevant items of the type rating skill test to include, if applicable : i. Airship systems ; ii. Operation of envelope pressure system. |
| g | Oral questions |

AMC  1.    FCL. 210. A     PPL ( A )  -  Experience   Requirements   and   Crediting

FLIGHT    INSTRUCTION    for    the    PPL ( A )

a )   Entry  to  Training :

Before  being  accepted  for  training  an  applicant  should  be  informed  that  the  appropriate  medical  certificate  must  be  obtained  before  solo  flying  is  permitted ;

b )   Flight   Instruction :

1 )  The  PPL( A )  flight  instruction  syllabus  takes  into  account  the  principles  of  threat  and  error  management  and  also  covers :

( i )    pre-flight  operations,  including  mass  and  balance  determination,  aircraft  inspection  and servicing ;

( ii )   aerodrome  and  traffic  pattern  operations,  collision  avoidance  precautions  and  procedures;

( iii )  control  of  the  aircraft  by  external  visual  reference ;

( iv )  flight  at  critically  low  air  speeds,  recognition  of,  and  recovery  from,  incipient  and  full stalls ;

( v )   flight  at  critically  high  air  speeds,  recognition  of,  and  recovery  from,  spiral  dive ;

( vi )  normal  and  crosswind  take-offs  and  landings ;

( vii ) maximum  performance  ( short  field  and  obstacle  clearance )  take- offs,  short-field  landings ;

( viii ) flight  by  reference  solely  to  instruments,  including  the  completion  of  a  level  180 ° turn;

( ix )   cross-country  flying  using  visual  reference,  dead  reckoning  and  radio  navigation  aids ;

( x )    emergency  operations,  including  simulated  aeroplane  equipment  malfunctions ;

( xi )   operations  to,  from  and  transiting  controlled  aerodromes,  compliance  with  air  traffic services  procedures,  communication  procedures  and  phraseology.

2 )   Before  allowing  the  applicant  for  a  PPL( A )  to  undertake  his / her  first  solo  flight,  the  FI should  ensure  that  the  applicant  can  use  R / T  communication.

c )   Syllabus   of   Flight   Instruction :

1 )  The  numbering  of  exercises  should  be  used  primarily  as  an  exercise  reference  list  and  as  a  broad  instructional  sequencing  guide ;   therefore  the  demonstrations  and  practices  need  not necessarily  be  given  in  the  order  listed.   The  actual  order  and  content  will  depend  upon  the following  interrelated  factors :

( i )    the  applicant’s  progress  and  ability ;

( ii )   the  weather  conditions  affecting  the  flight ;

( iii )  the  flight  time  available ;

( iv )  instructional  technique  considerations ;

( v )   the  local  operating  environment ;

( vi )  applicability  of  the  exercises  to  the  aeroplane.

2 )  Each  of  the  exercises  involves  the  need  for  the  applicant  to  be  aware  of  the  needs  of good  airmanship  and  look-out,  which  should  be  emphasized  at  all  times.

( i )    Exercise 1 a :   Familiarization  with  the  Aeroplane :

(A)   characteristics  of  the  aeroplane ;

(B)   cockpit  layout ;

(C)   systems ;

(D)  checklists,  drills  and  controls.

( ii )   Exercise  1 b :    Emergency   Drills :

(A)   action  if  fire  on  the  ground  and  in  the  air ;

(B)   engine  cabin  and  electrical  system  fire ;

(C)   systems  failure ;

(D)  escape  drills,  location  and  use  of  emergency  equipment  and  exits.

( iii )  Exercise  2 :    Preparation  for  and  Action  after  Flight :

(A)   flight  authorization  and  aeroplane  acceptance ;

(B)   serviceability  documents ;

(C)   equipment  required,  maps,  etc. ;

(D)  external  checks ;

(E)   internal  checks ;

(F)   harness,  seat  or  rudder  panel  adjustments ;

(G)  starting  and  warm-up  checks ;

(H)  power  checks ;

(I)   running  down  system  checks  and  switching  off  the  engine ;

(J)   parking,  security  and  picketing  ( for  example  tie  down ) ;

(K)   completion  of  authorization  sheet  and  serviceability  documents.

( iv )  Exercise  3 :    Air  Experience :   Flight  Exercise.

( v )   Exercise   4 :    Effects   of   Controls :

(A)   primary  effects  when  laterally  level  and  when  banked ;

(B)   further  effects  of  aileron  and  rudder ;

(C)   effects  of :

(a)   air  speed ;

(b)   slipstream ;

(c)   power ;

(d)   trimming  controls ;

(e)   flaps ;

(f)   other  controls,  as  applicable.

(D)  operation  of :

(a)   mixture  control ;

(b)   carburettor  heat ;

(c)   cabin  heating  or  ventilation.

( vi )  Exercise  5 a :    Taxiing :

(A)   pre - taxi  checks ;

(B)   starting,  control  of  speed  and  stopping ;

(C)   engine  handling ;

(D)  control  of  direction  and  turning ;

(E)   turning  in  confined  spaces ;

(F)   parking  area  procedure  and  precautions ;

(G)  effects  of  wind  and  use  of  flying  controls ;

(H)   effects  of  ground  surface ;

(I)    freedom  of  rudder  movement ;

(J)    marshalling  signals ;

(K)   instrument  checks ;

(L)   air  traffic  control  procedures.

( vii ) Exercise  5 b :    Emergencies :  brake  and  steering  failure.

( viii ) Exercise  6 :       Straight   and   Level :

(A)   at  normal  cruising  power,  attaining  and  maintaining  straight  and  level  flight ;

(B)   flight  at  critically  high  air  speeds ;

(C)   demonstration  of  inherent  stability ;

(D)   control  in  pitch,  including  use  of  trim ;

(E)   lateral  level,  direction  and  balance  and  trim ;

(F)   at  selected  air  speeds  ( use  of  power ) ;

(G)   during  speed  and  configuration  changes ;

(H)   use  of  instruments  for  precision.

( ix )  Exercise  7 :     Climbing :

(A)   entry,  maintaining  the  normal  and  max  rate  climb  and  leveling  off ;

(B)   leveling  off  at  selected  altitudes ;

(C)   en-route  climb  ( cruise  climb ) ;

(D)   climbing  with  flap  down ;

(E)    recovery  to  normal  climb ;

(F)    maximum  angle  of  climb ;

(G)   use  of  instruments  for  precision.

( x )   Exercise  8 :      Descending :

(A)   entry,  maintaining  and  leveling  off ;

(B)   leveling  off  at  selected  altitudes ;

(C)   glide,  powered  and  cruise  descent  ( including  effect  of  power  and  air  speed ) ;

(D)   side  slipping  ( on  suitable  types ) ;

(E)   use  of  instruments  for  precision  flight.

( xi )   Exercise  9 :      Turning :

(A)   entry  and  maintaining  medium  level  turns ;

(B)   resuming  straight  flight ;

(C)   faults  in  the  turn  ( for  example  in  correct  pitch,  bank  and  balance ) ;

(D)   climbing  turns ;

(E)   descending  turns ;

(F)   faults  in  the  turns  ( slipping  and  skidding  on  suitable  types ) ;

(G)   turns  onto  selected  headings,  use  of  gyro  heading  indicator  and  compass ;

(H)   use  of  instruments  for  precision.

( xii ) Exercise  10 a :     Slow   Flight :

Note :    the  objective  is  to  improve  the  student’s  ability  to  recognize  inadvertent  flight  at critically  low  speeds  and  provide  practice  in  maintaining  the  aeroplane  in  balance  while  returning  to  normal  air  speed.

(A)   safety  checks ;

(B)   introduction  to  slow  flight ;

(C)   controlled  flight  down  to  critically  slow  air  speed ;

(D)  application  of  full  power  with  correct  attitude  and  balance  to  achieve  normal  climb  speed.

( xiii )  Exercise  10 b :     Stalling :

(A)   safety  checks ;

(B)   symptoms ;

(C)   recognition ;

(D)  clean  stall  and  recovery  without  power  and  with  power ;

(E)   recovery  when  a  wing  drops ;

(F)   approach  to  stall  in  the  approach  and  in  the  landing  configurations,  with  and  without

power  and  recovery  at  the  incipient  stage.

( xiv )  Exercise  11 :       Spin   Avoidance :

(A)   safety  checks ;

(B)   stalling  and  recovery  at  the  incipient  spin  stage  ( stall  with  excessive  wing  drop,

about  45 ° ) ;

(C)   instructor  induced  distractions  during  the  stall.

Note  1:    at  least  two  hours  of  stall  awareness  and  spin  avoidance  flight  training  should

be  completed  during  the  course.

Note  2 :   consideration  of  manoeuvre  limitations  and  the  need  to  refer  to  the  aeroplane

manual  and  mass  and  balance  calculations.

( xv ) Exercise  12 :      Take-off  and  Climb  to  Downwind  Position :

(A)   pre-take-off  checks ;

(B)   into  wind  take-off ;

(C)   safeguarding  the  nose  wheel ;

(D)  crosswind  take-off ;

(E)   drills  during  and  after  take-off ;

(F)   short  take-off  and  soft  field  procedure / techniques  including  performance  calculations ;

(G)  noise  abatement  procedures.

( xvi )  Exercise  13 :        Circuit,   Approach   and   Landing :

(A)   circuit  procedures,  downwind  and  base  leg ;

(B)   powered  approach  and  landing ;

(C)   safeguarding  the  nose  wheel ;

(D)  effect  of  wind  on  approach  and  touchdown  speeds  and  use  of  flaps ;

(E)   crosswind  approach  and  landing ;

(F)   glide  approach  and  landing ;

(G)  short  landing  and  soft  field  procedures  or  techniques ;

(H)  flapless  approach  and  landing ;

(I)    wheel  landing  ( tail  wheel  aeroplanes ) ;

(J)   missed  approach  and  go-around ;

(K)   noise  abatement  procedures.

( xvii )  Exercise  12 / 13 :         Emergencies :

(A)   abandoned   take-off ;

(B)   engine  failure  after  take-off ;

(C)   mislanding  and  go-around ;

(D)   missed  approach.

Note :   in  the  interests  of  safety  it  will  be  necessary  for  pilots  trained  on  nose  wheel

aeroplanes  to  undergo  dual  conversion  training  before  flying  tail  wheel  aeroplanes,

and  vice-versa.

( xviii )  Exercise  14 :           First   Solo :

(A)   instructor’s  briefing,  observation  of  flight  and  de-briefing ;

Note :    during  flights  immediately  following  the  solo  circuit  consolidation  the  following

should  be  revised :

(B)   procedures  for  leaving  and  rejoining  the  circuit ;

(C)   the  local  area,  restrictions,  map  reading ;

(D)  use  of  radio  aids  for  homing ;

(E)   turns  using  magnetic  compass,  compass  errors.

( xix )  Exercise  15 :     Advanced   Turning :

(A)   steep  turns  ( 45 ° ),  level  and  descending ;

(B)   stalling  in  the  turn  and  recovery ;

(C)   recoveries  from  unusual  attitudes,  including  spiral  dives.

( xx )  Exercise  16 :       Forced   Landing   without   Power :

(A)   forced  landing  procedure ;

(B)   choice  of  landing  area,  provision  for  change  of  plan ;

(C)   gliding  distance ;

(D)  descent  plan ;

(E)   key  positions ;

(F)   engine  cooling ;

(G)  engine  failure  checks ;

(H)  use  of  radio ;

(I)    base  leg ;

(J)    final  approach ;

(K)   landing ;

(L)    actions  after  landing.

( xxi )  Exercise  17 :      Precautionary   Landing :

(A)   full  procedure  away  from  aerodrome  to  break-off  height ;

(B)   occasions  necessitating ;

(C)   in-flight  conditions ;

(D)  landing  area  selection :

(a)   normal  aerodrome ;

(b)   disused  aerodrome ;

(c)   ordinary  field.

(E)   circuit  and  approach ;

(F)   actions  after  landing.

( xxii )  Exercise  18 a :       Navigation :

(A)   flight  planning :

(a)   weather  forecast  and  actuals ;

(b)   map  selection  and  preparation :

(1)   choice  of  route ;

(2)   controlled  airspace ;

(3)   danger,  prohibited  and  restricted  areas ;

(4)   safety  altitudes.

(c)   calculations :

(1)   magnetic  heading(s)  and  time(s)  en-route ;

(2)   fuel  consumption ;

(3)   mass  and  balance ;

(4)   mass  and  performance.

(d)   flight  information :

(1)   NOTAMs  etc.. ;

(2)   radio  frequencies ;

(3)   selection  of  alternate  aerodromes.

(e)   aeroplane  documentation ;

(f)    notification  of  the  flight :

(1)   pre-flight  administrative  procedures ;

(2)   flight  plan  form.

(B)   departure :

(a)   organization  of  cockpit  workload ;

(b)   departure  procedures :

(1)   altimeter  settings ;

(2)   ATC  liaison  in  controlled  or  regulated  airspace ;

(3)   setting  heading  procedure ;

(4)   noting  of  ETAs.

(c)   maintenance  of  altitude  and  heading ;

(d)   revisions  of  ETA  and  heading ;

(e)   log  keeping ;

(f)    use  of  radio ;

(g)   use  of  Navaids ;

(h)   minimum  weather  conditions  for  continuation  of  flight ;

(i)    in-flight  decisions ;

(j)    transiting  controlled  or  regulated  airspace ;

(k)   diversion  procedures ;

(l)    uncertainty  of  position  procedure ;

(m)  lost  procedure.

(C)   arrival  and  aerodrome  joining  procedure :

(a)   ATC  liaison  in  controlled  or  regulated  airspace ;

(b)   altimeter  setting ;

(c)   entering  the  traffic  pattern ;

(d)   circuit  procedures ;

(e)   parking ;

(f)    security  of  aeroplane ;

(g)   refuelling ;

(h)   closing  of  flight  plan,  if  appropriate ;

(i)    post-flight  administrative  procedures.

( xxiii )  Exercise  18 b :   Navigation  Problems  at  Lower  Levels  and  in  Reduced  Visibility :

(A)   actions  before  descending ;

(B)   hazards  ( for  example  obstacles  and  terrain ) ;

(C)   difficulties  of  map  reading ;

(D)   effects  of  wind  and  turbulence ;

(E)   vertical  situational  awareness  ( avoidance  of  controlled  flight  into  terrain ) ;

(F)   avoidance  of  noise  sensitive  areas ;

(G)  joining  the  circuit ;

(H)  bad  weather  circuit  and  landing.

( xxiv )  Exercise  18 c :        Radio   Navigation :

(A)   use  of  GNSS :

(a)   selection  of  waypoints ;

(b)   to  or  from  indications  and  orientation ;

(c)   error  messages.

(B)   use  of  VHF  omni-range :

(a)   availability,  AIP  and  frequencies ;

(b)   selection  and  identification ;

(c)   OBS ;

(d)   to  or  from  indications  and  orientation ;

(e)   CDI ;

(f)    determination  of  radial ;

(g)   intercepting  and  maintaining  a  radial ;

(h)   VOR  passage ;

(i)    obtaining  a  fix  from  two  VORs.

(C)   use  of  ADF  equipment :  NDBs :

(a)   availability,  AIP  and  frequencies ;

(b)   selection  and  identification ;

(c)   orientation  relative  to  the  beacon ;

(d)   homing.

(D)  use  of  VHF / DF :

(a)   availability,  AIP,  frequencies ;

(b)   R / T  procedures  and  ATC  liaison ;

(c)   obtaining  a  QDM  and  homing.

(E)   use  of  en-route  or  terminal  radar :

(a)   availability  and  AIP ;

(b)   procedures  and  ATC  liaison ;

(c)   pilot’s  responsibilities ;

(d)   BITD ;

(d)   secondary  surveillance  radar :

(1)   transponders ;

(2)   code  selection ;

(3)   interrogation  and  reply.

(F)   use  of  DME :

(a)   station  selection  and  identification ;

(b)   modes  of  operation :   distance,  groundspeed  and  time  to  run.

( xxv )  Exercise  19 :      Basic   Instrument   Flight :

(A)   physiological  sensations ;

(B)   instrument  appreciation ;   attitude  instrument  flight ;

(C)   instrument  limitations ;

(D)   basic  manoeuvres ;

(a)   straight  and  level  at  various  air  speeds  and  configurations ;

(b)   climbing  and  descending ;

(c)   standard  rate  turns,  climbing  and  descending,  onto  selected  headings ;

(d)   recoveries  from  climbing  and  descending  turns.

(1)   a  BITD  may  be  used  for  flight  training  for :

(i)    flight  by  reference  solely  to  instruments ;

(ii)   navigation  using  radio  navigation  aids ;

(iii)  basic  instrument  flight.

(2)   The  use  of  the  BITD  should  be  subject  to  the  following :

(i)    the  training  should  be  complemented  by  exercises  on  an  aeroplane ;

(ii)   the  record  of  the  parameters  of  the  flight  must  be  available ;

(iii)  a  FI ( A )  or  STI ( A )  should  conduct  the  instruction.

AMC  1.    FCL. 210. H     PPL ( H )  -  Experience   Requirements   and   Crediting

FLIGHT    INSTRUCTION    for    the    PPL ( H )

a )   Entry   to   Training :

Before  being  accepted  for  training  an  applicant  should  be  informed  that  the  appropriate  medical  certificate  must  be  obtained  before  solo  flying  is  permitted ;

b )   Ground   Instruction :

Enhanced  ground  instruction  in  weather  interpretation,  planning  and  route  assessment,  decision  making  on   encountering   DVE  including  reversing  course  or  conducting  a  precautionary  landing ;

c )   Flight   Instruction :

1)  The  PPL( H )  flight  instruction  syllabus  should  take  into  account  the  principles  of  threat  and  error  management  and  cover :

(i)    pre-flight  operations,  including  mass  and  balance  determination,  helicopter  inspection  and servicing ;

(ii)   aerodrome  and  traffic  pattern  operations,  collision  avoidance  precautions  and  procedures ;

(iii)  control  of  the  helicopter  by  external  visual  reference ;

(iv)   take-offs,  landings,  hovering,  look-out  turns  and  normal  transitions  from  and  to  the  hover ;

(v)  emergency  procedures,  basic  autorotations,  simulated  engine  failure,  ground  resonance recovery  if  relevant  to  type ;

(vi)  sideways  and  backwards  flight,  turns  on  the  spot ;

(vii) incipient  vortex  ring  recognition  and  recovery ;

(viii) touchdown  autorotations,  simulated  engine-off  landings,  practice  forced  landings.  Simulated  equipment  malfunctions  and  emergency  procedures  relating  to  malfunctions  of  engines,  controls, electrical  and  hydraulic  circuits ;

(ix)  steep  turns ;

(x)   transitions,  quick  stops,  out  of  wind  manoeuvres,  sloping  ground  landings  and  take-offs ;

(xi)  limited  power  and  confined  area  operations,  including  selection  of  and  operations  to  and from  unprepared  sites ;

(xii) flight  by  sole  reference  to  basic  flight  instruments,  including  completion  of  a  level  180 ° turn  and  recovery  from  unusual  attitudes  to  simulate  inadvertent  entry  into  cloud  ( this  training may  be  conducted  by  an  FI( H ) ) ;

(xiii) cross-country  flying  by  using  visual  reference,  DR,  GNNS  and,  where  available,  radio navigation  aids ;   simulation  of  deteriorating  weather  conditions  and  actions  to  divert  or  conduct  precautionary  landing ;

(xiv) operations  to,  from  and  transiting  controlled  aerodromes ;   compliance  with  air  traffic services  procedures,  communication  procedures  and  phraseology ;

2 )  Before  allowing  the  applicant  for  a  PPL( H )  to  undertake  his / her  first  solo  flight,  the  FI should  ensure  that  the  applicant  can  use  R / T  communication ;

3 )  Wherever  possible,  flight  simulation  should  be  used  to  demonstrate  to  student  pilots  the  effects  of  flight  into  DVE  and  to  enhance  their  understanding  and  need  for  avoidance  of  this potentially  fatal  flight  regime.

d )   Syllabus   of   Flight   Instruction :

1 )  The  numbering  of  exercises  should  be  used  primarily  as  an  exercise  reference  list  and  as  a  broad  instructional  sequencing  guide ;   therefore  the  demonstrations  and  practices  need  not necessarily  be  given  in  the  order  listed.  The  actual  order  and  content  will  depend  upon  the following  interrelated  factors :

(i)    the  applicant’s  progress  and  ability ;

(ii)   the  weather  conditions  affecting  the  flight ;

(iii)  the  flight  time  available ;

(iv)  instructional  technique  considerations ;

(v)   the  local  operating  environment ;

(vi)  applicability  of  the  exercises  to  the  helicopter.

2 )  Each  of  the  exercises  involves  the  need  for  the  applicant  to  be  aware  of  the  needs  of good  airmanship  and  look-out,  which  should  be  emphasized  at  all  times.

( i )    Exercise 1 a :        Familiarization   with   the   Helicopter :

(A)   characteristics of the helicopter, external features;

(B)   cockpit layout;

(C)   systems;

(D)  checklists, procedures and controls.

( ii )   Exercise  1 b :       Emergency   Procedures :

(A)   action if fire on the ground and in the air;

(B)   engine, cabin and electrical system fire;

(C)   systems failures;

(D)  escape drills, location and use of emergency equipment and exits.

( iii )  Exercise  2 :      Preparation  for  and  action  after  flight :

(A)   flight authorization and helicopter acceptance;

(B)   serviceability documents;

(C)   equipment required, maps, etc.;

(D)  external checks;

(E)   internal checks;

(F)   seat, harness and flight controls adjustments;

(G)  starting  and  warm-up  checks  clutch  engagement  and starting rotors;

(H)  power checks;

(I)   running down system checks and switching off the engine;

(J)   parking, security and picketing;

(K)   completion  of  authorization  sheet  and  serviceability  documents.

( iv )   Exercise  3 :        Air   Experience :

(A)   to introduce the student to rotary wing flight;

(B)   flight  exercise.

( v )    Exercise  4 :         Effects   of   Controls :

(A)   function  of  flight  controls,  primary  and  secondary  effect;

(B)   effects  of :

(a)   air speed;

(b)   power changes (torque);

(c)   yaw (sideslip);

(d)   disc loading (bank and flare);

(e)   controls of selecting hydraulics on/off;

(f)    control friction.

(C)   instruments;

(D)  use of carburettor heat or anti-icing control.

( vi )   Exercise  5 :    Power   and   Attitude   Changes :

(A)   relationship between cyclic control position, disc attitude, fuselage attitude and air speed;

(B)   flapback;

(C)   power required diagram in relation to air speed;

(D)  power and air speed changes in level flight;

(E)   use of instruments for precision;

(F)   engine and air speed limitations.

( vii )  Exercise  6 :         Straight   and   Level :

(A)   at normal cruising power, attaining and maintaining straight and level flight;

(B)   control in pitch, including use of control friction or trim;

(C)   maintaining direction and balance, ( ball  or  yawstring  use ) ;

(D)  setting power for selected air speeds and speed changes;

(E)   use of instruments for precision.

( viii )  Exercise  7 :           Climbing :

(A)   optimum  climb  speed,  best  angle  or  rate  of  climb  from power required diagram;

(B)   initiation,  maintaining  the  normal  and  maximum  rate  of climb, levelling off;

(C)   levelling off at selected altitudes or heights; (D)  use of instruments for precision.

( ix )   Exercise  8 :             Descending :

(A)   optimum descent speed, best angle or rate of descent from power required diagram;

(B)   initiation, maintaining and leveling off;

(C)   leveling off at selected altitudes or heights;

(D)  descent (including effect of power and air speed);

(E)   use of instruments for precision.

( x )   Exercise  9 :            Turning :

(A)   initiation and maintaining medium level turns;

(B)   resuming straight flight;

(C)   altitude, bank and co-ordination;

(D)  climbing and descending turns and effect on rate of climb or descent;

(E)   turns onto selected headings, use of gyro heading indicator and compass;

(F)   use of instruments for precision.

( xi )   Exercise  10 :     Basic   Autorotation :

(A)   safety checks, verbal warning and look-out;

(B)   entry, development and characteristics;

(C)   control of air speed and RRPM, rotor and engine limitations;

(D)  effect  of  AUM,  IAS,  disc  loading,  G  forces  and  density altitude;

(E)   re-engagement  and  go-around procedures ( throttle  over- ride  or  ERPM  control ) ;

(F)   vortex condition during recovery;

(G)  gentle and medium turns in autorotation;

(H)  demonstration of variable flare simulated engine off landing.

( xii )  Exercise  11 a :       Hovering :

(A)  demonstrate hover IGE, importance of wind effect and attitude, ground cushion, stability in the

hover and effects of over controlling;

(B)   student holding cyclic stick only;

(C)   student handling collective lever  ( and  throttle )  only;

(D)   student handling collective lever, ( throttle ) and pedals;

(E)   student handling all controls;

(F)   demonstration of ground effect;

(G)  demonstration of wind effect;

(H)  demonstrate gentle forward running touchdown;

(I)   specific hazards for example snow, dust and litter.

( xiii )  Exercise  11 b :     Hover   Taxiing   and   Spot   Turns :

(A)   revise hovering;

(B)   precise ground speed and height control;

(C)   effect of wind direction on helicopter attitude and control margin;

(D)  control and co-ordination during spot turns;

(E)   carefully introduce gentle forward running touchdown.

( xiv )  Exercise  11 c :      Hovering   and   Taxiing   Emergencies :

(A)   revise  hovering  and  gentle  forward  running  touchdown, explain ( demonstrate  where

applicable )  effect of hydraulics failure in the hover;

(B)   demonstrate simulated engine failure in the hover and hover taxi

(C)   demonstrate dangers of mishandling and over-pitching.

( xv )  Exercise  12 :        Take - off   and   Landing :

(A)   pre-take-off checks or drills;

(B)   look-out;

(C)   lifting to hover;

(D)  after take-off checks;

(E)   danger of horizontal movement near ground;

(F)   danger of mishandling and overpitching;

(G)  landing ( without  sideways  or  backwards  movement ) ;

(H)  after landing checks or drills;

(I)   take-off and landing crosswind and downwind.

( xvi )  Exercise  13 :     Transitions  from  hover  to  climb  and  approach  to  hover :

(A)   look-out;

(B)   revise take-off and landing;

(C)   ground effect, translational lift and its effects;

(D)  flapback  and  its  effects;

(E)   effect of wind speed and direction during transitions from or to the hover;

(F)   the constant angle approach;

(G)  demonstration of variable flare simulated engine off landing.

( xvii )  Exercise  14 a :        Circuit,   Approach   and   Landing :

(A)   revise  transitions  from  hover  to  climb  and  approach  to hover;

(B)   circuit  procedures, downwind and base leg;

(C)   approach and landing with power;

(D)   pre-landing checks;

(E)   effect of wind on approach and IGE hover;

(F)   crosswind approach and landing;

(G)  go-around;

(H)  noise abatement procedures.

( xviii )   Exercise  14 b :    Steep  and  limited   power   approaches  and   Landings :

(A)   revise the constant angle approach;

(B)   the steep approach ( explain  danger  of  high  sink  rate  and  low  air  speed ) ;

(C)   limited  power approach ( explain  danger  of  high  speed  at  touch-down ) ;

(D)  use of the ground effect;

(E)   variable flare simulated engine off landing.

( xix)   Exercise  14 c :       Emergency   Procedures :

(A)   abandoned  take-off;

(B)   missed approach and go-around;

(C)   hydraulic off landing ( if  applicable );

(D)  tail rotor control or tail rotor drive failure ( briefing  only );

(E)   simulated emergencies in the circuit to include:

(a)   hydraulics failure;

(b)   simulated  engine  failure  on  take-off,  crosswind,  downwind  and  base leg;

(c)   governor failure.

( xx )  Exercise  15 :          First   Solo :

(A)   instructor’s briefing, observation of flight and debriefing;

(B)   warn  of  change  of  attitude  from  reduced  and  laterally displaced weight;

(C)   warn of low tail, low skid or wheel during hover, landing;

(D)  warn of dangers of loss of RRPM and overpitching;

(E)   pre-take-off checks;

(F)   into wind take-off;

(G)  procedures during and after take-off;

(H)  normal circuit, approaches and landings;

(I)   action if an emergency.

( xxi )  Exercise  16 :     Sideways   and   backwards   hover   manoeuvring :

(A)   manoeuvring sideways flight heading into wind;

(B)   manoeuvring backwards flight heading into wind;

(C)   combination of sideways and backwards manoeuvring;

(D)  manoeuvring sideways and backwards and heading out of wind;

(E)   stability and weather cocking;

(F)   recovery from backwards manoeuvring (pitch nose down);

(G)  limitations for sideways and backwards manoeuvring.

( xxii )  Exercise  17 :       Spot   Turns :

(A)   revise hovering into wind and downwind;

(B)   turn on spot through 360º:

(a)   around pilots position;

(b)   around tail rotor;

(c)   around helicopter geometric centre;

(d)   square and safe visibility clearing turn.

(C)   rotor RPM control, torque effect, cyclic limiting stops due to CG position and wind speed and

direction ;

( xxiii )  Exercise  18 :      Hover   OGE   and   vortex   ring :

(A)   establishing hover OGE;

(B)   drift, height or power control;

(C)   demonstration of incipient stage of vortex ring, recognition and recovery (from a safe altitude);

(D)  loss of tail rotor effectiveness.

( xxiv )  Exercise  19 :         Simulated   EOL :

(A)   the effect of weight, disc loading, density attitude and RRPM decay;

(B)   revise basic autorotation entry;

(C)   optimum use of cyclic and collective to control speed or RRPM;

(D)  variable flare simulated EOL;

(E)   demonstrate constant attitude simulated EOL;

(F)   demonstrate simulated EOL from hover or hover taxi;

(G)  demonstrate simulated EOL from transition and low level.

( xxv )  Exercise  20 :           Advanced   Autorotation :

(A)   over a selected point at various height and speed;

(B)   revise basic autorotation: note ground distance covered;

(C)   range autorotation;

(D)  low speed autorotation;

(E)   constant attitude autorotation (terminate at safe altitude);

(F)   ‘S’ turns;

(G)  turns through 180 ° and 360 °;

(H)  effects on angles of descent, IAS, RRPM and effect of AUM.

( xxvi )  Exercise  21 :       Practice   forced   landings :

(A)   procedure and choice of the forced landing area;

(B)   forced landing checks and crash action;

(C)   re-engagement and go-around procedures.

( xxvii )  Exercise  22 :         Steep   Turns :

(A)   steep (level) turns (30 ° bank);

(B)   maximum rate turns (45 ° bank if possible);

(C)   steep autorotative turns;

(D)  faults in the turn: balance, attitude, bank and co-ordination;

(E)   RRPM control and disc loading;

(F)   vibration and control feedback;

(G)  effect of wind at low level.

( xxviii )  Exercise  23 :         Transitions :

(A)   revise ground effect, translational lift and flapback;

(B)   maintaining constant height, (20-30 ft AGL);

(C)   transition from hover to minimum 50 knots IAS and back to hover;

(D)  demonstrate effect of wind.

( xxix )  Exercise  24 :            Quick   Stops :

(A)   use of power and controls;

(B)   effect of wind;

(C)   quick stops into wind;

(D)  quick stops from crosswind and downwind terminating into wind;

(E)   danger of vortex ring;

(F)   danger of high disc loading.

( xxx )  Exercise  25 a :          Navigation :

(A)   flight planning:

(a)   weather forecast and actuals;

(b)   map selection and preparation and use;

(1)   choice of route:

(2)   controlled airspace, danger and prohibited areas;

(3)   safety  altitudes  and  noise  abatement  considerations.

(c)   calculations:

(1)   magnetic heading(s) and time(s) en-route;

(2)   fuel consumption;

(3)   mass and balance.

(d)   flight information:

(1)   NOTAMs, etc.;

(2)   radio frequencies;

(3)   selection of alternate landing sites.

(e)   helicopter documentation;

(f)    notification of the flight:

(1)   pre-flight administrative procedures;

(2)   flight plan form (where appropriate).

(B)   departure:

(a)   organization of cockpit workload;

(b)   departure procedures:

(1)   altimeter settings;

(2)   ATC liaison in controlled or regulated airspace;

(3)   setting heading procedure;

(4)   noting of ETAs.

(c)   maintenance of height or altitude and heading;

(d)   revisions of ETA and heading:

(1)   10 ° line, double track and track error and closing angle;

(2)   1 in 60 rule;

(3)   amending an ETA.

(e)   log keeping;

(f)    use of radio;

(g)   use of navaids (if fitted);

(h)   minimum weather conditions for continuation of flight;

(i)    in-flight decisions;

(j)    transiting controlled or regulated airspace;

(k)   uncertainty of position procedure;

(l)    lost procedure.

(C)   arrival and aerodrome joining procedure:

(a)   ATC liaison in controlled or regulated airspace;

(b)   altimeter setting;

(c)   entering the traffic pattern;

(d)   circuit procedures.

(e)   parking;

(f)    security of helicopter;

(g)   refuelling;

(h)   closing of flight plan (if appropriate);

(i)    post-flight administrative procedures.

( xxxi )  Exercise  25 b :   Navigation  problems  at  low  heights  and  in  reduced  visibility :

(A)   actions before descending;

(B)   hazards (for example obstacles and other aircraft);

(C)   difficulties of map reading;

(D)  effects of wind and turbulence;

(E)   avoidance of noise sensitive areas;

(F)   actions in the event of encountering DVE;

(G)  decision to divert or conduct precautionary landing;

(H)  bad weather circuit and landing;

(I)   appropriate procedures and choice of landing area;

(J)   precautionary landing.

( xxxii )  Exercise  25 c :      Radio   Navigation :

(A)   use of GNSS:

(a)   selection of waypoints;

(b)   to or from indications and orientation;

(c)   error messages;

(d)   hazards of over-reliance on the use of GNSS in the continuation of flight in DVE.

(B)   use of VHF omni range:

(a)   availability, AIP and frequencies;

(b)   selection and identification;

(c)   OBS;

(d)   to or from indications and orientation;

(e)   CDI;

(f)    determination of radial;

(g)   intercepting and maintaining a radial;

(h)   VOR passage;

(i)    obtaining a fix from two VORs.

(C)   use of ADF equipment: NDBs:

(a)   availability, AIP and frequencies;

(b)   selection and identification;

(c)   orientation relative to the beacon;

(d)   homing.

(D)  use of VHF/DF:

(a)   availability, AIP and frequencies;

(b)   RTF procedures and ATC liaison;

(c)   obtaining a QDM and homing.

(E)   use of en-route or terminal radar:

(a)   availability and AIP;

(b)   procedures and ATC liaison;

(c)   pilots responsibilities;

(d)   secondary surveillance radar ( if  transponder  fitted ) :

(1)   transponders;

(2)   code selection;

(3)   interrogation and reply.

(F)   use of DME:

(a)   station selection and identification;

(b)   modes of operation: distance, groundspeed and time to run.

( xxxiii )  Exercise  26 :        Advanced   take-off,   landings   and   transitions :

(A)   landing and take-off out of wind (performance reduction);

(B)   ground  effect,  translational  lift  and  directional  stability variation when out of wind;

(C)   downwind transitions;

(D)  vertical take-off over obstacles;

(E)   running take-off;

(F)   cushion creep take-off;

(G)  reconnaissance of landing site;

(H)  running landing;

(I)   zero speed landing;

(J)   crosswind and downwind landings;

(K)   steep approach;

(L)   go-around.

( xxxiv )  Exercise  27 :       Sloping   Ground :

(A)   limitations and assessing slope angle;

(B)   wind and slope relationship: blade and control stops;

(C)   effect of CG when on slope;

(D)  ground effect on slope and power required;

(E)   right skid up slope;

(F)   left skid up slope;

(G)  nose up slope;

(H)  avoidance of dynamic roll over, dangers of soft ground and sideways movement on touchdown;

(I)   danger  of  striking  main  or  tail  rotor  by  harsh  control movement near ground.

( xxxv )  Exercise  28 :          Limited   Power :

(A)   take-off power check;

(B)   vertical take-off over obstacles;

(C)   in-flight power check;

(D)  running landing;

(E)   zero speed landing;

(F)   approach to low hover;

(G)  approach to hover;

(H)  approach to hover OGE;

(I)   steep approach;

(J)   go-around.

( xxxvi )  Exercise  29 :      Confined   Areas :

(A)   landing capability and performance assessment;

(B)   locating   landing   site   and   assessing   wind   speed   and direction;

(C)   reconnaissance of landing site;

(D)  select markers;

(E)   select direction and type of approach;

(F)   circuit;

(G)  approach to committed point and go-around;

(H)  approach;

(I)   clearing turn;

(J)   landing;

(K)   power check and performance assessment in and out of ground effect;

(L)   normal take-off to best angle of climb speed;

(M)  vertical take-off from hover.

( xxxvii )  Exercise  30 :      Basic   Instrument   Flight :

(A)   physiological sensations;

(B)   instrument appreciation:

(a)   attitude instrument flight;

(b)   instrument  scan.

(C)   instrument limitations;

(D)  basic manoeuvres:

(a)   straight   and   level   at   various   air   speeds   and configurations;

(b)   climbing and descending;

(c)   standard  rate  turns,  climbing  and  descending,  onto selected headings.

(E)   recoveries from climbing and descending turns; (F)   recoveries from unusual attitudes.

( xxxviii )  Exercise  31 a :      Night   flying  (  if  night  rating  required  ) :

(A)   pre-flight inspection using torch, pan lights, etc.;

(B)   take-off (no sideways or backwards manoeuvring);

(C)   hover taxi (higher and slower than by day);

(D)  transition to climb;

(E)   level flight;

(F)   approach and transition to hover;

(G)  landing;

(H)  autorotation;

(I)   practice  forced  landing  (  with  flares  if  appropriate :  simulated  ) ;

(J)   night emergencies  ( for  example  failure  of  lights,  etc. ).

( xxxix )  Exercise  31 b :      Night   cross - country  (  if  night   rating   required  ) :

(A)   navigation  principles as for day cross-country;

(B)   map marking (  highlighting  built-up  areas  with  thicker  lines,  etc.. ).

AMC  1.    FCL. 210. As     PPL ( As )  -  Experience   Requirements   and   Crediting

FLIGHT    INSTRUCTION    for    the    PPL ( As )

a )   Entry   to   Training :

Before  being  accepted  for  training  an  applicant  should  be  informed  that  the  appropriate  medical  certificate  must  be  obtained  before  solo  flying  is  permitted ;

b )   Flight   Instruction :

1 )  The  PPL( As )  flight  instruction  syllabus  should  take  into  account  the  principles  of  threat and  error  management  and  cover :

(i)    pre-flight  operations,  including  mass  and  balance  determination,  airship  inspection  and servicing ;

(ii)   ground  manoeuvring,  masting  and  unmasting  procedures ;

(iii)  aerodrome  and  traffic  pattern  operations,  collision  avoidance  precautions  and  procedures;

(iv)  control  of  the  airship  by  external  visual  reference ;

(v)   take-offs  and  landings ;

(vi)  flight  by  reference  solely  to  instruments,  including  the  completion  of  a  level  180 ° turn;

(vii)  cross-country  flying  using  visual  reference,  dead  reckoning  and  radio  navigation  aids ;

(viii) emergency  operations,  including  simulated  airship  equipment  malfunctions ;

(ix)   operations  to,  from  and  transiting  controlled  aerodromes,  compliance  with  air  traffic  services  procedures,  communication  procedures  and  phraseology.

2 )  Before  allowing  the  applicant  for  a  PPL( As )  to  undertake  his / her  first  solo  flight,  the  FI should  ensure  that  the  applicant  can  use  R / T  communication.

c )   Syllabus   of   Flight   Instruction :

1 )  The  numbering  of  exercises  should  be  used  primarily  as  an  exercise  reference  list  and  as  a  broad  instructional  sequencing  guide ;   therefore  the  demonstrations  and  practices  need  not necessarily  be  given  in  the  order  listed.  The  actual  order  and  content  will  depend  upon  the following  interrelated  factors :

(i)    the  applicant’s  progress  and  ability ;

(ii)   the  weather  conditions  affecting  the  flight ;

(iii)  the  flight  time  available ;

(iv)  instructional  technique  considerations ;

(v)   the  local  operating  environment ;

(vi)  applicability  of  the  exercises  to  the  airship.

2 )  Each  of  the  exercises  involves  the  need  for  the  applicant  to  be  aware  of  the  needs  of good  airmanship  and  look-out,  which  should  be  emphasized  at  all  times.

( i )    Exercise  1 a :     Familiarization   with   the   Airship :

(A)   characteristics of the airship;

(B)   cockpit layout;

(C)   systems;

(D)  checklists, drills and controls.

( ii )   Exercise  1 b :      Emergency   Drills :

(A)   action if fire on the ground and in the air;

(B)   engine cabin and electrical system fire;

(C)   systems failure;

(D)  escape drills, location and use of emergency equipment and exits.

( iii )  Exercise  2 :        Preparation   for   and   action   after   flight :

(A)   flight  authorization and airship acceptance;

(B)   serviceability documents;

(C)   equipment required, maps, etc.. ;

(D)  mass and balance;

(E)   external checks;

(F)   ground crew briefing;

(G)  internal checks;

(H)  harness, seat or rudder panel adjustments;

(I)   starting and warm-up checks;

(J)   power checks;

(K)   running down system checks and switching off the engine;

(L)   parking, security and masting;

(M)  completion   of   authorisation   sheet   and   serviceability documents.

( iv )  Exercise  3 :       Air  Experience :   flight   exercise.

( v )   Exercise  4 :        Effects   of   Controls :

(A)   primary effects; (B)   further effects;

(C)   effects of:

(a)   air speed;

(b)   power;

(c)   trimming controls;

(d)   other controls, as applicable.

(D)  operation  of :

(a)   mixture control;

(b)   carburettor heat;

(c)   cabin heating or ventilation.

( vi )  Exercise  5 :    Ground   manoeuvring :

(A)   pre-taxi checks;

(B)   starting, control of speed and stopping;

(C)   engine handling;

(D)  masting procedures;

(E)   control of direction and turning;

(F)   effects of wind;

(G)  effects of ground surface;

(H)  marshalling signals;

(I)   instrument checks;

(J)   air traffic control procedures;

(K)   emergencies.

(vii) Exercise 6a: Take-off procedures:

(A)   pre-take-off checks;

(B)   take-off with different static heaviness;

(C)   drills during and after take-off;

(D)  noise abatement procedures.

( viii )  Exercise  6 b :     Emergencies :

(A)   abandoned take-off;

(B)   engine failure after take-off;

(C)   malfunctions of thrust vector control;

(D)  aerodynamic control failures;

(E)   electrical and system failures.

( ix )   Exercise  7 :         Climbing :

(A)   entry,  maintaining  the  normal  and  max  rate  climb  and  leveling off;

(B)   levelling off at selected altitudes;

(C)   maximum angle of climb;

(D)  maximum rate of climb.

( x )     Exercise  8 :          Straight   and   Level :

(A)   attaining and maintaining straight and level flight;

(B)   flight at or close to pressure height;

(C)   control in pitch, including use of trim;

(D)  at selected air speeds  ( use  of  power ) ;

(E)   during speed changes;

(F)   use of instruments for precision.

( xi )    Exercise  9 :           Descending :

(A)   entry, maintaining and levelling off;

(B)   levelling off at selected altitudes;

(C)   maximum rate of descent;

(D)   maximum angle of descent;

(E)   use of instruments for precision flight.

( xii )    Exercise  10 :        Turning :

(A)   entry and maintaining level turns;

(B)   resuming straight flight;

(C)   faults in the turn;

(D)  climbing turns;

(E)   descending turns;

(F)   turns onto selected headings, use of gyro heading indicator and compass;

(G)  use of instruments for precision.

( xiii )    Exercise  11 :      Hovering :   hovering  manoeuvres  ( as  applicable ) ;

( xiv )    Exercise  12 a :    Approach   and   Landing :

(A)   effect of wind on approach and touchdown speeds;

(B)   landing with different static heaviness;

(C)   missed approach and go-around procedures;

(D)  noise abatement procedures.

( xv )      Exercise  12 b :       Emergencies :

(A)   aborted approach or go-around;

(B)   malfunction of thrust vector control;

(C)   envelope emergencies;

(D)  fire emergencies;

(E)   aerodynamic control failures;

(F)   electrical and system failures.

( xvi )    Exercise  13 :         Precautionary   Landing :

(A)   occasions necessitating;

(B)   in-flight conditions;

(C)   landing area selection;

(D)  circuit and approach;

(E)   actions after landing;

( xvii )    Exercise  14 a :        Navigation :

(A)   flight planning :

(a)   weather forecast and actuals;

(b)   map selection and preparation:

(1)   choice of route;

(2)   airspace structure;

(3)   sensitive areas;

(4)   safety altitudes.

(c)   calculations:

(1)   magnetic heading(s) and time(s) en-route;

(2)   fuel consumption;

(3)   mass and balance;

(4)   performance.

(d)   flight information:

(1)   NOTAMs etc.;

(2)   radio frequencies;

(3)   selection of alternate aerodromes.

(e)   airship documentation;

(f)    notification of the flight:

(1)   pre-flight administrative procedures;

(2)   flight plan form.

(B)   departure:

(a)   organisation of cockpit workload;

(b)   departure procedures:

(1)   altimeter settings;

(2)   ATC liaison in controlled or regulated airspace;

(3)   setting heading procedure;

(4)   noting of ETAs.

(c)   maintenance of altitude and heading;

(d)   revisions of ETA and heading;

(e)   log keeping;

(f)    use of radio;

(g)   use of navaids;

(h)   minimum weather conditions for continuation of flight;

(i)    in-flight decisions;

(j)    transiting controlled or regulated airspace;

(k)   diversion procedures;

(l)    uncertainty of position procedure;

(m)  lost procedure.

(C)   arrival, aerodrome joining procedure:

(a)   ATC liaison in controlled or regulated airspace;

(b)   altimeter setting;

(c)   entering the traffic pattern;

(d)   circuit procedures;

(e)   parking or on masting;

(f)    security of airship;

(g)   refuelling;

(h)   closing  of flight plan, if appropriate;

(i)    post-flight administrative procedures.

( xviii )  Exercise  14 b :   Navigation  problems  at  lower  levels  and  in  reduced  visibility :

(A)   actions before descending;

(B)   hazards (for example obstacles, and terrain);

(C)   difficulties of map reading;

(D)  effects of winds, turbulence and precipitation;

(E)   vertical situational awareness;

(F)   avoidance of noise sensitive areas;

(G)  joining the circuit;

(H)  bad weather circuit and landing.

( xix )    Exercise  14 c :      Radio   Navigation :

(A)   use  of  GNSS

(a)   selection of waypoints;

(b)   to or from indications and orientation;

(c)   error messages.

(B)   use of VHF omni range  ( if  applicable ) :

(a)   availability, AIP and frequencies;

(b)   selection and identification;

(c)   OBS;

(d)   to or from indications and orientation;

(e)   CDI;

(f)    determination of radial;

(g)   intercepting and maintaining a radial;

(h)   VOR passage;

(i)    obtaining a fix from two VORs.

(C)   use of ADF equipment: NDBs (if applicable):

(a)   availability, AIP and frequencies;

(b)   selection and identification;

(c)   orientation relative to the beacon;

(d)   homing.

(D)  use of VHF/DF:

(a)   availability, AIP and frequencies;

(b)   R/T procedures and ATC liaison;

(c)   obtaining a QDM and homing.

(E)   use of en-route or terminal radar:

(a)   availability and AIP;

(b)   procedures and ATC liaison;

(d)   BITD

(c)   pilot’s responsibilities;

(d)   secondary surveillance radar:

(1)   transponders;

(2)   code selection;

(3)   interrogation and reply.

(F)   use of DME (if applicable) ;

(a)   station selection and identification;

(b)   modes of operation: distance, groundspeed and time to run.

( xx )   Exercise  15 :   Basic  Instrument   Flight :

(A)   physiological sensations;

(B)   instrument appreciation: attitude instrument flight;

(C)   instrument limitations;

(D)  basic manoeuvres:

(a)   straight and level;

(b)   climbing and descending;

(c)   turns,   climbing   and   descending,   onto   selected  headings;

(d)   recoveries from climbing and descending turns.

(1)   A BITD may be used for flight training for :

(i)    flight by reference solely to instruments;

(ii)   navigation using radio navigation aids;

(iii)  basic instrument flight.

(2)   The use of the BITD should be subject to the following:

(i)    the training should be complemented by exercises on an airship;

(ii)   the record of the parameters of the flight must be available; and an FI(As) should conduct the  instruction.

AMC  1.    FCL. 205. S ( b )     SPL  -  Privileges   and   Conditions

CONTENTS    of    the    PROFICIENCY    CHECK    for    the    EXTENSION    of    SPL PRIVILEGES    to    EXERCISE     COMMERCIAL    PRIVILEGES    on    a    SAILPLANE

a )  The  applicant  should  be  responsible  for  the  flight  planning  and  should  ensure  that  all equipment  and  documentation  for  the  execution  of  the  flight  are  on  board ;

b )  An  applicant  should  indicate  to  the  FE  the  checks  and  duties  carried  out.

Checks  should  be  completed  in  accordance  with  the  authorized  checklist  for  the  sailplane  on which  the  test  is  being  taken.

FLIGHT    TEST    TOLERANCE

c )  The  applicant  should  demonstrate  the  ability  to :

1 )  operate  the  sailplane  within  its  limitations ;

2 )  complete  all  manoeuvres  with  smoothness  and  accuracy ;

3 )  exercise  good  judgment  and  airmanship ;

4 )  apply  aeronautical  knowledge ;

5 )  maintain  control  of  the  sailplane  at  all  times  in  such  a  manner  that  the  successful  outcome

of  a  procedure  or  manoeuvre  is  never  seriously  in  doubt.

CONTENT    of    the    SKILL    TEST

d )  The  applicant  should  demonstrate  his / her  skill  in  at  least  the  winch  or  aerotow  method  of launching.

| SECTION 1. PRE - FLIGHT OPERATIONS and TAKE - OFF |  |
| --- | --- |
| Use of checklist, airmanship, control of sailplane by external visual reference, look- out procedures, etc.. apply in all sections. |  |
| a | Pre-flight sailplane (daily) inspection, documentation, NOTAM and weather briefing |
| b | Verifying in-limits mass and balance and performance calculation |
| c | Passenger briefing |
| d | Sailplane servicing compliance |
| e | Pre-take-off checks |
| SECTION 2. LAUNCH METHOD |  |
| Note : at least for one of the three launch methods all the mentioned items are fully exercised during the skill test. |  |
| SECTION 2 a. WINCH or CAR LAUNCH |  |
| a | Signals before and during launch, including messages to winch driver |
| b | Initial roll and take-off climb |
| c | Adequate profile of winch launch |
| d | Launch failures (simulated) |
| e | Situational awareness |
| SECTION 2 b. AEROTOW LAUNCH |  |
| a | Signals before and during launch, including signals to or communications with tow plane pilot for any problems |
| b | Initial roll and take-off climb |
| c | Launch abandonment ( simulation only or “ talk - through “ ) |
| d | Correct positioning during straight flight and turns |
| e | Out of position and recovery |
| f | Correct release from tow |
| g | Lookout and airmanship through whole launch phase |
| SECTION 2 c. SELF LAUNCH ( TMGs excluded ) |  |
| a | ATC compliance |
| b | Aerodrome departure procedures |
| c | Initial roll and take-off climb |
| d | Simulated engine failure after take-off |
| e | Engine shut down and stowage |
| f | Lookout and airmanship through whole launch phase |
| SECTION 3. GENERAL AIRWORK |  |
| a | Maintain straight flight: attitude and speed control |
| b | Steep (45 ° bank) turns, look-out procedures and collision avoidance |
| c | Turning on to selected headings visually and with use of compass |
| d | Flight at high angle of attack (critically low air speed) |
| e | Clean stall and recovery |
| f | Spin avoidance and recovery |
| g | Local area navigation and awareness |
| SECTION 4. CIRCUIT, APPROACH and LANDING |  |
|  | Aerodrome circuit joining procedure |
|  | Collision avoidance : look-out procedures |
|  | Pre-landing checks |
|  | Circuit, approach control and landing |
|  | Precision landing ( simulation of out-landing : short field ) |
|  | Cross wind landing if suitable conditions available |

AMC  1.    FCL. 205. B ( b )     BPL  -  Privileges   and   Conditions

CONTENTS    of    the    PROFICIENCY    CHECK    for    the    EXTENSION    of    BPL PRIVILEGES    to    EXERCISE     COMMERCIAL    PRIVILEGES    on    a    BALLOON

a )  The  take-off  site  should  be  chosen  by  the  applicant  depending  on  the  actual  meteorological conditions,  the  area  which  has  to  be  overflown  and  the  possible  options  for  suitable  landing sites.  The  applicant  should  be  responsible  for  the  flight  planning  and  should  ensure  that  all equipment  and  documentation  for  the  execution  of  the  flight  are  on  board.  The  proficiency check  may  be  conducted  in  two  flights.  The  total  duration  of  the  flight(s)  should  be  at  least 60  minutes ;

b )   An  applicant  should  indicate  to  the  FE  the  checks  and  duties  carried  out.

Checks  should  be  completed  in  accordance  with  the  flight  manual  or  the  authorized  checklist for  the  balloon  on  which  the  test  is  being  taken.  During  pre-flight  preparation  for  the  test  the  applicant  should  be  required  to  perform  crew  and  passenger  briefings  and  demonstrate  crowd control.  The  load  calculation  should  be  performed  by  the  applicant  in  compliance  with  the operations  manual  or  flight  manual  for  the  balloon  used.

FLIGHT    TEST    TOLERANCE

c )  The  applicant  should  demonstrate  the  ability  to :

1)  operate  the  balloon  within  its  limitations ;

2 )  complete  all  manoeuvres  with  smoothness  and  accuracy ;

3 )  exercise  good  judgment  and  airmanship ;

4 )  apply  aeronautical  knowledge ;

5 )  maintain  control  of  the  balloon  at  all  times  in  such  a  manner  that  the  successful  outcome of  a  procedure  or  manoeuvre  is  never   seriously  in  doubt.

d )  The  following  limits  are  for  general  guidance.  The  FE  should  make  allowance  for  turbulent  conditions and  the  handling  qualities  and  performance  of  the  hot-air  balloon  used :

Height          ( 1 )    normal  flight                                         ±  100  ft

( 2 )    with  simulated   emergency                  ±  150  ft

CONTENT    of    the    SKILL    TEST

e )  The  contents  and  sections  of  the  proficiency  check  set  out  in  this  AMC  should  be  used  for  the  extension  of  BPL  privileges  to  exercise  commercial  privileges  on  a  hot-air  balloon.

| SECTION 1. PRE - FLIGHT OPERATIONS, INFLATION and TAKE - OFF |  |
| --- | --- |
| Use of checklist, airmanship, control of balloon by external visual reference, look- out procedures, etc.. apply in all sections. |  |
| a | Pre-flight documentation, flight planning, NOTAM and weather briefing |
| b | Balloon inspection and servicing |
| c | Load calculation |
| d | Crowd control and crew briefing |
| e | Passenger briefing |
| f | Assembly and layout |
| g | Inflation and pre-take-off procedures |
| h | Take-off |
| i | ATC compliance |
| SECTION 2. GENERAL AIRWORK |  |
| a | Climb to level flight |
| b | Level flight |

| SECTION 2. GENERAL AIRWORK - cont’d - |  |
| --- | --- |
| c | Descent to level flight |
| d | Operating at low level |
| e | ATC compliance |
| SECTION 3. EN - ROUTE PROCEDURES |  |
| a | Dead reckoning and map reading |
| b | Marking positions and time |
| c | Orientation, airspace structure |
| d | Maintenance of altitude |
| e | Fuel management |
| f | Communication with retrieve crew |
| g | ATC compliance or R / T communication |
| SECTION 4. APPROACH and LANDING PROCEDURES |  |
| a | Approach from low level and missed approach and fly on |
| b | Approach from high level and missed approach and fly on |
| c | Passenger pre-landing briefing |
| d | Pre-landing checks |
| e | Selection of landing field |
| f | Landing, dragging and deflation |
| g | ATC compliance or R/T communication |
| h | Actions after flight |
| SECTION 5. ABNORMAL and EMERGENCY PROCEDURES |  |
| This Section may be combined with Sections 1 through 4 |  |
| a | Simulated fire on the ground and in the air |
| b | Simulated pilot light and burner failures |
| c | Simulated passenger health problems |
| d | Other abnormal and emergency procedures as outlined in the appropriate flight manual |
| e | Oral questions |

f )   The  contents  and  Sections  of  the  proficiency  check  set  out  in  this  AMC  should  be  used for  the  extension  of  BPL  privileges  to  exercise  commercial  privileges  on  a  gas  balloon.

| SECTION 1. PRE - FLIGHT OPERATIONS, INFLATION and TAKE - OFF |  |
| --- | --- |
| Use of checklist, airmanship, control of balloon by external visual reference, look- out procedures, etc.. apply in all sections. |  |
| a | Pre-flight documentation, flight planning, NOTAM and weather briefing |
| b | Balloon inspection and servicing |
| c | Load calculation |
| d | Crowd control and crew briefing |
| e | Passenger briefing |
| f | Assembly and layout |
| g | Inflation and pre-take-off procedures |
| h | Take-off |
| i | ATC compliance |
| SECTION 2. GENERAL AIRWORK |  |
| a | Climb to level flight |
| b | Level flight |

| SECTION 2. GENERAL AIRWORK - cont’d - |  |
| --- | --- |
| c | Descent to level flight |
| d | Operating at low level |
| e | ATC liaison : compliance |
| SECTION 3. EN - ROUTE PROCEDURES |  |
| a | Dead reckoning and map reading |
| b | Marking positions and time |
| c | Orientation, airspace structure |
| d | Maintenance of altitude |
| e | Ballast management |
| f | Communication with retrieve crew |
| g | ATC compliance or R / T communication |
| SECTION 4. APPROACH and LANDING PROCEDURES |  |
| a | Approach from low level and missed approach and fly on |
| b | Approach from high level and missed approach and fly on |
| c | Passenger pre-landing briefing |
| d | Pre-landing checks |
| e | Selection of landing field |
| f | Landing, dragging and deflation |
| g | ATC compliance or R/T communication |
| h | Actions after flight |
| SECTION 5. ABNORMAL and EMERGENCY PROCEDURES |  |
| This Section may be combined with Sections 1 through 4 |  |
| a | Simulated closed appendix during take-off and climb |
| b | Simulated parachute or valve failure |
| c | Simulated passenger health problems |
| d | Other abnormal and emergency procedures as outlined in the appropriate flight manual |
| e | Oral questions |

AMC  1.   FCL. 225. B    BPL  -  Extension  of  Privileges  to  another  Balloon   Class  or  Group

a )  The  aim  of  the  flight  training  is  to  qualify  BPL  holders  to  exercise  the  privileges  on  a different  class  or  group  of  balloons ;

b )  The  following  Classes  should  be  recognized :

1 )   Hot - air  Balloons ;

2 )   Gas  Balloons ;

3 )   Hot - air  Airships.

c )  The  following  Groups  should  be  recognized :

1 )   Group  A :

( i )   Hot - air  Balloons  and  Hot - air  Airships  with  a  maximum  envelope  capacity  of

3 400m³ ;

( ii )  Gas  Balloons  with  a  maximum  envelope  capacity  of  1 260m³.

2 )    Group  B :

( i )   Hot - air  Balloons  and  Hot - air  Airship  with  an  envelope  capacity  between  3 401m³

and   6 000m³ ;

( ii )  Gas  Balloons  with  an  envelope  capacity  of  more  than  1 260m ³.

3 )    Group  C :

Hot - air  Balloons  and  Hot - air  Airship  with  an  envelope  capacity  between  6 001m³  and

10 500m³ .

4 )    Group  D :

Hot - air  Balloons  and  Hot - air  Airships  with  an  envelope  capacity  of  more  than

10 500m³.

d )  An  extension  to  Group  B  is  also  valid  for  group  A.

The  extension  for  the  Group  C  is  also  valid  for  the  groups  A  and  B.

An  extension  to  Group  D  will  include  the  privilege  for  the  other  3  ( three )  groups.

e )  The  ATO  should  issue  a  certificate  of  satisfactory  completion  of  the  instruction  to  licence endorsement.