Appendix    6.     Modular  Training   Courses  for  the  IR

A.     IR ( A )   —   Modular   Flying   Training   Course

GENERAL

1.   The  aim  of  the  IR ( A )  Modular  Flying  Training  course  is  to  train  pilots  to  the  level  of  proficiency  necessary  to  operate  aeroplanes  under   IFR   and   in   IMC.  The course consists  of

2  ( two )   modules, which  may  be   taken   separately  or  combined :

a )    Basic  Instrument  Flight   Module

This  comprises  10  hours  of  instrument  time  under  instruction,  of  which  up  to  5 hours  can  be instrument  ground  time  in  a  BITD,  FNPT  I  or  II,  or  an  FFS.  Upon  completion  of  the  Basic Instrument  Flight  Module,  the  candidate  shall  be  issued  a  Course  Completion  Certificate.

b )    Procedural   Instrument   Flight   Module

This  comprises  the  remainder  of  the  training  syllabus  for  the  IR ( A ),  40  hours  single - engine  or  45  hours  multi - engine  instrument  time  under  instruction,  and  the  theoretical  knowledge  course  for  the IR ( A ).

2.    An  applicant  for  a  modular  IR( A )  course  shall  be  the  holder  of  a  PPL( A )  or  a  CPL( A ). An  applicant  for  the  Procedural  Instrument  Flight  Module,  who  does  not  hold  a  CPL( A ), shall  be  holder  of  a  Course  Completion  Certificate  for  the  Basic  Instrument  Flight  Module.

The  ATO  shall  ensure  that  the  applicant  for  a  multi-engine  IR ( A )  course  who  has  not  held  a  multi-engine  aeroplane  class  or  type  rating  has  received  the  multi-engine  training  specified  in Subpart  H  prior  to  commencing  the  flight  training  for  the  IR ( A )  course.

3.   An  applicant  wishing  to  undertake  the  Procedural  Instrument  Flight  Module  of  a  modular  IR ( A ) course  shall  be  required  to   complete  all   the  instructional  stages  in  one  continuous  approved  course  of  training. Prior  to  commencing  the  Procedural  Instrument  Flight  Module,   the  ATO  shall  ensure  the competence  of  the  applicant  in  basic  instrument  flying  skills.

Refresher   training  shall  be  given  as required.

4.	  The course of  theoretical  instruction shall  be  completed  within 18  months.                                                                                              The  Procedural  Instrument  Flight  Module  and  the  Skill  Test  shall  be  completed  within  the  period  of validity  of  the  pass  in  theoretical  examinations.

5.	  The  course  shall  comprise:

a )  theoretical  knowledge  instruction  to  the  IR  knowledge   level ;

b )   instrument  flight   instruction.

THEORETICAL    KNOWLEDGE

6.   An  approved  modular  IR ( A ) course  shall  comprise at  least  150  hours  of  theoretical  knowledge instruction.

FLYING     TRAINING

7.  A  single - engine  IR ( A )  course  shall  comprise  at  least  50 hours  instrument  time  under  instruction of  which  up  to  20  hours  may  be  instrument  ground   time  in  an  FNPT I,  or  up  to  35  hours  in  an  FFS  or  FNPT  II.                                                                                                                A   maximum  of  10  hours of  FNPT  II  or  an  FFS  instrument  ground time  may  be  conducted   in  an  FNPT  I.

8.  A  multi - engine  IR ( A )  course  shall  comprise at  least  55 hours  instrument  time  under  instruction, of  which  up  to  25 hours  may  be  instrument  ground  time  in  an  FNPT  I,  or  up  to  40  hours  in  an FFS  or  FNPT  II.                                                                                                                                    A  maximum  of  10  hours  of  FNPT  II  or  an  FFS  instrument  ground  time  may  be  conducted   in  an FNPT  I.  The  remaining  instrument  flight  instruction  shall  include  at  least  15 hours  in  multi - engine aeroplanes.

9.  The  holder  of  a  single-engine IR ( A )  who  also  holds  a  multi - engine  Class  or  Type  Rating  wishing  to  obtain  a  multi - engine IR ( A )  for  the  first  time shall complete a course at  an  ATO comprising  at  least  5  hours  instruction in  instrument  flying   in  multi - engine   aeroplanes,   of  which            3  hours  may  be  in  an  FFS  or  FNPT  II.

10. 1.  The  holder  of  a  CPL ( A )  or  of  a  Course   Completion  Certificate  for  the  Basic Instrument  Flight Module  may  have  the  total  amount  of  training  required  in  paragraphs  7  or  8  above  reduced   by  10 hours.

10. 2.   The  holder  of  an  IR ( H )  may  have  the  total  amount  of  training  required  in  paragraphs  7  or  8  above  reduced  to  10  hours.

10. 3.  The  total  instrument flight  instruction  in  aeroplane  shall  comply  with  paragraph 7        or  8,  as  appropriate.

11.      The  flying  exercises  up  to  the  IR ( A )  Skill  Test  shall  comprise :

a )   Basic  Instrument  Flight  Module :                                                                                      Procedure  and  maneuver  for  basic  instrument  flight  covering  at  least :

basic  instrument  flight  without  external  visual  cues :

—   horizontal  flight ;

—   climbing ;

—   descent ;

—   turns  in  level  flight,  climbing,  descent ;

—    instrument  pattern ;

—    steep  turn ;

—    radio-navigation ;

—    recovery  from  unusual  attitudes ;

—    limited  panel ;

—    recognition  and  recovery  from  incipient  and  full  stalls ;

b )    Procedural    Instrument    Flight    Module :

( i )    pre - flight  procedures  for  IFR   flights,  including  the  use  of  the  Flight  Manual  and  appropriate air  traffic  services  documents  in  the  preparation  of  an  IFR  flight  plan ;

(  ii )  procedure  and  maneuvers  for  IFR  operation  under  normal,  abnormal  and  emergency  conditions  covering  at  least :

—   transition  from  visual   to  instrument  flight   on  Take-off ;

—   SID ( standard   instrument  departures )   and  STAR ( arrivals ) ;

—   en - route  IFR   procedures ;

—   holding   procedures ;

—   instrument  approaches  to  specified  minima ;

—   missed  approach  procedures ;

—   landings  from  instrument  approaches,  including  circling ;

( iii )   in - flight   maneuvers  and  particular  flight  characteristics ;

( iv )   if  required,  operation  of   a   multi - engine  aeroplane  in   the  above  exercises,  including  operation of   the  aeroplane  solely  by  reference  to   instruments  with  one  engine  simulated  inoperative  and  engine shutdown   and  restart  ( the  latter  exercise  to  be  carried  out  at  a  safe  altitude  unless carried  out  in  an FFS   or  FNPT  II ).

INTENTIONALLY            LEFT            BLANK

Aa.    IR ( A )  —    Competency - based   Modular   Flying   Training   Course  ( CBMFTC )

GENERAL

1.   The  aim  of  the  competency-based  modular  flying  training  course  is  to  train  PPL  or  CPL holders  for  the  instrument  rating,  taking  into  account  prior  instrument  flight  instruction  and experience.  It  is  designed  to  provide  the  level  of  proficiency  needed  to  operate  aeroplanes under  IFR  and  in  IMC.  The  course  shall  consist  of  a  combination  of  instrument  flight instruction  provided  by  an  IRI ( A )  or  an  FI ( A )  holding  the  privilege  to  provide  training  for the  IR  and  flight  instruction  within  an  ATO.

2.   An  applicant  for  such  a  competency-based  modular  IR ( A )  shall  be  the  holder  of  a                PPL ( A )  or  CPL ( A ).

3.   The  course  of  theoretical  instruction  shall  be  completed  within  18  months.  The  instrument flight  instruction  and  the  skill  test  shall  be  completed  within  the  period  of  validity  of  the  pass of  the  theoretical  knowledge  examinations.

4.   The  course  shall  comprise :

a )   Theoretical  knowledge  instruction  to  the  IR ( A )  knowledge  level ;

b )   Instrument  flight  instruction.

THEORETICAL    KNOWLEDGE

5.   An  approved  competency-based  modular  IR ( A )  course  shall  comprise  at  least  80  hours  of  theoretical  knowledge  instruction.  The  theoretical  knowledge  course  may  contain  computer-based  training  and  e-learning  elements.  A  minimum  amount  of  classroom  teaching  as  required by  ORA. ATO. 305  has  to  be  provided.

FLYING     TRAINING

6.   The  method  of  attaining  an  IR ( A )  following  this  modular  course  is  competency-based. However,  the  minimum  requirements  below  shall  be  completed  by  the  applicant.  Additional training  may  be  required  to  reach  required  competencies.

a )  A  single-engine  competency-based  modular  IR ( A )  course  shall  include  at  least  40  hours of  instrument  time  under  instruction,  of  which  up  to  10  hours  may  be  instrument  ground  time in  an  FNPT  I,  or  up  to  25  hours  in  an  FFS  or  FNPT  II.  A  maximum  of  5  hours  of  FNPT II or  FFS  instrument  ground  time  may  be  conducted  in  an  FNPT I.

( i )   when  the  applicant  has :

(A)  completed  instrument  flight  instruction  provided  by  an  IRI ( A )  or  an  FI ( A )  holding  the  privilege  to  provide  training  for  the  IR ;                                or

(B)  prior  flight  time  under  IFR  as  PIC  on  aeroplanes,  under  a  rating  providing  the  privileges to  fly  under  IFR  and  in  IMC   these  hours  may  be  credited  towards  the  40  hours  above  up  to maximum  of  30  hours ;

( ii )  when  the  applicant  has  prior  instrument  flight  time  under  instruction  other  than  specified in  point  ( a )( i ),  these  hours  may  be  credited  towards  the  required  40  hours  up  to  a  maximum  of  15  hours ;

( iii ) in  any  case,  the  flying  training  shall  include  at  least  10  hours  of  instrument  flight  time under  instruction  in  an  aeroplane  at  an  ATO ;

( iv ) the  total  amount  of  dual  instrument  instruction  shall  not  be  less  than  25  hours.

b )  A  multi-engine  competency-based  modular  IR ( A )  course  shall  include  at  least  45  hours instrument  time  under  instruction,  of  which  up  to  10  hours  may  be  instrument  ground  time  in an  FNPT  I,  or  up  to  30  hours  in  an  FFS  or  FNPT  II.  A  maximum  of  5  hours  of  FNPT  II  or  FFS  instrument  ground  time  may  be  conducted  in  an  FNPT  I.

( i )   when  the  applicant  has :

(A)  completed  instrument  flight  instruction  provided  by  an  IRI ( A )  or  an  FI ( A )  holding  the  privilege  to  provide  training  for  the  IR ;                           or

(B)  prior  flight  time  under  IFR  as  PIC  on  aeroplanes,  under  a  rating  giving  the  privileges  to  fly  under  IFR  and  in  IMC  these  hours  may  be  credited  towards  the  45  hours  above  up  to  a maximum  of  35  hours.

( ii )  when  the  applicant  has  prior  instrument  flight  time  under  instruction  other  than  specified in  point  ( b )( i ),  these  hours  may  be  credited  towards  the  required  45  hours  up  to  a  maximum  of  15  hours ;

( iii )  in  any  case,  the  flying  training  shall  include  at  least  10  hours  of  instrument  flight  time under  instruction  in  a  multi-engine  aeroplane  at  an  ATO ;

( iv )  the  total  amount  of  dual  instrument  instruction  shall  not  be  less  than  25  hours,  of  which at  least  15  hours  shall  be  completed  in  a  multi-engine  aeroplane.

c )  To  determine  the  amount  of  hours  credited  and  to  establish  the  training  needs,  the applicant  shall  complete  a  pre-entry  assessment  at  an  ATO ;

d )  The  completion  of  the  instrument  flight  instruction  provided  by  an  IRI ( A )  or  FI ( A )  in accordance  with   point  ( a )( i )  or  ( b )( i )  shall  be  documented  in  a  specific  training  record and  signed  by  the  instructor.

7.   The  flight  instruction  for  the  competency-based  modular  IR ( A )  shall  comprise :

a )   procedures  and  maneuvers  for  basic  instrument  flight  covering  at  least :

( i )      basic  instrument  flight  without  external  visual  cues ;

( ii )     horizontal  flight ;

( iii )    climbing ;

( iv )    descent ;

( v )     turns  in  level  flight,  climbing  and  descent ;

( vi )    instrument   pattern ;

( vii )   steep  turn ;

( viii )  radio  navigation ;

( ix )    recovery  from  unusual  attitudes ;

( x )     limited  panel ;                                             and

( xi )    recognition  and  recovery  from  incipient  and  full  stall.

b )  Pre - flight  procedures  for  IFR  flights,  including  the  use  of  the  flight  manual  and appropriate  air  traffic  services  documents  for  the  preparation  of  an  IFR  flight  plan ;

c )   Procedure  and  maneuvers  for  IFR  operation  under  normal,  abnormal,  and  emergency conditions  covering  at  least :

( i )     transition  from  visual  to  instrument  flight  on  take-off ;

( ii )    standard  instrument  departures  and  arrivals ;

( iii )   en - route  IFR  procedures ;

( iv )   holding  procedures ;

( v )    instrument  approaches  to  specified  minima ;

( vi )    missed  approach  procedures ;                                      and

( vii )   landings  from  instrument  approaches,  including  circling.

d )   in-flight  maneuvers  and  particular  flight  characteristics ;

e )   if  required,  operation  of  a  multi-engine  aeroplane  in  the  above  exercises,  including :

( i )   operation  of  the  aeroplane  solely  by  reference  to  instruments  with  one  engine  simulated  inoperative ;

( ii )  engine  shutdown  and  restart  ( to  be  carried  out  at  a  safe  altitude  unless  carried  out  in an  FFS  or  FNPT  II ).

8.    Applicants  for  the  competency-based  modular  IR ( A )  holding  a  Part - FCL  PPL  or  CPL and  a  valid  IR ( A )  issued  in  compliance  with  the  requirements  of  Annex  1  to  the  Chicago Convention  by  a  third  country  may  be  credited  in  full  towards  the  training  course  mentioned in  Paragraph  4.  In  order  to  be  issued  the  IR ( A ),  the  applicant  shall :

a )  successfully  complete  the  skill  test  for  the  IR ( A )  in  accordance  with  Appendix 7 ;

b )  demonstrate  to  the  examiner  during  the  skill  test  that  he / she  has  acquired  an  adequate level  of  theoretical  knowledge  of  air  law,  meteorology  and  flight  planning  and  performance        ( IR ) ;                                       and

c )  have  a  minimum  experience  of  at  least  50  hours  of  flight  time  under  IFR  as  PIC  on aeroplanes.

PRE  -  ENTRY      ASSESSMENT

9.   The  content  and  duration  of  the  pre-entry  assessment  shall  be  determined  by  the  ATO based  on  the  prior  instrument  experience  of  the  applicant.

MULTI  -  ENGINE

10.   The  holder  of  a  single-engine  IR ( A )  who  also  holds  a  multi-engine  class  or  type  rating wishing  to  obtain  a  multi-engine  IR ( A )  for  the  first  time  shall  complete  a  course  at  an  ATO comprising  at  least  5  hours  instrument  time  under  instruction  in  multi-engine  aeroplanes,  of which  3  hours  may  be in  an  FFS  or   FNPT  II  and  shall  pass  a  skill  test.

INTENTIONALLY            LEFT            BLANK

B.     IR ( H )   —   Modular   Flying   Training   Course

1.  The aim of the IR ( H ) modular flying training course is to train pilots to the level of proficiency necessary to operate helicopters under IFR and in IMC.

2.   An  applicant  for  a  modular  IR ( H )  course  shall  be  the  holder  of  a  PPL ( H ),  or  a                     CPL ( H )  or  an  ATPL ( H ).  Prior  to  commencing  the  aircraft  instruction  phase  of  the  IR ( H )  course,  the  applicant  shall  be  the  holder  of  the  helicopter  type  rating  used  for  the  IR ( H )  skill  test,  or  have  completed  approved  type  rating  training  on  that  type.  The  applicant  shall  hold  a  certificate  of  satisfactory  completion  of  MCC  if  the  skill  test  is  to   be  conducted  in  Multi- Pilot  conditions.

3.  An applicant wishing to undertake a modular IR ( H ) course shall be required to complete all the instructional stages in one continuous approved course of training.

4.  The course of theoretical instruction shall be completed within 18 months. The flight instruction and the skill test shall be completed within the period of validity of the pass in the theoretical examinations.

5.  The course shall comprise :

a )  theoretical knowledge instruction to the IR  knowledge level ;

b )  instrument flight  instruction.

THEORETICAL    KNOWLEDGE

6.	  An approved modular IR ( H )  course shall comprise at least 150 hours of instruction.

FLYING    TRAINING

7.  A single - engine  IR ( H )  course  shall  comprise  at  least  50 hours  instrument  time  under  instruction, of  which :

a )  up to 20 hours may be instrument ground time in an FNPT  I ( H )  or  ( A ).  These 20  hours  instruction time  in  FNPT I  ( H )  or  ( A )  may  be  substituted  by  20 hours  instruction  time  for  IR ( H )  in an aeroplane, approved  for  this  course ;            or

b )  up to 35 hours may be instrument ground time in a helicopter FTD 2/3, FNPT II/III or FFS. The instrument flight instruction shall include at least 10 hours in an IFR-certificated helicopter.

8.  A multi-engine IR ( H ) course shall comprise at least 55 hours instrument time under instruction of  which;

a )  up to 20 hours may be instrument ground time in an FNPT  I ( H ) or ( A ).  These 20 hours instruction  time  in  FNPT  I ( H )  or  ( A )  may  be  substituted  by  20 hours  instruction time  for  IR ( H )  in an aeroplane, approved  for  this course ;       or

b )  up to 40 hours may be instrument ground time in a helicopter FTD  2 / 3,  FNPT  II / III  or  FFS.

The  instrument flight  instruction shall  include  at  least  10 hours  in  an  IFR-certificated

multi - engine  helicopter.

9. 1.  Holders  of an ATPL (H)  shall  have  the  theoretical  knowledge  instruction hours reduced  by 50  hours.

9 .2.   The  holder  of  an  IR ( A )  may  have  the  amount  of  training  required  reduced  to  10 hours.

9 .3.  The  holder  of  a  PPL ( H )  with  a  helicopter  night  rating  or  a  CPL( H )  may  have  the total  amount  of  instrument  time  under  instruction  required  reduced  by  5  hours.

10.  The flying exercises up to the IR(H) skill test shall comprise :

a )   pre-flight procedures for IFR flights, including the use of the flight manual and appropriate air traffic  services  documents in the preparation of an IFR flight plan;

b )  procedure and maneuvers for IFR  operation under normal, abnormal and emergency conditions covering  at  least :

---   transition from visual to instrument flight on takeoff ;

---   standard instrument departures and arrivals ;

---   en - route  IFR  procedures ;

---   holding  procedures ;

---   instrument approaches to specified minima ;

---   missed approach procedures ;

---   landings from instrument approaches, including circling ;

c )   in - flight  maneuvers  and  particular  flight  characteristics ;

d )  if  required, operation of a multi-engine helicopter in the above exercises, including operation of the  helicopter solely by reference to instruments with one engine simulated  inoperative  and  engine shutdown  and  restart .

(  the  latter exercise to be carried out in an FFS  or  FNPT  II  or  FTD  2 / 3 ).

C.    IR ( As )   —  Modular   Flying   Training   Course

(  reserved  )

INTENTIONALLY            LEFT            BLANK