SUBPART   G.     INSTRUMENT    RATING   —   IR

AMC  1.    FCL. 625 ( c )     IR  -  Validity,   Revalidation   and   Renewal

RENEWAL   of   INSTRUMENT   RATING :    REFRESHER   TRAINING

a )   Paragraph  ( b )( 1 )  of  FCL. 740  determines  that  if  the  Instrument  Rating  has  lapsed,  the applicant  shall  go  through  refresher  training  at  an  ATO,  to  reach  the  level  of  proficiency needed  to  pass  the  instrument  element  of  the  Skill  Test  prescribed  in  Appendix  9  to  Part -FCL.  The  amount  of  refresher  training  needed  should  be  determined  on  a  case - by - case  basis by  the  ATO,  taking  into  account  the  following  factors :

1 )   the  experience  of  the  applicant.  To  determine  this,  the  ATO  should  evaluate  the  pilot’s  log  book,  and,  if  necessary,  conduct  a  test  in  an  FSTD ;

2 )   the  amount  of  time  lapsed  since  the  expiry  of  the  validity  period  of  the  rating.                   The  amount  of  training  needed  to  reach  the  desired  level  of  proficiency  should  increase  with the  time  lapsed.  In  some  cases,  after  evaluating  the  pilot,  and  when  the  time  lapsed  is  very limited  ( less  than  3  months ),  the  ATO  may  even  determine  that  no  further  refresher  training  is  necessary.  The  following  may  be  taken  as  guidance  when  determining  the  needs  of  the applicant :

( i )    expiry  for  a  period  shorter  than  3  months :   -   no  supplementary  requirements ;

( ii )   expiry  for  longer  than  3  months  but  shorter  than  1  year :   -  a  minimum  of  1  ( one )

training  session ;

( iii )  expiry  for  longer  than  1  year  but  shorter  than  7  years :   -   a  minimum  of  3  ( three )

training  sessions ;

( iv )  expiry  for  longer  than  7  years :    -   the  applicant  should  undergo  the  full  training  course

for  the  issue  of  the  IR.

b )   Once  the  ATO  has  determined  the  needs  of  the  applicant,  it  should  develop  an  individual training  programme,  which  should  be  based  on  the  initial  training  for  the  issue  of  Instrument Ratings  and  focus  on  the  aspects  where  the  applicant  has  shown  the  greatest  needs ;

c )   After  successful  completion  of  the  training,  the  ATO  should  give  a  Certificate  to  the applicant,  to  be  submitted  to  the  GDCA  of  RA  when  applying  for  the  renewal.

INTENTIONALLY            LEFT            BLANK

INTENTIONALLY            LEFT            BLANK

INTENTIONALLY            LEFT            BLANK