APPENDICES

Table   of   Content

APPENDIX   1.      Aeroplanes    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   3

APPENDIX   2.      Helicopters    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  21

APPENDIX   3.      MPL,   IR   Courses    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   41

INTENTIONALLY            LEFT            BLANK

APPENDIX   1.      AEROPLANES

AMC  1.   to  Appendix  3.     Training   Courses   for   the   issue   of   a   CPL   and   an   ATPL

GENERAL

a )   When  ensuring  that  the  applicant  complies  with  the  prerequisites  for  the  course,  in accordance  with  ORA. ATO. 145,  the  ATO  should  check   that  the  applicant  has  enough knowledge  of  mathematics,  physics  and  English  to  facilitate  the  understanding  of  the  theoretical  knowledge  instruction  content  of  the  course ;

b )   Whenever  reference  is  made  to  a  certain  amount  of  hours  of  training,  this  means  a  full hour.  Time  not  directly  assigned  to  training  (  such  as  breaks,  etc.. )  is  not  to  be  counted towards  the  total  amount  of  time  that  is  required.

INTENTIONALLY            LEFT            BLANK

A.      ATP   Integrated   Course :     Aeroplanes

a )   The  ATP  Integrated  Course  should  last  between  12  and  36  months.

This  period  may  be  extended  where  additional  flying  training  or  ground  instruction  is  provided by  the  ATO.

CREDITING

b )   Credit  for  previous  experience  given  to  an  applicant  who  already  holds  a  PPL  should  be entered  into  the  applicant’s  training  record.

In  the  case  of  a  student - pilot  who  does  not  hold  a  pilot  licence  and  with  the  approval  of  the GDCA  of  RA,  an  ATO  may  designate  certain  dual  exercises  to  be  flown  in  a  helicopter  or  a TMG  up  to  a  maximum  of  20  hours.

THEORETICAL    KNOWLEDGE

c )  The  750  hours  of  instruction  can  include  classroom  work,  interactive  video,  slide  or  tape presentation,  learning  carrels,  computer-based  training,  and  other  media  as  approved  by  the  GDCA,  in  suitable  proportions ;

The  750  hours  of  instruction  should  be  divided  in  such  a  way  that  in  each  subject  the minimum  hours  are :

1 )   Air  Law                                                              40  hours ;

2 )   Aircraft  General  Knowledge                             80  hours ;

3 )   Flight  Performance  and  Planning                     90  hours ;

4 )   Human  Performance  and  Limitations               50  hours ;

5 )   Meteorology                                                         60  hours ;

6 )   Navigation                                                          150  hours ;

7 )   Operational  Procedures                                       20  hours ;

8 )   Principles  of  Flight                                             30  hours ;

9 )   Communications                                                   30  hours.

Other  subdivision  of  hours  may  be  agreed  upon  between  the  GDCA  and  the  ATO.

FLYING    TRAINING

d )   The  Flying  Instruction  is  divided  into  5  ( five )  Phases :

(1)    Phase  1 :

Exercises  up  to  the  first  solo  flight  comprise  a  total  of  at  least  10  hours  dual  flight  instruction  on  an  SE  aeroplane  including :

( i )    pre - flight  operations,  mass  and  balance  determination,  aeroplane  inspection  and  servicing ;

( ii )   aerodrome  and  traffic  pattern  operations,  collision  avoidance  and  precautions ;

( iii )  control  of  the  aeroplane  by  external  visual  references ;

( iv )  normal  Take-offs  and  Landings ;

( v )   flight  at  critically  low  air  speeds,  recognition  of  recovery  from  incipient  and  full  stalls, spin  avoidance ;

( vi )  unusual  attitudes  and  simulated  engine  failure.

(2)    Phase  2 :

Exercises  up  to  the  first  solo  cross - country  flight  comprise  a  total  of  at  least  10  hours  of dual  flight  instruction  and  at  least  10  hours  solo  flight  including :

( i )     maximum  performance  ( short  field  and  obstacle  clearance )  take - offs  and  short - field  landings ;

( ii )    flight  by  reference  solely  to  instruments,  including  the  completion  of  a  180 °  turn ;

( iii )   dual  cross-country  flying  using  external  visual  references,  DR  (  Dead  Reckoning  )  and  radio  navigation  aids,  diversion  procedures ;

( iv )   aerodrome  and  traffic  pattern  operations  at  different  aerodromes ;

( v )    crosswind  take - offs  and  landings ;

( vi )   abnormal  and  emergency  procedures  and  maneuvers,  including  simulated  aeroplane equipment  malfunctions ;

( vii )  operations  to,  from  and  transiting  controlled  aerodromes,  compliance  with   ATS procedures,  R / T  procedures  and  phraseology ;

( viii ) knowledge  of  meteorological  briefing  arrangements,  evaluation  of  weather  conditions  for  flight  and  use  of  AIS.

(3)    Phase  3 :

Exercises  up  to  the  VFR  navigation  progress  test  comprise  a  total  of  at  least  5  hours  of  dual instruction  and  at  least  40  hours  as  PIC.

The  dual  instruction  and  testing  up  to  the  VFR  navigation  progress  test  should  comprise :

( i )    repetition  of  exercises  of  phases  1  and  2 ;

( ii )   VFR  flight  at  relatively  critical  high  air  speeds,  recognition  of  and  recovery  from  spiral  dives ;

( iii )  VFR  navigation  progress  test  conducted  by  an  FI  not  connected  with  the  applicant’s training ;

( iv )  night  flight  time  including  take-offs  and  landings  as  PIC.

(4)    Phase  4 :

Exercises  up  to  the  Instrument  Rating  Skill  Test  comprise :

( i )    at  least  55  hours  Instrument  Flight,  which  may  contain  up  to  25  hours  of  instrument ground  time  in  an  FNPT  I  or  up  to  40  hours  in  an  FNPT  II  or  FFS  which  should  be  conducted  by  an  FI  or  an  authorized  SFI ;

( ii )   20  hours  Instrument  Time  flown  as  SPIC ;

( iii )  pre - flight  procedures  for  IFR  flights,  including  the  use  of  the  Flight  Manual  and appropriate  ATS  documents  in  the  preparation  of  an  IFR  flight  plan ;

( iv ) procedures  and  maneuvers  for  IFR  operation  under  normal,  abnormal  and  emergency conditions   covering  at  least :

(A)   transition  from  visual  to  instrument  flight  on  Take - off ;

(B)   SID’s  and  STAR’s  /  arrivals ;

(C)   en - route  IFR  procedures ;

(D)  holding  procedures ;

(E)   instrument  approaches  to  specified  minima ;

(F)   missed  approach  procedures ;

(G)  landings  from  instrument  approaches,  including  circling.

( v )   in - flight  maneuvers  and  specific  flight  characteristics ;

( vi )  operation  of  an  ME  aeroplane  in  the  exercises  of  ( iv ),  including  operation  of  the aeroplane  solely  by  reference  to  instruments  with  one  engine  simulated  inoperative,  and  engine shut-down  and  restart  ( the  latter  training  should  be  at  a  safe  altitude  unless  carried  out  in  an FSTD ).

(5)    Phase  5 :

( i )    instruction  and  testing  in  MCC  comprise  the  relevant  training  requirements ;

( ii )   if  a  Type  Rating  for  MP  aeroplanes  is  not  required  on  completion  of  this  part,  the applicant  will  be  provided  with  a  Certificate  of  course  completion  for  MCC  Training.

INTENTIONALLY            LEFT            BLANK

B.      ATP   Modular   Theoretical   Knowledge   Course :     Aeroplanes

a )   The  aim  of  this  course  is  to  train   pilots  who  have  not  received  the  theoretical  knowledge instruction  during  an  integrated  course  to  the  level  of  theoretical  knowledge  required  for  the ATPL ;

b )   An  approved  course  should  include  formal  classroom  work  and  may  include  the  use  of such  facilities  as  interactive  video,  slide  or  tape  presentation,  learning  carrels  and  computer-based  training  and  other  media  distance  learning  ( correspondence )  courses  as  approved  by  the GDCA  of  RA.  Approved  distance  learning  ( correspondence )  courses  may  also  be  offered  as part  of  the  course ;

c )  The  ATP  Modular  Course  should  last  18  months.

This  period  may  be  extended  where  additional  training  is  provided  by  the  ATO.  The  flight instruction  and  skill  test  need  to  be  completed  within  the  period  of  validity  of  the  pass  in  the theoretical  examinations.

INTENTIONALLY            LEFT            BLANK

C.      CPL  /  IR   Integrated   Course :     Aeroplanes

a )   The  CPL / IR  Integrated  Course  should  last  between  9  and  30  months.  This  period  may  be  extended  where  additional  flying  training  or  ground  instruction  is  provided  by  the  ATO.

CREDITING

b)   Credit  for  previous  experience  given  to  an  applicant  who  already  holds  a  PPL  should  be entered  into  the  applicant’s  training  record.  In  the  case  of  a  student - pilot  who  does  not  hold a  pilot  licence  and  with  the  approval  of  the  GDCA,  an  ATO  may  designate  certain  dual exercises  to  be  flown  in  a  helicopter  or  a  TMG  up  to  a  maximum  of  20  hours.

THEORETICAL    KNOWLEDGE

c )   The  500  hours  of  instruction  can  include  classroom  work,  interactive  video,  slide  or  tape presentation,  learning  carrels,  computer-based  training,  and  other  media  as  approved  by  the  GDCA,  in  suitable  proportions.

The  500  hours  of  instruction  should  be  divided  in  such  a  way  that  in  each  subject  the minimum  hours  are :

1 )   Air  Law                                                                  30  hours ;

2 )   Aircraft  General  Knowledge                                 50  hours ;

3 )   Flight  Performance  and  Planning                         60  hours ;

4 )   Human  Performance  and  Limitations                   15  hours ;

5 )   Meteorology                                                             40  hours ;

6 )   Navigation                                                              100  hours ;

7 )   Operational  Procedures                                           10  hours ;

8 )   Principles  of  Flight                                                 25  hours ;

9 )   Communications                                                      30  hours.

Other  subdivisions  of  hours  may  be  agreed  upon  between  the  GDCA   and  the  ATO.

FLYING     TRAINING

d )   The  flying  instruction  is  divided  into  4  ( four )  Phases :

(1)    Phase  1 :

Exercises  up  to  the  first  solo  flight  comprise  a  total  of  at  least  10  hours  dual  flight  instruction  on  an  SE  aeroplane,  including :

( i )    pre-flight  operations,  mass  and  balance  determination,  aeroplane  inspection  and  servicing ;

( ii )   aerodrome  and  traffic  pattern  operations,  collision  avoidance  and  precautions ;

( iii )  control  of  the  aeroplane  by  external  visual  references ;

( iv )  normal  Take - offs  and  Landings ;

( v )   flight  at  critically  low  air  speeds,  recognition  of  and  recovery  from  incipient  and  full stalls,  spin  avoidance ;

( vi )  unusual  attitudes  and  simulated  engine  failure.

(2)    Phase  2 :

Exercises  up  to  the  first  solo  cross-country  flight  comprise  a  total  of  at  least  10  hours  of  dual flight  instruction  and  at  least  10  hours  solo  flight  including :

( i )      maximum  performance  ( short  field  and  obstacle  clearance )  Take- offs  and  short-field Landings ;

( ii )    flight  by  reference  solely  to  instruments,  including  the  completion  of  a  180 °  turn ;

( iii )   dual  cross-country  flying  using  external  visual  references,  DR  and  radio  navigation  aids, diversion  procedures ;

( iv )   aerodrome  and  traffic  pattern  operations  at  different  aerodromes ;

( v )    crosswind  take-offs  and  landings ;

( vi )   abnormal  and  emergency  operations  and  maneuvers,  including  simulated  aeroplane equipment  malfunctions ;

( vii )  operations  to,  from  and  transiting  controlled  aerodromes,  compliance  with  ATS  procedures,  R / T  procedures  and  phraseology ;

( viii ) knowledge  of  meteorological  briefing  arrangements,  evaluation  of  weather  conditions  for flight  and  use  of  AIS.

(3)    Phase  3 :

Exercises  up  to  the  VFR  navigation  progress  test  comprise  a  total  of  at  least  5  hours  of instruction  and  at  least  40  hours  as  PIC.

The  dual  instruction  and  testing  up  to  the  VFR  navigation  progress  test  and  the  skill  test should  contain  the  following :

( i )    repetition  of  exercises  of  Phases  1  and  2 ;

( ii )   VFR  flight  at  relatively  critical  high  air  speeds,  recognition  of  and  recovery  from  spiral  dives ;

( iii )  VFR  navigation  progress  test  conducted  by  an  FI  not  connected  with  the  applicant’s training ;

( iv )  night  flight  time  including  take-offs  and  landings  as  PIC.

(4)    Phase  4 :

Exercises  up  to  the  Instrument  Rating  Skill  Test  comprise :

( i )    at  least  55  hours  Instrument  Time,  which  may  contain  up  to  25  hours  of  Instrument Ground  Time  in  an  FNPT  I  or  up  to  40  hours  in  an  FNPT  II  or  FFS  which  should  be  conducted  by  an  FI  or  an  authorized  SFI ;

( ii )   20  hours  Instrument  Time  flown  as  SPIC ;

( iii )  pre-flight  procedures  for  IFR  flights,  including  the  use  of  the  Flight  Manual  and appropriate  ATS  documents  in  the  preparation  of  an  IFR  flight  plan ;

( iv ) procedures  and  maneuvers  for  IFR  operation  under  normal,  abnormal  and  emergency conditions  covering  at  least:

(A)   transition  from  visual  to  instrument  flight  on  take - off ;

(B)   SID’s  and  STAR’s  /  arrivals ;

(C)   en - route  IFR  procedures ;

(D)  holding  procedures ;

(E)   instrument  approaches  to  specified  minima ;

(F)   missed  approach  procedures ;

(G)  landings  from  instrument  approaches,  including  circling.

( v )   in - flight  maneuvers  and  particular  flight  characteristics ;

( vi )  operation  of  either  an  SE  or  an  ME  aeroplane  in  the  exercises  of  ( iv ),  including  in  the case  of  an  ME  aeroplane  operation  of  the  aeroplane  solely  by  reference  to  instruments  with one  engine  simulated  inoperative  and  engine  shut-down  and  restart.    The  latter  exercise  is  to  be conducted  at  a  safe  altitude  unless  carried  out  in  an  FSTD.

INTENTIONALLY            LEFT            BLANK

D.      CPL   Integrated   Course :     Aeroplanes

a )   The  CPL  Integrated  Course  should  last  between  9  and  24  months.

This  period  may  be  extended  where  additional  flying  training  or  ground  instruction  is  provided by  the  ATO.

CREDITING

b )   Credit  for  the  hours  flown  should  be  entered  into  the  applicant’s  training  record.  In  the case  of  a  student - pilot  who  does  not  hold  a  pilot  licence  and  with  the  approval  of  the  GDCA  of  RA,  an  ATO  may  designate  certain  dual  exercises  to  be  flown  in  a  helicopter  or  a TMG  up  to  a  maximum  of  20  hours.

THEORETICAL     KNOWLEDGE

c )  The  350  hours  of  instruction  can  include  classroom  work,  interactive  video,  slide  or  tape presentation,  learning  carrels,  computer-based  training,  and  other  media  as  approved  by  the  GDCA,  in  suitable  proportions.

FLYING    TRAINING

d )   The  flying  instruction  is  divided  into  4  ( four )  Phases :

(1)    Phase  1 :

Exercises  up  to  the  first  solo  flight  comprise  a  total  of  at  least  10  hours  dual  flight  instruction  on  an  SE  aeroplane,  including :

( i )    pre-flight  operations,  mass  and  balance  determination,  aeroplane  inspection  and  servicing ;

( ii )   aerodrome  and  traffic  pattern  operations,  collision  avoidance  and  precautions ;

( iii )  control  of  the  aeroplane  by  external  visual  references ;

( iv )  normal  take-offs  and  landings ;

( v )   flight  at  relatively  slow  air  speeds,  recognition  of  and  recovery  from  incipient  and  full stalls,  spin  avoidance ;

( vi )  unusual  attitudes  and  simulated  engine  failure.

(2)    Phase  2 :

Exercises  up  to  the  first  solo  cross-country  flight  comprise  a  total  of  at  least  10  hours  of  dual flight  instruction  and  at  least  10  hours  solo  flight  including :

( i )    maximum  performance  ( short  field  and  obstacle  clearance )  take- offs  and  short-field landings ;

( ii )   flight  by  reference  solely  to  instruments,  including  the  completion  of  a  180 °  turn ;

( iii )  dual  cross-country  flying  using  external  visual  references,  DR  and  radio  navigation  aids,  diversion  procedures ;

( iv )  aerodrome  and  traffic  pattern  operations  at  different  aerodromes ;

( v )   crosswind  take-offs  and  landings ;

( vi )  abnormal  and  emergency  procedures  and  maneuvers,  including  simulated  aeroplane equipment  malfunctions ;

( vii )  operations  to,  from  and  transiting  controlled  aerodromes,  compliance  with   ATS  procedures,  R / T  procedures  and  phraseology ;

( viii ) knowledge  of  meteorological  briefing  arrangements,  evaluation  of  weather  conditions  for flight  and  use  of  AIS.

(3)    Phase  3 :

Exercises  up  to  the  VFR  navigation  progress  test  comprise  a  total  of  at  least  30  hours instruction  and  at  least  58  hours  as  PIC,  including :

a )   at  least  10  hours  Instrument  Time,  which  may  contain  5  hours  of  Instrument  Ground  Time  in  an  FNPT  or  an  FFS  and  should  be  conducted  by  an  FI  or  an  authorized  SFI ;

b )   repetition  of  exercises  of  Phases  1  and  2,  which  should  include  at  least  5  hours  in  an aeroplane  certificated  for  the  carriage  of  at  least  four  persons  and  have  a  variable  pitch propeller  and  retractable  landing  gear ;

c )   VFR  flight  at  relatively  critical  high  air  speeds,  recognition  of  and  recovery  from  spiral dives ;

d )   night  flight  time  including  take-offs  and  landings  as  PIC.

(4)    Phase  4 :

The  Dual  Instruction  and  Testing  up  to  the  CPL ( A )  Skill  Test  contain  the  following :

( i )    up  to  30  hours  instruction  which  may  be  allocated  to  specialized  aerial  work  training ;

( ii )   repetition  of  exercises  in  Phase  3,  as  required ;

( iii )  in - flight  maneuvers  and  particular  flight  characteristics ;

( iv )  ME  training.

If  required,  operation  of  an  ME  aeroplane  including  operation  of  the  aeroplane  with  one  engine  simulated  inoperative,  and  engine  shutdown  and  restart  (  the  latter  exercise  at  a  safe altitude  unless  carried  out  in  an  FSTD  ).

E.      CPL   Modular   Course :    Aeroplanes

a )   The  CPL  Modular  Course  should  last  18  months.

This   period  may  be  extended  where  additional  training  is  provided  by  the  ATO.

The  flight  instruction  and  Skill  Test  need  to  be  completed  within  the  period  of  validity  of  the pass  in  the  theoretical  examinations ;

b )   An  approved  course  should  include  formal  classroom  work  and  may  include  the  use  of such  facilities  as  interactive  video,  slide  or  tape  presentation,  learning  carrels  and  computer-based  training  and  other  media  distance  learning  ( correspondence )  courses  as  approved  by  the GDCA  of  RA.  Approved  distance  learning  ( correspondence )  courses  may  also  be  offered  as part  of  the  course.

THEORETICAL     KNOWLEDGE

(c)  The 250 hours of instruction can include classroom work, interactive video, slide or tape presentation, learning carrels, computer-based training, and other  media  as  approved  by  the  competent  authority,  in  suitable proportions.

FLYING     TRAINING

d )  The  following  flight  time  is  suggested  for  the  flying  training :

| N 0 | Exercise N 0 | Training Item | Suggested Flight Time |
| --- | --- | --- | --- |
| 1. | Visual Flight Training : |  |  |
| Exercise 1 |  | Pre-flight operations : mass and balance determination, aeroplane inspection and servicing. | - |
| Exercise 2 |  | Take-off, traffic pattern, approach and landing, use of Checklist, collision avoidance and Checking procedures. | 0 : 45 hours |
| Exercise 3 |  | Traffic patterns : simulated engine failure during and after take - off. | 0 : 45 hours |
| Exercise 4 |  | Maximum performance ( short field and obstacle clearance ) take - offs and short - field landings. | 1 : 00 hours |
| Exercise 5 |  | Crosswind take - offs, landings and Go - around’s. | 1 : 00 hours |
| Exercise 6 |  | Flight at relatively critical high air speeds : recognition of and recovery from spiral dives. | 0 : 45 hours |
| Exercise 7 |  | Flight at critically slow air speeds, spin avoidance, recognition of and recovery from incipient and full stalls. | 0 : 45 hours |
| Exercise 8 |  | Cross - country flying using DR and radio navigation aids ; flight planning by the applicant ; filing of ATC flight plan ; evaluation of weather briefing documentation, NOTAM, etc..; R / T procedures and phraseology ; positioning by radio navigation aids; operation to, from and transiting controlled aerodromes, compliance with ATS procedures for VFR flights, simulated radio communication failure, weather deterioration, diversion procedures; simulated engine failure during cruise flight ; selection of an emergency landing strip. | 10 : 00 hours |
| 2. | Instrument Flight Training : |  |  |
| 2.1 | This module is identical to the 10 hours basic instrument flight module as set out in AMC 2 to Appendix 6. This module is focused on the basics of flying by sole reference to instruments, including limited panel and unusual attitudes. |  |  |
| 2.2 | All exercises may be performed in an FNPT I or II or an FFS. If Instrument Flight Training is in VMC, a suitable means of simulating IMC for the student should be used. |  |  |
| 2.3 | A BITD may be used for the following exercises : ( 9 ), ( 10 ), ( 11 ), ( 12 ), ( 14 ) and ( 16 ). |  |  |
| 2.4 | The use of the BITD is subject to the following : a. the training is complemented by exercises on an aeroplane b. the record of the parameters of the flight is available c. an FI ( A ) or IRI ( A ) conducts the instruction |  |  |
| Exercise 9 |  | Basic instrument flying without external visual cues ; horizontal flight ; power changes for acceleration or deceleration, maintaining straight and level flight ; turns in level flight with 15 ° and 25 ° bank, left and right ; roll - out onto predetermined headings. | 0 : 30 hours |
| Exercise 10 |  | Repetition of Exercise 9 ; additionally climbing and descending, maintaining heading and speed, transition to horizontal flight ; climbing and descending turns. | 0 : 45 hours |
| Exercise 11 |  | Instrument Pattern : | 0 : 45 hours |
| a | start exercise, decelerate to approach speed, flaps into approach configuration |  |  |
| b | initiate standard turn ( left or right ) |  |  |

| N 0 | Exercise N 0 | Training Item | Suggested Flight Time |
| --- | --- | --- | --- |
| Exercise 11 |  | Instrument Pattern / cont ‘ d / |  |
| c | roll out on opposite heading, maintain new heading for 1 minute |  |  |
| d | standard turn, gear down, descend 500 ft / min |  |  |
| e | roll out on initial heading, maintain descent ( 500 ft / min ) and new heading for 1 minute |  |  |
| f | transition to horizontal flight, 1.000 ft below initial flight level |  |  |
| g | initiate Go - around |  |  |
| h | climb at best rate of climb speed |  |  |
| Exercise 12 |  | Repetition of Exercise 9 and steep turns with 45° bank ; recovery from unusual attitudes. | 0 : 45 hours |
| Exercise 13 |  | Repetition of Exercise 12 | 0 : 45 hrs |
| Exercise 14 |  | Radio navigation using VOR, NDB or, if available, VDF ; interception of predetermined QDM and QDR. | 0 : 45 hours |
| Exercise 15 |  | Repetition of Exercise 9 and recovery from unusual attitudes. | 0 : 45 hrs |
| Exercise 16 |  | Repetition of Exercise 9, turns and level change and recovery from unusual attitudes with simulated failure of the artificial horizon or directional gyro. | 0 : 45 hours |
| Exercise 17 |  | Recognition of, and recovery from, incipient and full stalls. | 0 : 45 hrs |
| Exercise 18 |  | Repetition of Exercises (14 ), ( 16 ) and ( 17 ). | 3 : 30 hrs |
| 3. | ME Training |  |  |
|  | If required, operation of an ME aeroplane in the Exercises 1 through 18, including operation of the aeroplane with one engine simulated inoperative, and engine shutdown and restart. Before commencing training, the applicant should have complied with the Type and Class Ratings requirements as appropriate to the aeroplane used for the Test. |  |  |

INTENTIONALLY            LEFT            BLANK