SUBPART   H.     CLASS    and    TYPE    RATINGS

GM  1.    FCL. 700    Circumstances   in   which   Class   or   Type   Ratings   are   Required

LIST   of   CLASS   or   TYPE   RATINGS

The  following  tables  contain  lists  of  Aeroplanes  or  TMG  that  are  included  in  Class  Ratings.

a )   Class  Ratings  ( aeroplane ) :

SP  and  SEP  or  MEP  aeroplane  ( land  or  sea ) :

| Manufacturer | Aeroplanes |  | Licence Endorsement |
| --- | --- | --- | --- |
| All Manufacturers | SEP ( land ) | ( D ) | SEP ( land ) |
|  | SEP ( land ) with variable pitch propellers |  |  |
|  | SEP ( land ) with retractable undercarriage |  |  |
|  | SEP ( land ) with turbo or super charged engines |  |  |
|  | SEP ( land ) with cabin pressurization |  |  |
|  | SEP ( land ) with tail wheels |  |  |
|  | SEP ( land ) with EFIS |  |  |
|  | SEP ( land ) with SLPC |  |  |
|  | SEP ( sea ) with variable pitch propellers | ( D ) | SEP ( sea ) |
|  | SEP ( sea ) with turbo or super charged engines |  |  |
|  | SEP ( sea ) with cabin pressurization |  |  |
|  | SEP ( sea ) with EFIS |  |  |
|  | SEP ( sea ) with SLPC |  |  |
| All Manufacturers | MEP ( land ) | ( D ) | MEP ( land ) |
|  | MEP ( sea ) |  | MEP ( sea ) |

b )   Class  Ratings  ( aeroplane ) :

SP   and    SEP   TMG  ( land ) :

| Manufacturer | Aeroplanes |  | Licence Endorsement |
| --- | --- | --- | --- |
| All Manufacturers | All TMGs having an integrally mounted, non-retractable engine and a non- retractable propeller TMG |  | TMG |

c )  Additional  Class  and  Type  Rating  lists  and  endorsement  lists  are  published  by  the Agency ;

d )  Whenever  ( D ) is  indicated  in  one  of  the  lists  mentioned  in  paragraphs  ( a )  to  ( c ),  it indicates  that  differences  training  in  accordance  with  FCL. 710  is  required.

GM  1.    FCL. 710     Class   and   Type   Ratings  —  Variants

Differences   and   Familiarization   Training

a )   Differences  Training  requires  the  acquisition  of  additional  knowledge  and  training  on  an appropriate  training  device  or  the  aircraft ;

b )   Familiarization  Training  requires  the  acquisition  of  additional  knowledge.

AMC  1.    FCL. 725 ( a )   Requirements  for  the  Issue  of  Class  and  Type  Ratings

SYLLABUS   of   THEORETICAL   KNOWLEDGE   for   CLASS   or   TYPE   RATINGS

I.   SE   and   ME   Aeroplanes

a )   Detailed  listing  for  aeroplane  structure  and  equipment,  normal  operation  of  systems  and malfunctions :

1 )  dimensions :   minimum  required  runway  width  for  180 ° turn ;

2 )  engine  including  auxiliary  power  unit :

( i )    type  of  engine  or  engines ;

( ii )   in  general,  function  of  the  following  systems  or  components :

( A )  engine ;

( B )  auxiliary  power  unit ;

( C )  oil  system ;

( D )  fuel  system ;

( E )  ignition  system ;

( F )  starting  system ;

( G )  fire  warning  and  extinguishing  system ;

( H )  generators  and  generator  drives ;

( I )   power  indication ;

( J )  reverse  thrust ;

( K )  water  injection.

( iii )  on  piston  or  turbine-propeller  engines  additionally :

(A)  propeller  system ;

(B)  feathering  system.

( iv )  engine  controls  ( including  starter ),  engine  instruments  and  indications  in  the  cockpit,  their  function,  interrelation  and  interpretation ;

( v )   engine  operation,  including  APU,  during  engine  start,  start  and  engine  malfunctions, procedures  for  normal  operation  in  the  correct  sequence.

3 )   fuel  system :

( i )    location  of  the  fuel  tanks,  fuel  pumps,  fuel  lines  to  the  engines,  tank  capacities,  valves and  measuring ;

( ii )   location  of  the  following  systems :

(A)   filtering ;

(B)   heating ;

(C)   fuelling  and  defueling ;

(D)  dumping ;

(E)   venting.

( iii )  in  the  cockpit :

(A)   the  monitors  and  indicators  of  the  fuel  system ;

(B)   quantity  and  flow  indication,  interpretation.

( iv )  procedures :

(A)   fuel  procedures  distribution  into  the  various  tanks ;

(B)   fuel  supply,  temperature  control  and  fuel  dumping.

4 )   pressurization  and  air  conditioning :

( i )      components  of  the  system  and  protection  devices ;

( ii )     cockpit  monitors  and  indicators ;

( iii )     interpretation  about  the  operational  condition ;

( iv )     normal  operation  of  the  system  during  start,  cruise,  approach  and  landing,  air

conditioning  airflow  and  temperature  control.

5 )  ice  and  rain  protection,  windshield  wipers  and  rain  repellent :

( i )    ice  protected  components  of  the  aeroplane  including  engines,  heat  sources,  controls  and indications ;

( ii )   operation  of  the  anti-icing  or  de-icing  system  during  take-off,  climb,  cruise  and  descent, conditions  requiring  the  use  of  the  protection  systems ;

( iii )  controls  and  indications  of  the  windshield  wipers  and  rain  repellent  systems  operation.

6 )   hydraulic  system :

( i )    components  of  the  hydraulic  system(s),  quantities  and  system  pressure,  hydraulically actuated  components  associated  to  the  respective  hydraulic  system ;

( ii )   controls,  monitors  and  indicators  in  the  cockpit,  function  and  interrelation  and interpretation  of  indications.

7 )   landing  gear :

( i )    main  components  of  the :

(A)   main  landing  gear ;

(B)   nose  gear ;

(C)   gear  steering ;

(D)  wheel  brake  system,  including  anti-skid.

( ii )   gear  retraction  and  extension  ( including  changes  in  trim  and  drag  caused  by  gear operation ) ;

( iii )  required  tyre  pressure,  or  location  of  the  relevant  placard ;

( iv )  controls  and  indicators  including  warning  indicators  in  the  cockpit  in  relation  to  the retraction  or  extension  condition  of  the  landing  gear  and  brakes ;

( v )   components  of  the  emergency  extension  system.

8 )  flight  controls  and  high  lift  devices :

( i )      (A)   aileron  system ;

(B)   elevator  system ;

(C)   rudder  system ;

(D)   trim  system ;

(E)   spoiler  system ;

(F)   lift  devices ;

(G)  stall  warning  system ;

(H)  take-off  configuration  warning  system.

( ii )   flight  control  system  from  the  cockpit  controls  to  the  flight  control  or  surfaces ;

( iii )  controls,  monitors  and  indicators  including  warning  indicators  of  the  systems  mentioned

under  ( 8 ) ( i ),  interrelation  and  dependencies.

9 )  electrical  power  supply :

( i )    number,  power,  voltage,  frequency  and  location  of  the  main  power  system    ( AC  or  DC ),  auxiliary  power  system  location  and  external  power  system ;

( ii )   location  of  the  controls,  monitors  and  indicators  in  the  cockpit ;

( iii )  flight  instruments,  communication  and  navigation  systems,  main  and  back-up  power sources ;

( iv )  location  of  vital  circuit  breakers ;

( v )   generator  operation  and  monitoring  procedures  of  the  electrical  power  supply.

10 )  flight  instruments,  communication,  radar  and  navigation  equipment,  auto-flight  and  flight data  recorders :

( i )    visible  antennae ;

( ii )   controls  and  instruments  of  the  following  equipment  in  the  cockpit  during  normal operation :

(A)   flight  instruments ;

(B)   flight  management  systems ;

(C)   radar  equipment,  including  radio  altimeter ;

(D)  communication  and  navigation  systems ;

(E)   autopilot ;

(F)   flight  data  recorder,  cockpit  voice  recorder  and  data-link  communication  recording  function ;

(G)  TAWS ;

(H)   collision  avoidance  system ;

( I )   warning  systems.

11 )  cockpit,  cabin  and  cargo  compartment :

( i )    operation  of  the  exterior,  cockpit,  cabin  and  cargo  compartment  lighting  and  the emergency  lighting ;

( ii )   operation  of  the  cabin  and  cargo  doors,  stairs,  windows  and  emergency  exits ;

( iii )  main  components  of  the  oxygen  system  and  their  location,  oxygen  masks  and  operation of  the  oxygen  systems  for  the  crew  and  passengers,  required  amount  of  oxygen  by  means  of  a  table  or  diagram.

12 ) emergency  equipment  operation  and  correct  application  of  the  following  emergency equipment  in  the  aeroplane :

( i )    portable  fire  extinguisher ;

( ii )   first-aid  kits ;

( iii )  portable  oxygen  equipment ;

( iv )  emergency  ropes ;

( v )   life-jacket ;

( vi )  life  rafts ;

( vii ) emergency  transmitters ;

( viii ) crash  axes ;

( ix)   megaphones ;

( x )   emergency  signals.

13 )  pneumatic  system :

( i )    components  of  the  pneumatic  system,  pressure  source  and  actuated  components ;

( ii )   controls,  monitors  and  indicators  in  the  cockpit  and  function  of  the  system ;

( iii )  vacuum  system.

b )    Limitations :

1 )   general  limitations :

( i )  certification  of  the  aeroplane,  category  of  operation,  noise  certification  and  maximum  and minimum  performance  data  for  all  flight  profiles,  conditions  and  aircraft  systems :

( ii )   (A)   maximum  tail  and  crosswind-components  at  take-off  and  landing ;

(B)   maximum  speeds  for  flap  extension  vfo ;

(C)   at  various  flap  settings  vfe  ;

(D)  for  landing  gear  operation  vlo,  Mlo  ;

(E)   for  extended  landing  gear  vle,  Mle  ;

(F)   for  maximum  rudder  deflection   va,  Ma  ;

(G)  for  tyres ;

(H)  one  propeller  feathered.

(ii)   (A)   minimum  control  speed  air  v mca ;

(B)   minimum  control  speed  ground   v mcg  ;

(C)   stall  speed  under  various  conditions   v so,  v s1  ;

(D)  maximum  speed   v ne,   M ne  ;

(E)   maximum  speed  for  normal  operation   v mo,   M mo  ;

(F)   altitude  and  temperature  limitations ;

(G)  stick  shaker  activation.

(iii)    (A)   maximum  airport  pressure  altitude,  runway  slope ;

(B)   maximum  taxi  mass ;

(C)   maximum  take-off  mass ;

(D)  maximum  lift  off  mass ;

(E)   maximum  landing  mass ;

(F)   zero  fuel  mass ;

(G)  maximum  dumping  speed  v dco,   M dco,   v dce,   M dce  ;

(H)  maximum  load  factor  during  operation ;

(I)   certificated  range  of  centre  of  gravity.

2 )    engine  limitations :

( i )    operating  data  of  the  engines :

(A)   time  limits  and  maximum  temperatures ;

(B)   minimum  RPMs  and  temperatures ;

(C)   torque ;

(D)  maximum  power  for  take-off  and  go-around  on  pressure  altitude  or  flight  altitude  and temperature ;

(E)   piston  engines :  certified  range  of  mixture ;

(F)   minimum  and  maximum  oil  temperature  and  pressure ;

(G)  maximum  starter  time  and  required  cooling ;

(H)  time  between  two  start  attempts  for  engines  and  auxiliary  power  unit ;

(I)   for  propeller :  maximum  RPM  of  propeller  triggering  of  automatic  feathering  device.

( ii )   certified  oil  grades.

3 )   systems  limitations :

( i )    operating  data  of  the  following  systems :

(A)   pressurization,  air  conditioning  maximum   pressures ;

(B)   electrical  power  supply,  maximum  load  of  main  power  system  ( AC  or  DC ) ;

(C)   maximum  time  of  power  supply  by  battery  in  case  of  emergency ;

(D)  mach  trim  system  and  yaw  damper  speed  limits ;

(E)   autopilot  limitations  of  various  modes ;

(F)   ice  protection ;

(G)  speed  and  temperature  limits  of  window  heat ;

(H)  temperature  limits  of  engine  and  wing  anti-ice.

( ii )   fuel  system :   certified  fuel  specifications,  minimum  and  maximum   pressures  and

temperature  of  the  fuel.

4 )   Minimum  Equipment  List.

c )   Performance,  flight  planning  and  monitoring :

1 )   performance  calculation  about  speeds,  gradients,  masses  in  all  conditions  for  take-off,

en-route,  approach  and  landing  according  to  the  documentation  available  ( for  example  for  take-off  V 1,  V mbe,  V r,  V lof,  V 2,  take-off  distance,  maximum  take-off  mass  and  the  required  stop  distance )  on  the  following  factors :

( i )    accelerate  or  stop  distance ;

( ii )   take-off  run  and  distance  available  ( TORA,  TODA ) ;

( iii )  ground  temperature,  pressure  altitude,  slope,  wind ;

( iv )  maximum  load  and  maximum  mass  ( for  example  ZFM ) ;

( v )   minimum  climb  gradient  after  engine  failure ;

( vi )  influence  of  snow,  slush,  moisture  and  standing  water  on  the  runway ;

( vii ) possible  single  or  dual  engine  failure  during  cruise  flight ;

( viii ) use  of  anti-icing  systems ;

( ix )  failure  of  water  injection  system  or  antiskid  system ;

( x )   speeds  at  reduced  thrust,  V 1,  V 1red,  V mbe,  V mu,  V r,  V lof,  V 2  ;

( xi )  safe  approach  speed  V ref,  on  V mca  and  turbulent  conditions ;

( xii ) effects  of  excessive  approach  speed  and  abnormal  glideslope  on  the  landing  distance ;

( xiii ) minimum  climb  gradient  during  approach  and  landing ;

( xiv ) limiting  values  for  a  go-around  with  minimum  fuel ;

( xv )  maximum  allowable  landing  mass  and  the  landing  distance  for  the  destination  and alternate  aerodrome  on  the  following  factors :

(A)   available  landing  distance ;

(B)   ground  temperature,  pressure  altitude,  runway  slope  and  wind ;

(C)   fuel  consumption  to  destination  or  alternate  aerodrome ;

(D)   influence  of  moisture  on  the  runway,  snow,  slush  and  standing  water ;

(E)   failure  of  the  water  injection  system  or  the  anti  skid  system ;

(F)   influence  of  thrust  reverser  and  spoilers.

2 )   flight  planning  for  normal  and  abnormal  conditions :

( i )    optimum  or  maximum  flight  level ;

( ii )   minimum  required  flight  altitude ;

( iii )  drift  down  procedure  after  an  engine  failure  during  cruise  flight ;

( iv )  power  setting  of  the  engines  during  climb,  cruise  and  holding  under  various circumstances,  as  well  as  the  most  economic  cruising  flight  level ;

( v )   calculation  of  a  short  range  or  long  range  flight  plan ;

( vi )  optimum  and  maximum  flight  level  and  power  setting  of  the  engines  after  engine  failure.

3 )   flight  monitoring.

d )    Load  and  Balance  and  servicing :

1 )   load  and  balance :

( i )    load  and  trim  sheet  on  the  maximum  masses  for  take-off  and  landing ;

( ii )   centre  of  gravity  limits ;

( iii )  influence  of  fuel  consumption  on  the  centre  of  gravity ;

( iv )  lashing  points,  load  clamping,  maximum  ground  load.

2 )   servicing  on  ground,  servicing  connections  for :

( i )     fuel ;

( ii )    oil ;

( iii )   water ;

( iv)    hydraulic ;

( v )    oxygen ;

( vi )   nitrogen ;

( vii )  conditioned  air ;

( viii ) electric  power ;

( ix )   start  air ;

( x )    toilet  and  safety  regulations.

e )   Emergency  procedures :

1 )   recognition  of  the  situation  as  well  as  immediate  memory  actions  in  correct  sequence  and for  those  conditions  recognized  as  emergencies  by  the  manufacturer  and  competent  authority for  certification :

( i )     engine  failure  during  take-off  before  and  after  V1,  as  well  as  in - flight ;

( ii )    malfunctions  of  the  propeller  system ;

( iii )   engine  overheat,  engine  fire  on  ground  and  in-flight ;

( iv )   wheel  well  fire ;

( v )    electrical  smoke  or  fire ;

( vi )   rapid  decompression  and  emergency  descent ;

( vii )  air-conditioning  overheat,  anti-ice  system  overheat ;

( viii ) fuel  pump  failure ;

( ix )   fuel  freezing  overheat ;

( x )    electric  power  failure ;

( xi )   equipment  cooling  failure ;

( xii )  flight  instrument  failure ;

( xiii ) partial  or  total  hydraulic  failure ;

( xiv ) failures  at  the  lift  devices  and  flight  controls  including  boosters ;

( xv )  cargo  compartment  smoke  or  fire.

2 )   actions  according  to  the  approved  abnormal  and  emergency  checklist :

( i )    engine  restart  in - flight ;

( ii )   landing  gear  emergency  extension ;

( iii )  application  of  the  emergency  brake  system ;

( iv )  emergency  extension  of  lift  devices ;

( v )   fuel  dumping ;

( vi )  emergency  descent.

f )    Special  requirements  for  extension  of  a  type  rating   for  instrument  approaches  down  to Decision  Heights  of  less  than  200 ft  ( 60 m ) :

1 )   airborne  and  ground  equipment :

( i )    technical  requirements ;

( ii )   operational  requirements ;

( iii )  operational  reliability ;

( iv )  fail  operational ;

( v )   fail  passive ;

( vi )   equipment  reliability ;

( vii )  operating  procedures ;

( viii ) preparatory  measures ;

( ix )   operational  downgrading ;

( x )    communications.

2 )   procedures  and  limitations :

( i )    operational  procedures ;

( ii )   crew  coordination.

g )   Special  requirements  for  “ Glass  Cockpit “  aeroplanes  with  EFIS  Additional  learning objectives :

1 )   general  rules  of  aeroplanes  computer  hardware  and  software  design ;

2 )   logic  of  all  crew  information  and  alerting  systems  and  their  limitations ;

3 )   interaction   of   the   different   aeroplane   computer   systems,  their  limitations,  the  possibilities  of  computer  fault  recognition  and  the  actions  to  be  performed  on  computer  failures ;

4 )   normal  procedures  including  all  crew  coordination  duties ;

5 )   aeroplane  operation  with  different  computer  degradations  ( basic  flying ).

h )    Flight  Management  Systems  ( FMS ).

II.      SE   and   ME   Helicopters

a )   Detailed   listing   for   helicopters   structure,   transmissions,   rotors   and  equipment,  normal and  abnormal  operation  of  systems :

1 )   dimensions ;

2 )   engine  including  aux.  power  unit,  rotor  and  transmissions ;   if  an  initial  type  rating  for  a turbine  engine  helicopter  is  applied  for,  the  applicant  should  have  received  turbine  engine instruction :

(i)    type of engine or engines;

(ii)   in general, the function of the following systems or components :

(A)   engine ;

(B)   auxiliary power unite ;

(C)   oil system;

(D)  fuel system;

(E)   ignition system;

(F)   starting system;

(G)  fire warning and extinguishing system;

(H)  generators and generator drive;

(I)   power indication;

(J)   water or methanol  injection.

(iii) engine controls (including  starter), engine instruments and indications in the cockpit, their function and interrelation and interpretation ;

(iv)  engine operation, including APU, during engine start and engine malfunctions, procedures for normal operation in the correct sequence;

(v)   transmission system:

(A)   lubrication;

(B)   generators and generator drives;

(C)   freewheeling units;

(D)  hydraulic drives;

(E)   indication and warning systems.

(vi)  type of rotor systems: indication and warning systems.

3 )   fuel system:

(i)    location of the fuel tanks, fuel pumps, fuel lines to the engines tank capacities, valves and measuring;

(ii)   the following systems:

(A)   filtering;

(B)   fuelling and de-fuelling  heatings ;

(C)   dumping;

(D)  transferring;

(E)   venting.

(iii)  in the cockpit: the monitors and indicators of the fuel system, quantity and flow indication, interpretation ;

(iv)  fuel procedures distribution into the various tanks fuel supply and fuel dumping.

4 )   air conditioning:

(i)    components of the system and protection devices ;

(ii)   cockpit monitors and  indicators;

Note :   interpretation  about  the  operational  condition :   normal  operation  of  the  system  during  start,  cruise  approach  and  landing,  air  conditioning  airflow  and  temperature  control.

5 )   ice and rain protection, windshield  wipers  and  rain  repellent :

(i)    ice protected components of the helicopter, including engines and rotor systems, heat sources, controls and indications;

(ii)   operation of the anti-icing or de-icing system during take-off, climb, cruise and descent, conditions requiring the use of the protection systems;

(iii)  controls  and  indications  of  the  windshield  wipers  and  rain repellent system operation.

6 )   hydraulic  system :

(i)    components of the hydraulic system(s), quantities and system pressure, hydraulically actuated components associated to the respective hydraulic system;

(ii)   controls, monitors and indicators in the cockpit, function and interrelation and interpretation of indications.

7 )   landing gear, skids fixed and  floats :

(i)    main components of  the:

(A)   main landing gear;

(B)   nose gear;

(C)   tail gear;

(D)  gear steering;

(E)   wheel brake system.

(ii)   gear retraction and extension;

(iii)  required  tyre  pressure, or location of the relevant placard;

(iv)  controls and indicators including warning indicators in the cockpit in relation to the retraction or extension condition of the landing  gear ;

(v)   components of the emergency extension system.

8 )   flight controls, stab- and autopilot systems: controls, monitors and indicators including warning indicators of the systems, interrelation and dependencies.

9 )   electrical  power  supply :

(i)    number, power, voltage, frequency and if applicable phase and location of the main power system (AC or DC) auxiliary power system location and external power system;

(ii)   location of the controls, monitors and indicators in the cockpit;

(iii)  main  and  back-up  power  sources  flight  instruments, communication and navigation systems, main and back-up power sources;

(iv)  location of vital circuit breakers;

(v)   generator operation and monitoring procedures of the electrical power supply.

10 )  flight  instruments, communication, radar and navigation equipment, auto-flight  and  flight  data recorders :

(i)    antennas;

(ii)   controls  and  instruments  of  the  following  equipment  in  the cockpit:

(A)   flight instruments (for example air speed indicator, pitot static system, compass system, flight director);

(B)   flight management systems;

(C)   radar equipment (for example weather radar, transponder);

(D)  communication  and  navigation  system  (for  example  HF, VHF,  ADF,  VOR/DME,  ILS,  marker  beacon)  and  area navigation systems;

(E)   stabilisation and autopilot system;

(F)   flight  data  recorder,  cockpit  voice  recorder,  data-link communication recording function and radio altimeter;

(G)  collision avoidance system; (H)  TAWS;

( I )   HUMS.

11 )  cockpit,  cabin  and  cargo  compartment :

(i)    operation of the exterior, cockpit, cabin and cargo compartment lighting and the emergency lighting;

(ii)   operation of the cabin doors and emergency exits.

12 )  emergency  equipment :

(i)    operation   and   correct   application   of   the   following   mobile emergency equipment in the helicopter:

(A)   portable fire extinguisher;

(B)   first-aid kits;

(C)   portable oxygen equipment;

(D)  emergency ropes;

(E)   life-jacket;

(F)   life rafts;

(G)  emergency transmitters;

(H)  crash axes;

( I )   megaphones;

( J )   emergency  signals ;

(K)   torches.

(ii)   operation  and  correct  application  of  the  fixed  emergency equipment in the helicopter: emergency floats.

b )    Limitations :

1 )   general  limitations,  according  to  the  helicopter  flight  manual ;

2 )   minimum  equipment  list.

c )   Performance,  flight  planning  and  monitoring :

1 )   performance  calculation  about  speeds,  gradients,  masses  in  all  conditions  for  take-off,  en-route,  approach  and  landing :

(i)    take-off :

(A)   hover performance in and out of ground effect;

(B)   all approved profiles, cat A and B;

(C)   HV diagram;

(D)  take-off and rejected take-off distance;

(E)   take-off decision point (TDP) or (DPATO);

(F)   calculation of first and second segment distances;

(G)  climb performance.

(ii)   en-route:

(A)   air speed indicator correction;

(B)   service ceiling;

(C)   optimum or economic cruising altitude;

(D)  max endurance;

(E)   max range;

(F)   cruise climb performance.

(iii)  landing:

(A)   hovering in and out of ground effect;

(B)   landing distance;

(C)   landing decision point (LDP) or (DPBL).

(iv)  knowledge or calculation of :  Vlo,  Vle,  Vmo,  Vx,  Vy,  Vtoss,  Vne,  Vmax  range,  Vmini.

2 )    flight  planning  for  normal  and  abnormal  conditions :

(i)    optimum or maximum flight level;

(ii)   minimum required flight altitude;

(iii)  drift down procedure after an engine failure during cruise flight;

(iv)  power setting of the engines during climb, cruise and holding under various circumstances as well as at the most economic cruising flight level;

(v)   optimum and maximum flight level and power setting after an engine failure.

(3)   effect of optional equipment on performance.

d )   Load,  balance  and  servicing :

1 )   load  and  balance :

(i)    load and trim sheet on the maximum masses for take-off and landing;

(ii)   centre of gravity limits;

(iii)  influence of the fuel consumption on the centre of gravity;

(iv)  lashing points, load clamping, max ground load.

2 )   servicing  on  the  ground,  servicing  connections  for :

(i)    fuel;

(ii)   oil,  etc.. ;

(iii)  and  safety  regulations  for  servicing.

(e)   Emergency  procedures.

(f)    Special  requirements  for  extension  of  a  type  rating  for  instrument  approaches  down  to  a decision  height  of  less  than  200 ft  ( 60 m ) :

(1)   airborne and ground equipment :

(i)    technical requirements;

(ii)   operational requirements ;

(iii)   operational reliability;

(iv)   fail operational;

(v)    fail passive;

(vi)   equipment reliability;

(vii)   operating procedures;

(viii)  preparatory measures;

(ix)    operational downgrading;

(x)     communication.

2 )   procedures  and  limitations :

(i)    operational procedures ;

(ii)   crew co-ordination.

g )   Special  requirements  for  helicopters  with  EFIS.

h )   Optional  equipment.

III.       AIRSHIPS

a )   Detailed  listing  for  airship  structure  and  equipment,  normal  operation  of  systems  and malfunctions :

1 )   dimensions ;

(2)   structure  and  envelope :

(i)    internal structure;

(ii)   envelope;

(iii)  pressure system;

(iv)  gondola;

(v)   empennage.

3 )   flight  controls ;

4 )   systems :

(i)    hydraulic ;

(ii)   pneumatic.

5 )   landing  gear ;

6 )   fuel  system ;

7 )   fire  warning  and  extinguishing  system ;

8 )   emergency  equipment ;

9 )   electrical  systems ;

10 )  avionics,  radio  navigation  and  communication  equipment ;

11 )  instrumentation ;

12 )  engines  and  propellers ;

13 )  heating,  ventilation  and  air-condition ;

14 )  operational  procedures  during  start,  cruise,  approach  and  landing :

(i)    normal operations;

(ii)   abnormal  operations.

b )    Limitations :

1 )    general  limitations :

(i)    certification   of   the   airship,   category   of   operation,   noise certification and maximum and minimum performance data for all flight profiles, conditions  and  aircraft  systems ;

(ii)   speeds;

(iii)  altitudes.

2 )    engine  limitations ;

3 )    systems  limitations ;

4 )    minimum  equipment  list.

c )    Performance  and  flight  planning :

1 )   performance  calculation ;

2 )    flight  planning.

d )   Load  and  balance  and  servicing :

1 )   load  and  balance ;

2 )   servicing.

e )   Emergency  procedures :

1 )   recognition  of  emergency  situations ;

2 )   actions  according  to  the  approved  abnormal  and  emergency  checklist.

AMC  2.    FCL. 725 ( a )   Requirements   for   the   Issue   of   Class   and   Type   Ratings

TRAINING    COURSE

FLIGHT   INSTRUCTION   FOR   TYPE   RATINGS :    Helicopters

a )   The  amount  of  flight  instruction  depends  on :

(i)   complexity  of  the  helicopter  type,  handling  characteristics,  level  of  technology ;

(ii)   category  of  helicopter  ( SEP  or  SE  turbine  helicopter,  ME  turbine  and  MP  helicopter ) ;

(iii)  previous  experience  of  the  applicant ;

(iv)  the  availability  of  FSTDs.

b )   FSTDs :

The  level  of  qualification  and  the  complexity  of  the  type  will  determine  the  amount  of practical  training  that  may  be  accomplished  in  FSTDs,  including  completion  of  the  Skill  Test. Before  undertaking  the  skill  test,  a  student  should  demonstrate  competency  in  the  skill  test items  during  the  practical  training.

c )   Initial   Issue :

The  Flight  Instruction  ( excluding  Skill  Test )  should  comprise :

| Helicopter Types | In Helicopter | In Helicopter and FSTD associated Training Credits |
| --- | --- | --- |
| SEP ( H ) | 5 hours | Using FFS C / D : at least 2 hrs helicopter and at least 6 hrs total Using FTD 2 / 3 : at least 4 hrs helicopter and at least 6 hrs total |
| SET ( H ) under 3175 kg MTOM | 5 hours | Using FFS C / D : at least 2 hrs helicopter and at least 6 hrs total Using FTD 2 / 3 : at least 4 hrs helicopter and at least 6 hrs total |

| Helicopter Types | In Helicopter | In Helicopter and FSTD associated Training Credits |
| --- | --- | --- |
| SET ( H ) at or over 3175 kg MTOM | 8 hours | Using FFS C / D : at least 2 hrs helicopter and at least 10 hrs total Using FTD 2 / 3 : at least 4 hrs helicopter and at least 10 hrs total |
| SPH MET ( H ) CS and FAR 27 and 29 | 8 hours | Using FFS C / D : at least 2 hrs helicopter and at least 10 hrs total Using FTD 2 / 3 : at least 4 hrs helicopter and at least 10 hrs total |
| MPH | 10 hours | Using FFS C / D : at least 2 hrs helicopter and at least 12 hrs total Using FTD 2 / 3 : at least 4 hrs helicopter and at least 12 hrs total |

d )     Additional   Types

The   Flight  Instruction  ( excluding  Skill  Test )   should  comprise :

| Helicopter Types | In Helicopter | In Helicopter and FSTD associated Training Credits |
| --- | --- | --- |
| SEP ( H ) to SEP ( H ) within AMC 1. FCL.740. H ( a )( 3 ) | 2 hours | Using FFS C / D : at least 1 hrs helicopter and at least 3 hrs total Using FTD 2 / 3 : at least 1 hrs helicopter and at least 4 hrs total |
| SEP ( H ) to SEP ( H ) not included in AMC 1. FCL.740. H ( a )( 3 ) | 5 hours | Using FFS C / D : at least 1 hrs helicopter and at least 6 hrs total Using FTD 2 / 3 : at least 2 hrs helicopter and at least 7 hrs total |
| SET ( H ) to SET ( H ) | 2 hours | Using FFS C / D : at least 1 hrs helicopter and at least 3 hrs total Using FTD 2 / 3 : at least 1 hrs helicopter and at least 4 hrs total |
| SE ( H ) Differe - - nce Training | 1 hours | N / A |
| MET ( H ) to MET ( H ) | 3 hours | Using FFS C / D : at least 1 hrs helicopter and at least 4 hrs total Using FTD 2 / 3 : at least 2 hrs helicopter and at least 5 hrs total |
| ME ( H ) Differe - - nce Training | 1 hours | N / A |
| MPH to MPH | 5 hours | Using FFS C / D : at least 1 hrs helicopter and at least 6 hrs total Using FTD 2 / 3 : at least 2 hrs helicopter and at least 7 hrs total |
| Extend privileges on the same Type Rating from SPH to MPH ( except for initial MP issue ), or from MPH to SPH | 2 hours | Using FFS C / D : at least 1 hrs helicopter and at least 3 hrs total |

e )   Holders  of  an  IR ( H )  wishing  to  extend  the  IR ( H )  to  further  types  should  have additionally  2  hours  flight  training  on  type  by  sole  reference  to  instruments  according  to  IFR which  may  be  conducted  in  an  FFS  C / D  or  FTD  2 / 3.

Holders  of  an  SE  IR ( H )  wishing  to extend  the  IR  privileges  to  an  ME  IR ( H )  for  the  first  time  should  complete  at  least   5  hours  training.

AMC  1.   FCL. 740 ( b )( 1 )   Validity   and   Renewal   of   Class   and   Type   Ratings

RENEWAL   of   CLASS   and   TYPE   RATINGS :    Refresher   Training

a )   Paragraph  ( b )( 1 )  of  FCL.740  determines  that  if  a  Class  or  Type  Rating  has  lapsed,  the  applicant  shall  take  refresher  training  at  an  ATO.  The  objective  of  the  training  is  to  reach  the level  of  proficiency  necessary  to  safely  operate  the  relevant  type  or  class  of  aircraft.

The  amount  of  refresher  training  needed  should  be  determined  on  a  case-by-case  basis  by  the ATO,  taking  into  account  the  following  factors :

1 )  the  Experience  of  the  Applicant :

To  determine  this,  the  ATO  should  evaluate  the  pilot’s  Logbook,  and,  if  necessary,  conduct  a test  in  an  FSTD ;

2 )  the  Complexity  of  the  Aircraft ;

3 )  the  amount  of  time  lapsed  since  the  expiry  of  the  validity  period  of  the  Rating :

The  amount  of  training  needed  to  reach  the  desired  level  of  proficiency  should  increase  with the  time  lapsed.  In  some  cases,  after  evaluating  the  pilot,  and  when  the  time  lapsed  is  very limited ( less  than  3  months ),  the  ATO  may  even  determine  that  no  further  refresher  training is  necessary.  See  Table  1  &  2  below,  for  GDCA  recommended  practice.  When  determining  the  needs  of  the  pilot,  the  following  items  can  be  taken  into  consideration :

( i )    expiry  shorter  than  3  months :    no  supplementary  requirements ;

( ii )   expiry  longer  than  3  months,  but  shorter  than  1  year :     a  minimum  of  2  ( two )  training  sessions ;

( iii )  expiry  longer  than  1  year  but  shorter  than  3  years :    a  minimum  of  3  ( three )  training sessions, in  which  the  most  important  malfunctions  in  the  available  systems  are  covered;

( iv )  expiry  longer  than  3  years :    the  applicant  should  again  undergo  the  training  required  for  the  initial  issue  of  the  Rating  or,  in  case  of  helicopter,  the  training  required  for  the  “ additional  type  issue “,  according  to  other  valid  ratings  held.

b )   Once  the  ATO  has  determined  the  needs  of  the  applicant,  it  should  develop  an  individual training  programme  that  should  be  based  on  the  initial  training  for  the  issue  of  the  rating  and focus  on  the  aspects  where  the  applicant  has  shown  the  greatest  needs ;

c )   After  successful  completion  of  the  training,  the  ATO  should  give  a  Certificate,  or  other documental  evidence  that  the  training  has  been  successfully  achieved  to  the  applicant,  to  be submitted  to  the  GDCA  of  RA  when  applying  for  the  renewal.  The  certificate  or documental  evidence  needs  to  contain  a  description  of  the  training  programme.

Class  &  Type  Rating  Renewal  Policy  in  case  of  Flight  interruption

Minimum   required   Refresher   Training   Course   after   flight   interruption :

a )  for  fully  familiar  and  experienced  Crew  [ minimum  of  1 ( one )  year  operation’s  experience

and   completion   minimum   of   2  ( two )  Proficiency  Check  ].

Minimum  required  Refresher  Training,  which  included  FFS  Training  &  Flight  Training  Under  Supervision  on  aircraft  during  day  and  night  operation’s :

Phase  A   -    FFS   training :

Table  1

| After an interruption |  | Refresher Course + LFUS | Only Refresher Course |
| --- | --- | --- | --- |
| a | up to 3 months | no supplementary requirements |  |
| b | from 6 to 12 months | a minimum of 1 ( one ) FFS training session | a minimum of 2 ( two ) FFS training sessions |
| c | from 12 to 36 months | a minimum of 2 ( two ) FFS training sessions | a minimum of 3 ( three ) FFS training sessions |
| f | above 36 months | New Type Rating Course |  |

Phase  B  -  LFUS   training :

1 )  Minimum   LFUS  4  route  sector,  with  as  minimum   night  1  Take-off   &  1  Landing ;

2 )  2  route  sector  Line  Check   flight.

b )  for  low  experienced  Crew  [  below  1 ( one )  year  operation’s  experience,  and

without  of  the  completion  of  2  ( two )  Proficiency  Check  and  / or   accumulate  flight

experience  on  Type  below  than  300  hour’s.

Table  2

| After an interruption |  | Refresher Training Course |
| --- | --- | --- |
| a | up to 3 months | LFUS 2 route sector + 2 route sector Line Check flight |
| b | from 6 to 12 months | CBT 2 + 2 FFS + OPC FFS session |
| c | from 12 to 36 months | CBT 3 + 3 FFS + OPC FFS session |
| d | above 36 months | New Type Rating Course |

Note :   CBT  1  -   1 ( one )  day ,  8  hr’s  on  workstation ;

CBT  2  -   12  hr’s  on  workstation ;

CBT  3  -   18  hr’s  on  workstation ;

Duration  of  1  FFS  session,  as  minimum   is  4  hr’s.

AMC  1.    FCL. 720. A ( b )( 2 )( i )    Experience  Requirements  &  Prerequisites  for  the  Issue

of   Class  or  Type  Ratings  —  Aeroplanes

ADDITIONAL   THEORETICAL   KNOWLEDGE   for   a   CLASS   or   TYPE   RATING   for   HIGH   PERFORMANCE   SP   AEROPLANES

a )  A  number  of  aeroplanes  certificated  for  SP  operation  have  similar  performances,  systems and  navigation  capabilities  to  those  more  usually  associated  with  MP  types  of  aeroplanes,  and regularly  operate  within  the  same  airspace.  The  level  of  knowledge  required  to  operate  safely in  this  environment  is  not  part  of,  or  not  included  to  the  necessary  depth  of  knowledge  in  the training  syllabi  for  the  PPL,  CPL  or  IR ( A )  but  these  licence  holders  may  fly  as  PIC  of  such  aeroplanes.  The  additional  theoretical  knowledge  required  to  operate  such  aeroplanes  safely  is  obtained  by  completion  of  a  course  at  an  ATO ;

b )   The  aim  of  the  theoretical  knowledge  course  is  to  provide  the  applicant  with  sufficient knowledge  of  those  aspects  of  the  operation  of  aeroplanes  capable  of  operating  at  high  speeds  and  altitudes,  and  the  aircraft  systems  necessary  for  such  operation ;

c )   The  course  should  cover  at  least  the  following  items  of  the  aeroplane  syllabus  to  the ATPL ( A )  level :

| LO N 0 | LO TOPICS |
| --- | --- |
| 021 00 00 00 | AIRCRAFT GENERAL KNOWLEGDE : AIRFRAME and SYSTEMS, ELECTRICS, POWERPLANT and EMERGENCY EQUIPMENT |
| 021 02 02 01 to 021 02 02 03 | Alternating current : general Generators AC power distribution |
| 021 01 08 03 | Pressurization ( Air driven systems - piston engines ) |
| 021 01 09 04 | Pressurization ( Air driven systems - turbojet and turbo-propeller ) |
| 021 03 01 06 021 03 01 07 021 03 01 08 021 03 01 09 | Engine performance - piston engines Power augmentation ( turbo or supercharging ) Fuel Mixture |
| 021 03 02 00 to 021 03 04 09 | Turbine engines |
| 021 04 05 00 | Aircraft oxygen equipment |
| 032 03 00 00 | PERFORMANCE CLASS B : ME AEROPLANES |
| 032 03 01 00 to 032 03 04 01 | Performance of ME aeroplanes not certificated under CS and FAR to 25 : entire subject |
| 040 00 00 00 | HUMAN PERFORMANCE |
| 040 02 01 00 to 040 02 01 03 | Basic Human Physiology and High altitude environment |
| 050 00 00 00 | METEOROLOGY |
| 050 02 07 00 to 050 02 08 01 | Jet streams CAT Standing waves |
| 050 09 01 00 to 050 09 04 05 | Flight hazards Icing and turbulence Thunderstorms |

| LO N 0 | LO TOPICS |
| --- | --- |
| 062 02 00 00 | BASIC RADAR PRINCIPLES |
| 062 02 01 00 to 062 02 05 00 | Basic radar principles Airborne radar SSR |
| 081 00 00 00 | PRINCIPLES OF FLIGHT : AEROPLANES |
| 081 02 01 00 to 081 02 03 02 | Transonic aerodynamics : entire subject Mach number or shockwaves Buffet margin or aerodynamic ceiling |

d )   Demonstration  of  acquisition  of  this  knowledge  is  undertaken  by  passing  an  examination set  by  ATO.  A  successful  pass  of  this  examination  results  in  the  issue  of  a  certificate indicating  that  the  course  and  examination  have  been  completed ;

e )   The  certificate  represents a “ once  only “  qualification  and  satisfies  the  requirement  for  the addition  of  all  future  high  performance  aeroplanes  to  the  holder’s  licence.  The  certificate  is valid  indefinitely  and  is  to  be  submitted  with  the  application  for  the  first  HPA  type  or  class rating ;

f )    A  pass  in  any  theoretical  knowledge  subjects  as  part  of  the  HPA  course  will  not  be credited  against  meeting  future  theoretical  examination  requirements  for  issue  of  a  CPL ( A ), IR  ( A )  or  ATPL ( A ).

AMC  1.   FCL. 725. A ( b )    Theoretical   Knowledge  &  Flight  Instruction  for  the  Issue  of

Class  and  Type  Ratings  —  Aeroplanes

CLASS    RATING    SEA

a )   The  theoretical  knowledge  instruction  should  be  conducted  by  an  instructor  having appropriate  experience  of  class  rating  sea ;

b )   Depending  on  the  equipment  and  systems  installed,  the  instruction  should  include,  but  not be  limited  to,  the  following  content :

1 )   theoretical  knowledge :

( i )     the  aim  of  the  training  is  to  teach :

(A)  the  importance  of  preparation  for  flight  and  the  safe  planning  taking  into  consideration  all the  factors  for  manoeuvring  the  aircraft  on  the  wind,  tidal  currents,  high  and  low  water  times  and  water  movements  at  sea,  river  estuaries  and  lakes.  In  addition,  icing  conditions,  ice  covered  water  and  broken  ice  flows ;

(B)  the  techniques  about  the  most  critical  moments  at  take - off,  landing,  taxiing  and  mooring the  aircraft ;

(C)  the  construction  methods  and  characteristics  of  floats  and  water  rudders  and  the  importance  of  checking  for  leaks  in  the  floats ;

(D)  the  necessary  requirements  for  the  compliance  of  the  rules  for  the  avoidance  of  collisions at  sea,  in  regard  to  sea  charts,  buoys  and  lights  and  horns.

( ii )    after  completing  the  training,  the  student  should  be  able  to :

(A)  describe  the  factors  that  have  significance  for  planning  and  decision  about  initiation  of  seaplane  flying  and  alternative  measures  for  completion  of  flight ;

(B)  describe  how  the  water  level  is  affected  by  air  pressure,  wind,  tide,  regularizations  and  the  flight  safety  depending  on  changes  in  the  water  level ;

(C)  describe  the  origin  of  different  ice  conditions  in  water  areas ;

(D)  interpret  nautical  charts  and  maps  about  depths  and  shoals  and  risk  for  water  currents, shifts  of  the  wind,  turbulence ;

(E)  decide  what  required  equipment  to  bring  during  seaplane  flying  according  to  the operational  requirements ;

(F)  describe  the  origin  and  extension  of  water  waves,  swells  and  water  currents  and  their effect  on  the  aeroplane ;

(G)  describe  how  water  and  air  forces  effect  the  aeroplane  on  water ;

(H)  describe  the  effect  of  water  resistance  on  the  aeroplanes  performance  on  glassy  water  and during  different  wave  conditions ;

(I)  describe  the  consequences  of  taxiing  with  too  high  engine  RPM ;

(J)  describe  the  effect  of  pressure  and  temperature  on  performance  at  take-off  and  climb  from lakes  located  at  higher  altitude ;

(K) describe  the  effect  of  wind,  turbulence,  and  other  meteorological  conditions  of  special importance  for  flight  over  lakes,  islands  in  mountain  areas  and  other  broken  ground ;

(L)  describe  the  function  of  the  water  rudder  and  its  handling,  including  the  effect  of  lowered water  rudder  at  take-off  and  landing ;

(M) describe  the  parts  of  the  float  installation  and  their  function ;

(N)  describe  the  effect  of  the  floats  on  the  aeroplanes  aerodynamics  and  performance  in  water  and  in  air ;

(O) describe  the  consequences  of  water  in  the  floats  and  fouling  of  float  bottoms ;

(P)  describe  aviation  requirements  that  apply  specifically  for  the  conduct  of  aircraft  activity  on  water ;

(Q) describe  requirements  about  animal,  nature  and  environment  protection  of  significance  for flight  by  seaplane,  including  flight  in  national  parks ;

(R)  describe  the  meaning  of  navigation  buoys ;

(S)  describe  the  organization  and  working  methods  of  the  Sea  Rescue  Service ;

(T)  describe  the  requirements  in  ICAO  Annex  2  as  set  out  in  paragraph  3.2.6  “ Water  operation “,  including  relevant  parts  of  the  Convention  on  the  International  Regulations  for Preventing   Collisions  at  Sea.

(2)   practical  training :

( i )    the  aim  of  the  practical  training  is  to  learn :

(A)   the  skills  in  manoeuvring  aeroplanes  on  water  and  in  mooring  the  aeroplane ;

(B)   the  skills  required  for  the  reconnaissance  of  landing  and  mooring  areas  from  the  air,  including  the  take-off  area ;

(C)   the  skills  for  assessing  the  effects  of  different  water  depths,  shoals,  wind,  height  of  waves  and  swell ;

(D) the  skills  for  flying  with  floats  about  their  effect  on  performance  and  flight  characteristics ;

(E)   the  skills  for  flying  in  broken  ground  during  different  wind  and  turbulence  conditions ;

(F)   the  skills  for  take-off  and  landing  on  glassy  water, different   of  swell  and  water  current

conditions.

( ii )   after  the  training,  the  student  should  be  able  to :

(A)  handle  the  equipment  that  shall  be  brought   during  seaplane  flying ;

(B)  perform  pre-flight  daily  inspection  on  aeroplane,  float  installation  and  special  seaplane equipment,  including  emptying  of  floats ;

(C)  sail,  taxi  and  turn  the  aeroplane  at  swell  with  correct  handling  of  the  water  rudder ;

(D)  taxi  on  the  step  and  perform  turns ;

(E)   establish  the  wind  direction  with  the  aeroplane ;

(F)   take  necessary  actions  if  loss  of  steering  ability  and  person  falling  overboard ;

(G)  make  land  and  moor  aeroplane  at  bridge,  buoy  and  beach  with  the  use  of  appropriate knots  to  secure  the  aircraft ;

(H)  maintain  given  rate  of  descent  by  means  of  variometer  only ;

(I)    perform  take-off  and  landing  on  glassy  water  with  and  without  outer  references ;

(J)    perform  take-off  and  landing  under  swell ;

(K)   perform  power-off  landing ;

(L)   from  the  air,  reconnaissance  of  landing,  mooring  and  take-off  areas,  observing ;

(M)  wind  direction  and  strength  during  landing  and  take-off ;

(N)  surrounding  terrain ;

(O)  overhead  wires  and  other  obstacles  above  and  under  water ;

(P)   congested  areas ;

(Q)  determine  wind  direction  and  assess  wind  strength  from  water  level  and  when  airborne ;

(R)  state,  for  the  aeroplane  type  in  question ;

(a)   maximum  wave  height  allowed ;

(b)   maximum  number  of  ERPM  allowed  during  taxi ;

(S)   describe  how  flying  with  floats  affects  the  performance  and  flight  characteristics  of  the aeroplane ;

(T)   take  corrective  action  at  critical  moments  due  to  wind  shear  and  turbulence ;

(U)  navigate  on  the  water  with  reference  to  buoys  markers,  obstacles  and  other  traffic  on  the water.

c )   For  the  initial  issue  of  Class  Rating  sea  for  SP,  SE  and  ME  aeroplanes,  the  number  of multi-choice  questions  in  the  written  or  computer-based  examination  should  at  least  comprise 30  ( thirty )  questions,  and  may  be  conducted  by  the  training  organization.

The  pass  mark  should  be  75 %.

AMC  1.     FCL. 735. A ;    FCL. 735. H ;    FCL. 735. As

MULTI  -  CREW     COOPERATION     COURSE  (  MCC  )

a )  Competency  is  a  combination  of  knowledge,  skills  and  attitudes  required  to  perform  a  task to  the  prescribed  standard ;

b )  The  objectives  of  MCC  training  are  to  develop  the  technical  and  non- technical  components of  the  knowledge,  skills  and  attitudes  required  to  operate  a  multi-crew  aircraft ;

c )  Training  should  comprise  both  theoretical  and  practical  elements  and  be  designed  to  achieve  the  following  competencies :

| COMPETENCY | PERFORMANCE INDICATORS | KNOWLEDGE | PRACTICAL EXERCISES |
| --- | --- | --- | --- |
| Communication | a) Know what, how much and who to communicate to ; b) Ensure the recipient is ready and able to receive the information ; c) Pass messages and information clearly, accurately, timely and adequately ; d) Check if the other person has the correct understanding when passing important information ; e) Listen actively, patiently and demonstra-te understanding when receiving information ; f) Ask relevant and effective questions, and offer suggestions ; g) Use appropriate body language, eye contact and tone ; h) Open and receptive to other people’s view. | a) Human Factors, TEM and CRM ; b) Application of TEM and CRM principles to training. | In a commercial air transport environment, apply multi-crew procedures, including principles of TEM and CRM to the following : a) Pre-flight preparation : 1) FMS initialization; 2) radio and navigation equipment preparation ; 3) flight documentation 4) Computation of take-off performance data. b) Take-off and climb: 1) before take-off checks ; 2) normal take-offs; 3) rejected take-offs; 4) take-offs with abno-rmal and emergency situations included. c) Cruise: emergency descent. d) Descent and approach: 1) instrument flight procedures; 2) holding; 3) precision approach using raw data ; 4) precision approach using flight director ; 5) precision approach using autopilot ; 6) one-engine-inoperative approach; 7) non-precision and circling approaches ; 8) computation of approach and landing data ; 9) all engines GA ; 10) go-around with one engine inoperative; 11) wind shear during approach. |
| Leadership and Team Working | a) Friendly, enthusiastic, motivating and considerate of others; b) Use initiative, give direction and take responsibility when required; c) Open and honest about thoughts, concerns and intentions ; d) Give and receive criticism and praise well, and admit mistakes; e) Confidently do and say what is important to him or her ; f) Demonstrate respect and tolerance towards other people ; g) Involve others in planning and share activities fairly. |  |  |

| COMPETENCY | PERFORMANCE INDICATORS | KNOWLEDGE | PRACTICAL EXERCISES |
| --- | --- | --- | --- |
| Situation Awareness | a) Aware of what the aircraft and its systems are doing; b) Aware of where the aircraft is and its environment; c) Keep track of time and fuel ; d) Aware of the condition of people involved in the operation including passengers ; e) Recognize what is likely to happen, plan and stay ahead of the game ; f) Develop what-if scenarios and make pre-decisions ; g) Identify threats to the safety of the aircraft and of the people. |  | e) landing : transition from instrument to visual flight on reaching decision altitude or height or minimum descent altitude or height ; f) after landing and post flight procedures ; g) selected emergency and abnormal procedures |
| Workload Management | a) Calm, relaxed, careful and not impulsive ; b) Prepare, prioritize and schedule tasks effectively ; c) Use time efficiently when carrying out tasks; d) Offer and accept assistance, delegate when necessary and ask for help early ; e) Review and monitor and cross-check actions conscientiously ; f) Follow procedures appropriately and consistently ; g) Concentrate on one thing at a time, ensure tasks are completed and does not become distracted ; h) Carry out instructions as directed. |  |  |

| COMPETENCY | PERFORMANCE INDICATORS | KNOWLEDGE | PRACTICAL EXERCISES |
| --- | --- | --- | --- |
| Problem Solving and Decision Making | a) Identify and verify why things have gone wrong and do not jump to conclusions or make assumptions ; b) Seek accurate and adequate information from appropriate resources ; c) Persevere in working through a problem ; d) Use and agree an appropriate decision making process ; e) Agree essential and desirable criteria and prioritizes ; f) Consider as many options as practicable ; g) Make decisions when they need to, reviews and changes if required ; h) Consider risks but do not take unnecessary risks. |  |  |
| Monitoring and Cross - Checking | a) Monitor and cross-checks all actions ; b) Monitor aircraft trajectory in critical flight phases ; c) Take appropriate actions in response to deviations from the flight path. | a) SOPs ; b) Aircraft systems ; c) Undesired aircraft states. |  |
| Task Sharing | a) Apply SOPs in both PF and PNF roles ; b) Makes and responds to standard callouts. | a) PF and PNF roles ; b) SOPs. |  |
| Use of Competency Checklists | Utilize checklists appropriately according to SOPs. | a) SOPs ; b) Checklist philosophy |  |
| Briefings | Prepare and deliver appropriate briefings. | a) SOPs ; b) Interpretation of FMS data and in-flight documentation. |  |

| COMPETENCY | PERFORMANCE INDICATORS | KNOWLEDGE | PRACTICAL EXERCISES |
| --- | --- | --- | --- |
| Flight Management | a) Maintain a constant awareness of the aircraft automation state ; b) Manage automation to achieve optimum trajectory and minimum workload ; c) Take effective recovery actions from automation anomalies ; d) Manage aircraft navigation, terrain clearance ; e) Manage aircraft fuel state and take appropriate actions. | a) Understanding of aircraft performance and 0onfiguration ; b) Systems ; c) SOPs ; d) Interpretation of FMS data and in-flight documentation ; e) Minimum terrain clearance ; f) Fuel management IFR and VFR regulation. |  |
| FMS use | Programme, manage and monitor FMS in accordance with SOPs. | a) Systems ( FMS ) ; b) SOPs ; c) Automation. |  |
| Systems Normal Operations | Perform and monitor normal systems operation in accordance with SOPs | a) Systems ; b) SOPs. |  |
| Systems Abnormal and Emergency Operations | a) Perform and monitor abnormal systems operation in accordance with SOPs ; b) Utilize electronic and paper abnormal checklists in accordance with SOPs. | a) Systems ; b) SOPs. c) Emergency and abnormal procedures and checklists; d) Recall items. |  |
| Environment, Weather and ATC | a) Communicate effectively with ATC ; b) Avoid misunderstandings by requesting clarification ; c) Adhere to ATC instructions ; d) Construct a mental model of the local ATC and weather environment. | a) Systems ; b) SOPs ; c) ATC environment and phraseology ; d) Procedures for hazardous weather conditions. |  |

CERTIFICATE    OF    COMPLETION    FORM

| CERTIFICATE of COMPLETION of MCC - TRAINING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Applicant's Last Name(s) |  |  |  |  |  |  |  |  |  |  |  |  | First Name(s) |  |  |  |  |  |  |
| Type of Licence |  |  |  |  |  |  |  | Number |  |  |  |  |  |  |  |  | State |  |  |
| ME / IR |  |  |  |  |  |  |  | or |  | ME / IR Skill Test |  |  |  |  |  |  |  |  |  |
| Issued on |  |  |  |  |  |  |  |  |  |  | Passed on |  |  |  |  |  |  |  |  |
| Date |  |  |  |  |  |  |  |  | Signature of Applicant |  |  |  |  |  |  |  |  |  |  |
| The satisfactory completion of MCC - Training according to requirements is certified below : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRAINING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Multi - Crew Co - operation Training Received during Period : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From : |  |  |  |  |  | To : |  |  |  |  |  |  |  | At : | ATO / Operator * |  |  |  |  |
| State |  |  |  |  |  |  |  |  |  |  |  |  |  | CAA |  |  |  |  |  |
| Location |  |  |  |  |  |  |  |  |  |  |  |  |  | Date : |  |  |  |  |  |
| Signature of Head of ATO or Authorized Instructor * : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type & Number of Licence and State of Issue : _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ |  |  |  |  |  |  |  |  |  |  |  |  |  | Name(s) in Capital Letters of Authorized Instructor : _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ |  |  |  |  |  |

*  Delete   as   appropriate

AMC  1.    FCL. 740. H ( a )( 3 )    Revalidation   of   Type   Ratings  —  Helicopters

Only  the  following  SEP  helicopter  types  can  be  considered  for  crediting  of  the  Proficiency Check.

Other  SEP  helicopters  ( for  example  the  R22  and  R44 )  should  not  be  given  credit  for.

| Manufacturer | Helicopter Type and Licence Endorsement |
| --- | --- |
| Agusta - Bell |  |
| SEP | Bell 47 |
| Bell Helicopters |  |
| SEP | Bell 47 |
| Brantley |  |
| SEP | Brantley B 2 |
| Breda Nardl |  |
| SEP | HU 269 |
| Enstrom |  |
| SEP | ENF 28 |
| Hélicoptères Guimbal |  |
| SEP | Cabri G 2 |
| Hiller |  |
| SEP | UH 12 |
| Hughes or Schweizer |  |
| SEP | HU 269 |
| Westland |  |
| SEP | Bell 47 |

GM  1.    FCL. 720. PL    Experience  Requirements  and  Prerequisites  for  the  Issue  of

Type Ratings  —  Powered - Lift   Aircraft

The  endorsement  of  a  powered-lift  Type  Rating  to  an  aeroplane  or  helicopter  licence  does  not confer  upon  its  holder  the  privileges   to  fly  helicopters  or  aeroplanes,  respectively.

INTENTIONALLY            LEFT            BLANK