Հավելված

ՀՀ կառավարությանն առընթեր

քաղաքացիական ավիացիայի

գլխավոր վարչության պետի

2014 թվականի նոյեմբերի 6-ի

N 277-Ն հրամանի

<<Հավելված

ՀՀ կառավարությանն առընթեր

քաղաքացիական ավիացիայի

գլխավոր վարչության պետի

2010 թվականի հոկտեմբերի 20-ի

N 188-Ն հրամանի

ARM - AIR  OPS

/ ARM - AIR  OPERATIONS /

UNCONTROLLED   COPY   WHEN   DOUNLOADED  /  PRINTED

Check   with   GDCA  of  RA   Website

to   verify   current   version   before   using

INTENTIONALLY            LEFT            BLANK

FOREWORD

1    The   General   Department   of   Civil   Aviation   at   the   Government   of   Republic   of Armenia  ( GDCA  of  RA ),  known  in  these  regulations  as  the  “ Authority ”  has

implemented   ARMENIAN – AIR  OPERATION’S   (  ARM - AIR  OPS  )  based   on   the   European   Aviation   Safety   Agency  ( EASA )  Regulation,  with   a  view  to  harmonizing  legislation  and  to  regulate  all  Air   Transport  operations   of  aircrafts.

2    ICAO  Annex  6  has  been  selected  to  provide  the  basic  structure  of  ARM–AIR  OPS   and  for       Air  Operator,   NCC,  NCO  Certification  and  Private  Operator  Authorization,  but  with  additional  sub-division  where  considered appropriate.                                                                                                                                    The  content  of  Annex  6   has   been   used   and   added   to,   where   acceptable.

3   The  Authority  has  adopted  associated   compliance  or  interpretative  material  wherever  possible and,  unless  specifically  stated  otherwise,  clarification  will  be  based  on  this  material  or  other  EASA  documentation.

4    Future   development   of   the   requirements   of   ARM – AIR  OPS   will   be   in  accordance  with  Notice  of  Proposed  Amendment  ( NPA ) procedures ,  if  the  GDCA  of  RA  thinks  an  NPA  is  required.   These  procedures  allow  for  the  amendment  of  ARM – AIR  OPS   to  be  harmonized   with  amendments   to   EASA  and  ICAO  Annexes  in  a   timely  manner.   Typographical  errors  or  minor   changes  that  do  not  affect  the  industry  will  be  published  and   introduced  without  NPA  ( Notice  of  Proposed  Amendment ).

5    Amendments / Revision  history  table  is  included  to  every  Normal  or  Temporary  Revision,  so  that  the  changes  can  be  easily  tracked.

6    This  regulations  are  presented   in  Times  Roman   font   with   letter   size  of  12,  and   the  recent   changes   to   the   new   text   will   be   marked   by   Bold   Italics .

7    This   First  issue  is  dated  10  November  2014  edition.                                                                                                                   All  pages  of  this  edition  of  ARM – AIR  OPS  will  be  effective  from  16  December  2014.

All  pages  are  current  as  in  LEP  (  List  of  Effective  Page ),  the  blank  pages   are   marked   with   Intentionally  Left   Blank   as  it   is  controlled  by  LEP.

8.      Conformity   with   the   Guidance   Material   is   mandatory   unless   other   means  of  compliance   meet   the   equivalent   level   of   safety,   acceptable   to   the   Authority.

(  Purpose  of  change  to  ARM - OPS  1  issued  by  GDCA  of  RA  at  March  2013  by  new  ARM – AIR  OPS   Regulation’s   is  continue  to  harmonizing  legislation  and  to  regulate Commercial  Air  Transport,  Non - Commercial  Air  Transport   and  Private  Operations  of  Aircrafts  according  to  current  operations  and  market  requirements  ).

INTENTIONALY      LEFT      BLANK

REGULATIONS

COMMISSION   REGULATION   ( EU )  No  379 / 2014      of   7 April 2014

amending  Commission  Regulation  ( EU )  No  965 / 2012  laying  down  technical  requirements and  administrative  procedures  related  to  Air  Operations  pursuant  to  Regulation  ( EC )  No 216 / 2008  of   the  European  Parliament  and  of  the  Council

THE   EUROPEAN   COMMISSION,

Having  regard  to  the  Treaty  on  the  Functioning  of  the  European  Union,  having  regard  to     CR - EC ( Commission  Regulation  ( EC ) ) No  216 / 2008  of  the  European  Parliament  and  of  the Council  of  20  February  2008  on  common  rules  in  the  field  of  civil  aviation  and  establishing  a  European  Aviation  Safety  Agency,  and  repealing  Council  Directive  91 / 670 / EEC,  CR - EC  No  1592 / 2002  and  Directive  2004 / 36 / EC   thereof,  and  in  particular  Article  8 ( 5 ),

Whereas :

( 1 )  Operators  and  personnel  involved  in  the  operation  of certain  aircraft  have  to  comply  with  the  relevant  essential  requirements  set  out  in  Annex  IV  to  CR - EC  No  216 / 2008 ;

( 2 )   In  accordance  with  CR - EC  No  216 / 2008  the  Commission  should  adopt  the  necessary  implementing  rules  for  establishing  the  conditions  for  the  safe  operation  of  aircraft.  Those  rules  should  primarily  take  into  account  the  complexity  of  aircraft,  organizations  and  aircraft  operations,  as  well  as  the  risks  associated  with  the  different  types  of  operations ;

( 3 )  CR - EU )  No  965 / 2012  establishes  implementing  rules  for  Commercial  Air  Transport   Operations  with  aeroplanes  and  helicopters.  Rules  for  Commercial  Air  Transport  Operations  with  balloons  and  sailplanes  should  also  be  provided  in  order  to  comply  with  the  basic  principles  and  applicability  of  CR - EC  No  216 / 2008.  In  addition,  the  specificities  of  certain  commercial  operations  with  aeroplanes  and  helicopters,  starting  and  ending  at  the  same  aerodrome  or  operating  site,  need  to  be  appropriately  addressed  according to  their  scale  and  scope  and  the  risk  involved ;

( 4 )  CR - EU  No  800 / 2013  amends  CR - EU  No  965 / 2012  to  include  rules  for  non- commercial  operations  according  to  the  complexity  of  aircraft.   It  is  also  necessary  to  amend  CR - EU No  965 / 2012  in  order  to  reflect  the  current  state  of  the  art  and  to  ensure  proportionate  measures  for  certain  strictly  defined  activities  with  other - than - complex  aircraft  and  the  organizations  involved ;

( 5 )  Rules  for  specialized  operations  with  aeroplanes,  helicopters  balloons  and  sailplanes  should  also  be  included  taking  into  account  the  particular  aspects  of  such  operations  and  the  risk  involved.  For  reasons  of  proportionality  it  would  not  be  appropriate  to  subject  all commercial  operators  to  certification,  in  particular  commercial  specialized  operators.  Although  of  commercial  nature,  these  operators  would  be  subject  to  a  declaration  of  capability  instead

of  a  certificate.  Nevertheless,  conditions  for  certain  high  risk  commercial  specialized  operations,  which  endanger  third  parties  on  the  ground,  should  be  specified  in  the  interest  of safety  and  therefore  those  operations  should  be  submitted  to  authorization ;

( 6 )  CR - EU  No  965 / 2012  should  therefore  be  amended  accordingly ;

( 7 )  In  order  to  ensure  a  smooth  transition  and  a  high  level  of  civil  aviation  safety  in  the Union,  the  implementing  measures  should  reflect  the  state  of  the  art,  including  best  practices, and  scientific  and  technical  progress  in  the  field  of  air  operations.  Accordingly,  technical requirements  and  administrative  procedures  agreed  under  the  auspices  of  the  International  Civil Aviation  Organization  and  the  European  Joint  Aviation  Authorities  until  30  June  2009,  as  well as  existing  legislation  pertaining  to  a  specific  national  environment,  should  be  considered ;

HAS   ADOPTED   THIS   REGULATION :

Article  1       Subject   Matter   and   Scope

1.   This  Regulation  lays  down  detailed  rules  for  Air  Operations  with  aeroplanes,  helicopters, balloons  and  sailplanes,  including  ramp  inspections  of  aircraft  of  operators  under  the  safety oversight  of  another  State  when  landed  at  aerodromes  located  in  the  territory  subject  to  the provisions  of  the  Treaty ;

2.   This  Regulation  also  lays  down  detailed  rules  on  the  conditions  for  issuing,  maintaining, amending,  limiting,  suspending  or  revoking  the  certificates  of  operators  of  aircraft  referred  to in  Article  4  ( 1 ) ( b ) and  ( c )  of  CR - EC No  216 / 2008  engaged  in  Commercial  Air  Transport  Operation,  the  privileges  and  responsibilities  of  the  holders  of  certificates  as  well  as  conditions  under  which  operations  shall  be  prohibited,  limited  or  subject  to  certain  conditions

in  the  interest  of  safety ;

3. This  Regulation  also  lays  down  detailed  rules  on  the  conditions  and  procedures  for  the declaration  by,  and  for  the  oversight  of,  operators  engaged  in  commercial  specialized  operations  and  non - commercial  operation  of  complex   motor - powered  aircraft,  including  non -commercial  specialized  operations  of  complex  motor - powered  aircraft ;

4.  This  Regulation  also  lays  down  detailed  rules  on  the  conditions  under  which  certain  high risk  commercial  specialized  operations  shall  be  subject  to  authorization  in  the  interest  of  safety,  and  on  the  conditions  for  issuing,  maintaining,  amending,  limiting,  suspending  or revoking  the  authorizations ;

5.  This  Regulation  shall  not  apply  to  Air  Operations  within  the  scope  of  Article  1 ( 2 ) ( a ) of CR - EC No  216/2008 ;

6. This  Regulation  shall  not  apply  to  Air  Operations  with  tethered  balloons  and  airships  as  well  as  tethered  balloon  flights ;

Article  2     Definitions

For  the  purposes  of  this  Regulation :

( 1 )  “ Commercial  Air  Transport  “ ( CAT )   Operation    means   an  aircraft  operation  to  transport  passengers,  cargo  or  mail   for  remuneration  or  other  valuable   consideration ;

( 2 ) “ Performance  Class  B  Aeroplanes “   means  aeroplanes   powered  by  propeller   engines  with  a  maximum  operational  passenger  seating  configuration  of  9 ( nine )  or  less  and  a maximum  Take - off  mass  of  5 700 kg  or  less ;

( 3 ) “ Public  Interest  Site  ( PIS )  “   means  a  site  used  exclusively  for  operations  in  the  public  interest ;

( 4 ) “ Operation  in  Performance  Class  1 ”  means  an  operation  that,  in  the  event  of  failure  of the  critical  engine,  the  helicopter   is  able  to  land  within  the  rejected  take-off   distance available  or  safely  continue  the   flight  to  an  appropriate  landing  area,  depending  on when  the  failure  occurs ;

( 5 )  “ Performance - Based  Navigation  ( PBN ) “   means  area  navigation  based  on  performance requirements  for  aircraft  operating  along  an  ATS  route,  on  an  instrument  approach  procedure  or  in  a  designated  airspace.

( 6 )  “ Aeroplane “   means   an  engine - driven  fixed - wing  aircraft  heavier  than  air  that  is

supported  in  flight  by  the  dynamic  reaction  of  the  air  against  its  wings ;

( 7 )  “ Specialized  Operation ”   means  any  operation  other  than  commercial  air  transport  where the  aircraft  is  used  for  specialized  activities  such  as  agriculture,  construction,  photography, surveying,  observation  and  patrol,  aerial  advertisement ;

( 8 )  “ High  Risk  Commercial  Specialized  Operation ”    means  any  commercial  specialized aircraft  operation  carried  out  over  an  area  where  the  safety  of  third  parties  on  the  ground  is  likely  to  be  endangered  in  the  event  of  an  emergency,  or,  as  determined   by  the  Competent Authority  of  the  place  where  the  operation  is  conducted,  any  commercial  specialized  aircraft operation  that,  due  to  its  specific  nature  and  the  local  environment  in  which  it  is  conducted,  poses  a  high  risk,  in  particular  to  third  parties  on  the  ground ;

( 9 )  “ Introductory  Flight ”     means  any  flight  against  remuneration  or  other  valuable consideration  consisting  of  an  air  tour  of  short  duration,  offered  by  an  approved  training organization  or  an  organization  created  with  the  aim  of  promoting  aerial  sport  or  leisure aviation,  for  the  purpose  of  attracting  new  trainees  or  new  members ;

( 10 )  “ Competition  Flight ”   means  any  flying  activity  where  the  aircraft  is  used  in  air  races or  contests,  as  well  as  where  the  aircraft  is  used  to  practice  for  air  races  or  contests  and  to  fly  to  and  from  racing  or  contest  events ;

( 11 )  “ Flying  Display ”  means  any  flying  activity  deliberately  performed  for  the  purpose  of providing  an  exhibition  or  entertainment  at  an  advertised  event  open  to  the  public,  including  where  the  aircraft  is  used  to  practice  for  a  flying  display  and  to  fly  to  and  from  the  advertised  event.

Additional   definitions   are  laid  down  in  Annex  I   for  the  purposes  of  Annexes  II  to  V.

Article  3     Oversight  Capabilities

1.   Member  States  shall  designate  one  or  more  entities  as  the  Competent  Authority  within   that  Member  State  with   the  necessary  powers  and  allocated  responsibilities  for  the  certification  and  oversight  of  persons  and  organizations  subject  to  CR - EC  No 216 / 2008  and  its  implementing  rules ;

2.    If  a  Member  State  designates  more  than  one  entity  as  competent  authority :

( a )  the  areas  of  competence  of  each  competent  authority  shall  be  clearly  defined  in  terms of  responsibilities  and  geographic  limitation ;              and

( b )  coordination  shall  be  established  between  those  entities  to  ensure  effective  oversight  of  all  organizations  and  persons  subject  to  CR - EC  No  216 / 2008  and  its  implementing  rules,  within  their  respective  remits ;

3.      Member States shall  ensure  that  the  competent  authority(ies)  has(ve)  the   necessary  capability  to   ensure   the oversight of  all persons  and  organizations covered by  their oversight programme,  including  sufficient  resources to  fulfil  the requirements  of  this  Regulation ;

4.    Member  States  shall ensure that  Competent  Authority  personnel  do  not  perform  oversight activities  when  there  is  evidence that  this  could  result  directly  or  indirectly  in  a  conflict of  interest,  in  particular when  relating to  family  or  financial  interest ;

5.      Personnel  authorized  by  the  Competent  Authority  to  carry  out  certification  and  / or oversight  tasks  shall  be  empowered  to  perform  at  least  the  following  tasks :

( a )  examine  the   records,  data,  procedures  and   any  other  material  relevant  to  the   execution  of  the  certification  and  / or  oversight  task ;

( b )  take  copies  of  or  extracts  from  such  records,  data,  procedures  and  other  material ;

( c )  ask  for  an  oral  explanation   on  site ;

( d )  enter   relevant  premises,  operating   sites  or   means   of  transport ;

( e )  perform  audits,  investigations,  assessments,  inspections,  including  ramp  inspections  and unannounced  inspections ;

( f )    take  or  initiate  enforcement  measures  as  appropriate.

6.     The  tasks  under  paragraph  5  shall  be  carried  out  in  compliance  with  the  legal provisions  of  the  relevant  Member  State.

Article  4      Ramp  Inspections

Ramp  inspections  of  aircraft  of  operators  under  the  safety  oversight  of  another  Member State  or  of  a  third  country  shall  be  carried  out  in  accordance  with  Subpart  RAMP  of Annex  II.

Article  5      Air  Operations

1.   Operators  shall  only  operate  an  aircraft  for  the  purpose  of  Commercial  Air  Transport                     ( CAT  )  operations  as  specified  in  Annexes  III  and   IV ;

1 a.   Operators  engaged  in  CAT  operations  starting   and  ending  at  the  same  aerodrome  / operating  site  with  Performance  Class  B  aeroplanes  or  non - complex  helicopters  shall  comply with  the  relevant  provisions  of  Annexes  III  and  IV  ;

2.   CAT  operators  shall  comply  with  the  relevant  provisions  of  Annex  V when  operating :

a )  aeroplanes  and  helicopters  used  for :

( i )   operations  using  Performance - Based  Navigation  ( PBN ) ;

( ii )   operations  in  accordance  with  Minimum  Navigation  Performance  Specifications

( MNPS ) ;

( iii )  operations  in  airspace  with  Reduced  Vertical  Separation  Minima  ( RVSM ) ;

( iv )  Low  Visibility  Operations  ( LVO ) ;

b )   aeroplanes  and   helicopters  used  for   the   transport   of  Dangerous  Goods  ( DG ) ;

c )  two - engined  aeroplanes  used  for  Extended  Range  Operations  ( ETOPS )  in  Commercial  Air  Transport ;

d )   helicopters  used  for  commercial  air  transport  operations  with  the  aid  of  Night  Vision Imaging  Systems  ( NVIS ) ;

e )   helicopters  used  for  Commercial  Air  Transport  Hoist  Operations  ( HHO ) ;      and

f )     helicopters  used   for  Commercial  Air  Transport  Emergency  Medical  Service  Operations                    ( HEMS ).

3.   Operators  of  complex  motor - powered  aeroplanes  and  helicopters  involved  in  non -commercial  operations  shall  declare  their  capability  and  means  to  discharge  their  responsibilities  associated  with  the  operation  of  aircraft  and  operate  the  aircraft   in  accordance with  the  provisions  specified  in  Annex  III  and  Annex  VI.  Such  operators  when  engaged  in non - commercial  specialized  operations  shall  operate  the  aircraft  in  accordance  with  the

provisions  specified  in  Annex  III  and  VIII  instead ;

4.   Operators  of  other - than - complex  motor - powered  aeroplanes,  and  helicopters,  as  well  as balloons  and  sailplanes,  involved  in  non - commercial  operations,  including  non - commercial specialized  operations,  shall   operate  the  aircraft  in  accordance  with  the  provisions  specified  in Annex  VII ;

5.   Training  organizations  having  their  principal  place  of  business  in  a  Member  State  and approved  in  accordance  with  CR - EU No 1178 / 2011  when  conducting  flight  training  into, within  or  out  of  the  Union  shall  operate :

( a )   complex  motor - powered  aeroplanes  and  helicopters  in  accordance  with  the  provisions specified  in  Annex  VI ;

( b )   other - than - complex  motor - powered  aeroplanes  and  helicopters  as  well  as  balloons  and sailplanes  in  accordance  with  the  provisions  specified  in  Annex  VII ;

6.   Operators  shall  only  operate  an  aircraft  for  the  purpose  of  commercial  specialized operations  as  specified  in  Annexes  III  and  VIII ;

7.    Flights  taking  place  immediately  before,  during  or  immediately  after  specialized  operations and  directly  connected  to  those  operations  shall  be  operated  in  accordance  with  paragraphs  3,  4  and  6,  as  applicable.  Except  for  parachute  operations,  no  more  than  six  persons  indispensable  to  the  mission,  excluding  crew  members,  shall  be  carried  on  board ;

Article  6     Derogations

1.    By  way  of  derogation  from  Article 5(1),  aircraft  referred  to  in  Article  4  of  CR - EC  No  216 / 2008   shall  be  operated  under  the  conditions  set  out  in  Commission   Decision  C           ( 2009 ) 7633  of  14  October  2009  when  used  in  CAT  operations.  Any  change  to  the  operation  that  affects  the  conditions  set  out  in  that  Decision  shall  be  notified  to  the Commission   and  the  EASA  ( hereinafter  “ the  Agency “ )  before  the change  is  implemented.

A  Member  State, other  than  an  addressee  of  Decision  C  ( 2009 ) 7633,  which  intends  to  use  the  derogation  provided  for  in  that  Decision  shall  notify  its  intention  to  the Commission  and  the  Agency  before  the  derogation  is  implemented.  The  Commission  and  the  Agency shall  assess  to  what  extent  the  change  or  the  intended  use  deviates  from  the

conditions   of  Decision  C  ( 2009 ) 7633 or impacts  on  the  initial  safety  assessment  performed  in the  context  of  that  Decision.  If  the  assessment  shows  that  the  change  or

the  intended  use does  not   correspond  to   the  initial  safety  assessment   done   for  Decision  C ( 2009 ) 7633,  the  Member  State  concerned  shall  submit  a  new  derogation  request  in  accordance  with Article  14(6)  of  CR - EC  No  216 / 2008 ;

2.   By  way  of  derogation  from  Article  5(1),  flights  related  to  the  introduction  or modification  of  aircraft  types  conducted  by  design  or  production  organizations  within  the  scope  of  their  privileges  shall  continue  to  be operated  under  the  conditions  set  out  in Member  States  national  law.

3.   By  way  of  derogation  from  Article  5 (1), flights  related  to  the  introduction  or  modification of  aircraft  types  conducted  by  design  or  production  organizations  within  the  scope  of  their privileges  shall  continue  to  be  operated  under  the  conditions  set  out  in  Member  States  national law ;

4.    Notwithstanding  Article 5, Member  States  may  continue  to  require  a  specific  approval  and  additional  requirements  regarding  operational  procedures,  equipment,  crew  qualification  and  training  for  CAT  helicopter  offshore  operations  in  accordance  with  their  national  law. Member  States  shall  notify  the  Commission  and  the  Agency  of  the  additional  requirements being  applied  to  such  specific  approvals.  These  requirements  shall  not  be  less  restrictive than  those  of  Annexes  III  and  IV ;

4 a.    By  way  of  derogation  from  Article  5 (1)  and  (6),  the  following  operations  with  other -than - complex  motor - powered  aircraft  may  be  conducted  in  accordance  with  Annex  VII :

( a )  cost - shared  flights  by  private  individuals,  on  the  condition  that  the  direct  cost  is  shared by  all  the  occupants  of  the  aircraft,  pilot  included  and  the  number  of  persons  sharing  the  direct  costs  is  limited  to  six ;

( b )   competition  flights  or  flying  displays,  on  the  condition  that  the  remuneration  or  any valuable  consideration  given  for  such  flights  is  limited  to  recovery  of  direct  costs  and  a proportionate  contribution  to  annual  costs,  as  well  as  prizes  of  no  more  than  a  value  specified  by  the  Competent  A0uthority ;

( c )   introductory  flights,  parachute  dropping,  sailplane  towing  or  aerobatic  flights  performed either  by  a  training  organization  having  its  principal  place  of  business  in  a  Member  State  and  approved  in  accordance  with  CR - EU No  1178 / 2011,  or  by  an  organization  created  with  the  aim  of  promoting  aerial  sport  or  leisure  aviation,  on  the  condition  that  the  aircraft  is  operated by  the  organization  on  the  basis  of  ownership  or  dry  lease,  that  the  flight  does  not  generate profits  distributed  outside  of  the  organization,  and  that  whenever  non - members  of  the organization  are  involved,  such  flights  represent  only  a  marginal  activity  of  the  organization  ;

5.    By  way  of  derogation  from  CAT. POL. A. 300 ( a ) of  Annex  IV,  single - engined aeroplanes,  when  used  in  CAT  operations,  shall  be  operated  at  night  or  in  instrument meteorological  conditions  ( IMC )  under  the  conditions  set  out  in  the  existing  exemptions granted  by  Member  States  in  accordance  with  Article  8 ( 2 )  of  Regulation  ( EEC ) No  3922 / 91.  Any  change  to  the  operation  of  these  aeroplanes  that  affects  the  conditions  set  out  in  those exemptions  shall  be  notified  to  the  Commission  and  the  Agency  before  the  change  is implemented.  The  Commission  and  the  Agency  shall  assess  the  proposed  change  in  accordance  with  Article  14 ( 5 )  of  CR - EC  No 216 / 2008 ;

6.   Existing  helicopter  operations  to / from  a  Public  Interest  Site  ( PIS )  may  be  conducted  in  derogation  to  CAT. POL. H. 225  of  Annex  IV  whenever  the  size of the  PIS,  the  obstacle environment  or  the  helicopter  does  not  permit  compliance  with  the  requirements  for  operation  in  performance  class 1.  Such  operations   shall  be  conducted  under  conditions determined  by  Member  States.  Member  States  shall  notify  the  Commission  and  the  Agency  of  the  conditions  being  applied.

Article  7      Air  Operator  Certificates

1.   Air  Operator  Certificates  ( AOC’s )  issued  by  a  Member  State  to  CAT  operators  of aeroplanes  before  this  Regulation  applies  in  accordance  with  Regulation  ( EEC ) No  3922 / 91  shall  be  deemed  to  have  been  issued  in  accordance  with  this  Regulation.

However,  no  later  than  28  October  2014 :

( a ) operators  shall  adapt  their  management   system,  training  programmes,  procedures  and  manuals  to  be  compliant  with  Annexes  III,  IV and  V,  as  relevant ;

( b ) the  AOC  shall  be  replaced  by  certificates  issued  in  accordance  with  Annex  II  to  this Regulation.

2.    AOCs  issued  by  a  Member  State  to  CAT operators  of  helicopters  before  this  Regulation  applies  shall  be  converted  into  AOC’s  compliant  with  this  Regulation   in accordance  with  a  conversion   report  established  by  the  Member  State  that  issued  the  AOC,

in  consultation  with  the  Agency.

The  conversion  report  shall  describe :

( a ) the  national  requirements  on  the  basis  of  which  the  AOCs  were  issued ;

( b ) the  scope  of  privileges  that  were  given  to  the  operators ;

( c ) the  differences  between  the  national  requirements  on  the  basis  of  which  the  AOCs were  issued  and  the  requirements  of  Annexes  III,  IV and  V,  together  with  an  indication  of  how  and  when  the  operators  will be  required to  ensure  full  compliance  with  those Annexes.

The  conversion  report  shall  include  copies  of  all  documents  necessary  to  demonstrate  the elements  set  out  in  points  ( a )  to  ( c ),  including  copies  of  the  relevant  national requirements  and  procedures.

Article  8      Flight   Time   Limitations

- Flight  and   Duty  Time  Limitations  shall  be  subject  to   the  following :

( a )  for CAT  aeroplanes,  Article  8 ( 4 )  and  Subpart  Q  of  Annex  III   to   Regulation                 ( EEC )   No  3922 / 91 ;

( b )   for  helicopter  CAT  operations,  national  requirements ;

2.   Non - commercial  operations  with  complex  motor - powered  aeroplanes  and  helicopters  shall continue  to  be  conducted  in  accordance  with  applicable  national  flight  time  limitation legislation  until  the  related  implementing  rules  are  adopted  and  apply ;

3.   CAT  operations  with  helicopters,  CAT  operations  with  balloons  and  CAT  operations  with sailplanes  shall  comply  with  national  requirements ;

4.  Non - commercial  operations,  including  non - commercial  specialized  operations,  with  complex  motor - powered  aeroplanes  and  helicopters,  as  well  as  commercial  specialized  operations  with  aeroplanes,  helicopters,  balloons  and  sailplanes  shall  continue  to  be  conducted  in  accordance  with  applicable  national  flight  time  limitation  legislation  until  the  related  implementing  rules  are  adopted  and  apply.

Article  9      Minimum   Equipment   Lists

Minimum  Equipment  Lists  ( MEL)  approved  by  the  State  of  Operator  or  Registry  before  the application  of  this  Regulation,  are  deemed  to  be  approved  in  accordance  with  this  Regulation and  may  continue  to  be  used  by  the  Operator.  After  the  entry  into  force  of  this  Regulation any  change  to  the  MEL referred  to  in  the  first  Subparagraph  for  which  a  Master  Minimum

Equipment  List ( MMEL )  is  established  as  part  of  the  operational  suitability  data  in  accordance  with  CR - EU No 748 / 2012  shall  be  made  in  compliance  with  point   ORO. MLR. 105  of  Section  2  of  Annex  III  to  this  Regulation  at  the  earliest  opportunity  and  not  later  than 18  December  2017  or  two  years  after  the  operational  suitability  data  was  approved,  whichever is  the  latest.

Any  change  to  an  MEL  referred  to  in  the  first  Subparagraph,  for  which  an  MMEL  has  not been  established  as  part  of  the  operational  suitability  data,  shall  continue  to  be  made  in accordance  with  the  MMEL  accepted  by  the  State  of  Operator  or  Registry  as  applicable.

Article  10      Entry   into  Force

1.      This  Regulation  shall  enter  into  force  on  the  third  day  following  that  of its publication in  the  Official  Journal  of   the  European  Union.

It  shall  apply  from  28 October  2012.

2.      By  way  of  derogation  from  the  second  Subparagraph  of  paragraph  1,  Member  States may  decide  not  to  apply  the  provisions  of  Annexes  I  to V  until  28  October  2014.

When  a  Member  State  makes  use  of  that   possibility,  it  shall  notify  the  Commission  and  the  Agency.   This  notification  shall  describe  the  reasons  for  such  derogation  and  its duration  as  well  as  the  programme  for  implementation  containing  actions  envisaged  and related  timing.  This  Regulation  shall  be  binding  in  its  entirety  and  directly  applicable  in  all  Member  States.

3.   By  way  of  derogation  from  the  second  Subparagraph  of  paragraph  1,  Member  States  may decide  not  to  apply :

( a )   the  provisions  of  Annex  II  and  III  to  non - commercial  operations  with  complex  motor - powered  aeroplanes  and  helicopters  until  25  August  2016 ;     and

( b )   the  provisions  of  Annex  II,  V,  VI  and  VII  to  non - commercial  operations  with  aeroplanes,  helicopters,  sailplanes  and  balloons  until  25  August  2016.

4.   By  way  of  derogation  from  the  second  subparagraph  of  paragraph  1,  Member  States  may decide  not  to  apply  the  provisions  of  Annexes  II,  III,  VII  and  VIII  to  specialized  operations until  21  April  2017 ;

5.  By  way  of  derogation  from  the  second  subparagraph  of  paragraph  1,  Member  States  may decide  not  to  apply  the  provisions  of  Annexes  II,  III  and  IV  to :

( a )  CAT  operations  starting  and  ending  at  the  same  aerodrome  / operating  site  with Performance  Class  B  aeroplanes  or  non - complex  helicopters  until  21  April  2017 ;

( b )  CAT  operations  with  balloons  and  sailplanes  until  21  April  2017.

6.  When  a  Member  State  makes  use  of  the  derogation  provided  for  in  paragraph  5  point  ( a ),  the  following  rules  shall  apply :

( a )  for  aeroplanes,  Annex  III  to  Regulation  ( EEC )  No 3922 / 91  and  related  national exemptions  in  accordance  with  Article  8 ( 2 )  of  Regulation  ( EEC )  No  3922 / 91 ;

( b )  for  helicopters,  national  requirements ;

7.  When  a  Member  State  makes  use  of  the  derogations  provided  for  in  paragraphs  3,  4  and  5,  it  shall  notify  the  Commission  and  the  Agency.  This  notification  shall  describe  the  reasons for  the  derogation  and  its  duration,  as  well  as  the  programme  for  implementation  containing actions  envisaged  and  related  timing.

This  Regulation  shall  be  binding  in  its  entirety  and  directly  applicable  in  all  Member  States.

Done  at  Brussels,   7  April  2014.                                                      For  the  Commission

The  President

Jose  Manuel  BARROSO

INTENTIONALY      LEFT      BLANK

G  E  N  E  R  A  L      C  O  N  T  E  N  T

Title    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    1

Foreword  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    3

Commission  Regulation ( EU ) No 965 / 2012  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    5

General  Content   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

ANNEX   I.     PART - DEF.    /   DEFINITIONS

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  5

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    7

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   9

DEFINITIONS  for  TERMS  used  in  Annexes  II  to  VII    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   11

Section  I.   General   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  11

Section  II.  Definition  specified  for  Helicopters  only   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  19

ANNEX  I.  Guidance  Material  and  Acceptable  Means  &  Compliance  to  Part - DEF.

Guidance  Material  1.  to  Annex  I.   Definitions    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

Guidance  Material  2.  to  Annex  I.   Definitions    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   25

Guidance  Material  3.   to  Annex  I.   Definitions    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  35

Guidance  Material  4.   to  Annex  I.   Definition’s   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  36

Guidance  Material  5.   to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

Guidance  Material  6.   to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

Guidance  Material  7.   to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

Guidance  Material  8.   to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    36

Guidance  Material  9.   to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    36

Guidance  Material  10. to  Annex  I.   Definition’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    36

Definition’s  for  Terms  used  in  Annexes  II – VII - Amendment  1   .  .  .  .  .  .  .  .  .  .  .  .  .    36

ANNEX   II.   PART - ARO  /  AUTHORITY  REQUIREMENTS  for  AIR  OPERATORS

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   5

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     7

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

ARO. GEN. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  11

SUBPART   GEN.    General  Requirements    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   13

Section  I.      General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    13

Section  II.    Management    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     15

Section  III.   Oversight,  Certification  and  Enforcement    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    19

SUBPART   OPS.     Air  Operation’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

Section  I.     Certification  of  Commercial  Air  Transport  Operators   .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

Section  I a.  Authorization  of  High  Risk  Commercial  Specialized  Operations   .  .  .  .  .  .  .  .   25

Section  II.    Approvals   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     27

Section  III.   Oversight  of  Operations    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   29

SUBPART   RAMP.   Ramp  Inspections  of  Aircraft  of  Operators  under  the  Regulatory

Oversight  of  another  State   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

Annex  II.  Guidance  Material  and  Acceptable  Means  &  Compliance  to  Part - ARO  .  .   37

Appendix  I.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   107

Appendix  II.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  109

Appendix  III.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   111

Appendix  IV.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   113

Appendix  V.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  117

Appendix  VI.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    119

ANNEX  III.   PART - ORO  /  ORGANIZATION  REQUIREMENTS  for  AIR  OPERATIONS

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  7

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   11

ORO. GEN. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    15

SUBPART   GEN.   General  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

Section  I.      General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  17

Section  II.    Management    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    21

SUBPART   AOC.   Air  Operator  Certification    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

SUBPART   DEC.   Declaration    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  27

SUBPART   SPO.    Commercial   Specialized   Operations    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  29

SUBPART  MLR.   Manuals,  Logs  and   Records    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    31

SUBPART   SEC.    Security   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   35

SUBPART   FC.      Flight  Crew    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    37

ORO. FC. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   37

Section  1.    Common  Requirements    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     37

Section  2.    Additional  Requirements  for  Commercial  Air  Transport  Operations   .  .  .  .  .  .  .   41

Section  3.    Additional  Requirements  for  Commercial  Specialized  Operations   and

CAT  Operations  referred  to  in  ORO. FC. 005  ( b )( 1 )  and  ( 2 )  .  .  .  .  .  .  .  .  49

SUBPART   CC.      Cabin  Crew   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  51

ORO.  CC.  005     Scope    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  51

Section  I.     Common  Requirements    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   51

Section  II.    Additional  Requirements  for  Commercial  Air  Transport  Operations   .  .  .  .  .  .    57

SUBPART   FTL.   Flight  &  Duty  Time  Limitations  and  Rest  Requirements     .  .  .  .  .  .   61

Section  I.      General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    61

Section  II.    Commercial  Air  Transport   Operator’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    73

SUBPART   TC.   Technical  Crew  in  HEMS,  HHO  or  NVIS  Operations    .  .  .  .  .  .  .  .    89

Appendix.    Declaration   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   93

Annex  III.  Guidance  Material  and  Acceptable  Means  &  Compliance  to  Part - ORO  .  .  95

ANNEX   IV.   PART - CAT  /   COMMERCIAL  AIR  TRANSPORT  OPERATION’s

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    11

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   13

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  15

SUBPART   A.    GENERAL   REQUIREMENTS   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

CAT. GEN. 100   Competent  Authority   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   23

CAT. GEN. 105  Touring  Motor  Gliders,  Powered  Sailplanes  &  mixed  Balloons    .  .  .  .  .  .   23

Section  I.    Motor - Powered   Aircraft    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  25

Section  II.  Non  Motor - powered  Aircraft   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  33

SUBPART   B.     OPERATING    PROCEDURES    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    37

Section  I.    Motor - Powered   Aircraft   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   37

Section  II.   Non  Motor - powered  Aircraft    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   57

SUBPART  C.  AIRCRAFT  PERFORMANCE  and  OPERATING  LIMITATIONS    .  .  .  61

Section  I.    Aeroplanes   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    61

Chapter   1.   General   Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   61

Chapter   2.   Performance   Class  A   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    63

Chapter   3.   Performance   Class  B   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    71

Chapter   4.   Performance   Class  C   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    77

Section   II.  Helicopters   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    81

Chapter  1.    General   Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    81

Chapter  2.    Performance   Class  1   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   85

Chapter   3.   Performance   Class  2    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  89

Chapter   4.   Performance   Class  3    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  91

Section   III.  Mass  and  Balance   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  93

Chapter   1.    Motor - Powered  Aircraft   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  93

Section   IV.  Sailplanes   .  .  .  .  .  .  .  .  .  .  .  .  .   .  .  .  .  .  .  .  .  .  .  .  .  .   .  .  .  .  .  .  .  .  .  .  .  .   95

Section   V.    Balloons    .  .  .  .  .  .  .  .  .  .  .  .  .   .  .  .  .  .  .  .  .  .  .  .  .  .   .  .  .  .  .  .  .  .  .  .  .  .    97

SUBPART   D.    INSTRUMENTS,  DATA,  EQUIPMENT    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   99

Section   I.      Aeroplanes   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   99

Section   II.    Helicopters   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    119

Section   III.   Sailplanes   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   133

Section   IV.   Balloons    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    137

Annex  IV.  Guidance  Material  and  Acceptable  Means  &  Compliance  to  Part - CAT  .  .  141

ANNEX   V.   PART - SPA  /  SPECIFIC  APPROVALS

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   5

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     7

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

SUBPART  A.   General  Requirements    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   11

SUBPART  B.   Performance - Based   Navigation  (  PBN  )  Operations   .  .  .  .  .  .  .  .  .  .  .  .  13

SUBPART  C.   Operations  with   Specified   Minimum  Navigation   Performance    .  .  .  .  .  .   15

SUBPART  D.   Operations  in  Airspace  with  Reduced  Vertical  Separation  Minima    .  .  .  .    17

SUBPART  E.   Low  Visibility  Operations  (  LVO  )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

SUBPART  F.   Extended   Range   Operations   with   Two-Engined  Aeroplanes    .  .  .  .  .  .  .    23

SUBPART  G.   Transport   of   Dangerous   Goods    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    25

SUBPART  H.   Helicopter  Operations  with  Night  Vision  Imaging  Systems   .  .  .  .  .  .  .  .  .   27

SUBPART  I.    Helicopter   Hoist   Operations    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

SUBPART  J.   Helicopter  Emergency  Medical  Service  Operations   .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

Annex  V.   Guidance  Material  and  Acceptable  Means  &  Compliance  to  Part - CAT  .    41

INTENTIONALLY            LEFT            BLANK

INTENTIONALLY            LEFT            BLANK